IALA Recommendation E-108

On

The Surface Colours

Used As Visual Signals

On Aids To Navigation

Edition 1.1

December 2005

Edition 1

May 1998

AISM Association Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 01 Fax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      Website : www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| December 2005 | Entire document | Reformatted to reflect IALA Documentation Hierarchy |

Recommendation On The Surface Colours Used As Visual Signals On Aids To Navigation

(IALA Recommendation E-108, May, 1998)

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING the need to harmonise the surface colours used as visual signals on aids to navigation;

RECOGNISING ALSO that there is a need to specify both ordinary and fluorescent colours;

TAKING INTO CONSIDERATION the proposals of the IALA Engineering Committee;

ADOPTS the “Guidelines on the surface colours used as visual signals on aids to navigation”, set out in the Annex to this Recommendation;

RECOMMENDS that:

1.	The ordinary colours used as visual signals on aids to navigation be Red, Yellow, Green, Blue, White and Black, and Orange for special purposes requiring high conspicuity; and that their colour limits (extent of chromaticity and luminance factor) be as specified in Table 1 of the Guidelines on the surface colours used as visual signals on aids to navigation.

2.	That the fluorescent colours used as signals on aids to navigation be Red, Yellow and Green, and Orange for special purposes requiring high conspicuity; and that their colour limits be as specified in Table 3 of the Guidelines on the surface colours used as visual signals on aids to navigation.

3.	National Members take into account the advice given in the Guidelines on the surface colours used as visual signals on aids to navigation, particularly that contained in the “ Introduction”.

4.	Any other kind of surface colour be defined in accordance with the recommendations of the International Commission on Illumination (CIE)1 and in conformity, as far as practicable, with the colour limits of Table 1 of the Guidelines on the surface colours used as visual signals on aids to navigation.

5.	Any additional colours be defined in accordance with the recommendations of the CIE1 and be considered with extreme caution in regard to the confusions that might arise.

REVOKES IALA Recommendation for the surface colours used as visual signals on aids to navigation, dated May, 1980

1) CIE, Surface colours for visual signalling

Table of Contents

1	INTRODUCTION	6

1.1	General	6

2	SPECIFICATION OF THE COLOURS	7

3	CONSIDERATIONS OF PARTICULAR COLOURS	9

3.1	Red	9

3.2	Orange	9

3.3	Yellow and White	10

3.4	Green	10

3.5	Blue	10

3.6	Black	11

4	SYMBOLS AND ALPHANUMERIC CHARACTERS	12

5	COLOURS OF RETROREFLECTING MATERIALS	12

ANNEX

Guidelines On The Surface Colours Used As Visual Signals On Aids To Navigation

(Specification For Ordinary And Fluorescent Colours)

# INTRODUCTION

## General

A surface colour is a colour perceived to belong to a surface. The colour of an ordinary surface, such as an ordinary paint or an opaque plastics material, is the most common kind of surface colour and is known as an ordinary colour. Other kinds include fluorescent (or luminescent) colours, transilluminated colours ( for example, the colours of internally illuminated panels) and the colours of retroreflecting materials. A surface colour can be specified in terms of its chromaticity and its luminance factor. The chromaticity is defined by chromaticity coordinates, which may be plotted on a chromaticity diagram, and the luminance factor is a measure of the lightness of the colour relative to a pure white diffusing surface under the same illumination. As a specification must be made with respect to some type of illumination, the International Commission on Illumination (CIE) have precisely defined several standard illuminants; and since the results of the measurement of a colour can depend significantly on the degree of gloss on the surface, the CIE have also recommended various geometries of illumination and measurement1.

Two colours may be measured as having the same chromaticity and luminance factor under one illuminant but dissimilar ones under a different illuminant. This phenomenon is known as metamerism and its effect can be very significant. It is advisable to check that the appearance of a signal colour will remain reasonably constant under the various types of illumination by which the colour is expected to be seen.

A surface colour is usually seen in relation to other surface colours, and the perception of the colour can be quite markedly influenced by the presence of the other colours. Hence, a signal colour should always be checked, especially at a distance, for its appearance among the surrounding colours.

Deterioration of surface colours in use is a common occurrence, and care must be taken that signal colours always remain in compliance with their specifications. Particular attention should be given to fluorescent colours, because they are liable to undergo rapid changes of chromaticity and luminance factor on exposure to radiation and wear if they are not provided with special protective surfaces. Frequent inspections of fluorescent colours are advised until the normal useful life has been confidently ascertained for each typical situation where these colours are used. Special care may be needed if fluorescent and non-fluorescent colours of the same chromaticity are chosen to be used together, as different deteriorations might produce troublesome dissimilarities of the chromaticities.

# SPECIFICATION OF THE COLOURS

The recommendations in this document are based largely on experimental work involving the recognition and naming of colours, but they have also taken account of common practice and the limitation of currently available materials.

The method of specifying the colours is in conformity with the recommendations of the CIE1 and the specifications are expressed in terms of the CIE 1931 standard colorimetric system. the recommended limits of the chromaticity of a colour are specified by means of limiting boundaries that enclose a chromaticity region on a CIE standard chromaticity diagram.

The recommended ordinary and fluorescent colours are respectively specified in Tables 1 and 3, which provide the names of the colours, the equations and names of the boundary lines of the chromaticity regions, and the limits of the luminance factors. The coordinates of the corners of the chromaticity regions are respectively given, for convenience in plotting, in Tables 2 and 4, and the chromaticity regions are respectively shown on CIE standard chromaticity diagrams in Figures 1 and 2.

The standard illuminant specified for the measurement of a colour is D65, which represents a typical phase of daylight and has a correlated colour temperature of approximately 6504 kelvins. It is a tabulation of values across and beyond the visible spectrum1 and does not exist as a real light source although fairly close approximations to it can be realized. The chromaticity of standard illuminant D65 ( the illuminant point) is shown in the Figures.

The aim is to measure the colour and its appearance as the observer sees it. This depends on the structure of the surface, texture, gloss, pattern etc. To take all these effects into account a  45º/normal (45/0) geometry is used for measurements. The report CIE 15 (point 5.1.1) emphasis this as well. The d/8 geometry is used if the effects of surface characteristics shall be negated. A 45º/normal (45/0), means that the colour should be illuminated at an angle of 45º to the normal to the surface, and the colour should be measured in the direction of the normal. Measurement with a geometry of normal/45º will usually produce an identical result.

The 2° standard observer (CIE 15, point 6.1) is used for large observation distances and above all to consider the attitudes of the human eye: this is the area with the highest cone density and is therefore relevant for colour perception. This is the case for aids to navigation purposes. For other applications with shorter distances or larger observation angles like for the street traffic the 10° standard observer is used.

The chromaticity region recommended for each fluorescent colour is identical to the region of the corresponding ordinary colour. The colour of a fluorescent material should be measured with any protective surface that is normally used with the material.

The boundary lines of a chromaticity region, and the restrictions that may apply to the appropriate luminance factor, can together be referred to as the colour limits of a colour. The recommended colour limits are extreme values that should not be transgrested (except as mentioned in 3.1, 3.4, 3.5 and 3.6). more restrictive limits may be defined as appropriate to particular requirements; and they may be desirable for the signal colours used within one signaling system if substantial differences in appearance, either of chromaticity or luminance factor, are to be avoided. Also, the recommended colour limits of a colour are intended to apply throughout its entire service life, so examination of its condition may be required from time to time.

It should be noted that, with the exception of the purple boundary of Red, the specifications have not been designed to assist people with severely defective colour vision, most of whom will have great difficulty distinguishing between Red and Green.

3) Measurement of Colours

To measure colour different instruments of various companies are available on the market. An instrument with the following parameters according the remarks above should be chosen:

- measurement geometry: 45/0

- standard illuminant: D65

- standard observer: 2°

3a) Measurement Geometry:

The geometry belongs to the design of the instrument and is a unique attitude and it is fixed. It can not be changed by altering a value in the instrument set up or in the software. For the measurement of colours for aids to navigation the 45/0 geometry should be used. Instruments with such a geometry are offered by various companies worldwide.

3b) Aperture:

When measuring plastic samples the light from the instrument is not only reflected on the surface. Furthermore a certain amount of light is entering the material, is being absorbed, reflected and scattered back to the measuring sensor. If the aperture is too small, the amount of light to be measured to get reliable values might be too little. Therefore especially when plastic samples shall be measured an instrument with an aperture of more than 25 mm (at least 30 mm) diameter is recommended.

3c) Light Source:

Fluorescent colours need a certain amount of UV-radiation to create the high luminous factor. The fluorescent pigments absorb the UV-rays and emit rays at a lower energy level (longer wavelength), that are visible. Especially when measuring fluorescent colours it is important, that the light source in the measurement device emits enough UV-rays to excite the fluorescent process.

The best light source to measure fluorescent colours is a xenon arc lamp and a special filter (name?). Light sources like tungsten or halogen tungsten lamps produce no or not enough UV rays and lead to wrong results.

3d) Illuminant

The illuminant for the measurement of colours for aids to navigation is D65 and can easily be chosen in the set up of the instrument. Other illuminants like A or F can also be chosen but are of no interest for aids to navigation.

3e) Standard Observer:

According to the CIE two different values for the standard observer are possible. For aids to navigation the 2° standard observer should be used. This value can easily be adjusted in the set up program of the instrument (at least it should be possible).

3f) Miscellaneous:

Especially when measuring the colour of plastic materials the sample should be thick enough and as opaque as possible according to the final product. To erase any reflections from the surface below the plastic sample (e.g. measuring on a white table): use a black surface below the plastic sample. The black surface absorbs the transmitted light through the sample and it is not reflected back to the sensor. Thus measurement faults can be avoided.

4) Colour cards and samples:

Using a colour card (RAL card, NCS card etc.) or an original sample from a manufacturer to compare colours and to evaluate them may be helpful. It may give a rough evaluation but the result depends very much on the eye of the person comparing the colours. Furthermore it depends very much on the light that is used for the evaluation. Fluorescent lamps or tungsten lamps are not appropriate.

To minimise faults and misjudgement it necessary that the eye of the observer has no defects and can identify different colours (special tests are possible). The light used for evaluating colours should be as close to the D65 light as possible. This means using daylight at midday and a clear sky. The best way is to use a chamber with a xenon arc lamp and a filter to implement the D65 light. But these chambers seem no longer available.

The use of sample card can useful for the manufacturer when measuring and developing the varnish. For quality control purposes on the buoy yard it can not be recommended.

# 5 CONSIDERATIONS OF PARTICULAR COLOURS

## 5.1 Red

A minimum value of 0.07 is specified in Table 1 for the luminance factor of ordinary Red, but significantly higher values can usually be realized and, in most circumstances, a value greater than 0.10 should be maintained.

The chromaticity region of Red, which is identical for both ordinary and fluorescent colours, has been defined on the basis of achieving a very high probability of correct recognition for the colour, and it should prove to be quite practicable for ordinary reds with glossy surfaces and for fluorescent reds. There is doubt though, if their surfaces are matt or even semi-matt, whether serviceable materials of various kinds can always be manufactured in compliance with the restriction imposed by the white boundary of the chromaticity region for ordinary Red. Also , it is not yet certain that serviceable materials, with glossy surfaces when new, can necessarily be manufactured so that their compliance continues throughout a reasonable service life if considerable loss of gloss occurs. Therefore, it is proposed that the chromaticity region for ordinary Red may be extended, but only for materials with matt or semi-matt surfaces , to a revised white boundary of y=0.840 – x. This provision for ordinary red colours should not be used unless it is necessary, and then only with the understanding that the probability of correct recognition of the colour will be significantly reduced. The problem discussed here is not expected to arise with any of the other chromatic colours.

## 5.2 Orange

The probability of correct recognition of orange is usually not as high as that of red or yellow; moreover, when these colours subtend very small visual angles, orange and red, or orange and yellow, are very likely to be confused. Hence, in considering signal colours that need to be recognized at a distance, orange does not provide a satisfactory additional colour to a system that includes red and yellow. If orange is completely excluded from a system of signal colours for aids to navigation, the adjacent hue boundaries of Red and Yellow should remain as recommended in the Tables, since, otherwise, correct identification may not be made even at close ranges and the colours will not exhibit a reasonably consistent appearance world-wide.

Nevertheless, orange is probably the best ordinary colour for conspicuity against the sea, and it should preferably be reserved for those objects for which detection in the water is more important than recognition of their colours. The objects that require this consideration are items of emergency equipment, such as life-jackets and life rafts. The highest conspicuity will be obtained with fluorescent colours, and then fluorescent red-orange may be used and may, in some situations, be more conspicuous than fluorescent. Orange, but fluorescent red-orange is not likely to be seen as distinct from fluorescent Red.

## 5.3 Yellow and White

Discrimination between yellow and white is nor practicable when they subtend very small visual angles, so they should not be considered as separate colours except for close viewing. In particular, it would be inadvisable to create any circumstances that required unequivocal distinction between yellow and white in retroreflecting materials, whether by day or by night.

At sea, the probability of recognizing, or even detecting, white on its own will often be low.

## 5.4 Green

As an ordinary colour, green does not usually show well at sea. However, colours of fluorescent Green can be obtained with exceptionally high purities, and they will be very much more recognizable in most conditions.

It may be desirable, if green is required as a background colour on sign with symbols or alphanumeric characters, to use a special dark colour, that is, one having a value of luminance factor lower than the minimum value recommended in Table 1. in such circumstances, a value as low as 0.05 may be considered for this special dark green, which should anyway have a chromaticity conforming with the specification for ordinary Green, and which should never be used alone anywhere as a signal colour.

## 5.5 Blue

On inland waterways, and in estuaries and harbours, where colours may be seen at close range, Blue may prove to be a useful signal colour; but, at a distance, particularly at sea, it is unlikely to be easily recognized.

Although the recommended value of minimum luminance factor is Table 1 is 0.07, values significantly higher are attainable, and they should be required whenever possible if Blue is to be seen alone.

It may be desirable, if blue is required as a background colour on signs with symbols or alphanumeric character, to use a special dark colour, that is, one having a value of luminance factor lower than the minimum value recommended in Table 1. in such circumstances, a value as low as 0.05 may be considered for this special dark blue, which should anyway have a chromaticity conforming with the specification for ordinary Blue, and which should never be used alone anywhere as signal colour.

## 5.6 Black

A maximum value of 0.03, as specified in Table 1 is recommended for the luminance factor of ordinary black if surfaces are glossy, but, if surfaces are matt or semi-matt, then it may be necessary to allow a maximum value of 0.04 although the probability of correct recognition will thereby be lowered.

6 Considerations for bright and high saturated colours

6.1 General

It is generally known that the identification of buoys is determined by the shape, the size and by the colour of the buoys. As the shape (cylindrical or conical) is often neglected the importance of the selection of the surface colour increases.

A large investigation on the visibility and identification especially of buoys was made. The result was that the identification of the buoy increases with the saturation and luminance of its surface colour. Very bright and saturated colours lead to a high distance of recognition. Thus fluorescent colours are highly recommended.

As a result of the investigation new regions and luminance factors for colours on buoys, spars, topmarks and radar reflectors (to be coloured) were made. These determinations are valid for varnishes and sheetings. For plastic materials it is not yet investigated, if they are possible to achieve.

It is useful to determine two types of regions: one for the 'new condition' and one for the 'used condition' as the colour degrades during the use.

The 'new condition' (start box) defines the chromaticity limits when the colour on the buoy is new. It can be applied e.g. for measurements to give an approval or a denial. This region assures that only more saturated colours are used.

The 'used condition' means that for the whole service life of the buoy the colour must stay within this chromaticity limits. When it leaves the region, a new painting is required.

Starting form the region for the 'new condition' a colour may take same time to move out of this region and enter the region for 'used condition'. Leaving this region may last again some time until the colour must be renewed. By introducing a region for the 'new condition' only the colours with a high weather fastness and the more saturated colours are accepted.

6.2 Red:

Both regions (‘new’ and ‘used condition’) are for fluorescent and non-fluorescent colours the same. The upper border is slightly extended to the region of orange to allow brighter red hues. The luminance factor for non fluorescent colours is increased to the minimum of 12% to increase the brilliance and with it the distance of recognition. The border to purple is extended up to the purple line to allow more saturated red hues.

6.3 Yellow:

For the colour yellow only non-fluorescent colours are necessary as the luminance factor of minimal 0,5 is quite bright. Furthermore experiences revealed that a fluorescent yellow colour moves during the service life into the direction of the colour green.

In many countries the colour RAL 1023 “traffic yellow” as non-fluorescent colour is used. The ‘new condition’ lies symmetrical around the chromaticity coordinates of this colour. The ‘used condition’ is according the regular region for IALA Yellow and remains the same.

6.4 Green

For fluorescent and non-fluorescent colours the region for ‘new’ and ‘used condition’ are the same. The luminance factor for non-fluorescent colours is increased to the minimum of 12% to ensure bright colours (remark: see IALA rec from 1980). The border to the white region is moved to more saturated colours whereas the borders of the ‘used condition’ to yellow and blue were kept according the IALA preferred region. As a result of these new determinations only bright and saturated green colours are accepted and lead to a high distance of recognition.

# 7 SYMBOLS AND ALPHANUMERIC CHARACTERS

Good legibility requires that symbols and alphanumeric characters should have a good contrast with the colours against which they are seen. A contrast of luminance factors is usually of more advantage than one of hues, and the ratio of the luminance factors should be made as large as is possible. Thus Black should be applied on Yellow, and, in general, White should be used on Red, Green or Blue. However, if the luminance factors of Red or Green are particularly high, as they may be if these colours are fluorescent, then a contrast of Black may be more satisfactory. Sometimes a symbol or an alphanumeric character may be clearer if it is outlined in a contrasting colour or is shown on a distinct panel of contrasting colour.

# 8 COLOURS OF RETROREFLECTING MATERIALS

Two different specifications for the colours of retroreflecting materials are required if the colours are to be defined adequately for the purposes of this document. The specifications would define the colours for conditions of illumination that are representative of those occurring by day on the one hand and by night on the other. With regard to this document, a specification of the colours for night-time conditions is unquestionably the more useful, but the methods of measurement have not yet been internationally resolved. A specification of the colours for daytime conditions has been undertaken by the CIE², and the proposed chromaticity regions are, for the same conditions of illumination, largely identical with the recommended regions, in this document, for the corresponding ordinary colours completely identical regions for all the colours are feasible and would be preferable. A particular problem with a specification for daytime conditions relates to the geometry of measurement and the limits of the luminance factors. The two specifications for the colours of retroreflecting materials will have to await recommendations by IALA in the future.

Figure 1	Chromaticity regions for ordinary colours

Recommended regions for the ordinary colours specified in Table 1, the coordinates of the corner points being provided in Table 2.

Figure 2 - Chromaticity regions for fluorescent colours

Recommended regions for the fluorescent colours specified in Table 3, the coordinate of the corner points being provided in Table 4.

Figure 3 - Chromaticity Regions for Bright and High Saturated Colours

Recommended regions (new and used region) for colours specified in Table 5

Table 5 - Corners of the Chromaticity Regions of Bright and High Saturated Colours

New condition:

| Colour |  | Corners of the Chromaticity Regions |  |  |  | Luminance Factor |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | Ordinary | Fluorescent |
| Red | x | 0.649 | 0.602 | 0.701 | - | ≥ 0.12 | ≥ 0.25 |
|  | y | 0.351 | 0.348 | 0.249 | - |  |  |
| Yellow | x | 0.479 | 0.456 | 0.484 | 0.512 | ≥ 0.50 | - |
|  | y | 0.520 | 0.494 | 0.466 | 0.487 |  |  |
| Green | x | 0.100 | 0.222 | 0.300 | 0.300 | ≥ 0.12 | ≥ 0.25 |
|  | y | 0.831 | 0.543 | 0.605 | 0.693 |  |  |

Used condition:

| Colour |  | Corners of the Chromaticity Regions |  |  |  | Luminance Factor |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | Ordinary | Fluorescent |
| Red | x | 0.649 | 0.565 | 0.674 | - | ≥ 0.12 | ≥ 0.25 |
|  | y | 0.351 | 0.345 | 0.236 | - |  |  |
| Yellow | x | 0.465 | 0.427 | 0.470 | 0.522 | ≥ 0.50 | - |
|  | y | 0.534 | 0.483 | 0.440 | 0.477 |  |  |
| Green | x | 0.003 | 0.187 | 0.313 | 0.313 | ≥ 0.12 | ≥ 0.25 |
|  | y | 0.632 | 0.452 | 0.566 | 0.682 |  |  |

# References

English

Judd, D. B. and Yszecki, G. Color in Business, Science and Industry, 3rd Edition, New York, Viley, 1975.

Wright, W. D., The Measurement of Colour, 4th Edition, London, Hilger, 1969

CIE No. 39-2, Recommendations for Surface Colours for Visual Signalling, 1983

CIE No. 15, technical Report: Colorimetry, 2004

French

Kowaliski, P. Vision et Mesure de la Couleur, Paris, Masson, 1978

German

Richter, M., Einführung in die Farbmetrik, Berlin, de Gruyter, 1976.

Spanish

Lozano, R. D. El Color y su Medición, Buenos Aires, Editorial Americalee, 1978