AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline No. 1041

On

Sector Lights

Edition 1

December 2004

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      	Internet -  http://iala-aism.org

Table of Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	The Application of Sector Lights	3

2.1	Technical Considerations for Sector Lights	6

7

3	Basic Principles	7

3.1	Types of Sector Lights	7

3.2	Sector boundaries	9

3.3	Variation in luminous intensity between sectors	9

3.4	Colour integrity	11

3.5	Light Sources	11

4	Types of sector lights	12

4.1	Point Source Sector Lights	12

4.2	Projector Sector Lights	15

4.3	LED Sector lights	17

4.4	Slot Sector Lights	21

4.5	Range (directional) light	22

4.6	Diverged Beam (Laser) Sector Lights	23

4.7	Revolving pencil beam lights	23

5	Further considerations	24

5.1	Design and specification of sector lights	24

5.2	Support structures	25

5.3	Installation considerations	25

5.4	Sector checking	25

5.5	Effect of weather conditions	29

5.6	Photometry of sector lights	29

5.7	Application of sector lights as leading lights to mark a safe channel	32

6	References	34

ANNEX – Definitions	35

# Introduction

## Purpose and Scope

Sector lights have been used as an Aid to Navigation (AtoN) for almost 200 years. Many different methods are used to provide sector lights. In addition, there are several emerging technologies that may be used to provide sector lights.

The purpose of this document is to provide the IALA membership with guidance on the practical design of sector lights.

# The Application of Sector Lights

A sector light is an aid to navigation that displays different colours and/or rhythms over designated arcs. The colour of the light provides directional information to the mariner.

A sector or a limit between two sectors may indicate a fairway, a turning point, a junction with other channels, a hazard or something else of importance for the navigator.

When a fairway is covered by a white sector, the convention to a vessel approaching the light from the seaward side must be according to the IALA Maritime Buoyage System colour convention for Region A, and Region B.  The white sector indicates safe passage, however this does not always hold true in the entire radial length of the sector.  The fairway may alternatively be marked with lighted buoys or leading lights.

When designing fairways, consideration should be given to all possible types of aids to navigation to find the best solution for the particular site.

The design of sector lights can be a complex task. The process should be carried out with reference to a good quality chart of the area. In many cases, good local knowledge is also required and factors such as tidal flow, currents and background lighting may need to be considered (refer to 5).

A sector light may indicate one or more of the following boundaries of a navigable waterway

- change of course position

- shoals, banks, etc

- an area or position (eg. an anchorage)

- the deepest part of a waterway

- position checks for floating aids

Some examples of sector lights applications are illustrated in figures 2.1, 2.2 and 2.3.

Figure 2.1 Several applications for sector lights.

Light I is a coastal white light with a red sector indicating a danger. In this application high boundary accuracy is not essential.

Light II is a sector light obscured over the shore, with two white sectors indicating a safe channel. The boundary between the red and the green sector also indicates the position of a buoy.

Light III is a sector light with a red light and 4 white sectors indicating four anchorage positions.  It is obscured over the shore.

Light IV is a sector light with a white sector indicating a safe channel.

Figure 2.2 A single sector light marking a narrow channel

The white sector should, if possible, be wide enough to provide a margin of safety for a vessel that inadvertently leaves the white sector. Curves C and D indicate depth contours or limiting dangers that dictate the boundaries of sectors.

Figure 2.3 Multiple sectors arrangement marking a channel

Light V in figure 2.3 shows two sector lights arranged to create a parallel sector light system. This arrangement has five sectors. Each sector can have its own flash character or colour.

Light VI indicates oscillating boundaries which could be made up of five or seven sectors and can be used to improve identification of the vessels lateral position

## Technical Considerations for Sector Lights

The sector produced by a sector light should fulfil a number of essential requirements. It should have the right colour, its intensity has to be high enough to provide the required range, the luminous intensity has to fulfil the requirements and it has to cover the correct angle.

- There is no reference of the vessel’s lateral position within the channel until a sector boundary is reached. This may cause a problem in channels subject to a strong cross current.  For vessels with local knowledge, the zones defined by the angle of uncertainty can sometimes provide a useful guide to the vessel’s proximity to a sector boundary.

- Where Lateral information is required consideration of using oscillating boundaries.

- Where practicable, there should be a margin of safety between the sector boundary and adjacent hazards. If an appropriate safety margin cannot be achieved within the sector boundary, the hazards could be marked separately

- Zones defined by the angle of uncertainty should be considered an additional margin of safety over the actual sector boundary (see 5.7).

- The design process for a sector light needs to consider the speed and manoeuvrability of vessels likely to be negotiating the sector, how quickly they can respond once they cross a sector boundary and the situations that may develop when other vessels are in the vicinity (see 5.7).

- When using coloured filter in a sector light, the design should take account of the spectral distribution of the light source and the proportion of this light transmitted through the filter material.  The process should also check for potential for glare problems.

- The period of the light flash should be selected to provide ample time for a mariner to recognise the transitional phases that occur at the sector boundary(preferred Occulting or Isophase rhythm). This is particularly important for high-speed craft.

- Rhythmic characters cannot be used with oscillating boundaries as false signals can be generated.

- A white light is normally the first preference for a lighthouse or beacon.  If a single coloured sector is added, the preferred colour for the sector is red.

- If a white sector light is used to mark a navigation channel, coloured sectors may be used either side of the white to indicate the lateral limits.  In such cases it is common practice to use red and green sectors that follow the convention of the IALA Maritime Buoyage System.

- Multiple sectors can be used to provide a better indication of a vessel’s lateral position within the channel but at the expense of complexity for both the system designer and navigators.

- Bearings, directions of leading (range) lines and limits of sectors should always be stated in terms of the bearings that would be seen by the mariner. Bearings may carry a suffix ‘TBS” or True Bearing from Seaward as confirmation.

- The white sector indicates safe passage, however this does not always hold true for the entire length of the sector.

- The boundaries between sectors are always more or less diffuse. This also holds for the boundaries between light and dark sectors.

- When using coloured filters, the range of red and green light is approximately ¾ of the range of the white light, depending on light source and filter material.

- The coloured filter material used to create sectors must match the spectral characteristics of the light source to provide the required colour and intensity in the coloured sector.

- The range of red and green sectors may be enhanced at the design stage by the inclusion of intensifying prisms that are added to the main drum lens. The ranges of the red or green sectors may then be equal to the white sector.

- Ice or grime on the glazing of a lighthouse may cause coloured sectors to appear white. It may also make the light faint or invisible.

- The lantern glazing (or lantern panes) both absorbs and reflects light. The reflection of a beam against the inside of the glazing may give rise to false flashes of the wrong colour within a sector. Such anomalies can be minimised by tilting the glazing, by using curved panes or by using sector screens.

Basic Principles

## Types of Sector Lights

Many technical solutions exist for generating sectors. They may be categorized in a number of ways, one of which is by the optical geometry applied. In these terms, there are 5 basic types of sector lights:

- Point Source Sector Lights

- Projected Sector Lights

- Slot Sector Lights

- Range (directional) light

A comparison between these basic types of Sector Lights is made in table 3.1. A more detailed description of each is shown in Section 4.

|  | Practical Range | Angle of uncertainty / Boundary width | Max. Sector width | Typical Min. Sector width | Day operation |
| --- | --- | --- | --- | --- | --- |
| Point Source | Up to 20 nm | High (0.25º) | 360º | 1º | NO |
| Projected | Up to 30 nm | Very low (1 minute of arc, 0.017º) | 25º | 0.2 | YES |
| Slot | Up to 30 nm | Low Boundary width is constant with range | 45º | 1º | NO |
| Range light | Up to 23 nm | High | 30º | 3º | YES |

Table 3.1 Comparison of the basic types of sector lights

## Sector boundaries

The boundary region is perceived as a change from one character to another i.e. the mariner typically sees a mixture of the two sector characters. Each sector is either defined by its colour, its rhythmic character or a combination of these two. Between the sectors there is a boundary condition where the mariner is unable to distinguish between the two sectors.  This is a very important feature of a sector light.

When crossing the boundary, the mariner has a good indication of the bearing to the sector light, given that the boundary width is small. As the mariner traverses from one sector to an adjacent one, the colour change must be abrupt.  A vague boundary cannot provide useful information, as it requires a subjective assessment of colour saturation.

Being within a sector with a certain colour and/or rhythm the mariner only knows that the vessel is somewhere within the sector but will not know if he is in the centre of the sector or close to a boundary.

Lateral information can be provided within oscillating boundary sectors.

The width of the boundary region is therefore important, and the different types of Sector Lights have different characteristics in this respect.

Figure 3.2 Boundary width

The width of the boundary can be expressed as an angle of uncertainty if both the width and the range are known.

The basic mechanisms that determine the achievable minimum angle of uncertainty differ between types of Sector Lights.

A special form of sector boundary used in PEL lights is the oscillating boundary, where the individual sectors are defined by changing colours,.  The proportion of the colour signals provide lateral information where a vessel is across a sector. This type of boundary can only be produced by the projection type of Sector Lights.

## Variation in luminous intensity between sectors

In Sector Lights where optical filtering of a single light source is used, the luminous intensity, and thus the range, of each sector may vary considerably due to different transmission factors of the optical filters. There are a number of materials, including glass and plastic, suitable for filters. It is generally easier to get large filters in plastic rather than glass. When using plastic filters, the filter material must be ultraviolet stable and be suitably fitted to ensure that the heat generated by the lamps does not distort it..

The spectral characteristic of the light source has a major impact on the light output from a coloured filter. It is necessary to match the light source with a filter material having a suitable spectral transmission, thus providing a light whose chromaticity is within the IALA recommended colour region.

Some typical values of transmission factors are given in the tables below.

| Colour | Transmission |
| --- | --- |
| Red | 0.15 - 0.25 |
| Green | 0.15 - 0.25 |
| Yellow | 0.50 - 0.70 |

Table 3.3 Tungsten filament lamp (colour temperature approx. 3000 K)

| Colour | Transmission |
| --- | --- |
| Red | approx. 0.13 |
| Green | approx. 0.22 |
| Yellow | approx. 0.55 |

Table 3.4 Discharge lamp (colour temperature approx. 5500 K)

During the design of Sector Lights, with incandescent special attention should be paid to transmission factors and chromatic characteristics of the optical path. In a sector light having red, white and green sectors, the central white sector is about five times more intense than the colour sectors. It is possible to balance the luminous intensity between sectors using neutral density filters, which reduce the output intensity but does not change the colour, or using intensifying prisms. Where there is no background lighting, it may be appropriate to use a 25% neutral density filter in the white sector to provide the same intensities in all three sectors.  With moderate level of background lighting, a 50% neutral density filter can be used instead of clear glass in the white sector. Because most background lighting at night is white, this gives comparable conspicuity within all three sectors.

Attention should also be given to the fact that the atmosphere can affect light colour.

The spectral distribution of light passing through the atmosphere may change, depending on the spectral distribution of the light source. In some cases, white light sources have been observed to shift towards orange when viewed at a distance, also green sources containing a significant proportion of red have been perceived as being more red than green when viewed from distance.

There are three mechanisms involved: Atmospheric absorption, Mie scattering and Rayleigh scattering.

Atmospheric absorption occurs due to the absorption of energy in air molecules such as CO2 and H2O in the optical path. The absorption is wavelength dependent.

Mie scattering is scattering which is highly dependent on the size and shape of atmospheric particles, such as water molecules, in the optical path. This makes the scattering from fog have a similar spectral distribution as the illuminating source.

Rayleigh scattering is wavelength dependent, which causes blue light to be scattered more than red light. This is what makes the sunset red and the sky blue. In the visible range, the effect of Rayleigh scattering is less than the Mie Scattering.

Colour integrity

White , red and green are satisfactory colours for use in navigation lights.  If another colour is required, care must be taken to ensure that the chosen colour and filter material will maintain a consistent appearance in adverse weather conditions.

Yellow is not a suitable colour because it cannot be distinguished from white, especially with filament lamps used at low voltage.

Fog can be a problem with coloured lights, because fog scatters light of shorter wavelength (blue) more than of longer wavelength (red).  When a yellow, green or blue beacon is viewed through fog, if there is a significant red component in the light, then red may be the main colour that is seen.  This phenomenon has caused accidents at sea.  Purple-coloured beacons should also be avoided.

There are many blue and blue-green filter glasses and plastics that also freely transmit some red light.  Great care is required in the selection of filter glass for blue-green sectors to ensure that no red light is transmitted.  The use of colours other than red, white and green is not recommended for sectored navigation lights.  If more than three colours are required, an oscillating boundary or rhythmic lights could be used, rather than additional colours.

In general the optimum transmission in coloured filters is about 25%.

## Light Sources

Sector lights, like most other aids to navigation lights, are used in a flashing mode. There are three reasons for this. The first is that the sector light can be identified by its characteristic flash rhythm. The second reason is energy saving, particularly important for systems powered with primary batteries or solar photovoltaic power systems. The third reason is that by shortening the lamp’s on time, the maintenance intervals can be extended. The type of light source is restricted by these performance requirements.

Tungsten filament lamps have traditionally been used as the electric lamp light source. These have a very wide spectral distribution that allows use of a range of colour filter materials.

Modern lamps such as tungsten halogen and metal halide have a much narrower spectral distribution. It is therefore essential that filter material be carefully chosen to match the spectral characteristics of the lamp.

LED light sources are available in a range of monochromatic colours and do not need the use of coloured filters. Because of the spectral characteristics of white LED lights, coloured filters should not be used with these sources.

A summary of light sources may be found in Chapter 3 of the Navguide.

When a sector light is used for both day and night operation, the intensity for night viewing must be reduced to between 1% and 10% of the daylight intensity for equivalent conspicuity. This is more than can be achieved by voltage reduction at the lamp alone, without having the filament turn orange, and without interrupting the halogen cycle on tungsten-halogen lamps. A combination of automatic insertion of a neutral density filter (giving reduction down to 5%) and voltage reduction (giving reduction down to 20%) can be used to reduce intensity to 1% of daytime intensity. Alternatively a lampchanger can be used to automatically engage a low power lamp for night operation.4

# Types of sector lights

## Point Source Sector Lights

This type of sector light is sometimes referred to as a “traditional sector lights” due to the fact that they have been in use for more that 100 years. They can be generally categorised as shadow method sector lights, in which a coloured sector is generated by placing a piece of colourer glass or plastic (such as polycarbonate or acrylic) against the lantern house glazing or against the light itself. This casts a coloured shadow over the water at the required sector.

### Principle of operation

Point source sector lights generally take the form of a point light source usually a lamp within a drum lens with one or more optical filters placed at some distance, screening the light and producing each sector in the desired colour.

Figure 4.1 Basic operating principle of Point Source Sector Lights

The physical position of the optical filters should be mechanically adjustable for proper alignment of the individual sectors and sector boundaries.

In order to ensure sufficiently sharp boundaries between sectors, it is important to locate the optical filters at sufficient radial distance from the light source.

The optical filters may be located internally within the lighthouse glazing or external to a lantern.

| Internal filters in lantern house | External filters on small lantern |
| --- | --- |

Figure 4.2 Examples of internal (indoor) and external colour filters.

In Lighthouses and Beacons with a lantern house, the sector filters may be placed on the inside of the lantern house glazing, or the lantern house glazing can be replaced with a sector filter. In the latter case it is not possible to easily adjust the sector boundary.

Depending upon the available space within the lantern house, a good distance between the light source and the optical filter can be achieved. It is therefore possible to obtain sharper boundaries with external sector filters.

On small lantern sector lights consisting only of a lantern, the Sector filters can be mounted externally in a rack or the lantern manufacturer may provide fixed internal filters. In the latter case it is not possible to adjust the sector boundary.

### The light source

In a Point Source Sector light, the horizontal dimension of the light source is important to achieve small sectors as well as small angles of uncertainty in the boundaries of the sectors.   When considering a Point Sector Light the degree of uncertainty should be taken into account for the application.  A small light source results in smaller angle of uncertainty.

In the case of incandescent light sources, it is important to use only high grade lamps with pre-focused filaments. If the position of the filament varies between individual lamps, the sector positions also vary when a lamp is replaced. This problem can be overcome by incorporating a means of accurately aligning the filament as each new lamp is installed.

The spectral characteristics of the light source must match the desired spectrum of each coloured sector as mentioned in the section on basic principles.

Rotating lenses are not very suitable for Point Source Sector Lights due to the fact that the light source is very large and is constantly moving. A moving light source will result in moving/oscillating sector boundaries. Trials have shown that boundaries in the order of 5 are possible with small rotating optics.

### Angle of uncertainty

The shadow method can be used where very sharp sector boundaries are not required. The transition zone of uncertainty may be 1-3 and intensity may vary. As with any optical apparatus, precise positioning of the lamp is essential. The sector boundary will move if the lamp filament is moved, as it might when a lamp is replaced.

In the Point Source type of sector light the ratio between the horizontal dimension of the light source (d) and the distance between the light source and the optical filter (D), determines the angle of uncertainty (), and thus the width of the boundary region at any given distance from the sector light.

Coloured filter

D



d

Figure 4.3 Factors that determine the angle of uncertainty in point source type of sector lights

The mathematical relationship between the factors is as follows:

 = Angle of uncertainty = (d/D) x 57 (degrees)

where

d = Horizontal dimension of light source.

D = Distance from filter to light source.

This formula is known to be somewhat pessimistic. The observed angle of uncertainty is generally less than the calculated angle due to the relative intensities of sector colours (i.e. colour mixing) as the observer passes through the transition zone.

If space on the aid to navigation structure is not a limiting factor, it is usually possible to achieve an angle of uncertainty of around 0.5 with this type of sector light.

The angle of uncertainty can be reduced by decreasing the physical size of the light source or by increasing the radial distance to the coloured filter;

### Practical solutions for creating sharper sector boundaries

If large coloured sectors (>20º) are required, the size of the optical filter may become a problem. In this case the optical filter would preferably be placed close to the light source and perhaps inside the lens, thus reducing its size. In order to minimize the angle of uncertainty, the optical filter may be split up into two components, one being far away from the light source, and the other being close to the light source as illustrated in figure 4.4.

Figure 4.4 Illustration of an arrangement to decrease the angle of uncertainty

This will facilitate cleaning of the windows in Lantern House and in the case of an externally filtered lantern, it will reduce the filter surface exposed to the wind and grime.

### Minimum width of a sector

The physical dimension of a sector in the optical filter must exceed the physical horizontal dimension of the light source.

If the physical width of the white sector is smaller than the width of the light source, the sector will consist of light passing through two or more optical filters and the mariner will see multiple sectors simultaneously. This will result in an effective reduction in intensity of the white sector.

Figure 4.5 Big Light Source with Small Sector width

Minimum width of sector in relation to the horizontal dimension of the light source

### Projector Sector Lights

Projected light is a specialised form of sector light that can generate sharply defined sector boundaries. This feature is particularly useful for applications that require one or more narrow sectors.

### Principle of operation

In this type of sector light, an image of the desired sectors is produced, and projected through a lens. While the image of the desired sectors may be produced in a number of ways, the most common way of producing it is by precisely machined optical filters. The principle is the same as in a slide or an overhead projector, focused at infinity. Vertical strips of coloured filter glass, optically ground and highly polished on their edges to fit closely together, are used as the "slide" or "film" to divide the beam into different sectors. The condenser system collects light radiating from the lamp and spreads it uniformly across the coloured filter.

An image of the filter is projected out to infinity. Sector boundaries may appear blurred within the first few hundred metres as they are out of focus, but will be very sharp at working distances.

Small changes in lamp filament position may cause minor changes in intensity within the beam, but will have no effect on the boundary positions, which are fixed by the projection (or objective) lens system.

Projection lens

Coloured filter

Night filter

Figure 4.7 Basic operating principle of Projector Sector Lights

The objective lens and the filter assembly together determine the total subtense. Different objective systems are used to obtain all the different subtenses with optimal efficiency. Generally smaller subtenses require larger objective lenses and longer barrel lengths. A smaller subtense with the same size lamp would provide a greater intensity and range.

.

A PEL Sector Light is a typical example of a projection light.

A projector Sector Light uses colour to convey information to the mariner about his angular position relative to the light. The process of "colouring" a beam involves filtering out many colours and only allowing the desired colour to pass.

If the filter does not block off enough undesired colours (wavelengths), there can be problems with the light appearing to change colour in fog. If the selection of wavelengths that is passed is too narrow, the light will not be intense enough.

The Sector Light is bright enough to use by day if sufficient power is available to operate the necessary size of lamp.

The size of lamp that can be fitted is limited by physical size constraints of the optical arrangement. If greater ranges are required than can be achieved with the largest lamp then it is possible to operate two projector light units in a bi-form arrangement.

Greater range is achievable by using a smaller subtense for the projector.

### Angle of uncertainty

Some sector projectors can overcome boundary inaccuracies by the design of the projector optics. These projector sector lights are so precise that a complete colour change at a sector boundary occurs over an angle of less than 1 minute (0.02°) in most models. This corresponds to a lateral distance of just 1 metre at a viewing distance of 3.5 km. This angle is as small as the human eye can resolve.

In addition the intensity is maintained right to the edge of the beam, and does not reduce the further the observer is away from the axis.

## LED Sector lights

Light emitting diodes (LED’s) have revolutionised signalling using light in all areas of traffic and are now used in many applications including traffic lights, braking lights, and aids to navigation lights. This has been made possible mainly by the development of high power diodes with approximately 1-watt output (output of standard diodes is in the region of 0.1 watt).

### Principle of operation

LED’s coloured light sources are available in a range of monochromatic colours and do not need the use of coloured filters. They can provide a very energy efficient light source for a coloured sector light, as the light is generated directly in the desired colour and no light is wasted by absorption in sector filters. The filter cost is avoided, as are the cleaning problems often associated with filter installations. Because of the spectral characteristics of white LED lights, coloured filters should not be used with these sources.

However there are a number of factors that limit the application of LED’s for use in sector lights.

### Size of LED sources

Since the luminous intensity of LED’s is relatively low, they are normally used in arrays in order to produce a usable range. They are therefore a large light source and most suited to sector lights where large light sources are used such as slot sector lights. A detailed description of an experimental slot sector light using LED’s is given in reference 1.

### Spectral characteristics of LED’s

Using the conventional sector light projector system with incandescent lamp and filters, the lamp could be replaced by LEDs (figure 1). In such an arrangement, LEDs technology doesn't present its full intensity in red and green colours as the filters reduce the light of the coloured sectors to 20% of the white sector). (Reference 2)

Figure 4.8 LED projector light

The luminous efficiency (emitted luminous flux per electrical power) of white diodes is only comparable with that of incandescent lamps. The projector in figure 4.8 then corresponds to a sector light with a 1-watt incandescent lamp, producing a very low intensity and hardly meaningfully usable light.

An improvement is possible by using an individual projector for every sector (figure 4.9). The luminous intensity of the coloured sectors could clearly be raised since the colour doesn't arise from filtration any more.

Figure 4.9 Multi beam LED projector sector light

However, this construction has two disadvantages:

- The sector limits must be adjusted very carefully relative to each other. It is almost impossible to adjust the sectors on-site. Since the white diodes are not as bright (at same electrical power) compared with the coloured LED’s, either the power of the coloured diodes must be reduced or a less intense white sector must be accepted.

- The light produced by a white LED often has a high colour temperature (over 5000 K). This is especially true when the LED is viewed along its optical axis, where correlated colour temperature as high as 20,000 K has been measured. The blue component of the spectrum is significant. A sector light uses three different colours that have to be separated from each other at all times. Red and white can generally be easily separated, while green and white are not so easily separated. (Reference 1).

### Colour mixing LED sector light

To overcome these disadvantages, the white sector can be made by colour mixing from green and red LED’s (figure 4.10). To produce the colour white by so called additive colour mixture, it is necessary to mix the three colours red, green and blue.

Figure 4.10 Mixing of colours to produce white light

The green colour used in LED traffic signals is a bluish green and therefore contains blue colour. At careful choice of the luminous intensity of red and green therefore produces a warm white, which fulfils the IALA requirements for white signal lights

A colour factor is a semi permeable mirror that reflects the light or lets through depending on colour. The colour factor is set at an angle of 45° to reflect red light and to pass blue/green through. It is put between the frame for the sector limits and the projection optics. The two light sources are arranged at the colour factor so that both beams are brought together again in the projection lens (figure 4.11). (Reference 2)

Figure 4.11 LED sector light using colour mixing

The following comparison is from the measures between a commercial sector light in incandescent lamp technology and the above experimental LED light.

| Parameter | incandescent lamp technology | LED-technology |
| --- | --- | --- |
| electrical Power | 5 Watt | 2 Watt |
| luminous intensity (white) | 1500 cd | 1300 cd |
| luminous intensity (red) | 300 cd | 650 cd |
| luminous intensity (green) | 300 cd | 650 cd |

With improvements in the optic system as well as in the diodes it should be possible to replace sector lights of up to 50 watts by LED with half of the power consumption.

## Slot Sector Lights

### Principle of operation

Slot sector lights consist of one or more light beams directed to a focal aperture (slot) at some distance. The ratio between the focal aperture width and the distance between the light source (assuming a point source) and the slot aperture defines the horizontal angle of the sector.

Light source

Slot aperture

Focal aperture

Figure 4.12 Basic operating principle of Slot Sector Lights

In the above example, there is only one sector. The principle may however be extended to a number of light sources of different colours yielding a number of sectors of different colours, and if need be different rhythmical characters.

Light source array

Slot

Figure 4.13 Light Source Array

Figure 4.14 Basic operating principle of Slot Sector Lights

(Three light sources and three sectors)

Coloured sectors are arranged with coloured glass mounted directly in channels in front of the lamp.

The boundaries are defined with baffle plates. The baffle plates are some millimetres wider than the focal apertures in the lantern house, to compensate for refraction of light around the baffle plate edge and provide the light exactly at the boundary crossing. The baffle plates are mounted in the same way as the filter glass.

Using an array of lamps as the light source, the luminous intensity in different sectors can be altered depending on the number of lamps used in that sector. The lamps can be connected in several separate groups, which are switched on in sequence, thus giving a stable power load and a character like a rotating light. With a Slot Sector Light it is possible to create very distinct boundaries and sectors if the lamps are placed a long distance from the slot.

### The light source

The light source depends on the application. Sealed beam lamps, PAR 56 200W 30V, mounted in simple stands are typically used as a light source. Systems with up to 2 x 96 lamps have been build, but smaller systems also exist with only a couple of boundaries and 3-6 lamps.

An advantage of such a lamp array is that a burned out lamp will not lead to a total loss of light in boundaries or sectors, but only a reduction in luminous intensity in that part of a sector as the boundary is illuminated by several overlapping lamps.

## Range (directional) light

Reflector Range Lights may be used to provide a moderate cost sector light to illuminate an arc covering an area of danger.

### Level of performance

Range lights are marketed by several of the major AtoN manufacturers. The type under consideration is built to a US Coastguard specification and has the lamp mounted at the focus of a very accurate silvered glass parabolic mirror. This produces a nearly parallel beam of light that is diverged by means of a reeded beam spreader. These beam spreaders are usually available to provide beam angles of 3, 8 , 11 , 20 and 28.

Filters are available in red, green and yellow for coloured sectors.

These commercial products are optically very efficient and provide a low cost means of illuminating single sectors, provided that beam spreaders are available for the required angle.

## Revolving pencil beam lights

The application of accurate coloured sectors with a revolving pencil beam is not easy to achieve.

The whole surface of the optical panel would serve as a virtual light source and because of its large horizontal dimension the filter would have to be placed at an unacceptably great distance from the optic in order to produce a properly defined boundary line. If a coloured sector is needed in a major light provided with a revolving pencil beam, recourse is usually taken to the installation of a subsidiary light, using a fixed lens with a coloured filter. Otherwise, the major light might be provided with a fixed lens of large focal distance and a coloured filter but the fixed lens would not produce the same high luminous intensity as the pencil beam.

In application as where a sector is required to mark a hazard but the boundaries of the sector are not critical, a coloured filter can be used with a rotating pencil beam to create the sector.

It may be noted however, that in modern revolving optics of small diameter and yet of high luminous intensity such as obtained with sealed beam lamps, arranged in a vertical stack, coloured sectors have been successfully applied.

# Further considerations

## Design and specification of sector lights

The following steps are recommended when preparing a specification for a sector light.

Night Only or Day + Night

The first decision is whether a night-only signal is required, or a day and night signal.  If a day and night system is required, the method of intensity reduction must be determined. This may affect the choice of optic apparatus.

Determine Required Intensity

Intensity is determined from the range required and the conditions under which viewing occurs (transmissivity of atmosphere, level of background lighting).  IALA recommendations should be consulted.

Choose Total Subtense

There is a tradeoff between subtense and intensity – the greater the subtense the less the intensity (for a given lamp) (this is specific for projectors).

Choose Lamp Size

The required range, optic apparatus and the available power source will influence determination lamp type and size. Sector lights powered from renewable energy sources such as PV solar or wind generally use low power lamps or LEDs in combination with high performance lens systems.

Oscillating Boundary Option

This option used in the PEL optic apparatus is useful in critical applications where early warning of deviation from a sector (or approach to a sector) is required.  On energy-critical sites it may be less attractive because of the long on-periods required for its effective operation.

Determine Individual Sector Angles

Each sector light is configured for its end use.  For most applications red, white and green sectors are chosen.  Sectors are specified reading from left to right when looking towards the light from seaward, with red on left (System A) or on right (System B). Determine the bearing and angle required for each sector.

Check Vertical Divergence

Sketch a vertical profile through the light and viewing area.  Check that mariners on highest and lowest vessels at closest and furthest points can all fall within the vertical divergence of the light.

Specify Flash Character

Determine the character for the sector light, bearing in mind the characters of other nearby AonN lights. In general, a longer flash length provides better performance. For renewable energy systems, the additional energy consumption arising from long flash length must be balanced against the ability of the power system to provide the required energy.  Flash characters are not suitable to use with oscillating boundaries.

Specify Night Intensity Reduction (if applicable)

When the sector light is available both day and night, some night intensity reduction is usual.  Night intensity is a percentage of day intensity.

Reduced Intensity in White Sector:  Yes or No

Intensity in the white sector can be reduced by 30% using neutral density filter to match the intensities in the coloured sectors.

Reflections

Ensure that there are no unwanted reflections from lantern glazing, sash bars, obstructions, etc.

Connections to Other Devices

Determine if connection or synchronization is required with other lights, special controllers, monitoring.

## Support structures

As with all AtoN structures, the design and materials used in the construction of sector lights should be capable of withstanding the forces imposed at the site in storms and be resistant to corrosion due to the salt laden marine environment. The design of structures should also facilitate maintenance access. The amount of twist and sway of the structure need to be considered in the overall design as excessive movement could undermine the application of the sector light.

Since the bearing of a sector light is critical, structures supporting sector lights must be particularly resistant to twisting to ensure that the sector lights remain accurate.

## Installation considerations

Reflections in sector lights can be particularly important because of the risk of the reflected light being mistaken for the true sector light by the mariner. When installing sector lights care must be taken to avoid reflections from lantern glazing or obstructions in/ near the sector.

Reliable operation of a sector light depends on periodic maintenance as well as good initial design. Provision must be made for easy access to the sector light so that lamps can be changed, lens cleaned, etc.

## Sector checking

As sector lights are generally used to ensure vessels navigate in very precise channels, thorough and precise checking of sector lights (range checking???) to ensure their accuracy is essential.

### Objective

The objective in checking of sectors is to determine the sectors direction and / or to measure the distance from the sector boundary to a hazard or turning point.

The navigable sector in many sector lights is a white sector. This is a fairway free from hazards or indicating a turning point. In figure 5.1, the navigable sector is marked by , while the non-navigable sectors are marked by .

Sector of uncertainty

=Leading white sector

=Non navigable sector

White leading sector

 A circle indicate suitable place for inspections (observations) for leading sectors. Her shall clear light determent

(Square indicate suitable place for inspections (observations) for not leading sectors. Her determines the border where change in colour will occur

Figure within ring or square gives the number of observations

Figure 5.1

A lighthouse or beacon is regarded as usable when the navigator is able to determine his position without difficulty under clear visibility and without binoculars,

### Preparation for measurement

Measurement of sectors should be carried out by at least two experienced observers. Testing should be conducted under good weather and water conditions, preferably clear and transparent air. Observations made from a long distance should be carried out in complete darkness.

Prior to testing, the sector light system should be inspected to ensure that it is functioning correctly and glazing, lens, etc are clean. It is recommended when the light have long eclipses (period) with darkness to alter the character to fixed light, and undoubtedly make a notes to mariner. If determination of the sector boundaries is made in cold weather, there is a risk of snow or ice on the light, making accurate determination more difficult and possibly leading to incorrect results.

If testing gives rise to any interference with normal ship traffic due to movements of the testing boat, new or unusual light signals, or working with lighthouses, care should be taken to inform mariners via Notice to Mariners and inform the nearest pilot station or VTS-centre.

### Position determination

The availability of GPS and DGPS has greatly simplified the process of accurately plotting the results of field viewing trials of the effective positions of sector boundaries.

### Method of measurement

If the sector boundary angle of uncertainty is to be measured, the boundary for both bright white and distinctly coloured light must be determined. In other cases, the boundary between two coloured sectors where change of colour takes place is determined in the centre area between the sectors. Between white and coloured sectors, the boundary for clear coloured light should be measured.

Sector of uncertainty shall be determined for all lead sectors and also where, from a navigational point of view, it is desired to carefully stipulate the border line between two sectors. In short distance from the lighthouse the sector of uncertainty can sometime be very small and is accordingly not necessary to measure.

### Measuring on land/shore

When measuring from land, the observers pass laterally across the border and carefully check the change-over between lights. The positions of the boundary change-over are recorded and plotted on a chart. Measurement can take place in daylight if the lights are visible.

### Measuring from boat

Measurement is carried out at a number of different positions as indicated in figure 5.2 to ensure that the sectors are correctly marked throughout the usable part of the sectors.

Figure 5.2 Sector light testing using a boat

The procedure for measurement is to move laterally across the boundary between the white sector light and the adjacent coloured sector light. On each transition, note the positions, using GPS latitude and longitude, at which the white light is clearly identifiable and coloured light is clearly identifiable. The distance between the observations in each place must be big enough to be seen on the chart. This is repeated at various positions along the sectors as suggested in figure 5.2.

These positions are then plotted on a chart in order to determine the precise boundaries of the sectors and angle of uncertainty.

Measurement near to hazard protected by the sector boundary

Determined boundary for white light

Sector of uncertainty

Boundary for clear white light

The number of observations varies depending on the length of the border.

At least 2 (normally 3-4) locations should be tested, one of which is close and one at the outer end of the sector. The spread of observations is important to determine the angle of uncertainty.

Every measurement group contains of about 4 observations

Figure 5.3 Measurement procedure

Where electronic positioning of the test vessel is not available, the boundary of the sector is marked using on shore posts or buoys moored with triple leg moorings so that position can be maintained. The test vessel is then lined up with these “fixed “ marks and the boundary of the sector is adjusted to line up with the observer on the vessel.

## Effect of weather conditions

Navigators navigating with the help of multi-coloured sector lights are warned about the certain risks prevailing during the cold season. There are always uncertain angles between the sectors, within which it is difficult to determine the colour of the light.

During the cold season the range of these uncertain angles may increase substantially due to snow and ice coating or formation of mist. The light could then be interpreted as white in sectors where it under normal conditions will be seen as coloured. Furthermore a false light in dark sectors, originating from adjacent sectors, might be observed under these same conditions.

When navigating with the help of sector lights it is therefore very important to make sure that the vessel is on the course line by taking frequent and careful positions, especially during the cold season.

Photometry of sector lights

It is possible to define and measure the angle of uncertainty for a projector sector light with a distance photometer in a laboratory.

(see IALA Recommendation On Determination Of The Luminous Intensity Of A Marine Aid To Navigation Light, December 1977, for the photometry of marine-aids-to-navigation signal lights)

The horizontal light distribution I() is measured as a function of an angle. In most cases the luminous intensity will change when moving from one sector to the next (The expected region of uncertainty is named F , the angular resolution S.).

Figure 5.4 Intensity as a function of angle

### Sectors with different characters:

When the sectors are distinguished by characters only, it is necessary to switch the measured sector on and all others off. The luminous intensity will then change from 100% to 0% when moving through the angle of uncertainty.

The angle of uncertainty can be defined by the change of luminous intensity: the photometric definition of the angle of uncertainty U is the angular distance in which the luminous intensity changes from 90% to 10%.

Figure 5.5 Change in intensity across the sector boundary

### Sectors with different colours:

When sectors are distinguished by colour there will be a change in intensity and the colour will move continuously from one colour to the next. The angle of uncertainty should be defined as the angle distance from the point the colour leaves the IALA preferred region of the first sector colour to the point the colour enters the preferred region of the second colour.

Figure 5.6 Change in colour across the sector boundary for Incadescent lamps

### Geometric requirements for measuring the uncertainty angle of a projector

An exact photometric measurement of the uncertainty angle depends on the angle resolution of the goniometer. To measure the region of uncertainty it is necessary that the angular resolution S of the goniometer is much smaller than the expected angle of the uncertainty F (figure 5.4).

s << F

The minimum ratio should be 5: 	s  F / 5

In order to avoid blurring of the intensity distribution by geometric effects, it is recommended that

- the diameter of the photometer head builds an angle smaller than s as seen from the exit pupil of the sector light

- the diameter of the exit pupil of the sector light builds an angle smaller than s as seen from the photometer head

Figure 5.7 Photometric measurement

This results in the requirements:

- Dph / d = ph  S

- 2R / d = obj  S

Dph 	diameter of the active sensor area of the photometer head

d	measuring distance / crossover distance

R	Radius of the exit pupil / lens of the projector

## Application of sector lights as leading lights to mark a safe channel

The fairway is regarded as a rectangle (reference IALA Recommendation for Leading Lights E-112: useful segment). The sector width of the (white) leading sector increases with the distance between the light and the observer. To ensure that the leading sector shows the safe channel, the leading sector must fit in the width of the fairway  at the furthest point of the useful segment .

Figure 5.8 Sector definition

However the white sector at the furthest point should be smaller than the width of the fairway.

- The angular region of uncertainty B(U) should be inside the width.

- The maximum width of the vessels Bvessel should be considered. When the observer in the axis of the ship recognizes the change of the sector one half of the width of the vessel is outside the leading sector.

- There should be additional aspects considered. For example the accuracy of the adjustment and measurement of the position, the reaction of the vessels changing their direction. These influences should be included in a value Badd.

The leading sector of the light Bsecure (angle of uncertainty not included) should fulfil the following requirement

Bsecure  B - Bvessel - 2*B(U) - 2*Badd.

It is recommended that at the end of the channel closest to the direction light the width of the white sector should be at least twice the beam of the largest ship expected to use the light. The light should therefore be located at some considerable distance from the near end of the channel if a very narrow angular width of the central sector is to be employed. For example, if the angular width is to be low, and the beam of the largest ship is 20 m, the lateral width should be at least 40 m and the light should be placed at a distance of at least 2300 m from the near end of the channel.

# References

- Pasi Orrevetelainen, Boundary Issues of an LED Sector Light, Helsinki University of Technology, 2003.

- IALA Engineering Environment and Preservation of Historic Lighthouses Committee, EEP4 Input EEP4/6l/2, LED Sector light, F. Hermann, C.Mauel, Traffic Technologies Centre, Koblenz, Germany.

- PEL Sector Lights, Vega Industries, 21 Heriot Drive, Porirua 5022, New Zealand.

- IALA Recommendation for a definition of the nominal daytime range of maritime signal lights intended for the guidance of shipping by day, April 1974. IALA.

- IALA Navguide 2001 Edition 4, Section 3.4.1. IALA, 2001.

- IALA Recommendation on the Determination of the Luminous intensity of a marine aid to navigation light, IALA December 1977.

# ANNEX – Definitions

Sector Light. A sector light is an aid to navigation that displays different colours and/or rhythms over designated arcs. The colour of the light provides directional information to the mariner. A sector or a limit between two sectors may indicate a fairway, a turning point, a junction with other channels, a hazard or some other point of importance for navigation.

Sector Boundary Region. The Sector Boundary Region is perceived as a change from one character to another i.e. the mariner typically sees a mixture of the two sector characters. Each sector is either defined by its colour, its rhythmic character or a combination of these two. Between the sectors there is a boundary condition where the mariner is unable to distinguish between the two sectors.

For Sector Lights using only colour as the character, the specified values are defined by the recommended IALA colour coordinates.

In practice however the boundary region is defined by observing if the individual sector characters can be determined by an observer.

Boundary Resolution. On the boundary between a white and a coloured sector (eg red) there is a small transition angle within which the colour is neither completely white nor completely red.  Boundary resolution is the smallest angle over which a complete colour change occurs.

Angle of uncertainty. The horizontal angle of the region of indefinite character near the boundaries of a sector of a sector light.

The limits or boundaries of a sector are not always precisely cut off due to the characteristics of the light source and merging of colours or changing rhythms between adjacent sectors. The transition zone is defined by an "angle of uncertainty"

Sector Screen /Cut Screen. An opaque screen so placed as to provide a sharp cut-off to a beam in sector light, and to reduce the angle of uncertainty.

Vertical Divergence. Vertical divergence is the total projected angle in the vertical plane. The associated intensity reduction factor must be given with vertical divergence (usually 10%).

Oscillating Boundary. A method of giving more information of position near the sector boundary to the mariner by creating a light character with alternately flashing colours.

Total Subtense is the total projected angle in the horizontal plane.  In projectors the basic light beam is circular in cross-section, and is masked down to a rectangle as shown in the diagram.

Cross-section of Projected Beam

Projected Beam without Filter Assembly in place

Subtense

Plan view

Subtense angle of projected beam

Sector Accuracy. Due to manufacturing tolerances on lenses, filter glasses and lens-mounting systems, the actual location of a sector boundary may be displaced a small amount from the intended place.  Accuracy is the maximum angle between intended and actual location.

Intensity and Range. Intensity is the amount of light energy emitted from a light in a given direction, and is measured in candela.  Increased intensity gives increased viewing range, but typically, range also depends on atmospheric transmissivity and level of background lighting.

See also IALA Recommendation E 110, The Rhythmic Character of Lights on Aids to Navigation, Paragraph 2.4.

IALA Dictionary 2-5-230

IALA Dictionary 2-4-125

IALA Dictionary 2-1-100