IALA Guideline No. XXXX

On

Synchronisation of Lights

Draft

April 2008

Rev 4.1

AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      Internet -  http://iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

1	INTRODUCTION	5

2	PURPOSE AND SCOPE	5

3	APPLICATION	6

4	BENEFITS OF SYNCHRONISATION	6

5	CONSIDERATIONS FOR IMPLEMENTATION OF SYNCHRONISATION	7

5.1	Synchronisation Configuration for Optimum Conspicuity	7

5.2	Logical Grouping of lights	7

5.3	Use of different characters	8

5.4	Sequential	8

5.5	Leading Lights	8

5.6	Other considerations	8

6	LIMITATIONS OF SYNCHRONISATION	9

6.1	Environmental limitations	9

6.2	Maximum slippage time	9

6.3	Minimum angular separation	9

7	SYNCHRONISATION METHODS	10

7.1	Introduction to Synchronisation	11

7.2	Timing Options	12

7.3	Centralised Synchronisation - Master-slave	13

7.3.1	External master	13

7.3.2	Internal master	13

7.4	Decentralised synchronisation - Peer-to-peer	13

7.4.1	Protocol example	14

8	TRAINING & QUALITY MANAGEMENT	14

9	SYNCHRONISATION EXAMPLES	15

9.1	FMA - Fairway of Vuosaari	15

9.2	SMA – Synchronised fairways passage with buoys	17

9.3	AMSA – Synchronised lights on pile beacons in Port Hedland (AUS)	18

9.4	TTC – Bremerhaven Synchronised Buoys	20

9.5	GLA Synchronised lights viewing trial	22

9.6	The Frigg Field Cessation Project	25

10	DEFINITIONS	27

10.1	Terminology	28

10.2	Symbology	28

11	References	28

APPENDIX 1 –	29

12	TECHNICAL DESCRIPTION OF SYNCHRONISING METHODS	29

12.1.1	GPS	29

12.1.2	GSM network time	30

12.1.3	DCF77 – Frankfurt time – frequency standard stations	31

12.1.4	Digital television (DVB)	31

12.1.5	Radio network (Yleisradio, Finnish National Radio Organisation)	32

12.1.6	TV line frequency	32

12.1.7	e-Loran	32

12.1.8	Synchronisation using other radio signals	33

13	Appendix 2 – GLA Synchronised Lights Viewing Trial	33

IALA GUIDELINE FOR SYNCHRONISATION OF LIGHTS MARKING CHANNELS AND WATERWAYS

# INTRODUCTION

In the provision of marine aids to navigation (AtoN), advances in technology, both in light sources and communications, have enabled lights to be synchronised far more readily and accurately than before.  Previously, synchronisation has been achieved using fixed wiring or radio systems.  Radio systems have included the use of general radio timing signals or local radio systems established for the purpose.  The additional effort and cost in establishing and maintaining fixed wiring and radio systems meant that these systems were used on a limited basis. However, now that cost effective, robust and accurate timing options are available, synchronisation is a practical proposition.

Additionally, advances in technology have made synchronisation of AtoN lights readily available for use on fixed and floating aids.  This can enable enhancements, aimed at improving the overall conspicuity of aids to navigation lights, especially in built-up areas and areas with background lighting.

A number of AtoN providers have already conducted demonstrations.  Some have established synchronised or sequential light systems in particular locations.  The results of a number of these demonstrations are attached to this Guideline as Annexes.

The introduction of GNNS timing signals has led to readily available, relative low cost, self contained AtoN light units, produced by a number of manufacturers in the global market that can be used world wide.

The use of synchronised and sequential lights should be assessed on the basis of recognised risk management meeting the navigational requirement.

In addition to the objective of this Guideline, there are other applications where synchronization / sequentialisation may apply, as reflected in the IALA Recommendation O-116 (The Marking of Aquaculture Farms) (?? This is Ed 2, June 2007), or on other types of aids, such leading lines, or when the identification of the "geometry" is relevant to the mariner.

# PURPOSE AND SCOPE

The purpose of this Guideline is to provide IALA Members and AtoN providers with guidance on the provision of synchronised lights as part of an AtoN system for marking channels and waterways, with regard to improving the conspicuity of AtoNs, especially in areas where good conspicuity of aids to navigation is difficult to achieve due, for instance, to background lighting.

This document applies to marine aids to navigation signal lights on fixed and floating applications.

# APPLICATION

It has long been a concern to authorities, service providers and mariners that the growing background lighting in coastal, port approach and harbour areas is reaching critical levels and often overwhelming aids to navigation when seen against this background.  It is often difficult to pick out and identify the aids or it is otherwise difficult for the mariner to   channel lights.  Therefore, there is a particular need to emphasise the navigable channel or marked routes.

In seeking to meet the navigational requirement as identified by risk assessment, the option of using synchronised and / or sequential lights provides a useful augmentation / enhancement to conventional AtoN lights in enhancing the aids when viewed against background lighting.

# BENEFITS OF SYNCHRONISATION

Synchronising of two or more flashing lights is already in use in signalling systems for various transport systems, including road, rail, aviation, and maritime. Historically, synchronisation has been used in the maritime world for leading lights. The purpose of synchronising is to increase the conspicuity of the signal, and / or to indicate that the two or more lights are associated in some manner.  For example if two buoys form a “gate” in a channel, the lights on them might be synchronised to make that gate pair more conspicuous.

Sequencing of lights is when a series of lights is flashed in a time sequence to show the geographical relationship between them.  Such a set of lights is sometimes called a “flare path” and provides the user with directional information.

It is also possible to combine the two effects, so that, for example, if there is a channel marked by pairs of buoys, the lights on each pair are synchronised, and in addition the pairs light are set to be in a time sequence along the length of the channel.

In each case, the objective is to help the mariner distinguish which lights are pairs of buoys (or beacons) marking a channel, and in addition to indicate which pairs are closest and which are more distant.

Following extensive trials and experience gained with both synchronised and sequential aids to navigation, it is clear that two key benefits can result from the use of such AtoNs:

- Synchronised lights provide high impact conspicuity.  They draw the observers eye to their presence and overcome background lighting due to their regular and combined effect; and

- Sequential lights provide directional awareness and information when within a system, e.g. waterway.  The observer experiences visual movement in the horizontal plane.

The results of a number of these trials are included in the Annexes to this Guideline.

# CONSIDERATIONS FOR IMPLEMENTATION OF SYNCHRONISATION

The flashing sequences of AtoN can be synchronised in a number of different ways. The devices should flash in a particular order for the fairway to be the most conspicuous to the mariner.  Solutions tested to date indicate that having the lights of a complete fairway section flashing simultaneously is a good solution for optimum conspicuity.

(the working group suggests this section should tie to the practical examples in part 11, where examples correlate closely to the considerations below)

## Synchronisation Configuration for Optimum Conspicuity

It is important to test or simulate the synchronisation of flashing lights on one fairway in different conditions before making decisions on the synchronisation method to be introduced.  This will allow an assessment to be made as to the extent that the visibility of the fairway can be improved. Tests should also involve affected mariners in order to ensure that the optimum system of synchronisation is implemented for a particular application.

## Logical Grouping of lights

Fairway turning points can be used to divide the fairway into sections.  Lateral marks of the same fairway section could be synchronised, after which the aids marking the next section could be turned on.  In order for the various fairway sections to stand out they should have similar characters.  If this is not possible, the flashing sequences should be in multiples of each other's light periods.

Another possible option would be to follow the example of airport runway lights, which are turned in sequence (consecutively).  This alternative could also be utilised to guide vessels to port, however, this may look peculiar when leaving port.  In this alternative the flashing sequences at the ends of the fairway should be different from those in the middle.  This method is not advisable on a shipping lane as the AtoN are not placed at equal intervals.  As the distances vary, it is not possible to create an “approach effect” similar to an airport.

A third option would be to synchronise the different sides of the fairway. Using this method, the direction of the fairway could be highly visible but detecting its width would be difficult.

## Use of different characters

Different characters could be used effectively to identify the beginning of a fairway or change in the fairway.  For example the first two buoys or channel markers could have a different character from the rest of the channel, whilst remaining synchronised.

Reference should be made to existing IALA Recommendations and Guidelines when considering the use of different characters `vis: E-110 (For the rhythmic characters of lights on aids to navigation (Edition 2.0) and the IALA Maritime Buoyage System.

The character period of synchronised lights should be sufficiently short such that the observer can see those aids as frequently as possible.

Synchronised lights, with the same character, in ‘gated pairs’ are particularly conspicuous when marking bends in a channel.

## Sequential

For lights flashing sequentially, the sync delay needs to be determined taking into account the geometry of the channel and in particular the paired buoy spacing. As a guide – “the closer the buoys are together the shorter the delay may be”.

The geographical spacing between synchronised lights within a group, on fixed or floating aids, should be such that the group as a whole is within the observer’s field of view.

The flash character of sequencing lights and/or pairs should be unique to ensure independent identification in case of sync failure. Ie/ first pair group flash / second pair group flash.

## Leading Lights

Selection of characters and management of synchronised leading lights should be such that the front and rear lights can be identified easily, and an overlap of their flash ‘on’ time can still occur in the event of sync failure.

Special consideration should be taken in the design of synchronised leading lights to ensure that in the event of the failure of one lead, a single light is not mistakenly paired with an apparently synced external source such as its reflection on water.  This consideration could include an automatic disabling of the second light.

## Other considerations

1. The geographical layout and tapestry of the aids to navigation, channel or port approach where synchronised aids are deployed should be considered as a whole before decisions are made to establish such aids.

2. The likely sea state and prevailing visibility – i.e. local conditions – should be considered when planning to use synchronised or sequential lights in a waterway.

3. At dusk when the lights fist turn on, and on occasion due to synchronisation signal loss, there may be a period of time where one of more of the lights will not be in synch, therefore, consideration should be made to the general layout of the navigation aids to ensure the mariner can still identify the channel.

4. A preference to using grouped synchronisation over sequential synchronisation may be made to avoid confusion to the mariner when transiting in an opposite direction to the sequential flashing.

5.

Promulgation of information by way of notice to mariners must be carried out informing the mariners when a synchronized system is put in place.

6. Affected stakeholders should be consulted when designing the synchronized system.

# LIMITATIONS OF SYNCHRONISATION

## Environmental limitations

The application of synchronised and / or sequential aids to navigation does not necessarily provide the mariner with positional information. Synchronised lights provide spatial awareness and orientation within a channel or system of aids to navigation.

There are physical limitations with regard to the installation of equipment required for synchronisation systems.  E.g. Lights synchronised using GPS must ensure that the GPS sensor has an un-obscured view of the sky in order to receive regular timing signal updates. Atmospheric conditions may affect the signal strength for radio synchronisation systems.

Power requirements to provide a synchronised light system will in general, be somewhat greater.  Solar systems may not produce adequate power at high latitudes, and especially with respect to minor aids and/or where other AtoN are also deployed at the site eg AIS, racons.

The impact of synchronised / sequential lights can be adversely affected by; buoy stability, visibility, excessive height of eye vis-a-vis. vertical divergence, and general adverse weather and sea conditions (in a manner similar to conventional marking).

## Maximum slippage time

To ensure the mariner can clearly ascertain synchronised groupings the timing error between synchronised lights should not be greater than 50ms. (Refer to GLA Report Version RPT-09-03-MN-IT-07.)

## Minimum angular separation

To ensure clear separation of individual synchronised lights can still be made, it is recommended that there should be a minimum angular separation of 5 minutes of arc, subtended at the observer, within the arc of utilisation of the synchronised lights. Lights too close together may be appear as a single light of a unique and different colour.

# SYNCHRONISATION METHODS

Synchronisation and sequencing require identical and accurate time information available at all the lights which are to be synchronised or sequenced.  This can be accomplished in at least two ways:

- An accurate internal timing mechanism at each light location

- Reception at each light location of the same timing data from a reliable source

In practice, the latter is usually employed, and implementation is done by a radio broadcast system set up for the purpose, or by reception of an existing timing signal, or by cable connection.  Early synchronisation systems often employed a special local radio transmitter, emitting a timing signal, which was received at each light.

Nowadays, various local or global time signals are available and dedicated radio transmitters are not required. The most commonly used timing signal is that contained within a Global Navigation Satellite System GNSS system, such as GPS or GLONASS.

In some countries, radio timing signals are broadcast at VLF and these have been used for synchronisation of lights.  In Japan, the FM commercial broadcasting of 76-90Mhz had been used, with a whip antenna attached to the superstructure of the buoy or lantern house of the lighthouse, however such systems are largely phased out in favour of GPS.

Note that synchronisation of the timing of the lights in the group is all that is needed.  Synchronisation to GPS time or other standard is not needed.

Timing by a GNSS implies that the synchronised light system will not be fully redundant. If the GNSS for any reason should be unavailable, the synchronised light system will have to degrade to an ordinary light system. The probability for this to happen is low. Typically, each light is fitted with an integral timing device that provides the rhythmic flash character. It is common practice to adjust this integral timer by synchronising it with an external pulse from a reference timing source (GNSS Clock or Local Radio Transmitter). It is important that the timing accuracy of the lights integral timer is maintained to within 50ms between external synchronisation pulses i.e. the clock contained in the light must be sufficiently accurate in the short term to stay in step with the GNSS clock or other timing source.

In the event of a timing source failure, the accuracy of the light should be maintained for sufficient time period for marine use.

The most practical method of maintaining synchronised flashing is to control the internal frequency source of the flasher by means of external synchronisation data. Most commercial products work this way.  The data obtained externally may be an exact time code, a reference frequency or even a synchronisation pulse.  Maintaining a continuous link with a source of synchronisation data, however, consumes a lot of power.  By using an internal frequency source, it is also possible to prevent an error where the light stops flashing as the connection to an external source of synchronisation data is interrupted.

The prerequisites for the synchronised flashing of AtoN are a light source, flasher and a synchronisation device.  The flasher determines when the light source flashes at the accuracy of its internal clock (frequency source).  In practice, there are no other functional alternatives.  The internal clock of a flasher usually is a crystal oscillator.  The most important factor affecting the stability of a crystal oscillator's frequency is temperature variations, but as temperature changes are slow on the water surface, the frequency of a crystal oscillator remains relatively stable.

This Guideline only addresses wireless synchronisation methods.  Beacons have been synchronised using cables led along the seabed, but this has not been considered a good solution especially for floating aids to navigation, as they can shift.  In addition, the costs are high, and synchronisation relying on a cable will make it more difficult to move and add beacons.

## Introduction to Synchronisation

The internal clocks of various parts of the system differ from each other.  Even if the clocks are set on the same time, over time there will be an offset between them.  The following formula describes the difference between the time of two clocks C1(t) and C2(t):

C1(t) = a12 . C2(t) + b12 ,

In which a12 is the relative drift of the clocks and b12 the relative offset.  Correcting the momentary values is not adequate for synchronisation, as after the correction the clock times will continue drifting.  That is why, when planning synchronisation, it is necessary to decide if both the drift and offset will be corrected, or if the offset will be corrected repeatedly.

The better the internal clock of the device is, the longer it stays on the correct time without synchronisation.  When planning the synchronisation frequency, the failure of one or several synchronisations should also be taken into account: the system should still remain within the "synchronisation limits".

Time synchronisation can be divided into three different accuracy levels.  The first one is accuracy of order (one part in the system knows that it precedes another), the second is accuracy of relative time (the same time unit is used inside the system) and the third one is global accuracy (the time is also accurate in proportion to the rest of the world).  In the synchronisation of fairways, relative time is a sufficient accuracy level.  The components do not need to be perfectly synchronised, as the human eye will perceive two events as simultaneous, if the difference between them is no more than in the range of 50ms.  Consequently, the accuracy requirements are not high, and far more accurate synchronisation is required such as wireless data communication systems.  With a GPS system, it is even possible to achieve "overly" accurate synchronisation, but on the other hand, the synchronisation could be performed less frequently.

## Timing Options

When selecting a frequency reference, it is a good idea to favour solutions that offer good reception in the area of operation, good price-quality relationship and reliability.  In terms of the reception of the frequency reference, it is important that the whole group of beacons to be synchronised picks up the same reference.  Systems requiring unconventional aerial solutions are not excluded, as every beacon does not need to have a receiver.  There are other ways of forwarding the signals (see Section 9.3).

The GPS system is widely used for receiving an accurate time, and there are plenty of existing solutions.  In case there are plans to equip beacons with GPS positioning equipment, synchronisation through GPS is the best solution.

In Central Europe, the radio signal DCF77 offers good reception and it is frequently used in clock applications.

In the future, the television and radio network could possibly be used (especially digital networks), but currently picking up a time signal from a TV or radio signal is not quite so simple.

Others methods may include, but are not limited to, services such as the eLORAN operated in the UK, and the NW European chain in Norway.

Table 1 - Comparison of Frequency References

| Timing Method | Advantages | Disadvantages |
| --- | --- | --- |
| GPS (1,5 GHz) | Not limited by distance between lights. Low price, stable and accurate receiver, accurate atom-clock as a reference, small receiving antenna | Requires direct line of sight low power signal – subject to jamming and , to a less extent, spoofing |
| GSM-network (900 MHz, 1800 MHz) | Low price if GSM monitoring of beacon is already installed | Not all GSM-networks are UTC *- synchronized. Not all carriers provide access to the NITZ clock. No known commercial receiversapplications available |
| Radio signal (DCF77) (77,5 kHz) | stable and accurate | regional availability, atmospheric conditions may affect the signal |
| DVB (470–790 MHz) | works (available ?) all over the world | no commercial receivers available |
| Broadcast radio network | works all over the world | no commercial receivers available |
| TV line frequency | works all over the world | no commercial receivers available |
| Loran-C | Stable receiver, accurate atom-clock as a reference | more expensive than products using GPS, restricted receiving area, uncertain future. no commercial receivers available |

* UTC (Coordinated Universal Time)

Detailed explanations of synchronisation timing methods are outlined in Appendix 1.

The internal timing device within the synchronised light unit provides the essential back-up in the event of timing source failure.  This redundancy facility provides integrity in the synchronisation or sequencing of lights that allows this component of the AtoN to have availability sufficient to the needs of the service provider of the aids.

Centralised Synchronisation - Master-slave

The system used for centralised synchronisation is hierarchic.  One or more masters of a higher level control slaves of a lower level.  Receiving messages takes up less power than transmitting, and calculation or processing information takes up even less [7].  As low power consumption is important in the case of aids to navigation, a system should be favoured in which frequency references are processed locally, and the master is external.  In case a master inside the system is used, greater power consumption should be allowed for the master than for the slaves.

### External master

The time data used for synchronisation can be obtained from outside the system, for example from GPS satellites or a GSM network.  In other words, each beacon updates its internal clock according to the GPS or GSM network clock.  An external device near the group of beacons, which is capable of receiving one of the frequency references mentioned in the previous Section, could also work as an external master.  The master would transmit the correction data to the other devices that are part of the beacon group.

### Internal master

In case one of the devices in the beacon group receives an external frequency reference and, based on this, transmits correction messages to other devices in the group, we can talk about an internal master.  In case there is no need to use an external frequency reference, the clock of the internal master can be used as the reference for the whole group of beacons.  In this case, however, the distances must be sufficiently short in order for all the devices in the same beacon group to listen to the master.  When using an internal master, it should also be ensured that each device only listens to one master.

### Decentralised synchronisation - Peer-to-peer

The group to be synchronised can also be decentralised in its network topology.  In this case, there is no master in the group of devices, but the synchronisation is based on messages between the devices.  The network nodes can communicate with all of their neighbours, and synchronisation is not prevented by one node not working (as in master-slave synchronisation, if the master is out of order).

The following Sections discuss examples of how this type of synchronisation could be implemented.

### Protocol example

A RBS synchronisation method (Reference Broadcast Synchronization) has been suggested for sensor networks. [5].  In this method, the network nodes transmit reference messages to their neighbours, by which the operation of nodes is synchronised.  In the synchronisation process, the local reception times of nodes are compared to each other (the messages do not contain time stamps).  The greatest advantage of this protocol is that the "critical path" is shorter.  The critical path contains all those transactions that affect synchronisation accuracy.  The recipients must estimate the total time spent on sending the reference message, including such as the processing time of the message and waiting time, before sending the message.  All these affect the time accuracy.  The critical path of the RBS protocol only comprises the differences in travelling time and reception processing of the message.  The RBS protocol requires the sending of several messages. In case the system has n parts, the protocol requires O(n2) message exchanges.

Statistical clock synchronisation for sensor networks [6] is an extension of the RBS protocol.  By lowering the synchronisation accuracy, the complexity of calculations and power consumption can be reduced.  This is practical, especially if great synchronisation accuracy is not needed, and the power supply to the system is limited. In this method, the devices send several reference message to each other.  The reception times of messages are independently decentralised and thus describe the Gaussian distribution with the average of zero.  It can be assumed that the width of the Gaussian density function describes the synchronisation accuracy, and through this we can deduce how many reference messages must be sent to achieve the target accuracy.

# TRAINING & QUALITY MANAGEMENT

Consideration for preferred methods of installation, service and maintenance must be taken.

Reference manufacturers datasheet, and organisations standard operating procedures.

SYNCHRONISATION EXAMPLES

## FMA - Fairway of Vuosaari

A lighted fairway is currently being built to the new port of Vuosaari, a draft of which is included in Figure 10.  As the Figure is a draft only, the numbers of aids to navigation do not quite tally with Table 13.  The fairway can be divided into three sections.  The first section extends from the port to the first turn, and the second one from here to the turn between two islands, Eestinluoto and Itä-Tonttu. The third section extends from this turn to the open sea.

The first section is marked with two left and two right marks (all ice buoys).  The direction of the second section is first shown with two leading marks, in addition to which the fairway is marked with six left marks (one edge mark and five ice buoys) and ten right marks (ice buoys).  The second section also includes a wide part, which is marked with three east marks (ice buoys).  There also is a crossing fairway in the second section. A south mark (edge mark) has been placed at the crossing to improve clarity.  The direction of the third section is marked with two boards. In addition, the fairway is marked with two left marks (edge marks), one right mark (ice buoy), one east mark (buoy beacon) and a radar target.

For lateral and cardinal marks in the various sections of the fairway, see Table 13.  The flashing sequences of these marks are of different lengths, and the prerequisite for most synchronisation methods would be that all flashing sequences would be multiples of each other.  There is one south mark (sequence 10s) and three east marks (sequence 5s) in Vuosaari fairway. The sequence of the lateral marks is 3s.

Table 2- Lateral and Cardinal Marks on Vuosaari Fairway

|  | Lateral Marks |  | Cardinal Marks |  |
| --- | --- | --- | --- | --- |
|  | Left | Right | East | West |
| Section 1 | 2 ice buoys | 2 ice buoys |  |  |
| Section 2 | 1 edge mark 5 ice buoys | 10 ice buoys | 3 ice buoys | 1 edge mark |
| Section 3 | 2 edge marks | 1 ice buoy | 1 beacon buoy |  |

Figure 1 - Aids to Navigation in Vuosaari Fairway

## SMA – Synchronised fairways passage with buoys

Galten

The fairway passage is situated in Mälaren just west to Stockholm.  The synchronising was introduced as a permanent solution as the first place in Sweden in year 2000.

The fairway passage changes directions three times and is marked with 14 buoys.

From the beginning before the synchronising was installed the character was Fl R 3s and Fl (2) G 6s. The existing character was first used when synchronising was tested and it was then perceived as inconspicuous.  Test was then conducting with Fl G 3s and Fl R 3s in the straight part and with Q where the fairway turns.  This solution was considered as very good and is still in use today.

GALTEN 1:25000

## AMSA – Synchronised lights on pile beacons in Port Hedland (AUS)

Port Hedland is one of Australia’s busiest ports mainly handling iron ore exports.  The port is located on the remote, sparsely populated north-west coastline of Australia.

The area is characterised by shallow depths, significant tidal ranges (up to 7.5m) and strong cross currents.  The channel is some 23 nautical miles long, maintained to depths of 14-16 metres and contains a number of bends.  The main channel is primarily used by vessels leaving the port fully laden.  Vessels entering the port in ballast generally use an alternative route that is marked by 3 buoys (E1, E2 and E3).  Vessels using the channel commonly have a bridge elevation of around 30 metres.

Figure 2 - Aids to Navigation in Port Hedland Channel

The channel is marked by 30 beacons of which 13 are operated by AMSA with the remainder being inside the port limits and operated by the Port Hedland Port Authority. The lighting equipment comprises 12 x LED155 beacons and 1 x MLED350 beacon at C1 to provide a longer range to assist vessels to locate the outer end of the channel.  The beacons are located on piles that have an elevation of about 8 metres (above MHWS).

AMSA installed the current synchronisation system on the lights in October 2006 replacing an older system installed by the Port Authority where each light flashed together at a frequency of 2.5 seconds (0.5 flash / 2.0 second eclipse).  Using the characters as depicted in the table below, the lights are synchronised with critical turning points identified by white lights.  These revised synchronisation arrangements have been extremely well received by the port pilots.

Table 3 - Port Hedland Synchronised Beacons – New Characters and Nominal Range

| AMSA Ref | Name | Colour | Intensity Cd | Character | Nom. Range nM |
| --- | --- | --- | --- | --- | --- |
| AN183 | C1 | White | 1700 | Fl 2s (0.5,1.5) | 10 |
| AN184 | C2 | Red | 220 | Fl 2s (0.5,1.5) | 6 |
| AN185 | C3 | Green | 220 | Fl 2s (0.5,1.5) | 6 |
| AN186 | C4 | Red | 220 | Fl 2s (0.5,1.5) | 6 |
| AN188 | C6 | Red | 220 | Fl 2s (0.5,1.5) | 6 |
| AN189 | C7 | Green | 220 | Fl 2s (0.5,1.5) | 6 |
| AN190 | C8 | White | 220 | Fl 2s (0.5,1.5) | 6 |
| AN191 | C9 | White | 220 | Fl 2s (0.5,1.5) | 6 |
| AN192 | C10 | Red | 220 | Fl 2s (0.5,1.5) | 6 |
| AN193 | C11 | Green | 220 | Fl 2s (0.5,1.5) | 6 |
| AN194 | C12 | Red | 220 | Fl 2s (0.5,1.5) | 6 |
| AN195 | C13 | Green | 220 | Fl 2s (0.5,1.5) | 6 |
| AN196 | C14 | Red | 220 | Fl 2s (0.5,1.5) | 6 |

.

## TTC – Bremerhaven Synchronised Buoys

In September 2006 the TTC discussed with the local office of Bremerhaven several ideas to increase the conspicuity of the lighted buoys in the approach of Bremerhaven. This was necessary because a new container terminal causes high background lighting and therefore the lighted buoys could hardly be seen against the relatively bright background.

Situation:

In the approach of Bremerhaven 6 pairs of lighted buoys are concerned. The distance of the lighted buoys is between 1.5 km and 2 km. All of them are in pairs and they have three different characters due to different meaning and their position.

In Germany lateral marks are allowed to use only selected characters according to a national regulation (Erlass vom 24.11.77):

General lateral marks: fl (flash) or oc (occulting)

Lateral mark to show “change of course”: q (quick) 0.5+ (0.5)

The time schedule of the characters for each pair of buoys is shown in fig.1.

Proposals:

Different proposals according the character of the buoys were made:

oc 4s: 3 + (1)), fl. 4s: (1 + (3) and iso 4s: (2 + (2). To install a “runway effect” each buoy should get the same character. These ideas could be simulated by a program on screen.

Result:

According to the demands of the marines (position identification by flash character) it was decided that all pairs of buoys shall be kept with their own characters.

- Due to the relatively great distances between the buoys and the weather condition (light haze) during the test period only the next pair was visible. The situation that you can observe all 6 buoys pairs at the same time can be simulated on the screen only.

- According to this experience that only one pair can be seen no further experiments were made. A “runway light” as used on airports was not examined and would bring no advantage in this case.

- All lights shall be synchronized: they start at the same time (“point of synchronisation” in fig.1) with their characters so that you see the next pair of buoys switching on and off at the same time.

Summary:

In this application the most important result is that you get the best improvement by synchronizing a pair of buoys. There was nearly no benefit of synchronizing a whole chain of buoys.

Remark:

Visual aids like buoys and the GPS system are considered to be redundant to each other. In case of a synchronisation by GPS a certain dependency arises so that in case of a failure of the GPS system the advantage of the synchronisation is lost.

This is especially remarkable for leading lights.

Q : 0,5s + (0,5)s

Oc(2) 9s : (1)s + 2s + (1)s + 5s

Fl(2) 9s : 1s + (2)s + 1s + (5)s

Pair: 41/42

Pair: 43/44

Pair: 45/46

Pair: 47/48

Pair: 49/50

Pair: 51/52

Oc(2) 9s : (1)s + 2s + (1)s + 5s

Fl(2) 9s : 1s + (2)s + 1s + (5)s

Q : 0,5s + (0,5)s

Point of Synchronization

## GLA Synchronised lights viewing trial

Trinity House - Trials of Synchronised and sequential lights on buoys - River Swale -2008

Location:

The River Swale lies at the Southern side of the Thames Estuary to the East of the Isle of Sheppey.

Additional buoys were deployed in the river to ensure a satisfactory spacing was achieved to best demonstrate the concept of synchronised and sequential lights on buoys.

Trial 1 which was witnessed by a total of 59 stakeholders from all parts of the maritime community, including press representatives.

All buoys were programmed with characteristic Fl.G 2s or Fl.R 2s,  0.3s on + 1.7s off, and arranged to be synchronised.

After the trials vessel had entered the river the two gate buoys, Columbine and Whitstable Street were altered to flash for 1 second. This enabled the participants to view the differentiating conspicuity that is possible within the timing sequence, but maintaining overall synchronisation

Trial 2 took place about 1 month later when the lights on the buoys had been programmed to flash sequentially up the river. The first, gate pair had a longer flash and the eclipse between buoys was zero, as shown in the table below

|  | Columbine & Whitstable Street |  | Swale 1 & Swale 2 |  | Columbine Spit & Swale 4 |  | Swale 5 & Swale 6 |  | Ham Gat & Pollard Spit |  | Swale 7 & Swale 8 |  | Sand End & South Oaze |  | Receptive |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flash | 1.0 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  |
| Eclipse |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 1.9 |
| Sync delay (sec) | 0.0 |  | 1.0 |  | 1.3 |  | 1.6 |  | 1.9 |  | 2.2 |  | 2.5 |  | 2.8 |  |
| TIME |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The final, trial 3, involved reprogramming the lights to flash with both synchronised and sequential combinations. An extra pair of gate buoys, the Ham Gat and Pollard Spit had a longer flash length for individual conspicuity. After all buoys in the river flashing together, extended on the two pairs of gates, the first 4 pairs of buoys flashed together followed by one flash again of all buoys and then as the river curves the buoys in this section flashed in sequence with a general eclipse of 0.25 seconds before a final eclipse of 0.5 seconds when the run starts again.

When results of all three trials were analysed the main factors identified were:-

- The requirement for individual identification of certain buoys must be assessed when considering the waterway as a whole.

- The eclipse between respective buoys, or pairs of buoys, will depend on buoy spacing.

- Where curves in a channel occur, breaks in synchronisation and use of sequential systems should be considered.

## The Frigg Field Cessation Project

In the Frigg Field Cessation Project a simulation of aids to navigation to mark the disused structures were carried out by Ålesund University College on behalf of the concessionaire and coordinated by the Norwegian Coastal Administration.

Figure 3 - The Frigg Field disused structures after decommissioning (CDP1, TP1 and TCP2).

Table 4 - Proposed light characters

| Platform | Character | Light (s) | Dark (s) | Duty Cycle |
| --- | --- | --- | --- | --- |
| CDP1 | Fl W 10s | 0.8 | 9.2 | 0.087 |
| TP1 | LFl W 15s | 2.4 | 12.6 | 0.190 |
| TCP2 | Fl(3) W 15s | 0.8 | 1.2 / 10.2 | 0.194 |

Figure 4 - Sequencing scheme. Start of sequence is synchronized each 30 seconds.

For most simulations an 80 000 ton tanker with 20 m eye height was used. Different approaches from 10 n miles to 1 n mile were carried out (CPA = 1 n mile), and with different density of shipping and fishing activity in the vicinity.

To ensure validity, reliability and objectivity in the assessment of different navigation light configurations, 20 navigators was used as a reference group.

The group was exposed to three scenarios of the final stage of the cessation. First they experienced to see the Frigg Field at 5 nautical mile range with non-synchronous light characteristics, and then with synchronous light characteristics. Thereafter they experienced the field and the different light regimes from the same position, but now with the lights from eight fishing vessels around the substructures, and with the lights from the Heimdal Field at a greater distance.

Replying to specific questions the group concluded the following:

- Synchronized lights is the best alternative with background lighting (94 %)

- Synchronized lights is the best alternative without background lighting (88 %)

# DEFINITIONS

All appropriate definitions can be found in the IALA Dictionary.

DGPS		Differential Global Positioning System

DVB-S 		Dif Satellite Digital Video Broadcasting

DVB-T 		Terrestrial Digital Video Broadcasting

GSM 		Groupe Spécial Mobile

Global System for Mobile Communication

GPS 		Global Positioning System

HDTV 		High Definition Television

IALA 		International Association of Marine Aids to Navigation and

Lighthouse Authorities

IEEE 		Institute of Electrical and Electronics Engineers

IMSI 		International Mobile Subscriber Identity

ISO 		International Organization for Standardization

LED 		Light Emitting Diodes

LF 		Low Frequency

MSC 		Mobile Services Switching Center

NASA 		National Aeronautics and Space Administration

NITZ 		Network Identity and Timezone

NTP 		Network Time Protocol

PAL 		Phase Alternation Lines

RBS 		Reference Broadcast Synchronization

RDS 		Radio Data System

SCTS 		Service Center Time Stamp

SDH 		Synchronous Digital Hierarchy

SMCS 		Short Message Service Centre

SMS 		Short Message Service

TCP 		Transmission Control Protocol

TPSN 		Timing Synch Protocol for Sensor Networks

UHF 		Ultra High Frequency

UTC 		Coordinated Universal Time

VLR 		Visitor Location Register

WLAN 		Wireless Local Area Network

## Terminology

Synchronised lights – all flash at the same time

Sequential lights – flash one after another

Synchronisation slippage – timing error between synchronised lights

Temporal separation – time difference between flashes

Conspicuity of lights – the ability of lights to stand out from their surroundings

Flare path – runway effect of sequential lights

Synchronisation delay – deliberate delay in synchronised timing

Angular subtense – the observed angular separation between two lights

Angular separation – the geometrical separation between two lights

## Symbology

Charting; The IHO’s Chart Standardisation and Paper Chart Working Group have considered how to chart synchronised lights.

The Working Group noted that the use of synchronised and sequential lighting patterns is increasing.  It agreed that these should be seen on the nautical chart and details given in chart notes or associated publications.

The Working Group decided that the abbreviation ‘sync’ was to be used, but that ‘seq’ would not be used.  Although sequential lights are not strictly synchronised, it was decided that the qualifying term ‘sync’ would be used to cover both cases.

Synchronised lights often occur on buoys or beacons.  Details of their synchronicity will be given in the List of Lights, Sailing Directions and/or a chart note.  The international abbreviation ‘sync’ may be added to the light description

Fl.R. 3s (sync)

# References

[1] FMA, FMA´s Navguide, 2003.

[2] FMA, Synchronisation of Beacons on a Fairway, Report 2006

[3]

[4] S. Ganeriwal, R. Kumar, M. Srivastava, Timing-sync protocol for sensor

networks, Proc. First Int. Conf. on Embedded Network Sensor Systems, Los

Angeles, California, Nov. 2003.

[5] J. Elson , L. Girod , D. Estrin, Fine-grained network time synchronization using

reference broadcasts, Proc. of the 5th symposium on Operating systems design

and implementation, Boston, Massachusetts, December 09-11, 2002.

[6] S. PalChaudhuri, A. Saha, D. B. Johnson, Probabilistic clock synchronization

service in sensor networks, Technical Report TR 03-418, Department of

Computer Science, Rice University, 2003.

[7] G. Pottie, W. Kaiser, Wireless integrated network sensors, Communications of

the ACM, 43(5), pp 51-58, May 2000.

[8] Synchronised Lights Viewing Trial, GLA Report Version RPT-09-03-MN-IT-07

# APPENDIX 1 –

# TECHNICAL DESCRIPTION OF SYNCHRONISING METHODS

GPS

The GPS satellites continuously transmit scattered spectrum signals at the frequencies of 1575.42 MHz (L1) and 1227.60 MHz (L2).  The code intended for civilians (the C/A code) is transmitted at the frequency L1 and the P code used by the military at frequencies L1 and L2.  The codes transmit satellite position navigation message and time.  The GPS receiver calculates its position by measuring the relative reception times of the satellite signals, which means it has to have a direct line of sight to the satellites.  A signal that is reflected (multi-path, as it is known) for example from water may distort the results.

The accuracy of a GPS receiver can be improved by using base stations positioned at known locations, which transmit real-time correction data to the mobile receiver.  This method is called differential GPS (DGPS), and it permits achieving positioning accuracies of 2 – 4 metres.  The correction data can be transmitted using various systems, such as GSM network support stations or MF (approx 300 kHz) using radio masts.  The correction data is usually transmitted in the RTCM format.  An RTCM message only includes correction data, and this is why the time to be corrected still has to be obtained from a GPS satellite.

The most direct way of making use of GPS signals in synchronisation is to equip each device to be synchronised with its own GPS receiver.  There are finished chipsets in the market, which can take data received through an aerial to an internal frequency source.  The circuits intended for calculating the GPS position naturally are more complicated than those only receiving a clock time or synchronisation pulse and transmitting it further.  Thanks to their commonness, the circuits used in GPS applications are cheap and their power consumption is no longer very high.  No other costs are incurred in this case, as receiving the signal is free, and the buoys do not need to communicate with each other.

The GPS signal would enable continuous synchronisation, but this would be unnecessary and impractical.  The internal clocks or frequency sources of the lights usually are good enough to stay well synchronised for minutes, tens of minutes or even hours depending on the product (oscillators).  The synchronisation could be performed at certain intervals (activation by GPS receiver) in order to maintain the required accuracy.  This would also reduce the power consumption.

Recent GPS jamming trials by the GLA R&RNav Directorate in the UK have demonstrated that synchronised lights will continue to free run in synchronisation for a significant period of time in the presence of GPS jamming so long as the lights are synchronised at the commencement of the jamming. However if jamming occurs before lighting up time so that the lights come on in the absence of valid GPS timing signals, synchronisation will not occur. Flashers using GPS timing for synchronisation should therefore be designed so that the flasher timers remain synchronised even when the light is off during daytime.

### GSM network time

In the GSM network, it is possible to obtain the local time and date (including summer time) from a base station.  In the case of the TeliaSonera (Telecommunications provider in the Nordic and Baltic countries) network, the clock time is read from the Mobile Switching Center (MSC), and it is accurate.  The MSC reads the clock's SDH (Synchronous Digital Hierarchy) from an atomic clock.  In the SDH network, a caesium clock works as the synchronizer at the frequency of 2.048 MHZ.  The clock time is transmitted as a code in data signals, and the time data is regenerated in the network nodes.  Automatic updating of time and the NITZ feature (Network Identity and Timezone) can be selected both in the network and in mobile communication devices according to GSM standards.  In the oldest phones, it is not possible to make use of this data, but more recent models usually can update their own internal clock when contacting the network.

Time updates are normally performed when registering with the network, when the mobile phone moves to another geographical time zone or the network time changes (for instance between summer and winter time).  The base station can also transmit the NITZ data when the network changes its identity, or at any time during the signalling connection.

A mobile communication network with an NITZ feature can transmit the local time IMSI identifier (International Mobile Subscriber Identity) as an attachment.  An IMSI identifier is transmitted e.g. when the phone is switched on.  As an attachment to an IMSI identifier, time data is transmitted only in case that the subscriber's data is found in the mobile phone network VLR (Visitor Location Register).  In case the subscriber data are missing in the visitor register, this can usually be rectified by switching the phone off and on again.

In case there is a need to update the internal clock or oscillator of the buoys using a GSM receiver and NITZ data, rebooting the terminal device once should be sufficient.  It should be noted, however, that NITZ only works if supported by the terminal device.  In case the terminal supports the GPRS functionality and the GPRS connection is always on, automatic updating of time will not work.  In other words, if the terminal device has GPRS capability, the device cannot be on the "always on" mode.

The network time can also be obtained in a roundabout way for example by sending a text message to one's own number, in which case the SMS message received will include the Service Center Time Stamp (SCTS) of the Short Message Service Centre (SMSC).  In other words, all beacons should send an SMS message to themselves, which makes this an expensive and impractical synchronisation method.  Additionally, the processing of SMS messages is associated with specific delays.  Another possible method would be to establish a connection to a network time server (e.g. pool.ntp.org) using a TCP protocol (Transmission Control Protocol) commonly used on the Internet.  This method, however, would be very expensive and impractical.  The third alternative would be to use a dedicated channel in the cell (Cell Broadcast channel) to transmit the clock data.  The members of the same beacon group would probably also belong to the same GSM cell, which would make this easy to implement.  In TeliaSonera network, however, Cell Broadcasting channels are not in use.

### DCF77 – Frankfurt time – frequency standard stations

In Germany, Central European time (UTC+1 in the winter, UTC+2 in the summer) is transmitted at the frequency of 77.5 kHz.  The DCF77 transmitter aerial transmits the Braunschweig atomic clock time from the vicinity of Frankfurt am Main (50:01N, 09:00E) and the signal can be received within a radius of 2,000 km.  In Finland, this signal should be received up till Oulu.  In Denmark, a company called Danaton has implemented synchronisation using a DCF77 reference station.  The company does not, however, offer a separate synchronisation device, nor are their flashers and light sources suitable for all aids to navigation.  In addition, it is uncertain if the signal is strong enough in Finland for the company's equipment.

### Digital television (DVB)

In HDTV (High Definition Television), not only video and audio signals but also other information can be transmitted, including a time code.  The time code is transmitted in accordance with the DVB-T (Terrestrial Digital Video Broadcasting) and DVB-S (Satellite Digital Video Broadcasting) standards.

In Digita's networks, the time code is usually transmitted along wires from Pasila to TV transmitters located elsewhere in Finland.  This time code is corrected periodically. The only exceptions are those TV transmitters that are so close to each other that their coverage areas are partly overlapping.  These transmitters, however, are equipped with GPS receivers, from which they receive the accurate time.  If the whole group of beacons receives the time message transmitted by the same transmitter, the time differences are too insignificant to affect synchronisation.  If the various beacons in a group listen to different transmitters, there is a high probability that the accuracy of the time data received still remains adequate, as the transmitters use a common GPS time.

Digital TV broadcasts are within television's UHF range (470-790 MHz) in three channel bundles (three landline networks).  In 2005, the channel bundles A and B will cover 99.9% of the Finnish population and bundle C 78%.  The outer sea areas are not in the range at the moment.  The size of the aerial needed to receive TV network signals is too large to be suitable for floating aids to navigation.  In addition, the aerial must be directional.  In fixed aids to navigation, the size and directionality are not as great a problem as in case of floating aids.

### Radio network (Yleisradio, Finnish National Radio Organisation)

By means of FM RDS (Radio Data System) it is possible to transmit useful information to the recipients, such as the name of the radio channel and programme, traffic status reports or time.  The RDS clock of Yleisradio is locked to the GPS clock, and the transmitter codes transmit the time once a minute.  In case an error occurs in the reception of the clock time, no correction will be possible until after one minute.  Some receiver models read the time from the first message received when turned on.  If the receiver is tuned to a local radio frequency transmitting a wrong clock time, the receiver is left with the wrong time.  In this case, the receiver will not read the new time when changing to Yle's channel, unless it is turned off and on again on this channel.

There are a number of radio stations in the world with the task of transmitting an accurate UTC time signal at different frequencies.  Some of the best known ones of these are the WWW station in Colorado, USA and WWH in Hawaii, USA.  These radio stations transmit data at the frequencies 2.5, 5, 10, 15, and 20 MHz.  Unfortunately, these radio stations cannot be picked up in Finland.  In the LF frequency area, however, there is a standard DCF77 station that can be received in Finland (see Chapter 3.2.3).

### TV line frequency

TV messages also include the line frequency 15625 Hz, which is locked to an accurate frequency reference.  In the PAL system (Phase Alternation Lines) used in Europe, each whole image consists of 625 lines, and 25 whole images are transmitted to the recipient every second (625 x 25 = 15625).  The frequency references of large national companies, such as Yleisradio, are based on caesium or rubidium clocks.  In principle, the internal oscillator of the buoy flasher could be calibrated using the reference frequency of the TV signal.

### e-Loran

Loran-C, which operates at LF frequencies, was originally designed for radio navigation in the US coastal areas.  The overland Loran-C base stations transmit signals simultaneously.  As the references for Loran-C transmissions, caesium oscillators are used, and this is why the signals are often used for frequency calibrations.  Using a Loran-C receiver, an accurate UTC pulse (or counter) can be obtained, but no clock time.  A Loran-C Nels has been working in Europe since the 1950's, but in January 2006 this system ceased to operate.

Some countries, notably US and UK have upgraded (or are in the process of upgrading) their LORAN stations to eLORAN’.

### Synchronisation using other radio signals

No time code or frequency reference is necessarily needed for synchronisation, as it can also be performed using a wakeup signal.  A prerequisite for this in a group of beacons is a device that, working as a master, transmits a wakeup signal.  The other devices work as slaves and correct their clock times based on the wakeup signal.  This is the most simple synchronisation method based on a radio signal that has been commonly used to synchronise lights prior to GPS.  In addition, the functioning of the devices can be synchronised by sending messages between them.

# Appendix 2 – GLA Synchronised Lights Viewing Trial

***********************