AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

DRAFT

IALA Recommendation E-200-0

On

Marine Signal Lights

Part 0 - Overview

Edition 0.3

March 2008

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 01 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      		Internet -  http://iala-aism.org

Recommendation on Marine Signal Lights, Part 0 - Overview

(Recommendation E-200-0)

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING the need to provide guidance within which the colours and colour boundaries of lights on aids to navigation should be determined;

RECOGNISING ALSO that that such guidance should enable a common approach to be made world-wide, thus greatly assisting mariners, who, while passing through waters of different authorities, should not be confused by light colours that are ambiguous;

RECOGNISING FURTHER that this document supersedes the IALA “Recommendations for the Colours of Light Signals on Aids-to-Navigation” dated December 1977;

NOTING this document applies only to marine Aid-to-Navigation lights installed after its published date;

NOTING ALSO that from three years after the date of this Recommendation, all lanterns placed in service should have colour coordinates in either the IALA Optimum or IALA Temporary regions but that to avoid colour confusion, the IALA Optimum region is preferred;

NOTING FURTHER that from ten years after the date of this Recommendation, all lanterns placed in service should have colour coordinates in the IALA Optimum region;

CONSIDERING that to be reviewed by Secretariat - for example.

ADOPTS the Recommendation on Marine Signal Lights in the annexes of this recommendation; and,

RECOMMENDS that National Members and other appropriate Authorities providing marine aids to navigation services [action to be taken].

*    *    *

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.  [as required]

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| 30 June 2005 | First draft version | Revised by specialized working group tasked with amalgamating a number of older IALA recommendations and look into the usage of blue lights. |
| 13 February 2008 | Second draft version | Updated slightly after Ad Hoc Group meeting in Koblenz January 2008 |
| 2008-02-21 | proposal | Frank |
| 2008-03-20 | Third draft | Omar |

Table of content

1	INTRODUCTION	5

1.1	Background	5

1.2	Superseded documents	5

1.3	Scope of the E-200 series or recommendations	6

1.4	Purpose of this document	6

2	Overview of the E-200 series of Recommendations	7

2.1	General	7

2.2	A short description of the individual parts of the E-200 series	7

2.3	How to use the E-200 series of Recommendations	8

3	Index of selected topics	10

4	General Definitions (TBC)	10

5	References	10

Annex 1

The E-200 series of Recommendations

on Marine Signal Lights

# INTRODUCTION

## Background

Mariners of all times have needed Visual Aids to Navigation. The first application of a system of signal lights for navigational purposes is believed to have been made for maritime purposes. AtoN technology developed over time and major developments in light sources and optics had been achieved before the beginning of the 20th Century. A lot of investigation in physics, meteorology and physiology were carried out for marine lights and later the results were used e.g. for railroad and aeronautical lights.

Throughout its 50 years history IALA has provided guidance on both engineering aspects and management aspects of Aids to Navigation to its members. Consequently IALA has published numerous recommendations and guidelines on the technical aspects of visual aids. The number of relevant documents has made it increasingly complicated for members to get a thorough understanding of the basic theory and recommended methods and mathematical models used. Furthermore the developments in light source technologies (Light Emitting Diodes) has revealed some shortcomings of the model recommended for calculating effective intensity.

This development has led to the need to review and amalgamate a number of IALA documents into one set of interrelated documents, making it easier for members to acquire relevant information on various aspects of visual aids to navigation.

Need to mention new aspects of colour and intensity measurement and in particular issues related to Effective Intensity.

## Superseded documents

This series of recommendations supersede the following IALA recommendations:

- IALA Recommendation for the colours of light signals on aids to navigation (Dec. 1977);

- IALA Recommendation on the determination of the luminous intensity of a marine aid to navigation light (Dec. 1977);

- IALA Recommendation on the calculation of the effective intensity of a rhythmic light (Nov. 1980);

- Draft IALA Recommendation E-122 on the photometry of marine aids to navigation signal lights (Oct 2004);

- Recommendation for the notation of luminous intensity and range of lights (1966);

- Recommendation for a definition of the nominal daytime range of maritime signal lights intended for the guidance of shipping by day (1974).

The content of the abovementioned recommendations has been updated and amalgamated into this new series of recommendation.

## Scope of the E-200 series or recommendations

The E-200 series of recommendations gives guidance on the recommended basic characteristics of marine signal lights i.e. luminous intensity (brightness), spectral properties (colour), how to measure their temporal characteristics and how to estimate how well the signal is perceived by the mariner from a distance.

This series of recommendations does not give guidance on the technical design of marine signal lights, their operation or management.

## Purpose of this document

The purpose of this first part of the E-200 series of recommendations is to give some background information and an overview of the recommendations in the series as well as to give guidance to readers on where to find specific topics.

# Overview of the E-200 series of Recommendations

## General

The series consists of:

- Part 0 – Overview

- Part 1 – Colours

- Part 2 – Notation of Luminous Intensity and Range

- Part 3 – Measurement

- Part 4 – Determination and Calculation of Effective Intensity

- Part 5 – Calculation of the Performance of Optical Apparatus

## A short description of the individual parts of the E-200 series

Part 0 – Overview (this document) gives some background information and an overview of the recommendations in the series as well as to give guidance to readers on where to find specific topics.

Part 1 – Colours describes the recommended spectral characteristics i.e. recommended colour chromaticity regions of marine signal lights. Information on how and why these regions have been adjusted is given. A new recommended chromaticity region for blue lights is defined and some guidance on the use of blue lights is given.

Part 2 –Notation of Luminous Intensity and Range describes how the illumination at the eye of an observer varies with distance and how to quantify Luminous Range. A definition of Luminous Intensity is given as well as criteria for calculating Nominal Luminous Range. The necessary formulae to be used for these calculations are described. These are useful when calculating the luminous range of an existing light as well as when calculating the required luminous intensity of a new light with a given required range.

Part 3 – Measurement describes the recommended principles for measuring the characteristics of marine signal lights. Recommendations are given on laboratory procedures and equipment as well as details of methods such as zero length photometry, telephotometry, tristimulus colorimetry and spectroradiometry.

Part 4 – Determination and Calculation of Effective Intensity describes how to calculate the Effective Intensity of a flashing signal light. It gives an overview of existing methods for calculating effective intensity and recommends that the Schmidt Clausen method be used for this purpose.

Note that this is the old stuff and that the work of the ad-hoc group has revealed that we need to recommend another method or perhaps multiple methods for determining effective intensity. Hopefully a completely rewritten E-200-4 will be submitted to EEP11 as a late paper. Certainly a discussion will tale place in WG4 at EEP11.

Part 5 – Calculation of the Performance of Optical Apparatus describes how to calculate the performance of optical apparatus (intensity) when direct measurement is impossible or impractical. It is intended as a guide to estimation of the luminous intensity and angle of divergence of the beam from various types of beam projection apparatus when data can be obtained by direct measurement on similar but not identical combinations of light source and optical system. The methods described are referred to as comparison or “ratio-ing” techniques.

## How to use the E-200 series of Recommendations

(Section needs to be developed further)

There are two main ways of using the E-200 series of recommendations, Top Down and Bottom Up.

Top-Down

From the nautical requirements, the required effective luminous intensity under given service conditions can be derived. If the design of the optical apparatus is known the photometric intensity can hereafter be calculated.

Bottom-Up

For a given beacon the photometric intensity can be measured or calculated by the tools of part 3 or 5. The result is then used to obtain the values for the effective intensity under given service conditions and the luminous range.

The figure below gives the link between the parts of the recommendation:

In most cases the colour of a signal light (part 1) can be treated separately. It has however influence on the description of the nautical requirements and on the measurement (part 3).

| Action | What part of E-200 to use |
| --- | --- |
| Decide Operational requirement | Not covered by E-200 |
| Selecting the colour of a light source | 1 |
| Calculate the Luminous Range | 2 |
| Calculate the Nominal Luminous Range | 2 |
| Estimate the Required Intensity | 2 |
| Estimate the influence of background lighting | 2 |
| Estimate the influence of meteorological visibility | 2 |

# Index of selected topics

Selected topics – in what part can they be found?

(A table of content for the whole series basically)

Background Lighting

# General Definitions (TBC)

(in Koblenz 2008 the WG agreed that definitions should be in the individual parts and/or the revised IALA Dictionary. This has later been challenged and is under consideration).

# References

[1] IALA Recommendation for the colours of light signals on aids to navigation (Dec. 1977);

[2] IALA Recommendation on the determination of the luminous intensity of a marine aid to navigation light (Dec. 1977);

[3] IALA Recommendation on the calculation of the effective intensity of a rhythmic light (Nov. 1980);

[4] IALA Recommendation E-122 on the photometry of marine aids to navigation signal lights (Oct 2004);

[5] Recommendation for the notation of luminous intensity and range of lights (1966);

[6] Recommendation for a definition of the nominal daytime range of maritime signal lights intended for the guidance of shipping by day (1974).