AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

DRAFT

IALA Recommendation [E200-2]

On

Marine Signal Lights

Part 2 - Calculation, definition and notation of luminous range

Edition 0.57

[Date approved by Council]

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      		Internet -  http://iala-aism.org

Recommendation on XXX [ Title Here]

(Recommendation [leave number blank, to be filled by IALA Secretariat])

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING the need to publish the performance of marine signal lights;

RECOGNISING ALSO the need to specify, design and quantify the perormance of marine signal lights worldwide;

RECOGNISING FURTHER that;

NOTING this document only applies to marine Aid-to-Navigation lights installed after the date of this publication;

NOTING ALSO that;

NOTING FURTHER that;

CONSIDERING that

ADOPTS the tables and charts in the annex of this recommendation; and,

RECOMMENDS that IALA Members and other appropriate Authorities providing marine aids to navigation services design, specify and publish the performance of marine Aid-to-Navigation signal lights in accordance with this recommendation.

*    *    *

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.  [as required]

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| May 2004 [example only] | Section 3.4; Tables 1-4; Graphs – Chromaticity Regions | Revised to reflect CIE No. 39-2, 1983– Recommendations for Surface Colours for Visual Signalling. |
| October 2008 | General | Revised during IALABATT/IALALITE Copenhagen |

Document Revisions	3

Annex	5

IALA Recommendation E-200-2	5

Marine Signal Lights	5

Part 2 - Notation of Luminous Intensity and Range	5

1	INTRODUCTION	5

1.1	Scope / Purpose	5

1.2	Background / History	5

2	Physical basics – Allard’s Law	6

2.1	Allard’s law	6

2.2	Allard's law using the atmospheric transmissivity T	6

2.3	Allard's law using the transmissivity   for 1 nautical mile	7

2.4	Meteorological Visibility	8

2.5	Allard’s Law based on Meteorological Visibility	9

3	Luminous Range	9

4	Nominal Luminous Range	10

4.1	Definition of the nominal luminous range of lights intended for the guidance of shipping	10

4.2	Notation of the nominal luminous range of lights intended for the guidance of shipping	10

5	Important Factors in the Design of Marine Signal Lights	10

5.1	Service Condition Factor	10

5.2	Local Conditions	11

5.3	Zone of Utilisation	11

5.4	Background Luminance	11

5.5	Leading Lights	11

Appendix 1 Diagrams and Tables	12

A1.1 Meteorological Visibility and Transmissivity	12

A1.2 Luminous range for night time	13

A1.3 Luminous range for daytime	16

References	21

Annex

IALA Recommendation E-200-2

Marine Signal Lights

Part 2 - Notation of Luminous Intensity and Range

# INTRODUCTION

## Scope / Purpose

The scope of this recommendation is to permit providers and manufacturers of marine AtoN lights, as well as mariners, to determine the luminous range of lights as a function of their intensity and of the meteorological visibility. This recommendation provides a link between the physical and photometric features of marine AtoN lights and the luminous range information given to the mariner.

For providers of marine AtoN lights, this[include as appropriate – the scope of the document / benefit of adhering to the information contained in the document]

recommendation should be used to estimate the required luminous intensity when designing lights.

Manufacturers of marine AtoN lights should quote the nominal luminous range (also known as the ‘nominal range’) of their lights in accordance with this recommendation.

## Background / History

The IALA definition of the luminous range of lights was first introduced by a Recommendation in 1966 [1]. For many years this definition has been an important basis for the description of marine AtoN lights.

However, since 1966 several additions have been made to the definition of luminous range. These additions were spread over five IALA documents ([2], [3], [4], [5], [6]).

To avoid confusion the information from these six documents has been collated into a single document for the calculation, definition and notation of luminous range of lights, which helps to distinguish between the different threshold values and their application.

The two basic recommendations [1] and [3] included nomograms for luminous range estimation. These nomograms are still supported. However, the wide-spread use of computer modelling makes it feasible to base the estimation of luminous range on formulae. Therefore, these have been provided in this recommendation.

Previous IALA Recommendations and the IALA Dictionary use the “sea mile” as the unit of measure for luminous range, nominal luminous range, and meteorological visibility.  This document replaces the sea mile with the nautical mile as the preferred unit of measure and as the unit of measure used in definitions.  The difference between a sea mile (about 1853.2 m) and a nautical mile (1852 m) is small, and of no practical consequence for these calculations.  The nautical mile has been chosen as the unit of measure because it is used more widely than the sea mile.

For many countries the use of SI-units is obligatory. The 'nautical mile' and the units derived from it are outside this International System of Units. One aim of this recommendation is to give the reader the necessary formulae to convert between the SI-units and the units used in navigation.

# Physical basics – Allard’s Law

The illuminance of a signal light at the observer’s eye can be calculated by a physical law called Allard’s law.

## Allard’s law

Allard’s law allows the calculation of the illuminance E as a function of distance d, luminous intensity I and an exponential factor z.

Ed=Ie−zdd2				(equation 1)

The exponential factor z describes the atmospheric absorption and scattering (extinction).  In practice, there are alternative ways of characterizing the prevailing atmosphere as follows.

## Allard's law using the atmospheric transmissivity T

Atmospheric transmissivity (T) is defined as the ratio of the luminous flux transmitted by the atmosphere over a unit distance to the luminous flux which would be transmitted along the same path in a vacuum.

T=ΦdUΦvacuumdU				(equation 2)

Where:

T is the atmospheric transmissivity (dimensionless)

(dU) is the luminous flux at the unit distance after passing through the atmosphere

vacuum(dU) is the theoretical luminous flux at the unit distance after passing through a vacuum

dU is the unit distance

Because the ratio of the luminous fluxes in equation 2 is the same as the ratio of the corresponding illuminance values, equation 2 can be rewritten as

T=EdUEvacuumdU				(equation 3)

Where:

E(dU) is the illuminance at the unit distance after passing through the atmosphere

Evacuum(dU) is the theoretical illuminance at the unit distance after passing through a vacuum

Inserting expressions for E(dU) and Evacuum(dU) from equation 1 into equation 3, and noting that for Evacuum(dU) z = 0, equation 3 becomes

T=e−zdU					(equation 4)

Combining equation 1 and 4 yields

Ed=ITddUd2				(equation 5)

## Allard's law using the transmissivity   for 1 nautical mile

The unit distance for transmissitity is chosen to be one nautical mile.  Expressed in all metric units equation 5 takes the form

Ed=ITMddUd2				(equation 6)

Where:

E(d) is the illuminance at distance d in metres

I is the luminous intensity in candela

TM is the atmospheric transmissivity [dimensionless] for 1 nautical mile

d is the distance in metres

dU is the unit distance that corresponds to the transmissivity [1852 m]

In practice the distance d is expressed in nautical miles.  Using the fact that one nautical mile equals 1852 metres and suppressing the unit distance in the exponent equation 6 can be written as

Ed=ITMd1852 metresnautical mile×d2			(equation 7)

where d is the distance in nautical miles.

Simplifying and suppressing all units yields

Ed=I(3.43×106)TMdd2 			(equation 8)

Where:

E(d) is the illuminance at the eye of the observer in lm/m2 [lx]

I is the luminous intensity of the light [cd]

TM is the transmissivity for one nautical mile of the atmosphere

d is the numerical value of the distance in nautical miles

## Meteorological Visibility

The meteorological visibility is an alternative way to describe the extinction by the atmosphere of the object being viewed, which in the development above is quantitatively characterised by the atmospheric transmissivity.

Meteorological visibility is the greatest distance at which a black object of suitable dimensions can be seen and recognized by day against the horizon sky, or, in the case of night observations, could be seen and recognized if the general illumination were raised to daylight level [2].

By definition the relationship between the meteorological visibility (V) and the transmissivity is

V=ln 0.05ln TM×dU				(equation 9)

Where:

V is the meteorological visibility in nautical miles

TM is the transmissivity [dimensionless] for one nautical mile

dU is the unit distance of 1 nautical mile

Suppressing the units and suppressing the unit distance yields:

V=ln 0.05ln TM					(equation 10)

## Allard’s Law based on Meteorological Visibility

It is recommended in the IALA dictionary that the atmospheric extinction be described by using meteorological visibility V rather than the transmissivity TM.

Allard's law can be expressed using meteorological visibility V by combining equations 8 and 10.

Ed=I(3.43×106)0.05dVd2				(equation 11)

Where:

E(d) is the illuminance at the eye of the observer [lx]

I is the luminous intensity of the light [cd]

d is the distance in nautical miles

V is the meteorological visibility in nautical miles

the units (not shown) associated with (3.43106) are m2/M2

# Luminous Range

In the case of a light that appears as a point source, the luminous range D is defined as the maximum distance at which a light can be seen, as determined by the luminous intensity I of the light, the meteorological visibility V and the threshold of illuminance Et at the eye of the observer. At this distance, the illuminance E at the observer’s eye is reduced to the threshold value Et .

Inserting these parameters into equation 11 and rearranging yields:

I=(3.43×106)EtD20.05−DV 			(equation 12)

Where:

I is the luminous intensity of the light [cd]

Et is the threshold of illuminance at the eye of the observer [lx] (see 4.1)

D is the luminous range in nautical miles

V is the meteorological visibility in nautical miles

Equation 12 is recommended for the calculation of the luminous range of signal lights.  Due to the numerical character of equation 12, numerical iteration is necessary in order to calculate the luminous range D.  A rough estimation of D can be derived from the nomograms provided in this recommendation.

# Nominal Luminous Range

## Definition of the nominal luminous range of lights intended for the guidance of shipping

IALA recommends that the nominal luminous range of maritime signal lights intended for the guidance of shipping should be defined as follows:

The nominal luminous range of a maritime signal light is the distance in nautical miles at which this light produces an illumination at the eye of the observer:

- of 2  10-7 lx for night time range

- of 1  10-3 lx for day time range

It should be assumed that meteorological visibility V equals 10 nautical miles    (TM = 0.7411) and that the atmosphere is homogenous.

Note: Please see A1.2.3 for further considerations of threshold illumination values.

Notation of the nominal luminous range of lights intended for the guidance of shipping

IALA recommends that the following information on nominal range of lights intended for the guidance of shipping should be published in the “Lists of Lights”:

- The nominal range of lights intended for the guidance of shipping by night;

- Where applicable, the nominal range of lights intended for the guidance of shipping by day;

- Nomograms permitting mariners to estimate the luminous range of lights intended for the guidance of shipping by day or by night as a function of their nominal range, the prevailing meteorological visibility and, where applicable, the sky luminance in the direction of observation.

# Important Factors in the Design of Marine Signal Lights

The following important factors should be taken into account when selecting marine AtoN signal lights for installation. As a result, a different luminous intensity may be required to achieve the required range.

## Service Condition Factor

In practical installations, the degradation of luminous intensity under service conditions, due to light source degradation, dirt and salting of lanterns etc., should be taken into consideration.  It is recommended that the intensity used to calculate the nominal range for publication should include a service factor.  It is recommended that this service factor be taken as 0.75 (corresponding to a reduction in intensity of 25%). Although this service conditions factor includes degradation of the light source output, it should be noted that some light sources, such as discharge lamps and LEDs, can degrade significantly more than 25% over their lifetime. When designing and installing AtoN signal lights, degradation over the service period as well as the lifetime of the equiment should be taken into consideration.

## Local Conditions

The prevailing visibility conditions will vary over different geographical locations.  Therefore, when selecting a light, this should be taken into account. Selection should be based on a practical luminous range value and not on nominal range.

## Zone of Utilisation

The required range may vary over the zone of utilization of the light.

## Background Luminance

Different levels of background luminance  may require different values of illuminance threshold values (see Appendix 1).

## Leading Lights

It is important to note that a leading light, like any other night-time light, will have a nominal luminous range that corresponds to the distance at which the illumination at the eye of the observer is 2  10-7 lx.  However, IALA recommendation E-112 for leading lights [5] states that the illumination required for an observer to use the leading lights for alignment at night must be at least 1  10-6 lx.  Because the illumination level that corresponds to nominal luminous range is 5 times less than the level needed to align the lights in the useful segment, the concept of nominal range for a leading light is not typically used.

Et=(0.242×10−6))×1+0.4L 2

- Appendix 1
Diagrams and Tables

## A1.1 Meteorological Visibility and Transmissivity

V=ln 0.05ln TM∗dU=−3 Mln TM		TM=0.05dUV=0.051 MV

## A1.2 	Luminous range for night time

### A1.2.1 Diagram Luminous Range

Threshold value for illuminance Et =  210-7 lx

I = 0.686 D2 (0.05)-D/V   where I is in candela, and D & V are numerical values in M

### A1.2.2 Table (night time)

Table to be used to determine the night time nominal range

rounded off to the nearest nautical mile

| Luminous | Nominal | Luminous | Nominal | Luminous | Nominal |
| --- | --- | --- | --- | --- | --- |
| intensity | range | intensity | range | intensity | range |
|  | (rounded) |  | (rounded) |  | (rounded) |
| candelas | nautical miles | kilocandelas (103 cd) | nautical miles | Megacandelas (106 cd) | nautical miles |
| 1 - 2 | 1 | 0.633 – 1.06 | 9 | 0.927 – 1.35 | 26 |
| 3 - 9 | 2 | 1.07 – 1.75 | 10 | 1.36 – 1.96 | 27 |
| 10 - 23 | 3 | 1.76 – 2.84 | 11 | 1.97 – 2.84 | 28 |
| 24 - 53 | 4 | 2.85 – 4.53 | 12 | 2.85 – 4.11 | 29 |
| 54 - 107 | 5 | 4.54 – 7.13 | 13 | 4.12 – 5.93 | 30 |
| 108 - 203 | 6 | 7.14 – 11.1 | 14 | 5.94 – 8.53 | 31 |
| 204 - 364 | 7 | 11.2 – 17.1 | 15 | 8.54 – 12.2 | 32 |
| 365 - 632 | 8 | 17.2 – 26.1 | 16 | 12.3 – 17.5 | 33 |
|  |  | 26.2 - 39.7 | 17 | 17.6 – 25.1 | 34 |
|  |  | 39.8 – 59.9 | 18 | 25.2 – 35.9 | 35 |
|  |  | 60.0 – 89.8 | 19 | 36.0 – 51.2 | 36 |
|  |  | 89.9 - 133 | 20 | 51.3 – 72.9 | 37 |
|  |  | 134 -198 | 21 | 73.0 - 103 | 38 |
|  |  | 199 - 293 | 22 | 104 -147 | 39 |
|  |  | 294 - 432 | 23 | 148 - 209 | 40 |
|  |  | 433 - 634 | 24 |  |  |
|  |  | 635 - 926 | 25 |  |  |

Threshold value for illuminance Et =  2  10-7 lx

### A1.2.3 Compensation for Background Lighting (night time)

### The threshold of detection of 210-7 lx corresponds to a situation with no background lighting. In most real situations the lights are viewed against a background that does have lights. This will reduce the luminous range.

The recommended method for compensating for background lighting is to use different values for the threshold of illuminance.

### Two different values should be used:

minor background lighting:	2  10-6 lx	factor 10 

substantial background lighting:	2  10-5 lx	factor 100 

According to equation 12 of the recommendation, background lighting will increase the required intensity by the factor above when the luminous range is fixed.

Regarding a light with a fixed intensity the consideration of background lighting will reduce the luminous range.

### The graph in A2.2.1 has been drawn for a threshold of 210-7 lx. For the other values of threshold (minor and substantial background lighting) mark off along the scale of abscissae the distance between 'No Background lighting (NONE)' and that under consideration as it appears on the auxiliary scale.

### Example:

Suppose that it is required to find the luminous range of a light with a nominal luminous range of 17 M and a luminous intensity of 32,300 cd for substantial background lighting and a visibility of 5 M.

Measure the distance A separating 'no background lighting (NONE)' and 'substantial background lighting (SUBSTANTIAL)'. Transfer this distance to the scale of abscissae from graduation to 17 M (32,300 cd) in the same sense. A point slightly to the right of graduation corresponding to 7 nautical miles is obtained. Erect from this point a parallel to the axis of ordinates to meet the curve for 5 nautical miles visibilty. Read off the luminous range on the vertical scale against the point so obtained. We read approx. 5 nautical miles.

## A1.3 Luminous range for daytime

### A1.3.1: Estimation of Threshold of Illuminance for Daytime Range

The mariner should be able to estimate the luminous range of lights by day for different sky luminances.  However, the threshold of illuminance Et in lx, produced by a light, depends on the luminance L of the sky in candelas per square metre, in the direction of observation according to the formula:

Et=(0.242×10−6))×1+0.4L 2 			(equation 13)

Where:

Et is the threshold illuminance at the observer’s eye in lm/m2 [lx]

L is the sky (background) luminance in cd/m2

The threshold Et of 110-3 lx thus corresponds to a sky luminance of 10,000 candelas per square metre. The calculated threshold Et should be inserted in equation 12

### A1.3.2 Diagram Luminous Range

Threshold value for illuminance Et =  110-3 lx

I =  3430 D2 (0.05)-D/V   where I is in candela, and D & V are numerical values in M

### A1.3.3 Explanation of daytime diagram

Calculation of threshold at day

Et=(0.242×10−6 lx)×1+0.4×L(cdm²)2

Et : 	threshold of illuminance

L : 	luminance of sky in the direction of observation

| Meteorological condition | Luminance in cd/m² | threshold Et in 10-3 lx |
| --- | --- | --- |
| Very dark overcast sky | 100 | 0.013 |
| Dark overcast sky | 200 | 0.024 |
| Ordinary overcast sky | 1 000 | 0.107 |
| Bright overcast sky or clear sky away from the direction of the sun | 5 000 | 0.506 |
| Bright cloud or clear sky close to the direction of the sun | 10 000 | 1 |
| Very bright cloud | 20 000 | 1.98 |
| Glaring cloud | 50 000 | 4.91 |

Use of the graph (A1.3.2):

The graph has been drawn for a sky luminance of 10 000 cd/m². For  other values of sky luminance mark off along the scale of abscissae the distance between the luminance of 10 000 cd/m² and that under consideration as it appears on the auxilliary scale.

Example:

Suppose that it is required to calculate the luminous range of a light of 2 000 000 cd for a meteorological visibility of 2 nautical miles under an ordinary overcast sky (luminance 1 000 cd/m²).

Measure the distance A separating graduations 10 000 cd and 1 000 cd on the auxiliary scale. Transfer this distance to the scale of abscissae from graduation corresponding to 2 000 000 cd (2106 cd) in the same sense. A point slightly to the right of graduation corresponding to 12 nautical miles is obtained. Erect from this point a parallel to the axis of ordinates to meet the curve for 2 nautical miles visibility. Read off the luminous range on the vertical scale against the point so obtained. We read approx. 4 nautical miles.

### A1.3.4 Table (daytime)

Table to be used to determine the daytime nominal range

rounded off to the nearest nautical mile

| Luminous | Nominal | Luminous | Nominal |
| --- | --- | --- | --- |
| intensity | range | intensity | range |
|  | (rounded) |  | (rounded) |
| kilocandelas (103 cd) | nautical miles | Megacandelas (106 cd) | nautical miles |
| 1 – 12.0 | 1 | 1.02 – 1.82 | 7 |
| 12.1 – 45.3 | 2 | 1.83 – 3.16 | 8 |
| 45.4 – 119 | 3 | 3.17 – 5.32 | 9 |
| 120 – 267 | 4 | 5.33 – 8.78 | 10 |
| 268 – 538 | 5 | 8.79 – 14.2 | 11 |
| 539 – 1010 | 6 | 14.3 – 22.6 | 12 |
|  |  | 22.7 – 35.6 | 13 |
|  |  | 35.7 – 55.5 | 14 |
|  |  | 55.6 – 85.6 | 15 |
|  |  | 85.7 – 130 | 16 |
|  |  | 131 – 198 | 17 |
|  |  | 199 – 299 | 18 |
|  |  | 300 – 449 | 19 |
|  |  | 450 – 669 | 20 |
|  |  | 670 – 993 | 21 |
|  |  | 994 – 1460 | 22 |

Threshold value for illuminance Et =  110-3lx

A1.3.5 Table (Night and day with background)

# References

[1]	Recommendation for the notation of luminous intensity and range of lights.

(IALA, November 1966)

[2]	International Dictionary of Aids to Marine Navigation, Chapter 2, Visual Aids

2-1-265 to 2-1-285

(IALA 1970)

[3]	Recommendation  for a definition of the nominal daytime range of marine

signal lights intended for the guidance of shipping by day

(IALA 1974)

[4]	Recommendations on the determination of the luminous intensity of a marine

aid-to-navigation light

(IALA 1977)

[5]	Recommendation for leading lights

(IALA, E-112, May 1998)

[6]	Recommendation on the photometry of marine aids to navigation Signal lights

(IALA, E-122, June 2001)