# Review of the IALA Maritime Buoyage System

## Background

One of the conclusions of the XVIth IALA Conference (held in China in May 2006) was

Although the maritime buoyage system has served the maritime community well since its inception in the 1970s, it is time to review the system in light of changes in the shipping environment and the development of digital aids to navigation.

This led to a recommendation that

IALA review the maritime buoyage system in consultation with all related organizations and user groups.

The task to review the IALA Maritime Buoyage System, as first published in 1980, was placed on the ANM Committee’s 2006-2010 Work Programme.

## Stakeholder Consultation

The ANM Committee began discussing this task immediately after the Shanghai Conference.  Since that time, members have been collecting and analyzing opinions and options for the future of the MBS.  All of this has fed into the collation of stakeholder consultation that followed.

The ANM Committee initiated a consultation process in April 2007, and continued with a second stage, later in the year.  This resulted in numerous responses which have been collated and analysed at ANM11 (April 2008).

Over six hundred responses to the MBS questionnaire were received from national members; varying from individual users, pilot associations, harbour master associations, shipping interests, hydrographic offices, and maritime and aids to navigation authorities.  Responses were received from all major regions of the world.

Consolidated responses were provided by national authorities and some individual users. These responses were collated by a special working group of the ANM committee and the general findings are shown below.

## Findings

The results of the widespread consultation endorse the view of the committee that the fundamental principles of the MBS should be retained. However, due to changes in shipping patterns, innovations and technological developments, some enhancements to the MBS are required.

Ideally, a unified marking arrangement would, in principle, be desirable for Regions A and B.  However, almost all view this change as impractical, costly, detrimental to safety and possibly unachievable.

A particular suggestion for more commonality between the two marking regions was identified and will be progressed by the committee.

More details on the responses to the MBS questionnaire, as produced by the committee, is given at the Annex.

## Proposed Revision of the IALA MBS Booklet

Having considered the responses to the MBS survey, the committee has, following much debate, captured the following points as key elements that will shape the revision of the MBS:

- The MBS is effective and should not be fundamentally altered. Responses indicate general satisfaction with the existing MBS, noting the substantial contribution made to the safety of navigation since its inception.

- There is significant opportunity to improve the scope of the MBS, taking advantage of innovations and new technologies.

- It is unrealistic to consider progressing to a unified system (Region A and B)

- The IALA Emergency Wreck Marking Buoy (EWMB), currently on trial, is gaining acceptance and should be incorporated fully into the MBS.

- There is considerable support for the view that the EWMB when deployed should remain on station for as long as required.

- There is some support that the EWMB should be re-designated as an Emergency Danger Marking Buoy.

- There has been a proliferation in the use of Special Marks as an ‘all-purpose’ aid; it is desirable to distinguish the different uses of Special Marks.

- There is support for re-defining Special Marks to allow its use as a navigational mark.

- There is support that Special Marks have additional IHO approved symbology attached to the buoy.

- There has been a proliferation in the use of Cardinal Marks with consequent confusion or inability to distinguish the meaning of individual AtoNs.

- There is support that the Isolated Danger Mark should have clearer guidance on its use and that there should be means of indicating the extent of the hazard.

- It is desirable that the revised MBS document makes reference to existing IALA guidance on the efficient disposition and types of buoyage.

- The revised MBS booklet should also make reference to innovations and integration of new technologies (such as sequential and synchronised lights, blue lights, radio aids and AIS as AtoN).

- The information contained in the MBS Booklet ought to be widely promulgated so that the mariner may use it in conjunction with the appropriate nautical chart or publication and other systems.

## Action Plan

### The IALA Council is invited to

- Note the results of the widespread consultation carried out by IALA

- Accept the findings as the basis for a revision of the IALA MBS Booklet

- Approve the circulation of this document to all the technical committees of IALA in time for the Sep/Oct 2008 meetings.

- Approve the production and distribution of a revised IALA MBS Booklet (and the resources required)

- Approve a draft information paper for input to IMO NAV 55 (July 2009), outlining the update of the MBS Booklet.

- Advise the ANM Committee of their decision, after their May 2008 meeting (43rd Council Session).

### The aim is to

- Present the final MBS booklet to the IALA Council in December 2009 for approval.

- Circulate the final MBS booklet at the 2010 IALA Conference (to be held in March 2010, Cape Town, South Africa).

- Encourage the widest possible circulation of the new MBS booklet, including the mandatory carriage requirements on board vessels.

*********

Annex

Responses to MBS Questionnaire

Introduction

Over six hundred responses to the MBS questionnaire were received from national members; varying from individual users, pilot associations, harbour master associations, shipping interests, hydrographic offices, and maritime and aids to navigation authorities.  Responses were received from all major regions of the world.

Consolidated responses were provided by national authorities and some individual users. These responses were collated by a special working group of the ANM committee and the general findings are shown below.

Do you think the MBS requires modification?

- MBS does not requires major modification

- MBS can be improved though the use of:

  - clarification of definitions

  - additions of some marks

- There is a general preference for unification of IALA A and B systems:

  - However, implementation difficulties were noted by many respondents:

    - Too difficult and time consuming due to scale of system

    - Changes would be cost prohibitive for Competent Authorities

    - Transition period would create confusion

  - One suggestion was that IALA could consider guidelines for characteristics that could designate the Port or Starboard side of channel.  For example, Port-Hand (or Starboard-Hand Side) could utilize a new standardized  light characteristic:

    - Double flash

    - Two light

    - Some new characteristic (not yet defined)

    - Changes could be implemented over a long period

    - A change in color scheme would not be required, since the newly defined characteristic would indicate the port or starboard side

Are you satisfied with the existing characteristics of lights in use in the MBS?

- Overall, there is general satisfaction with existing system of lights

- Mariners have a need to quickly identifying lights, this need is most prevalent in regions that have:

  - Fast vessels (e.g. high speed ferries)

  - High traffic density

  - Use of Cardinal Marks in small areas

Are you aware of any marine accidents or incidents due to the misinterpretation of / confusion with buoyage within a region?

- Not a widespread problem.

- Only 2 reported accidents; 1 in US and 1 in France

- The response to this survey is not considered reliable for collecting this data.  If necessary, additional analysis using accident reporting data (i.e. Lloyds and Competent Authorities) could be pursued to further determine if this is truly a problem.

Do you believe there is a need for additional or new types of floating aids to navigation?

- There is general support for the following new types of lights

  - Synchronization of lights (to include sequencing) and use of Flicker

  - Utilization of new light characteristics (not currently defined by MBS)

  - Utilization of new combination of light colors

  - Many responses indicated this was an issue that was worthy of pursuit

  - Standards and guidelines need to be developed to help Authorities determine how best to utilize these types of lights

  - Some doubts were expressed and IALA was urged to use caution before publication of new guidelines that could potentially confuse the mariner

- Emergency Wreck Marking Buoy (EWMB)

  - This mark could be generalized to be used to mark all types of emerging dangers requiring an immediate response, and be renamed the “Emergency Danger Marking Buoy” to better reflect its purpose.

  - Of the 15 responses that addressed the use of the EWMB, 12 were in favor of implementation in the MBS.  There were 3 responses from National Authorities that indicated they would not implement the EWMB within their system, however they did not oppose its inclusion in the MBS.

  - The period of use should not be prescribed by MBS guidelines

- Special Marks

  - The Special Mark has been widely used, perhaps to close an unfilled gap in the MBS

  - The definition of the Special Mark in the MBS that they “are not primarily intended to assist navigation” should be reconsidered; in many areas Special Marks are assisting navigation

  - Utilization of standard symbols used in official nautical documents (e.g. charts and publications) should be displayed on the body of the special mark to indicate the nature of the special area.  Development of a standard picturegram by the MBS is encouraged.

- Isolated Danger Mark

  - Difficult to lay and maintain

  - The minimum safe distance to pass is not indicated

  - Authorities do not have guidance on the maximum size of the danger for which this mark is appropriate

  - One response was received to eliminate the use of this mark

Do you have any other comments or suggestions on the MBS?

- Expanded use of AIS as AtoN is encouraged, in particular as a way to mark areas of danger (e.g. isolated danger, restricted areas, etc)

- The format of the MBS guidelines should be aligned to the IMO Highway of the Sea framework:

FAIRWAY MARKS

Lateral Marks

Safe Water Marks

OBSTRUCTION MARKS

Cardinal Marks

Isolated Danger Marks

New Danger Mark (e.g. Emergency Wreck Marking Buoy)

SPECIAL MARKS

- Changes made to MBS guidelines should be made in consultation to standards developed by other maritime organizations (e.g. IMO, CEVNI, etc).

- Changes to MBS guidelines should consider interface with standards established by Competent Authorities for Inland systems

- Additional consideration of electronic and radio navigation (e.g. GPS, AIS, etc) should consider the question as to whether the MBS is solely a physical system of buoyage?

- The preamble to the MBS guideline should be updated to address the following National Authority concerns:

  - The responsibility of mariners for safe navigation

  - The appropriate use of aids to navigation in conjunction with official nautical documents SOLAS Chapter V and IMO Resolutions

  - The appropriate context of the MBS as it relates to electronic navigation

  - A recommendation concerning the above concerns could be issued before the MBS guidelines are updated