IALA Guideline No. XXXX

On

Synchronisation of Lights

Draft

April 2008

Rev 3

AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      Internet -  http://iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

1	INTRODUCTION	6

2	DEFINITIONS	6

3	BENEFITS OF SYNCHRONISATION	7

4	Limitations/Considerations of Synchronisation	9

4.1	Environmental limitations	9

4.2	Maximum slippage time	10

4.3	Minimum angular separation	10

5	Synchronisation configuration for optimum conspicuity	10

5.1	Logical Grouping of lights	10

5.2	Use of different characters	11

6	Synchronization Methods	11

6.1	General methods of synchronisation	12

6.2	Timing Options	13

6.2.1	GPS	14

6.2.2	GSM-networks time	14

6.2.3	DCF77 – Frankfurt clock – frequency standard stations	14

6.2.4	Digital television (DVB)	14

6.2.5	Radio network (Yleisradio, Finnish National Radio Organisation)	14

6.2.6	TV stripe frequency	14

6.2.7	e-Loran	14

6.2.8	Synchronization using other radio signals	14

6.3	Master-slave synchronization	14

6.3.1	External master	14

6.3.2	Internal master	14

6.3.3	Protocol example	14

6.4	Peer-to-peer synchronization	14

6.4.1	Night time switch	14

6.4.2	Protocol example	14

7	Experiences with synchronization	15

7.1	Finnish Maritime Administration (FMA)	15

7.2	Farvandsvæsenet – Royal Danish Administration of Navigation and Hydrography	15

7.3	Sjöfartsverket – Swedish Maritime Administration	15

7.4	Kystverket – The Norwegian Coastal Administration	15

8	Synchronisation Examples	15

8.1	FMA - Fairway of Vuosaari	15

8.2	SMA – Synchronised fairways passage with buoys	16

8.3	AMSA  – Synchronised Channel lights	17

8.4	TTC – Synchronised Buoys	18

9	Summary	20

10	References	20

11	Annex 1 – GLA Synchronised Lights Viewing Trial	20

IALA GUIDELINE FOR SYNCHRONISATION OF LIGHTS FOR MARKING CHANNELS AND WATERWAYS

# INTRODUCTION

In the provision of marine aids to navigation (AtoN), advances in technology, both in light sources and communications, have enabled lights to be synchronised far more readily and accurately than before. Previously the use of synchronised lights was limited due to the lack of external local, regional or global communications systems providing adequate timing accuracy. However, now that robust and accurate timing options are available, synchronisation is now a practical proposition.

Additionally, advances in manufacturing have made synchronisation of AtoN lights readily available for use on fixed and floating applications. This can provide the mariner with new enhancements, aimed at improving the overall conspicuity of aids to navigation lights, especially in built-up areas and areas with confusing background lightings.

A number of AtoN providers have already conducted extensive demonstrations, some having established synchronised or sequential light systems in particular locations. The results of a number of these demonstrations are attached to this Guideline as Annexes.

This has led to readily available, relative low cost, self contained AtoN light units produced by a number of manufacturers in the global market that can be used world wide.

The use of synchronised and sequential lights should be assessed on the basis of recognised risk management meeting the navigational requirement.

In addition to the objective of this Guideline, there are other applications where synchronization / sequentialisation may apply, as reflected in the Recommendation O-116 on Marking of Aquaculture Farms (recommendation 1.4), or other types of aids such leading lines or when the identification of the "geometry" is relevant to the mariner

# DEFINITIONS

All appropriate definitions can be found in the IALA Dictionary.

- Terminology

Synchronised lights – all flash at the same time

Sequential lights – flash one after another

Synchronisation slippage – timing error between synchronised lights

Temporal separation – time difference between flashes

Conspicuity of lights – the ability of lights to stand out from their surroundings

Flare path – runway effect of sequential lights

Synchronisation delay – deliberate delay in synchronised timing

Angular subtense – the observed angular separation between two lights

Angular separation – the geometrical separation between two lights

- Symbology

Charting ;  The IHO’s Chart Standardisation and Paper Chart Working Group have considered how to chart synchronised lights.

The Working Group noted that the use of synchronised and sequential lighting patterns is increasing. It agreed that these should be seen on the nautical chart and details given in chart notes or associated publications.

The Working Group decided that the abbreviation ‘sync’ was to be used, but that ‘seq’ would not be used. Although sequential lights are not strictly synchronised, it was decided that the qualifying term ‘sync’ would be used to cover both cases.

Synchronised lights often occur on buoys or beacons. Details of their synchronicity will be given in the List of Lights, Sailing Directions and/or a chart note. The international abbreviation ‘sync’ may be added to the light description

Fl.R. 3s (sync)

# PURPOSE AND SCOPE

The purpose of this Guideline is to provide IALA Members and AtoN providers with guidance on the provision of synchronised lights as part of an AtoN system for marking channels and waterways , with regard to improving the conspicuity of the AtoN especially in areas where aids are difficult to distinguish.

This document applies to marine aids to navigation signal lights in fixed and floating applications.

# APPLICATIONS

It has long been a concern to authorities, service providers and mariners that the growing light pollution in coastal, port approach and harbour areas is reaching critical levels and often overwhelming aids to navigation when seen against this background. It is often difficult to pick out and identify the aids or it is otherwise impossible for the mariner to distinguish the channel , and, therefore, there is a particular need to emphasise the navigable channel or recommended routes.

In seeking to meet the navigational requirement as identified by risk assessment, the option of using synchronised and / or sequential lights provides a useful alternative to conventional AtoN lights in enhancing the aids when viewed against background lighting.

Trials that have been carried out have produced a number of interesting results that should be taken into consideration when applying synchronisation.

For example:

- Synchronised lights, with the same character, in gated pairs are particularly conspicuous especially when marking bends in a channel.

- For lights flashing sequentially, the sync delay needs to be determined taking into account the geometry of the channel and in particular the paired buoy spacing. As a guide – “the closer the buoys are together the shorter the delay may be”.

- The geographical spacing between synchronised lights within a group, on fixed or floating aids, should be such that the group as a whole is within the observer’s field of view.

- The character period of synchronised lights should be sufficiently short such that the observer can see those aids as frequently as possible.

- Selection of characters and management of synchronised leading lights should be  such that the front and rear lights can be individually identifiable.

- The geographical layout and tapestry of the aids to navigation, channel or port approach where synchronised aids are deployed should be considered as a whole before decisions are made to establish such aids.

- The height of eye of the observer should be considered when selecting the light units as a narrow divergence may well cause obscuration from the bridge of large vessels when close to the aid.

- The likely sea state and prevailing visibility – ie local conditions – should be considered when planning to use synchronised or sequential lights in a waterway.

- The application of synchronised and / or sequential aids to navigation does not necessarily provide the mariner with positional information.

- Synchronised lights provide spatial awareness and orientation within a channel or system of aids to navigation.

- It is envisaged that the development of e-Navigation in providing high integrity positional information will further allow the concept and application of synchronised and sequential aids to navigation in the future.

- A complete notice to mariners must be published informing the mariners when a synchronized system is put in place.

- Affected mariners should be consulted when designing the synchronized system.

## 5.	BENEFITS OF SYNCHRONISATION

Synchronising of two or more flashing lights is already in use in signalling systems for various transport systems, including road, rail, aviation, and maritime. The purpose of synchronising is to increase the conspicuity of the signal, and or to indicate that the two or more lights are associated in some manner.  For example if two buoys form a “gate” in a channel, the lights on them might be synchronised to make that gate pair more conspicuous.

Sequencing of lights is when a series of lights is flashed in a time sequence to show the geographical relationship between them.  Such a set of lights is sometimes called a “flare path” and provides the user with directional information.

It is also possible to combine the two effects, so that, for example, if there is a channel marked by pairs of buoys, the lights on each pair are synchronised, and in addition the pairs light in a time sequence along the channel.

In each case the objective is to help the mariner distinguish which lights are pairs of buoys (or beacons) marking a channel, and in addition to indicate which pairs are closest and which are more distant.

Following extensive trials and experience gained with both synchronised and sequential aids to navigation it is clear that two key advantages can result from the use of such AtoN:

- Synchronised lights provide high impact conspicuity. Drawing the observers eye to their presence and overcoming background lighting due to their regular and combined effect

- Sequential lights provide directional awareness and information when within a system, eg waterway, the observer experiencing visual movement in the horizontal plane.

The results of a number of these trials are included in the Annexes to this Guideline.

# Limitations/Considerations of Synchronisation

## Environmental limitations

- In certain locations it  the synchronisation of several lights together may be considered as light pollution by non-users.

- Power requirements to provide a synchronised light system will to a limited degree in general be greater. Solar systems may not produce adequate power at high latitudes, and especially with respect to minor aids and/or where other AtoN are also deployed at the site eg AIS, racons.

- The impact of synchronised / sequential lights can be adversely affected by; buoy stability, visibility, excessive height of eye v. vertical divergence, and general adverse weather and sea conditions (in a manner similar to conventional marking)

## Maximum slippage time

Based on the reports from the General Lighthouse Authorities of the United Kingdom and Ireland , it is recommended  that the timing error (synchronisation slippage) between synchronised lights should not be greater than 50ms. (Refer to GLA Report Version RPT-09-03-MN-IT-07.)

## Minimum angular separation

It is recommended that there should be a minimum angular separation of 5 minutes of arc, subtended at the observer, within the arc of utilisation of the synchronised lights.

# 7	Synchronisation configuration for optimum conspicuity

The flashing sequences of aids to navigation can be synchronised in a number of different ways, and the devices should flash in order for the fairway to be the most visible to the mariner. Solutions tested to date indicate that having the lights of a complete fairway section flashing simultaneously is a good solution for optimum conspicuity.

It is important to test or simulate the synchronisation of flashing lights on one fairway in different conditions before making decisions on the synchronisation method to be introduced. This will enable the extent that the visibility of the fairway can be improved in proportion to the power consumption needed for synchronisation to be ascertained. Tests should also involve affected mariners.

## Logical grouping of lights

Fairway turning points can be used to divide the fairway into sections. Lateral marks of the same fairway section could be synchronised, after which the aids marking the next section could be turned on. In order for the various fairway sections to stand out they should have similar characteristics.

Another possible alternative would be to simulate airport runway lights, which are turned in sequence (consecutively). This alternative could also be utilised to guide vessels to port. However, this may look peculiar when leaving the port unless the lights can be turned on in the opposite order.  In this alternative the flashing sequences at the ends of the fairway should be different from those in the middle. This method is not advisable on a shipping lane as the aids to navigation are not placed at equal intervals.

A third alternative would be to synchronise the different sides of the fairway. Using this method, the direction of the fairway could be highly visible but detecting its width would be difficult.

Use of different characters

## Different characters could be used effectively to identify the beginning of a fairway or change in the fairway. For example the first two buoys or channel markers could have a different character from the rest of the channel, whilst remaining synchronised.

Reference should be made to existing IALA Recommendations and Guidelines when considering the use of different characters `vis: E-110 and the IALA MBS

# 8	Synchronisation Methods

Synchronisation and sequencing require identical time information being available at all the lights which are to be synchronised or sequenced.  This can be accomplished in at least two ways:

- An accurate internal time mechanism at each light location

- Reception at each light location of the same timing data from a trusted source

In practice the latter is usually employed, and implementation is done by a radio broadcast system set up for the purpose, or by reception of an existing timing signal, or by cable connection.  Early synchronisation systems often employed a special local radio transmitter, emitting a timing signal received at each light.

Nowadays various local or global time signals are available and dedicated radio transmitters are not required. The most commonly used timing signal is that contained within a GNSS system, such as GPS or GLONASS.

In some countries, radio timing signals are broadcast at VLF and these have been used for synchronisation of lights.  In Japan, the JJY signals at 40kHz and 60kHz have been used, with a small H-field antenna built into the lantern, but such systems are now largely phased out in favour of GPS.

Note that synchronisation of the timing of the lights in the group is all that is needed.  Synchronisation to GPS time or other standard is not needed.  Synchronisation accuracy of 100 parts per million is acceptable in most cases . .

Timing by a Global Navigation Satellite System (GNSS) implies that the synchronised light system will not be fully. If the GNSS for any reason should be unavailable the synchronised light system will have to degrade to an ordinary light system. The probability for this to happen is low.

Typically, each light is fitted with an integral timing device that provides the rhythmic flash character. It is common practice to adjust this integral timer by synchronising it with an external pulse from a reference timing source (GNSS Clock or Local Radio Transmitter). It is important that the timing accuracy of the lights integral timer is maintained to within 50 ms between external synchronisation pulses i.e. the clock contained in the light must be sufficiently accurate in the short term to stay in step with the GNSS clock or other timing source.

In the event of a timing source failure, the accuracy of the light should be maintained for sufficient time period for marine use. eg three hours(?)

## TRAINING

## QUALITY MANAGEMENT

## 9.	General methods of synchronisation

## Timing Options

General information regarding different timing options to be added here…..

| Timing Method | Benefits | Disadvantages |
| --- | --- | --- |
| GPS (1,5 GHz) | Low price, stable and accurate receiver, accurate atom-clock as a reference, small receiving antenna | needs direct visual contact |
| GSM-network (900 MHz, 1800 MHz) | Low price | Not all GSM-networks are UTC *- synchronized |
| response / impulse signal (selectable frequency) | many options, stable system | synchronising unit may have large power consumption |
| Radio signal (DCF77) (77,5 kHz) | stable and accurate | regional availability, atmospheric conditions may affect the signal |
| DVB-T (470–790 MHz) | works all over the world | no commercial receivers available |
| Broadcast radio network | works all over the world | no commercial receivers available |
| TV stripe frequency | works all over the world | no commercial receivers available |
| Loran-C | Stabile receiver, accurate atom-clock as a reference | more expensive than products using GPS, restricted receiving area, uncertain future |

* UTC (Coordinated Universal Time)

The integral timing device within the synchronised light unit provides the essential back-up in the event of timing source failure. This redundancy facility provides integrity in the synchronisation or sequencing of lights that allows this component of the aid to navigation to have availability sufficient to the needs of the service provider of the aids.

### GPS

### GSM-networks time

### DCF77 – Frankfurt clock – frequency standard stations

### terrestrial digital television (DVB-T - Digital Video Broadcasting Terrestrial)

### Radio network (Yleisradio, Finnish National Radio Organisation)

### TV stripe frequency

### e-Loran

### Synchronization using other radio signals

## Master-slave synchronization

### External master

### Internal master

### Protocol example

## Peer-to-peer synchronization

### Night time switch

### Protocol example

# Annex	1  -  Experiences with synchronization

## Finnish Maritime Administration (FMA)

## Farvandsvæsenet – Danish Maritime Safety Administration

## Sjöfartsverket – Swedish Maritime Administration (SMA)

## Kystverket – The Norwegian Coastal Administration

Annex 2 	Synchronisation Examples

Example 1

## FMA - Fairway of Vuosaari

Example 2

## SMA – Synchronised fairways passage with buoys

Galten

The fairway passage is situated in Mälaren just west to Stockholm.  The

synchronising was introduced as a permanent solution as the first place in

Sweden in year 2000.

The fairway passage changes directions three times and is marked with 14

buoys.

From the beginning before the synchronising was installed the character was

Fl R 3s and Fl (2) G 6s. The existing character was first used when

synchronising was tested and it was then perceived as inconspicuous.  Test was then conducting with Fl G 3s and Fl R 3s in the straight part and with Q where the fairway turns. This solution was considered as very good and is still in use today.

GALTEN 1:25000

Example 3

## AMSA  – Synchronised Channel lights

In January 2005 trials of a sequenced (“flare path”) light system was carried out in the Point Richards Channel in the Port of Geelong, Australia.  Feedback from the pilots using the channel indicated that a system of synchronisation that allowed all of the lights to flash together would be preferred instead of the sequenced lights approach.

Closely following this trial was a project to upgrade the main channel beacons.  In consultation with the pilots a new improved lightning scheme for the channel that included synchronisation was agreed.  The channel was reviewed and it was decided to split the channel into four sections for the purpose of synchronising the lights, with sections 1 and 3 synchronised and sections 2 and 4 synchronised.

At the start of each section of the channel a fixed light plus a synchronised flashing light was implemented to clearly mark the start of that new section.

Example 4

## TTC Bremerhaven  – Synchronised Buoys

In September 2006 the TTC discussed with the local office of Bremerhaven several ideas to increase the conspicuity of the lighted buoys in the approach of Bremerhaven. This was necessary because a new container terminal causes high background lighting and therefore the lighted buoys could hardly be seen against the relatively bright background.

Situation:

In the approach of Bremerhaven 6 pairs of lighted buoys are concerned. The distance of the lighted buoys is between 1.5 km and 2 km. All of them are in pairs and they have three different characters due to different meaning and their position.

In Germany lateral marks are allowed to use only selected characters according to a national regulation (Erlass vom 24.11.77):

General lateral marks: Fl (flash) or Oc (occulting)

Lateral mark to show “change of course”: Q (quick) 0.5+ (0.5)

The time schedule of the characters for each pair of buoys is shown in fig.1.

Proposals:

Different proposals according the character of the buoys were made:

Oc 4s 3 + (1)), Fl 4s (1 + (3)) and Iso 4s (2 + (2)) ?. To install a “runway effect” each buoy should get the same character. These ideas could be simulated by a program on screen.

Result:

According to the demands of the mariners (position identification by flash character) it was decided that all pairs of buoys shall be kept with their individual ? own characters.

- Due to the relatively great distances between the buoys and the weather condition (light haze) during the test period only the next pair was visible. The situation that you can observe all 6 buoys pairs at the same time can be simulated on the screen only.

- According to this experience that only one pair can be seen no further experiments were made. A “runway light” as used on airports was not examined and would bring no advantage in this case.

- All lights shall be synchronized: they start at the same time (“point of synchronisation” in fig.1) with their characters so that you see the next pair of buoys switching on and off at the same time.

Summary:

In this application the most important result is that you get the best improvement by synchronizing a pair of buoys. There was nearly no benefit of synchronizing a whole chain of buoys.

Remark:

Visual aids like buoys and the GPS system are considered to be redundant to each other. In case of a synchronisation by GPS a certain dependency arises so that in case of a failure of the GPS system the advantage of the synchronisation is lost.

This is especially remarkable for leading lights.

Q : 0,5s + (0,5)s

Oc(2) 9s : (1)s + 2s + (1)s + 5s

Fl(2) 9s : 1s + (2)s + 1s + (5)s

Pair: 41/42

Pair: 43/44

Pair: 45/46

Pair: 47/48

Pair: 49/50

Pair: 51/52

Oc(2) 9s : (1)s + 2s + (1)s + 5s

Fl(2) 9s : 1s + (2)s + 1s + (5)s

Q : 0,5s + (0,5)s

Point of Synchronization

Example 5

# GLA Synchronised Lights Viewing Trial

Trinity House - Trials of Synchronised and sequential lights on buoys - River Swale -2008

Location:

The River Swale lies at the Southern side of the Thames Estuary to the East of the Isle of Sheppey.

Additional buoys were deployed in the river to ensure a satisfactory spacing was achieved to best demonstrate the concept of synchronised and sequential lights on buoys.

Trial 1 which was witnessed by a total of 59 stakeholders from all parts of the maritime community, including press representatives.

All buoys were programmed with characteristic Fl.G 2s or Fl.R 2s,  0.3s on + 1.7s off, and arranged to be synchronised.

After the trials vessel had entered the river the two gate buoys, Columbine and Whitstable Street were altered to flash for 1 second. This enabled the participants to view the differentiating conspicuity that is possible within the timing sequence, but maintaining overall synchronisation

Trial 2 took place about 1 month later when the lights on the buoys had been programmed to flash sequentially up the river. The first, gate pair had a longer flash and the eclipse between buoys was zero, as shown in the table below

|  | Columbine & Whitstable Street |  | Swale 1 & Swale 2 |  | Columbine Spit & Swale 4 |  | Swale 5 & Swale 6 |  | Ham Gat & Pollard Spit |  | Swale 7 & Swale 8 |  | Sand End & South Oaze |  | Receptive |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flash | 1.0 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  |
| Eclipse |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 1.9 |
| Sync delay (sec) | 0.0 |  | 1.0 |  | 1.3 |  | 1.6 |  | 1.9 |  | 2.2 |  | 2.5 |  | 2.8 |  |
| TIME |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The final, trial 3, involved reprogramming the lights to flash with both synchronised and sequential combinations. An extra pair of gate buoys, the Ham Gat and Pollard Spit had a longer flash length for individual conspicuity. After all buoys in the river flashing together, extended on the two pairs of gates, the first 4 pairs of buoys flashed together followed by one flash again of all buoys and then as the river curves the buoys in this section flashed in sequence with a general eclipse of 0.25 seconds before a final eclipse of 0.5 seconds when the run starts again.

When results of all three trials were analysed the main factors identified were:-

- The requirement for individual identification of certain buoys must be assessed when considering the waterway as a whole.

- The eclipse between respective buoys, or pairs of buoys, will depend on buoy spacing.

- Where curves in a channel occur, breaks in synchronisation and use of sequential systems should be considered.

Example 6

The Frigg Field Cessation Project

In the Frigg Field Cessation Project a simulation of aids to navigation to mark the disused structures were carried out by Ålesund University College on behalf of the concessionaire and coordinated by the Norwegian Coastal Administration.

Figure: The Frigg Field disused structures after decommissioning (CDP1, TP1 and TCP2).

Table: Proposed light characters

| Platform | Character | Light (s) | Dark (s) | Duty Cycle |
| --- | --- | --- | --- | --- |
| CDP1 | Fl W 10s | 0.8 | 9.2 | 0.087 |
| TP1 | LFl W 15s | 2.4 | 12.6 | 0.190 |
| TCP2 | Fl(3) W 15s | 0.8 | 1.2 / 10.2 | 0.194 |

Figure: Sequencing scheme. Start of sequence is synchronized each 30 seconds.

For most simulations an 80 000 ton tanker with 20 m eye height was used. Different approaches from 10 n miles to 1 n mile were carried out (CPA = 1 n mile), and with different density of shipping and fishing activity in the vicinity.

To ensure validity, reliability and objectivity in the assessment of different navigation light configurations, 20 navigators was used as a reference group.

The group was exposed to three scenarios of the final stage of the cessation. First they experienced to see the Frigg Field at 5 nautical mile range with non-synchronous light characteristics, and then with synchronous light characteristics. Thereafter they experienced the field and the different light regimes from the same position, but now with the lights from eight fishing vessels around the substructures, and with the lights from the Heimdal Field at a greater distance.

Replying to specific questions the group concluded the following:

- Synchronized lights is the best alternative with background lighting (94 %)

- Synchronized lights is the best alternative without background lighting (88 %)

# Example 7

# AMSA – Synchronised lights on pile beacons in the main channel at Port Hedland (AUS)

Port Hedland is one of Australia’s busiest ports mainly handling iron ore exports.  The port is located on the remote, sparsely populated north-west coastline of Australia.

The area is characterised by shallow depths, significant tidal ranges (up to 7.5m) and strong cross currents.  The channel is some 23 nautical miles long, maintained to depths of 14-16 metres and contains a number of bends.  The main channel is primarily used by vessels leaving the port fully laden. Vessels entering the port in ballast generally use an alternative route that is marked by 3 buoys (E1, E2 and E3). Vessels using the channel commonly have a bridge elevation of around 30 metres.

The channel is marked by 30 beacons of which 13 are operated by AMSA with the remainder being inside the port limits and operated by the Port Hedland Port Authority. The lighting equipment comprises 12x LED155 beacons and 1x MLED350 beacon at C1 to provide a longer range to assist vessels to locate the outer end of the channel.  The beacons are located on piles that have an elevation of about 8 metres (above MHWS).

AMSA installed the current synchronisation system on the lights in October 2006 replacing an older system installed by the Port Authority where each light flashed together at a frequency of 2.5 seconds (0.5 flash / 2.0 second eclipse). Using the characters as depicted in the table below, the lights are synchronised with the Port Hedland Port Authority lights that mark the inner portion of the channel so as to flash in unison. These revised synchronisation arrangements have been extremely well received by the port pilots:

Table – Port Hedland Synchronised Beacons – New Characters and Nominal Range

| AMSA Ref | Name | Colour | Intensity Cd | Character | Nom. Range nM |
| --- | --- | --- | --- | --- | --- |
| AN183 | C1 | White | 1700 | Fl 2s (0.5,1.5) | 10 |
| AN184 | C2 | Red | 220 | Fl 2s (0.5,1.5) | 6 |
| AN185 | C3 | Green | 220 | Fl 2s (0.5,1.5) | 6 |
| AN186 | C4 | Red | 220 | Fl 2s (0.5,1.5) | 6 |
| AN188 | C6 | Red | 220 | Fl 2s (0.5,1.5) | 6 |
| AN189 | C7 | Green | 220 | Fl 2s (0.5,1.5) | 6 |
| AN190 | C8 | White | 220 | Fl 2s (0.5,1.5) | 6 |
| AN191 | C9 | White | 220 | Fl 2s (0.5,1.5) | 6 |
| AN192 | C10 | Red | 220 | Fl 2s (0.5,1.5) | 6 |
| AN193 | C11 | Green | 220 | Fl 2s (0.5,1.5) | 6 |
| AN194 | C12 | Red | 220 | Fl 2s (0.5,1.5) | 6 |
| AN195 | C13 | Green | 220 | Fl 2s (0.5,1.5) | 6 |
| AN196 | C14 | Red | 220 | Fl 2s (0.5,1.5) | 6 |

# Summary

# References

[1] FMA, FMA´s Navguide, 2003.

[2] K-E Löfgren, Veneilijän merenkulkuoppi I saaristonavigointi, Suomen

Navigaatioliitto, 2003. (Book about Navigation theory)

[3] D. L. Mills, Internet time synchronization: the network time protocol, IEEE

Transactions on Communications, vol. 39, no. 10, pp. 1482-1493, Oct. 1991.

[4] S. Ganeriwal, R. Kumar, M. Srivastava, Timing-sync protocol for sensor

networks, Proc. First Int. Conf. on Embedded Network Sensor Systems, Los

Angeles, California, Nov. 2003.

[5] J. Elson , L. Girod , D. Estrin, Fine-grained network time synchronization using

reference broadcasts, Proc. of the 5th symposium on Operating systems design

and implementation, Boston, Massachusetts, December 09-11, 2002.

[6] S. PalChaudhuri, A. Saha, D. B. Johnson, Probabilistic clock synchronization

service in sensor networks, Technical Report TR 03-418, Department of

Computer Science, Rice University, 2003.

[7] G. Pottie, W. Kaiser, Wireless integrated network sensors, Communications of

the ACM, 43(5), pp 51-58, May 2000.

[8] http://www.sabik.fi/

[9] http://www.solarmarinelights.com/

[10] http://www.automaticpower.com/

[11] http://www.oi-electric.dk/

[12] http://www.idman.fi/page.asp?pageid=air,intro&languageid=EN

[13] http://www.fma.fi/media/merivayla/2004-2.pdf

Appendix A: List of people interviewed

Finnish Research Institute, ICT section - Risto Mutanen - Synchronising methods and equipment solutions

Finnish Research Institute, ICT section - Jan Holmberg - Synchronising methods and equipment solutions

Finnish Research Institute, ICT section - Heikki Seppä - Synchronising methods and equipment solutions

Royal Danish Administration of Navigation and Hydrography - Jørgen Petersen

Swedish Maritime Administration - Hans Rosén

The Norwegian Coastal Administration - Oddbjørn N. Vik

Digita Ltd - Henri Viljasjärvi

TeliaSonera Ltd - Kirsi Lepistö

***********************