EEP 12    Proposed  Navguide Updates      Adrian Wilkins

Mooring Design

This section will require the removal of refs. to E107 and replace with refs. to the new guideline on mooring design (presuming that it is approved).

The swinging radius formulae will need replacing with the formulae from the new paper.

The approximation for the minimum mooring length is incorrect, This could be expressed as ---

Minimum length of mooring = 3H for water depths up to 50m    This may be decreased over 50m water depth to 2H  but is dependent on local sea conditions.

Shallow moorings, where breaking waves occur may require mooring lengths up to 7H.   ---

Topmarks

This section will need updating in line with the work that WG4 is doing on buoy visibility. It should provide guidance on the size of topmarks and of the buoy superstructure. It presently only provides a guide to the topmark size as a proportion of the buoy body diameter.

Positioning

The term ‘sinker’ should be included alongside ‘anchor’.

Racons and RTE

This section probably needs updating now that new technology radars have been approved for commercial use.