Product Template 5a – Racon

# Purpose

This product template identifies the parameters that should be defined and the standards against which each parameter should be tested to enable a realistic comparison to be made between competing products.

# Scope

The template is applicable for racons used as aids to navigation.

## Product Certification Template 5a – Racon

| Ref. No | Parameter category | Measurement | Test Method or Reference | Comments |
| --- | --- | --- | --- | --- |
| 1. | Regulatory |  |  | A racon is an intentional radio transmitter. Use within most jurisdictions requires a permit to operate. The permit is granted dependent on the Operator and the racon being acceptable to the jurisdiction. |
| 1.1 | Permit To Operate |  |  | The Operator of the racon is generally responsible for obtaining the permit to operate. The Manufacturer may be called upon to assist by providing information under parameters 1.2 and 1.3. |
| 1.2 | Type Acceptance |  |  | The jurisdiction within which the racon will be operated may require type acceptance in some form. |
| 1.2.1 |  | Type Acceptance available for jurisdiction under which the racon will be operated |  | Manufacturer to state type acceptance certificates. |
| 1.2.2 |  | Type Acceptance available from another jurisdiction that is recognized by the jurisdiction within which the racon will be operated |  | Manufacturer to state type acceptance certificates. |
| 1.2.3 |  | Equivalent to Type Acceptance such as the European CE mark which includes the requirements of the Radio and Telecommuni-cations Terminal Equipment (R&TTE) directive |  | Manufacturer to state certificates and any other relevant material. |
| 1.3 | Other Regulatory Issues | Anything not already included for Type Acceptance, but still required for other regulatory reasons such as environmental (e.g. European Restriction of Hazardous Substances (ROHS)) directive, electromagnetic compatibility, safety, etc. |  | Manufacturer to state certificates and any other relevant materials. |
| 2.0 | Non-regulatory Radio Performance |  |  |  |
| 2.1 |  | Electromagnetic Compatibility beyond that which is required by regulation | Various international standards, national standards, Operator requirements, etc. | Manufacturer to state certificates and any other relevant material. |
| 3. | Racon Performance |  |  |  |
| 3.1 | IALA Recommendation R-101 |  |  |  |
| 3.1.1 |  | Antenna Polarization per Band | Reference R101 Table 1,1 | Manufacturer to state antenna polarization. |
| 3.1.2 |  | Frequency Band | Reference R101 Table 1,2,1 | Manufacturer to state. |
| 3.1.3 |  | Blocking Period | There is no international standard test procedure for this item; reference R101 Table 1,2,2 | Manufacturer to state. |
| 3.1.4 |  | Primary Radar Pulse Limit | There is no international standard test procedure for this item; reference R101 Table 1,2,3 | Manufacturer to state range of pulse width the racon will respond to. |
| 3.1.5 |  | Frequency Accuracy per Band, Pulses Less Than 200 usec. in Width | There is no international standard test procedure for this item; reference R101 Table 1,3 | Manufacturer to state. |
| 3.1.6 |  | Frequency Accuracy per Band, Pulses Greater Than or Equal to 200 usec. in Width | There is no international standard test procedure for this item; reference R101 Table 1,3 | Manufacturer to state. |
| 3.1.7 |  | Response Delay | There is no international standard test procedure for this item; reference R101 Table 1,4,1 | Manufacturer to state. |
| 3.1.8 |  | Identity, Morse Codes with Leading Dash | There is no international standard test procedure for this item; reference R101 Table 1,4,2 | Manufacturer to state. |
| 3.1.9 |  | Duration of Response | There is no international standard test procedure for this item; reference R101 Table 1,4,2 | Manufacturer to state. |
| 3.1.10 |  | Antenna Gain per Band | reference R101 Note 1 | Manufacturer to state. |
| 3.1.11 |  | Receiver Sensitivity per Band | reference R101 Note 1 | Manufacturer to state. |
| 3.1.12 |  | Transmitter Power per Band | reference R101 Note 1 | Manufacturer to state. |
| 3.1.13 |  | Duty Cycle (ON, OFF Periods) | reference R101 Note 1 | Manufacturer to state range of adjustment. |
| 3.1.14 |  | Side Lobe Supression | There is no international standard test procedure for this item; reference R101 Note 1 | Manufacturer to state. |
| 3.2 | Other |  |  |  |
| 3.2.1 |  | Antenna Azimuth Response per Band |  | Manufacturer to state. |
| 3.2.2 |  | Antenna Vertical Response per Band |  | Manufacturer to state. |
| 3.2.3 |  | Response Proportional Scaling | There is no international standard test procedure for this item. | Manufacturer to state. |
| 3.3 | Programming |  |  | Manufacturer to state. |
| 4. | Electrical |  |  |  |
| 4.1 |  | Power supply normal and extreme voltage | IEC 60945 section 7 | State voltage tolerance |
| 4.2 |  | Reverse polarity circuit protection | IEC 60945 section 7.2 |  |
| 4.3 | Power Consumption |  | Power consumption measurements are made with the racon set to respond with a “T” code and a response length of 20 usec. |  |
| 4.3.1 |  | Power consumption - quiescent | There is no international standard test procedure for this item. Appendix A offers a definition of racon loading. |  |
| 4.3.2 |  | Power consumption – at 33% duty cycle with no radars present | There is no international standard test procedure for this item. Appendix A offers a definition of racon loading. |  |
| 4.3.3 |  | Power consumption – at 33% duty cycle, lightly loaded | There is no international standard test procedure for this item. Appendix A offers a definition of racon loading. |  |
| 4.3.4 |  | Power consumption – at 33% duty cycle, heavily loaded | There is no international standard test procedure for this item. Appendix A offers a definition of racon loading. |  |
| 4.3.5 |  | Power consumption – at 33% duty cycle, at maximum capacity of the racon | There is no international standard test procedure for this item. Appendix A offers a definition of racon loading. | Manufacturer to state maximum capacity of the racon. |
| 4.4 |  | Power supply terminations/connectors |  | Manufacturer to state |
| 4.5 |  | Monitoring communications interface |  | Manufacturer to state |
| 4.6 |  | Monitoring data output and format | No test method | Manufacturer to state |
| 4.7 |  | Monitoring connector type | No test method | Manufacturer to state |
| 5. | Physical |  |  |  |
| 5.1 |  | Maximum Height | No test method | Manufacturer to state |
| 5.2 |  | Maximum Diameter | No test method | Manufacturer to state |
| 5.3 |  | Maximum Weight | No test method | Manufacturer to state |
| 5.4 |  | Method of fixing | No test method | Manufacturer to state |
| 5.5 |  | Location of antenna(s) - Distance from reference level | No test method | Manufacturer to state |
| 5.6 |  | External package materials | No test method | Manufacturer to state |
| 5.7 |  | Description of fixing |  |  |
| 6. | Environmental |  |  |  |
| 6.1 |  | Temperature – operational and storage |  |  |
| 6.2 |  | Humidity – operational and storage | IEC 60945 section 8.3; IEC 60068-2-3 |  |
| 6.3 |  | IP Rating |  |  |
| 6.4 |  | Immersion | IEC 60945 section 8.9 |  |
| 6.5 |  | Salt air and sea water spray | IEC 60945 section 8.12 |  |
| 6.6 |  | Special requirements - Inert gas charging - Other | No test method | Manufacturer to state |
| 6.7 |  | Ultraviolet exposure |  |  |
| 6.8 |  | Shock and vibration | IEC 60945 section 8.7 Or Vibration MIL-STD-810F method 514.5 category 10 And shock MIL-STD-202G method 213B |  |
| 6.9 |  | Hazardous Area rating |  | Manufacturer to state and supply certificate |
| 7. | Service |  |  |  |
| 7.1 |  | Maintenance free period |  |  |
| 7.2 |  | Rework at stated intervals |  |  |
| 7.3 |  | Assured service life of all components |  |  |
| 8. | Disposal |  |  |  |
| 8.1 |  | Obligatory and recommended disposal method |  |  |

Appendix A

Racon Loading Definitions for Power Consumption Measurements

Often, purchasing tenders may use specifications such as “power consumption while responding to ten radars” while giving no definition of “responding to ten radars.” Further, while in busy harbours there may be many active radars, there is no generally accepted value for how many radars that is. In order to offer a standard method for determining power consumption, this Appendix defines racon loading.

Various formulae for estimating the number of interrogations for a given number of radars have been used over the years. In general, the arithmetic is the same, but the constant values differ. Any constant values are based on the “typical radar” which has evaded agreed upon definition.

The general form of the equation for net interrogation rate is:

I = (RW/S)N

where:

I	is net interrogation rate in pulses per second

N	is number of radars present

R	is radar pulse repetition rate in pulses per second

S	is period of antenna sweep in seconds

W	is width of radar signal main lobe in seconds

For the purposes of this document, the typical radar has the following characteristics:

R	is 1000 pulses per second

S	is 3 seconds

W	is 0.003 seconds (this value is a function of the antenna directivity and pulse signal strength at the racon)

The definition also assumes that multiple radars are synchronized and their pulses do not overlap at the racon. While this assumption is unrealistic in the field, it does provide the worst case for racon loading.

Applying the typical radar yields the net interrogation rate:

I = 1N

This may at first seem surprising. What should be noted is that the racon may respond to about 10 pulses or less from a radar during each of that radar’s sweeps.

For the purposes of this document, racon loading is defined as follows:

Lightly loaded	10 radars present	10 pulses per second net interrogation rate

Heavily loaded	100 radars present	100 pulses per second net interrogation rate

One racon may have a different maximum capacity, rated in pulses per second, than another. Power consumption at maximum capacity is a value useful for power supply sizing, as a racon could operate at its maximum capacity for short periods of time because of radar activity in the areas. The maximum capacity conditions will never be present for an extended length of time.

Typical racons will have maximum capacities of from 5000 pulses per second to 10000 pulses per second. When compared against the loading definitions above, racons may seem overspecified. However, for best service, the racon must at least have the capacity to respond at the pulse repetition rate of the typical radar. High maximum capacity also acts to increase the number of responses to multiple overlapping radar interrogations in busy areas.