IALA Guideline No. ####

On

Conspicuity

Edition 1

December 2009

AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  iala-aism@wanadoo.fr       Internet:  www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

Document Revisions	2

Table of Contents	3

Index of Tables	4

Index of Figures	4

1	Introduction	5

2	Purpose	5

3	Definitions	5

4	Scope	5

5	Conspicuity Diagram and Factors	6

5.1	Observer	6

5.1.1	Eye State and Adaptation	6

5.1.2	Knowledge/Experience	7

5.1.3	Colour Contrast	7

5.1.4	Illuminance Contrast	8

5.1.5	Illuminance (Int/Dist2)	8

5.1.6	Angular Subtense	8

5.1.7	Perceived Shape	9

5.2	Atmosphere	9

5.2.1	Visibility	9

5.2.2	Distance	11

5.3	Target (AtoN Light)	11

5.3.1	Colour	11

5.3.2	Intensity	11

5.3.3	Size	12

5.3.4	Light Source Shape	12

5.3.5	Rhythmic Character and Flash Profile	12

5.3.6	Synchronisation	13

5.3.7	Floodlighting?	13

5.3.8	Movement	13

5.4	Background	13

5.4.1	General Background Lighting	14

5.4.2	Rival Lights (Intensity)	14

5.4.3	Colour	15

6	Methods of Designing an AtoN Light using Existing IALA Documents.	15

7	Methods of Improving the Conspicuity of an AtoN Light	15

7.1	Basic Guide to find Required Intensity	15

7.1.1	Define Operational Requirements	15

7.2	Further Guidance to Improve Conspicuity against a Lit Background	15

7.2.1	Review Operational Requirements to define limits of position, colour, rhythmic character etc.	15

7.2.1.4	Flash AtoN sequentially (lights down a channel).	16

7.2.1.5	Temporal intensity profile	16

ANNEX 1	Annex Title	18

8	Heading 1	18

8.1	Heading 2	18

8.1.1	Heading 3	18

APPENDIX 1	Appendix title	19

9	Heading 1	19

Index of Tables

No table of figures entries found.

Index of Figures

Figure 1	Conspicuity diagram and factors	6

Figure 2	Daytime (Photopic) and Dark Adapted (Scotopic) Eye Response	7

Figure 3	CIE 1931 Chromaticity Diagram	8

Figure 4	Diagram explaining Angular Subtense	9

Figure 5	Required Intensity Wavelength Dependent, according to Ångström, for Photopic Vision	10

Figure 6	Required Intensity Wavelength Dependent, according to Ångström, for Scotopic Vision	11

Figure 7	Title required	14

Figure 8	Title required	14

# Introduction

With increasing light pollution and the proliferation of built up shorelines, the mariner often has difficulty in detecting and identifying marine Aid-to-Navigation (AtoN) lights against a background scene of general lighting and individual bright light sources.  Features such as streetlighting, harbour area floodlighting, architectural lighting and lit signage may look enchanting but can cause serious problems for a mariner trying to identify an important AtoN light.

This guideline looks at ways to increase the conspicuity of marine AtoN lights in the presence of unwanted background lighting.

# Purpose

This Guideline documents methods of increasing the conspicuity of a marine Aid-to-Navigation (AtoN) light, or lights, when viewed against a background of general lighting or rival lights.

# Definitions

Joint IEC and CIE document

CIE Publication 17.4 International Lighting Vocabulary (1987)

IEC Publication 50(845) International Electrotechnical Vocabulary (1987)

845-11-30 Conspicuity:

Quality of an object or a light source to appear prominent in the surroundings.

Background Luminance (Specialist grp):

A general area or large patch of light of roughly uniform brightness, directly behind an AtoN light, that impairs the mariner’s view of the AtoN light of interest.

Rival light (Specialist grp):

A point source of light, not required for navigational purposes, that impairs the mariner’s view of an Aid-to-Navigation light thereby reducing its effectiveness. The intensity of a rival light can be measured in candelas or as the illuminance it causes at the eye of the observer at a given distance.

Intensity:	)

Illuminance:	)Definitions from CIE or IALA E-200

Luminance:	)

Notes:

Background Lighting

(Take definition from existing  1051 Guideline?)

General Background Lighting:

Alternative terms: Uniform Background Lighting, Background Glow

# Scope

The scope of this Guideline is marine Aid-to-Navigation (AtoN) lights viewed at night against a background scene of unwanted lighting.

Address the conspicuity of a night time light when viewed against a background of general lighting or rival lights.  It does not address daytime lights or daymarks.

# Conspicuity Diagram and Factors

Adaptation (eye state)

Knowledge/Experience

Colour Contrast

Illuminance Contrast

Illumination (Int/Dist2)

Angular subtense

Perceived Shape

Visibility

Distance

Flash Profile

Colour

Intensity

Size

Shape

Rhythmic Character

Synchronisation

Floodlighting

Movement

The Target

The Background

The Atmosphere

Background Luminance

Rival Lights (Intensity)

Colour

- Conspicuity diagram and factors

## Observer

### Eye State and Adaptation

It is known that the performance of the eye varies with the ambient light level to which it is exposed.  In full daylight the normal observer has full colour vision in direct view (photopic vision) and is very sensitive to movement (or apparent movement) in peripheral view.  After twenty minutes or so in full darkness, the eye adapts to the dark and becomes very sensitive.  In this very sensitive state at low light levels, the colour sensing elements of direct vision are not active and objects appear grey.  The most sensitive region of the eye in this (scotopic) state is the periphery, about 10 degrees away from direct vision.  In scotopic vision, the peak spectral response of the eye shifts towards shorter wavelengths.  This ‘blue-shift’ causes blue-white lights to be more pronounced at night when compared to yellow-white lights.  Therefore changing an AtoN light to a higher colour temperature will increase its conspicuity under scotopic or mesopic conditions, even though its intensity remains the same [REFERENCE R&D REPORT - MALCOLM??].

- Daytime (Photopic) and Dark Adapted (Scotopic) Eye Response

At low levels of light (e.g. twilight), the eye reaches a state midway between photopic and scotopic that exhibits elements of both states (mesopic).  The mariner at night on the low-lit bridge of a ship is probably in mesopic vision.  There is limited direct colour vision and enhanced sensitivity in peripheral vision.  Since peripheral vision is sensitive to movement, and since flashing (especially sequenced) lights constitute a form of movement, initial detection of a flash

At low levels of incident light, when the observer strains to see an AtoN, the observer’s visual response is much slower than at higher levels of light.  There is also much more chance of colour confusion, especially for flashing lights, especially between green, blue and white.  A longer flash duration will reduce colour confusion at these low levels.

Age affects the performance of an observer’s vision.  Older observers have a reduced sensitivity to blue because of the degradation of the cornea.  Typically, a 50 year-old observer has between 10% and 20% of sensitivity to blue compared with a 20 year-old observer.  This clearly has implications on the conspicuity of blue lights.  In observers of all age groups, tiredness adversely affects the performance of the visual system.

Colour deficient observers can experience colour confusion.  The most common forms of deficiency result in red-green confusion.  Most colour deficient observers are male.  More information is given in IALA E-200-1.

### Knowledge/Experience

When a mariner has prior knowledge or experience of a particular port entry or navigational channel, he or she will be able to discern the relevant AtoN lights more easily.  The more times a mariner navigates a particular channel, the easier becomes the identification and recognition of AtoNs. AtoNs that appear conspicuous and obvious to an experienced mariner can often appear indistinct or be missed altogether by an inexperienced one.

### Colour Contrast

The ability of the human observer to detect slight differences in colour is well-known.  For surface colour, a slight difference in hue or chroma can be easily detected by the average person.  However, when it comes to discerning differences in emitted light, knowledge is less well documented.

At very low levels of illuminance, when an observer can barely detect a light, the colour of the light and the duration of the light are difficult to discern.  As illuminance levels increase, colour recognition and the duration of a flash become clearer.  But the appearance of a point source of light is more difficult to predict than that of an area of light.

When an AtoN light is seen against a background luminance, it is more easily discernible when the colours of background light and AtoN light are different.  The greater the difference, as shown on the chromaticity diagram, the easier it is to detect one against the other.

- CIE 1931 Chromaticity Diagram

### Illuminance Contrast

When an AtoN light is brighter than a neighbouring light, it is more conspicuous.  The ration required for an observer to notice a difference in eye illuminance is roughly 4:1.  However, when dealing with background luminance, the relationship is approximately to the power of 2.  For example, a three-fold increase in background luminance should be matched with a nine-fold increase in AtoN light intensity.

### Illuminance (Int/Dist2)

The amount of illuminance an AtoN light casts upon the eye of the observer depends upon its intensity and the distance between AtoN and observer.  The minimum discernible illuminance at the eye of a photopic observer is often quoted as 0.05 microlux (for a dark-adapted observer in scotopic vision however, it can be as low as 0.0015microlux).  At these levels, i.e. at the threshold of visual perception, the chances of seeing a light are little better than 50%.  The minimum recommended illuminance level for an AtoN light in darkness is 0.2 microlux, based on international agreement in 1933.  This is the value that the nominal range figure is calculated from. At this level, the colour and rhythmic character of an AtoN can be recognised with confidence.

However, with high levels of background lighting, a higher value of illuminance is required in order to see the AtoN light.  At these higher levels the human visual system behaves quite differently than at low illuminance levels.  Phenomena such as short flashes (strobes), fast repeating flashes and flicker become more conspicuous as illuminance levels increase.

### Angular Subtense

Viewed from far away, most AtoN lights are point sources and therefore have no discernible size.  As the observer gets closer to the AtoN light, the size increases so that the size and shape of the light source become noticeable.  If the angle subtending the eye, in other words the angle at the eye from one side of the viewed object to the other side, is less than one minute of arc, it can be said to be a point source.

- Diagram explaining Angular Subtense

### Perceived Shape

Following on from angular subtense, in order for the shape of a light source to be recognised, it needs to subtend an angle of greater than one minute of arc.  Such things as extended light sources or floodlit areas are generally designed to given shape information.  Therefore, the extremities of such lights need to be greater than that angular subtense to be useful as AtoNs.  The physical size of such a light source would therefore determine its useful range.

[NEED FORMULA FOR RATIO OF DISTANCE TO SIZE FOR MINIMUM DISCERNIBLE SHAPE – E.G. 1500:1. ANY IDEAS???]

## Atmosphere

### Visibility

Visibility is usually thought of as meteorological visibility which is defined as:

the greatest distance at which a black object of suitable dimensions can be seen and recognized by day against the horizon sky. [IALA E-200]

For night time viewing:

the greatest distance at which a black object of suitable dimensions could be seen and recognized if the general illumination were raised to daylight level. [IALA E-200]

The state of the atmosphere will determine the greatest distance at which a light of given intensity can be seen at night.  This distance is known as the ‘luminous range’ of the light.  The method by which luminous range is determined is Allard’s law, which takes into account the intensity of the light, the distance of the observer from that light and the visibility.

Another way of defining the state of the atmosphere is by its ‘transmissivity’, which can be thought of as the throughput of light for one nautical mile of atmosphere.  Transmissivity is usually given as a factor, e.g. 0.74, which means that light exiting one nautical mile of atmosphere will be 74% of the light input after the inverse square law has been applied (see more in IALA E-200-2).

Both meteorological visibility and transmissivity assume that the atmosphere is spectrally neutral. In other words, all wavelength of visible light are affected equally.  Unfortunately, the atmosphere is somewhat more complicated than this and will often affect wavelengths within the visible spectrum unequally.

The two phenomena traditionally used to describe the way the atmosphere scatters light are Rayleigh scatter and Mie scatter.  These are traditional used to describe how sunlight is affected.  A model that more completely describes how light at sea level is affected is that of Ångström.  This model includes the effects of Rayleigh and Mie scattering as well as the effect of aerosol particulates such as salt.

Generally, Rayleigh scattering occurs when visibility is moderate to good where light is scattered preferentially by wavelength to the fourth power of the wavelength.  Therefore, a blue light will be scattered more, and will have less luminous range, than a red light of the same intensity.  Mie scattering occurs during conditions of poor visibility when water droplets within the atmosphere are larger.  Mie scattering is spectrally neutral.  The Ångström model combines both Mie and Rayleigh and includes other factors for aerosols present in the atmosphere close to the sea. [XAVIER, CAN YOU PROVIDE SOME INPUT OR REFERENCES??]

The chart below (figure 4) shows how required intensity for a given luminous range can very with wavelength.  It is valid for a photopic (daytime) observer (see Eye State and Adaptation). This chart is intended for guidance only.  Luminous range calculations should be carried out using IALA RecommendationE-200.

- Required Intensity Wavelength Dependent, according to Ångström, for Photopic Vision

FOR GUIDANCE ONLY

For an observer in scotopic vision, which has a different spectral response to photopic, these required intensity values will change, as seen below in figure 5.

- Required Intensity Wavelength Dependent, according to Ångström, for Scotopic Vision

FOR GUIDANCE ONLY

### Distance

[INPUT REQUIRED]

## Target (AtoN Light)

### Colour

AtoN lights are often restricted by their navigational requirements.  A starboard hand buoy must exhibit a green light, and so on.  However, since most background luminance is white or near-white, the AtoNs most susceptible to masking or interference are white lights.  It is possible therefore, to change the colour temperature of the white AtoN light within the IALA Optimum colour region.  For example, a white LED could have a colour temperature of around 7,000ºK.  This would stand out markedly against a background of high pressure sodium lamps at 2,500 ºK.

One exception to this however, is yellow special mark AtoNs.  If these are viewed against a background of low pressure sodium, the difference in colour (hence the discernibility of the AtoN) would be insignificant.

### Intensity

The intensity of an AtoN light determines its nominal range and its luminous range for a given visibility, as described in IALA E-200-2.  However, to make a light more conspicuous by increasing the intensity, that increase usually needs to be substantial, typically tenfold.  This is because the intensity required of a light to be perceived against a background luminance is proportional to the square of the background luminance (Langmuir & Westendorp 1931).  Guidance is given in IALA E-200-2 for required intensities at different levels of background lighting.

When considering a flashing light, effective intensity is the relevant parameter to use at threshold illuminance levels.  However, when considering short flashes at higher levels of illuminance, and this is usually the case when viewed against background luminance, the value of effective intensity is not proportionate to conspicuity.  Work is continuing to try and find a model that can quantify the effects of flashing lights at higher than threshold levels, a so-called ‘apparent intensity’ model.

### Size

The size of an AtoN light will determine its angular subtense for an observer at a given distance (see Angular Subtense).  For an extended light source, it is important to make the size large enough so that it may be recognised at the maximum operational distance.  Furthermore, if the light source is to be viewed obliquely within its arc of utilisation, the width of the light source should be wide enough to account for the oblique view.  The cosine of the angle of view may be used to calculate the required increase in width.

[DIAGRAM REQUIRED]

Because it is not viewed as a point source, the luminance of the source, rather than its intensity, is relevant to the observer.  Therefore the luminance of the extended source should be at least four times greater than the background luminance. [IS EVERYONE AGREED ON THIS?  DOES ANYONE HAVE BETTER INFORMATION]

For a light that is viewed as a point source, its size does have an effect on its performance at distance in one respect - scintillation.  When observing the difference between a large light source and a small light source of similar intensity at a distance, the difference in scintillation is quite marked.  On a clear night, when distant lights scintillate or twinkle, a large light source, for instance a large lens panel, twinkles much less.  Trying to find a flashing light among twinkling lights can be difficult.  A large light source, even when it is flashing, twinkles less and stands out from its surroundings as a result.

### Light Source Shape

The shape of a light source is irrelevant when it is viewed as a point source but if it is intended to be viewed as a light area or extended light source, a given shape can be instantly recognisable to an observer. A lit shape can provide a very conspicuous marker, used either as a pointer to an AtoN or as an AtoN in its own right.

### Rhythmic Character and Flash Profile

The rhythmic character of an AtoN light is what distinguishes it from its surroundings and from other AtoN lights.  Generally, the rhythmic character cannot be altered for operational reasons.  However, it may be possible to alter the timing of the rhythm, for example to increase the repetition rate of flashes within a quick flash to improve conspicuity.  At high levels of illuminance quickly repeated short flashes are generally more conspicuous than longer flashes with longer eclipse times between them.  Care should be taken to ensure that the character is still suitable in conditions of poor visibility, when illuminance levels at the eye of the observer may be low and the visual response slows down as a result (see Eye State and Adaptation).

The shape of the flash, or flash profile, is also important to conspicuity.  A rectangular flash shape, i.e. one with a fast rise and fall time, is more conspicuous than, say, a Gaussian flash shape (such as observed from a rotating lens system) [REFERENCE TO SAME R&D REPORT – MALCOLM??].

Placing a high intensity strobe light next to the AtoN light can have the desired effect of drawing the observer's eye to the AtoN.  Once the AtoN position has been located, the lower intensity rhythmic character can then be identified.  This technique can be useful in areas with high levels of background lighting and rival lights and, although it may require and increase in overall power consumption, it may be a less 'power-hungry' solution than increasing the intensity of the AtoN light; this due to the brevity of the strobe flash duration.  This technique is more effective when the strobe is synchronised with the AtoN rhythmic character.

Flickering the AtoN light within the flash profile at a frequency of around 10Hz, is a technique that has had some success in Japan. [REFERENCE JCG PAPER ON FLICKER]  In situations where background luminance and rival light interference are high, flickering the AtoN light can increase conspicuity greatly.  However, this technique is only effective when illuminance levels at the eye of the observer are high.  At low levels of observer illuminance, the advantage of the flicker effect is lost and some observers fail to notice any flicker at all.  More study is needed to find optimum flicker frequencies, optimum flicker duty cycles and the effects of flicker on various colours of lights.

### Synchronisation

Synchronising several AtoN lights can increase conspicuity considerably.  This technique is very effective for several buoys marking a channel, for port and starboard port entry lights or for leading light.  When viewed at a distance, the effect of several lights all coming on together over a wide area is highly conspicuous.  However, if all AtoN lights in the synchronised group have the same rhythmic character, there can be problems in identifying individual AtoNs within the group.  His is particularly relevant in poor visibility, when perhaps only one of the AtoNs can be seen at any one time.

It is possible to retain individual characters within a group of AtoNs and synchronise them at the start of rhythmic character with the longest period.  For this to be practicable, the AtoN characters need to have a common period or exact divisions of the longest period (or exact multiples of the shortest period). [REFERENCE TO IALA GUIDELINE ON SYNCHRONISED LIGHTS]

### Floodlighting?

Floodlighting can be considered as an extended light source.  A close range an illuminated structure can provide a conspicuous mark for an AtoN light. If the structure is uniquely recognisable, it can provide instant and reliable recognition to the mariner.  However, floodlighting usually requires high-powered lamps and may not be practicable for AtoNs with low-energy systems.

At distances of a few miles, the effects of floodlighting usually starts to fade as the angular subtense of the area illuminated becomes smaller and the recognition of the object becomes more difficult.  Therefore floodlighting is recommended for improving conspicuity at relatively short distances, although this will depend on the illuminated area and the degree of illumination.  [REFERENCE IALA GUIDELINE ON ILLUMINATED STRUCTURES]

### Movement

Movement is known to be conspicuous to the human observer being part of our primeval defence system.  Movement of an AtoN light however is undesirable since its purpose is to mark a fixed, designated position for the mariner.  Lights on floating AtoNs may move but such movement is unplanned and usually too slow to affect conspicuity.

There are methods of creating apparent movement and one of them is by sequencing a row of lights to flash on and off consecutively.  This technique is commonly used on roads to warn of roadworks.  A row of lamps flashing in sequence can be very conspicuous and can give directional information.  However, using such techniques, for instance on buoys marking a channel can have a limited effect when used over a large area. [REFERENCE TO IALA GUIDELINE ON SYNCHRONISED LIGHTS]

## Background

### General Background Lighting

General Background Lighting: A general area or large patch of light of roughly uniform brightness, directly behind an AtoN light, that impairs the mariner’s view of the AtoN light of interest.  The luminance of the background lighting can be measured in candelas per square metre.

When a large mass of shore lighting forms a backdrop to an AtoN light and the individual light sources within the background are indistinguishable, it may be taken as a homogenous background luminance.

- Title required

To the observer, shore lights tend to merge at a distance, for instance, when the mariner is some way offshore.  Therefore, the AtoN lights most affected by background luminance are usually long-range lights.

### Rival Lights (Intensity)

Rival light: A point source of light, not required for navigational purposes, that impairs the mariner’s view of a nearby Aid-to-Navigation light thereby reducing its effectiveness.  The intensity of a rival light can be measured in candelas or as the illuminance it causes at the eye of the observer at a given distance.

Rival lights can be considered as individual point sources of unwanted light that mask or obscure an AtoN light and prevent easy detection or recognition of the AtoN.  Sometimes a slight change in position of the mariner within a channel or the height of observation can make the difference between seeing an AtoN or not seeing it.

- Title required

Background scenes are often comprised of a mixture of general background lighting and rival lights.

### Colour

The colour of general background lighting is a mixture of all the lights in view but usually takes on the colour of the dominant light source. For instance, if the background scene is comprised mainly of high pressure sodium, the overall colour seen from a distance will be yellow-white with a hint of orange. If the background is a busy floodlit container port however, the background lighting could be predominantly metal-halide lamps giving a blue-white hue.

A rival light, on the other hand, could consist of an individual light source of any colour.

# Methods of Designing an AtoN Light using Existing IALA Documents.

# Methods of Improving the Conspicuity of an AtoN Light

## Basic Guide to find Required Intensity

### Define Operational Requirements

#### Intensity/Illuminance

(This could be addressed briefly or very thoroughly.  A more thorough discussion would get into a discussion of things like vertical divergence.  This section should include a discussion of apparent intensity.  It should also address the x10 and x100 intensity factors in IALA Recommendations and in the NAVGUIDE.)

## Further Guidance to Improve Conspicuity against a Lit Background

### Review Operational Requirements to define limits of position, colour, rhythmic character etc.

#### Relocation

- Move the AtoN away from rival lights, or for a new installation locate AtoN away from rival light (discussed in IALA Guideline 1051).  A small vertical shift can often separate the AtoN light from the background or rival light from the observer's viewpoint.  However, conflict between rival lights and AtoNs can often occur in specific locations within the AtoN's zone of utilisation and the whole zone should be considered over a range of observation heights.

- Relocate, shade, or redirect rival lights.

(See GLA Development Department Report No. 5/IT/1999 “Lowestoft Background Lighting Investigation”, 19 April 1999).

#### Colour

Colour change is often limited with AtoN lights because navigational requirements dictate the colour of the AtoN light but most background and rival lights are white.  The usual problem is therefore trying to see a white light against a white background.  Since many street lights are sodium, they tend to be yellowy-white.  The change of AtoN light source from incandescent filament to a lamp of high colour-temperature (e.g. metal halide or white LED) often provides a significant colour contrast to the background.

There are two exceptions to this:

- when the background is for example a marshalling yard or football stadium where high colour temperature floodlights are used;

- when the AtoN light is yellow ( for example, a special mark) and the background is low-pressure sodium light.

#### Flash AtoN simultaneously (most often used for leading lights)

#### Flash AtoN sequentially (lights down a channel).

#### Temporal intensity profile

Simple flash characteristics (e.g. a Fl 4 light on an AtoN is more conspicuous against a background of fixed rival lights than an AtoN with a fixed light characteristic).

Flicker.  This is what the JCG has been working on.  After our meeting in Tokyo we have the current state-of-knowledge about flicker.

Flash Profile – a sharp rise time (and fall-time?) is generally more conspicuous than a slow rise-time.

Some work has been done on how the repetition rate of a flashing light affects its conspicuity but this is almost certainly dependent on the illuminance at the eye.

#### Shape of light (example: a “light pipe”)

As discussed in floodlighting, shapes are recognised more quickly and effectively that a flashing point source. Japan Coast Guard has done some work on illuminated numbers on buoys. This has the advantage, not only of fast recognition but also enables the mariner to gauge his/her distance from the buoy more effectively. This has reportedly reduced the number of collisions with buoys. Spain is also experimenting with shaped ‘Pointer’ lights to accentuate the position of an AtoN against heavy background lighting.

Two advantages of viewing a shape rather than a point source:

quick recognition;

better gauging of distance between AtoN and observer.

#### Movement of light

(either a light that moves or a group of lights that turn on and off in a manner that makes it look the AtoN is moving).  Does anyone know of an AtoN that uses movement to increase conspicuity?

#### Flood lighting?

Looking forward: address our goal to eventually develop quantitative conspicuity models.

Notes:

As we address each of the above methods we would include a discussion of how that method of increasing conspicuity is affected by:

- Illuminance level;

- The passage of light through the atmosphere;

- Differences between observers.

Three examples:

- Illuminance level.  Work by the French has shown that increased conspicuity associated with flicker is greatly diminished – or even lost – at low illuminance levels.

- The passage of light through the atmosphere.  As Ian has pointed out, as blue light passes though the atmosphere it tends to be selectively filtered and after a distance may ultimately appear to be red.  So picking a blue light because blue is more conspicuous against a given background may not work so well if the blue light needs to be seen at a distance.  (Note: passage of light through the atmosphere will obviously reduce the illuminance.  If all the atmosphere does is reduce illuminance then I would include under “Illuminance level” and not under “Passage of light through the atmosphere.”)

- Differences between observers.  For observers with colour problems one may not be able to achieve an expected increase in conspicuity by picking different colours (i.e.  to many male observers a red light will appear no different than a yellow light, so picking a red AtoN light for use against a background of yellow lights will not achieve the desired conspicuity results for a portion of the population).

It is known that older observers have significantly impaired blue vision caused by yellowing of the cornea. The blue sensitivity of a typical 50 year-old is between 10% and 20% of that of a typical 20 year-old.

For information on colour deficient observers, including lines of colour confusion, see E-200-1. Blue-yellow confusion is very rare (1:63000), reinforcing the advantage of blue-white lights against a yellowy sodium background.

- Annex Title

Guidelines should have Annexes.  Appendices are attached to Annexes.

# Heading 1

Body text

To restart the numbering at 1:

- Office 2003, go to Format / Bullets and Numbering / Restart numbering (lower left in the box)

- Office 2007, go to down arrow next to Numbering icon and select Set Numbering Value

## Heading 2

Body text

### Heading 3

Body Text Indent

#### Heading 4

Body Text Indent 2

- Appendix title

# Heading 1

Followed by Body Text