#### Night Operations

Refer to IALA publication:

Recommendation E200-2 On Marine Signal Lights Part 2 – Calculation, definition and notation of luminous range (December 2008)

##### Nominal Range and Luminous Intensity

Figure 1 and Table 1 are extracts of the IALA Recommendation E200-2 On Marine Signal Lights Part 2 – Calculation, definition and notation of luminous range and provide a conversion between nominal range and luminous intensity.

Figure 1 – Night time Luminous Range Diagram

Threshold value for illuminance Et =  2  10-7 lx

Table 1 – IALA conversion table for Luminous Intensity and Nominal Range for night observations.

Note - This assumes an atmospheric transmissivity of T=0.74 and a threshold of illumination of 0.2 microlux.

| Luminous | Nominal | Luminous | Nominal | Luminous | Nominal |
| --- | --- | --- | --- | --- | --- |
| intensity | range | intensity | range | intensity | range |
|  | (rounded) |  | (rounded) |  | (rounded) |
| candelas | nautical miles (M) | kilocandelas (103 cd) | nautical miles (M) | Megacandelas (106 cd) | nautical miles (M) |
| 1 - 2 | 1 | 0.633 – 1.06 | 9 | 0.927 – 1.35 | 26 |
| 3 - 9 | 2 | 1.07 – 1.75 | 10 | 1.36 – 1.96 | 27 |
| 10 - 23 | 3 | 1.76 – 2.84 | 11 | 1.97 – 2.84 | 28 |
| 24 - 53 | 4 | 2.85 – 4.53 | 12 | 2.85 – 4.11 | 29 |
| 54 - 107 | 5 | 4.54 – 7.13 | 13 | 4.12 – 5.93 | 30 |
| 108 - 203 | 6 | 7.14 – 11.1 | 14 | 5.94 – 8.53 | 31 |
| 204 - 364 | 7 | 11.2 – 17.1 | 15 | 8.54 – 12.2 | 32 |
| 365 - 632 | 8 | 17.2 – 26.1 | 16 | 12.3 – 17.5 | 33 |
|  |  | 26.2 - 39.7 | 17 | 17.6 – 25.1 | 34 |
|  |  | 39.8 – 59.9 | 18 | 25.2 – 35.9 | 35 |
|  |  | 60.0 – 89.8 | 19 | 36.0 – 51.2 | 36 |
|  |  | 89.9 - 133 | 20 | 51.3 – 72.9 | 37 |
|  |  | 134 -198 | 21 | 73.0 - 103 | 38 |
|  |  | 199 - 293 | 22 | 104 -147 | 39 |
|  |  | 294 - 432 | 23 | 148 - 209 | 40 |
|  |  | 433 - 634 | 24 |  |  |
|  |  | 635 - 926 | 25 |  |  |

##### Luminous Range Diagram for Night time

The Luminous Range Diagram, shown in figure 1, enables the mariner to determine the approximate range at which a light may be sighted, by night in the meteorological conditions prevailing at the time, and for various levels of background lighting.

Example:

Suppose that it is required to find the luminous range of a light with a nominal range of 20 M and a luminous intensity of 110,000 cd with no background lighting and a visibility of 5 M.

Read off 20 M on the Nominal Luminous Range scale. Erect from this point a parallel to the axis of ordinates to meet the curve for 5 nautical miles visibilty. Read off the luminous range on the vertical scale against the point so obtained. We read approx. 12 nautical miles.

##### Background Lighting

Nominal range at night is calculated with no allowance for glare from background lighting.  Excessive background lighting, from street lights, neon signs etc., frequently makes an aid to navigation light less effective and, in some cases, it becomes completely lost in the general background clutter.

The recommended method for compensating for background lighting is to use different values for the required illuminance.

Two different values should be used:

minor background lighting:	2  10-6 lx	factor 10 

substantial background lighting:	2  10-5 lx	factor 100 

The diagram in Diagram 1  has been drawn for a required illuminance of 210-7 lx. For the other values of required illuminance (minor and substantial background lighting) mark off along the scale of abscissae the distance between 'No Background lighting (NONE)' and that under consideration as it appears on the auxiliary scale.

Example:

Suppose that it is required to find the luminous range of a light with a nominal range of 17 M and a luminous intensity of 32,300 cd for substantial background lighting and a visibility of 5 M.

Measure the distance A separating 'no background lighting (NONE)' and 'substantial background lighting (SUBSTANTIAL)'. Transfer this distance to the scale of abscissae from graduation to 17 M (32,300 cd) in the same sense. A point slightly to the right of graduation corresponding to 7 nautical miles is obtained. Erect from this point a parallel to the axis of ordinates to meet the curve for 5 nautical miles visibilty. Read off the luminous range on the vertical scale against the point so obtained. We read approx. 5 nautical miles.

##### Glare

Glare can be caused by bright lights emitted from the shore, such as car headlights, or from another vessel indiscreetly using a search-light.  An aid to navigation light can also cause glare if it is too bright for the shortest viewing distance, especially when the focal plane of the light and the observer's eye are at the same height.  This situation can arise with two station leading lines.

For aids to navigation lights it is generally accepted that the illuminance at the eye of the navigator from the light:

- should not exceed 0.1 lux, and;

- should be reduced to 0.01 lux if the background is very dark .

Refer to IALA publications:

Recommendation E-112 for leading lights (including excel program);

Guideline 1023 for the design of leading lines.

In situations where glare is a problem, one or more of the following alterations may lead to a satisfactory result:

- raise the focal plane of the light so that the mariner uses the loom of the light or a less intense part of the vertical distribution of the light;

- reduce the intensity of the light by:

- reducing the illuminance of the light source,

- reducing the size of the optic,

- masking the optic with, for example, perforated metal sheet;

- screen unnecessary arcs of the light;

- use two or more lower intensity lights instead of one higher intensity light.

Whatever methods are used, it will be necessary to measure or calculate the intensity and distribution of the modified light or lighting system.

##### Intensity Losses

Some lighting equipment has to be installed inside a protective lantern housing.  Unless it is practicable to measure the luminous intensity of the complete installation, it is normal practice to apply a de-rating factor to the intensity of the lighting equipment to allow for the reflection and transmission losses at the lantern glazing.  Generally referred to as the glazing loss factor.  Glazing bars or astragals may reduce the intensity of the light at certain bearings.  The installation of non-vertical astragals will overcome this reduction to a certain extent.  The focal plane of the light should be positioned away from any horizontal glazing bars or intersection.

IALA recommends that, in the absence of more definitive information, the glazing loss factor be taken as 0.85 for a system in clean condition.

Refer to IALA publication:

Recommendation E200-5 On Marine Signal Lights Part -5 Estimation of the Performance of Optical Apparatus (December 2008)

##### Service Conditions Factor

Under normal operating conditions the luminous intensity of a light is likely to degrade between service (maintenance) intervals.  There are several components to this degradation:

- meteorological conditions (which may only be temporary);

- dirt and salt deposition (which can be minimised by an efficient regular programme of cleaning of the optical system and housing), and;

- progressive deterioration of the light source over the service interval.

It is clearly impossible to represent such a complex array of factors in any simple way, and a proper assessment of the various effects could only be made by measurements on site at regular intervals.  However, in order to give a more realistic figure for the performance of the light under normal operating conditions than when the luminous intensity is measured in a laboratory or on a photometric range, it may be appropriate to apply a service conditions factor to the measured intensity.

IALA recommends that, in the absence of more definitive information, the service condition factor be taken as 0.75.

Refer to IALA publication:

Recommendation E200-5 On Marine Signal Lights Part -5 Estimation of the Performance of Optical Apparatus (December 2008)