#### Day Operations

A number of authorities have established daytime lighted leading lines in major ports and waterways to achieve a more consistent performance than is possible with dayboards.

##### Nominal Daytime Range and Luminous Intensity

Refer to IALA publication:

Recommendation E200-2 On Marine Signal Lights Part 2 – Calculation, definition and notation of luminous range (December 2008)

Recommendation E-111 on Port Traffic Signals

Figure 2 and Table  are extracts of Recommendation E200-2 On Marine Signal Lights Part 2 – Calculation, definition and notation of luminous range (December 2008) and provides a conversion between nominal daytime range and luminous intensity.

Figure 2 – Daytime Luminous Range Diagram

Threshold value for illuminance Et =  110-3lx

Table 2 – IALA conversion table for Luminous Intensity and Nominal Daytime Range.

| Luminous | Nominal | Luminous | Nominal |
| --- | --- | --- | --- |
| intensity | range | intensity | range |
|  | (rounded) |  | (rounded) |
| kilocandelas (103 cd) | nautical miles (M) | Megacandelas (106 cd) | nautical miles (M) |
| 1 – 12.0 | 1 | 1.02 – 1.82 | 7 |
| 12.1 – 45.3 | 2 | 1.83 – 3.16 | 8 |
| 45.4 – 119 | 3 | 3.17 – 5.32 | 9 |
| 120 – 267 | 4 | 5.33 – 8.78 | 10 |
| 268 – 538 | 5 | 8.79 – 14.2 | 11 |
| 539 – 1010 | 6 | 14.3 – 22.6 | 12 |
|  |  | 22.7 – 35.6 | 13 |
|  |  | 35.7 – 55.5 | 14 |
|  |  | 55.6 – 85.6 | 15 |
|  |  | 85.7 – 130 | 16 |
|  |  | 131 – 198 | 17 |
|  |  | 199 – 299 | 18 |
|  |  | 300 – 449 | 19 |
|  |  | 450 – 669 | 20 |
|  |  | 670 – 993 | 21 |
|  |  | 994 – 1460 | 22 |

##### Luminous Range Diagram for Daytime use

The Luminous Range Diagram, shown in Figure 2, enables the mariner to determine the approximate range at which a light may be sighted, by day in the meteorological conditions prevailing at the time, and for various levels of sky luminance.

| Meteorological condition | Luminance in cd/m² | required illuminance Et in 10-3 lx |
| --- | --- | --- |
| Very dark overcast sky | 100 | 0.013 |
| Dark overcast sky | 200 | 0.024 |
| Ordinary overcast sky | 1 000 | 0.107 |
| Bright overcast sky or clear sky away from the direction of the sun | 5 000 | 0.506 |
| Bright cloud or clear sky close to the direction of the sun | 10 000 | 1 |
| Very bright cloud | 20 000 | 1.98 |
| Glaring cloud | 50 000 | 4.91 |

The graph has been drawn for a sky luminance of 10 000 cd/m². For  other values of sky luminance mark off along the scale of abscissae the distance between the luminance of 10 000 cd/m² and that under consideration as it appears on the auxilliary scale.

Example:

Suppose that it is required to calculate the luminous range of a light of 2 000 000 cd for a meteorological visibility of 2 nautical miles under an ordinary overcast sky (luminance 1 000 cd/m²).

Measure the distance A separating graduations 10 000 cd and 1 000 cd on the auxiliary scale. Transfer this distance to the scale of abscissae from graduation corresponding to 2 000 000 cd (2106 cd) in the same sense. A point slightly to the right of graduation corresponding to 12 nautical miles is obtained. Erect from this point a parallel to the axis of ordinates to meet the curve for 2 nautical miles visibility. Read off the luminous range on the vertical scale against the point so obtained. We read approx. 4 nautical miles.

##### Daymarks (Dayboards)

The size of a dayboard should be determined for the maximum useful viewing distance and minimum visibility conditions.  Daymarks used on leading lines are typically rectangular with the long side vertical.  The aspect ratio for the rectangle is commonly 2:1(height = 2 x width).

The typical operational range of daymarks under different visibility conditions is shown in Table 1.

Table 1 – Typical operational range of daymarks.

| Operational Range of Daymarks (Nautical Miles) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Daymark height (metres). Aspect ratio h=2w |  |  |  |  |
| Minimum visibility (Nautical Miles) | 1.8 | 2.4 | 3.7 | 4.9 | 7.3 |
| 1 | 0.5 | 0.7 | 0.9 | 1.0 | 1.1 |
| 2 | 0.6 | 0.9 | 1.2 | 1.4 | 1.5 |
| 3 | 0.6 | 1.1 | 1.5 | 1.9 | 2.1 |
| 4 | 0.7 | 1.3 | 1.8 | 2.3 | 2.7 |
| 5 | 0.8 | 1.5 | 2.1 | 2.7 | 3.3 |
| 6 | 0.8 | 1.6 | 2.3 | 2.9 | 3.6 |
| 7 | 0.9 | 1.7 | 2.4 | 3.3 | 4.0 |
| 8 | 0.9 | 1.7 | 2.6 | 3.5 | 4.2 |
| 9 | 0.9 | 1.9 | 2.8 | 3.8 | 4.5 |
| 10 | 1.0 | 2.0 | 3.0 | 4.0 | 5.0 |

Table 1 (Night and day with background) For Guidance Only – not to be used for Nominal Range Publication