IALA Guideline  XXXX

DRAFT

On

Daymarks for floating aids to Navigation

Draft Edition 1ab

4. April 2008

AISM Association Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 01   Telefax +33 1 34 51 82 05

e-mail - iala-aism@wanadoo.fr     Internet -  www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

IALA Guideline on Daymarks for
Floating Aids to Navigation

1	Introduction	3

2	Overview	3

3	Definitions	3

4	Visual perception of Daymarks – Basic Principles.	4

4.1	Introduction	4

4.2	Identification and recognition of daymarks	4

5	Limits of visual Perception of daymarks	7

5.1	General	7

5.2	Contrast	7

5.3	Contrast at short distances	7

5.3.1	Contrast as a function of distance	7

5.4	Visibility law of Daymark	7

5.5	General comment on limits of visual range	8

5.6	Range - Visibility distance	8

6	Advantages of using highly saturated colours for daymarks (SABINA)	10

7	Factors that reduce visual range (TG2 Section 5.)	13

8	Service factors (TG3 Section 6)	15

9	Other considerations (TG3 Section 8.)	15

10	Use of Multiple Marks (TG2 Section 9.)	16

11	Climate effects (TG3 Section 10 )	16

12	References	18

12.1.1	Contrast at short distances	21

IALA Guideline on Daymarks for
Floating Aids to Navigation

# Introduction

Existing IALA documents provide information on the proportions of buoy daymarks and their associated topmarks but only limited information information is given on the range at which daymarks can be effectively detected and recognised.

This guideline provides a general informative overview of the principle factors affecting the performance of the daymark function of a floating aid to navigation and how to determine daytime range for simple case. In practice we are unable to integrate all parameters for determination of daymark range and perfect theory is still unknown.

# Overview

When designing a daymark… Methodology for simple case

- First determine the required range (service distance i.e. at what distance should daymark be detected)

- Type of mark (cardinal, lateral) and select the colour and shape of the daymark

- Conditions of utilisation of daymark (height of vessel bridge, direction of observations) [include remark about modification of background with distance of observation after]

- Calculate photometric contrast at short distance using formula (XX) (see formula annexe 2) [modification of name of short distance in all document]

- Determine the fraction of time, the service is to be provided (policy matter - 95% could be chosen)

- Determine the available Meteorological visibility that prevails (95%) of the time in the service area of the aid. (remember that Sea state also affects service and is not taken into account in the equations.)

- Calculate contrast as a function of distance, using selected metorological visibility using formula (YY) (see formula annexe 2)

- Calculate the required (equivalent area) size of daymark (Using formula CC)…. (see formula annexe 2)

- Select the appropriate shape of the daymark while ensuring that the size requirements are met

The results must be adjusted while taking into account the “other” factors mentioned later in this document.

# Definitions

| Daymark | A daymark is that part of a navigational mark whose purpose is to enable visual identification of the mark in daytime. For the purpose of this guideline, a daymark includes shape, colour and topmark (defined in MBS) as well as any identifying markings. |
| --- | --- |
|  | (see also Navguide 3.13 page 8.2) |
| Photometric contrast | Difference of light issued from daymark and the background without taking into account of atmospheric extinction |
| Observation contrast | Difference of light issued from daymark and the background with taking into account of atmospheric extinction |

# Visual perception of Daymarks – Basic Principles.

Provide basic information on visual recognition of daymarks. Contrast with colour and luminance. Temporaral contrast. IALA paper on colour contrast from basement (Omar)

The contrast of the object, relative to its background is important for its visual function

The efficiency… Size of the area

## Introduction

There are three degrees of visibility of daymarks:

(a) When, because of the distance, the daymark is no more than a blotch on the background and neither shape nor colour can be distinguished (dotted outline) - the object is visible;

(b) When the Daymark is visible and its shape and outline can be seen, but the colour remains unclear – object is partly recognizable;

(c) When the shape and colour of the sign can be seen distinctly – the object is identified.

For each phenomena of transmission, we have

Something which creates 	signal Source

Something which conducts 	signal Media

Something which receipts 	signal Sensor

To study way of identification of a daymark is to define whole parameters which characterized these 3 things.

Daymark is defined by size, luminance, colours and shape. But have to take into account background just behind daymark. Background luminance and colour depend on geographical and meteorological conditions. Reference background depends on height of vessel bridge and direction of observation. These parameters vary and are “hazardous arbitrary.” Are difficult to quantify exactly

Atmosphere is characterized by meteorological visibility which is hazardous arbitrary.

At the end, observer behaviour depends on physiologic and complex mind phenomena.

We are able to characterize the visual appearance of an object

We are able to characterize why an object is appears, but don’t know very well how that object is distinguished and identified. This is why there isn’t here some accurate vision law.

Limit between physics and physiology

## Identification and recognition Detection and identification of daymarks

Geographical range

Evaluation of the geographical range using the equation presented in Nav guide.

Rg=2.03×ho×hb

Where:

Rg = geographical range (nautical miles)

ho = elevation of observer’s eye (metres)

Hb = elevation of the base of the daymark (metres)

We have to control that geographical range of lower part of the daymark is greater than the required range.

Table 1 - Geographical Range in Nautical Miles

| Observer eye height metres | Elevation of Mark / metres |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 10 | 15 | 20 |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |  |  |  |  |

Shape

If we want that mark is to be seen in all direction in horizontal plane, we have to control it has same aspect in all direction of observation. (Symmetrical)

In case of many navigational marks close to each other, they should differ in shape in order to facilitate the identification of individual marks. Different shapes may be defined to increase contrast, or increase luminance (flat surfaces reflect better than surfaces where there may be shadowing effects).

There are a lot of possible different shapes.

The equations in this guideline were derived from experiments performed at a range of 1000 times the size of the objects and using only simple shapes.

Practical criteria for recognition

If both conditions for visual detection (contrast and solid angle re. para HHH see part XXX) are fulfilled,  letters and symbols can be recognized at a distance of 500 times the height of the letter or symbol (assuming a width to height ratio of about 1:1). Similarly, the shape of an object can be recognized at a distance of 1000 times the height of the object.

The formulas given in this guideline can be used to determined whether or not the contrast of the object in question is sufficient for it to be perceived at a given distance.

The influence of colour on detection and recognition identification from a distance

If one is able to see the colour

About colour, visibility depends on contrast and identification of colour depends on amount of light issued of daymark (with eye adaptation of sky).

Detection depends on luminance difference of light issued of daymark and background (observation contrast) and colour difference between daymark an background.

Recognition (identification) depends on saturation of colour (less important fading effect), depends on luminance difference of light issued of daymark and background (observation contrast).

d = height of daymark

X = Observation distance

Colour propertie at distance of observation d/X<1 (non fluorescent colours)

Properties of non fluorescent colours when viewed at distance

| Colour | Detection | Visibility Shape identification | Colour identification |
| --- | --- | --- | --- |
| Black | Good (high contrast value) if viewed against the sky. Bad if viewed against a dark background such as a dark coastline In case of foggy conditions, a dark background becomes gray at distance, hence a black object is seen better in foggy conditions. | Same properties than for visibility | Good, The colour does not change with different illumination conditions. |
| White | Good if viewed in conditions with clear sky, illuminated directly or from the side against any type of background. Bad for cloudy sky when viewed against the sky or sea as background. In case of foggy conditions, background becomes gray, hence a white object presents a bad colour difference with background. | Same properties than for detection | Bad if viewed in the direction of the sun (object appears black) Good |
| Ordinary Red | Very good. Good colour difference with sky and coast | Medium shape identification | Bad if you are in front of the sun (seems to be black) Very good colour identification |
| Fluorescent red | Very good colour difference with sky and coast | Low shape identification | Very good colour identification |
| Green | Not so good because colour degradation is important Medium to low detection because of meteological and physiological effect | Same properties than for detection | Low colour recognition Difficult identification of colour. Problem with vegetation on the coast. If object is viewed at large distance (appears as a point source), it can be difficult to distinguish between green and blue. |
| Fluorescent green | Good colour difference with background | Low shape identification | Good colour identification |
| Yellow | Good if viewed in conditions with clear sky. Bad for cloudy sky when viewed against the sky or sea as background. In case of foggy conditions, background becomes grey, hence a white object presents a bad colour difference with background. | Same properties than for detection | Good if viewed in conditions with clear sky. In cloudy sky or foggy conditions bad colour identification Difficult Identification of colour. Yellow seems to white. Good colour identification if just near a black dark colour |
| Blue | Not very good because of atmospheric absorption It seems to be not very good (meteological and physiological effects) but need to be confirmed | Same properties than for detection | Risk of confusion with sea and sky in sunshine |

For all colours except black: when viewed in the direction of the sun, the object may appear to be black.

Luminance of fluorescent colours is greater than that of ordinary colours since a part of the short wavelength incident light is transformed to lower wavelengths contributing to the intended colour (blue to red). More important problem is lifetime of fluorescent colour in comparison of ordinary colour.

Fluorescent colour

Luminance of fluorescent colours is greater than that of ordinary colours since a part of the short wavelength incident light is transformed to lower wavelengths contributing to the intended colour (blue to red).

Result is very good in red (3 or 4 time more light), good in yellow ( 2 or 3 time more light) and not so good in green (less than 2 time light) but interesting in some cases.

More important problem is lifetime of fluorescent colour in comparison of ordinary colour.

Effect of partially transparent daymarks

[see if there is IALA definition for transparent daymark?]

The equations are directly applicable to daymarks with solid surfaces.

If parts of the daymark surface is transparent (holes or gaps), the contrast decreases, and thus the range reduced.

There is at present no verified method available for contrast correction due to partial transparency, but a rough estimate can be achieved by taking a medium contrast value.  multiplying the contrast value with the ratio of the transparent area divided by the total area

[add an illustration of the problem]

Remember to mention the nominal vs. “real” range when designing

# Limits of visual Perception of daymarks

Deal with question of visual acuity.

There is 2 different approach to determine range of daymark :

  - approach based on contrast

  - approach based on colour difference

The true method would a mix of both

## General Calculation based on observation contrast

Specific information on the limits of recognition of buoy daymarks.  Cardinal towers, lateral shapes etc. Omar paper.

Information on the limits of recognition of cardinal topmarks.

XAVIER

Math calc for contrast colour with different sky colour .

Math def of range of daymark (incouding size).

[Describe French method]

[Include table of size for different range]

## General Calculation based on colour difference

[Describe German method]

[Include table of size for different range]

# Advantages of using highly saturated colours for daymarks (SABINA)

Details on the different recognition ranges that arise from contrast such as colour, luminance, temporal. Review work from Sabina paper. Old EEP paper from Germany on conspicuity of surface colours. Effect of incident light on daymark recognition.

General comments of colour perception

Red is very good for detection at distance, but if viewed with direct sunlight, it will be perceived at black.

Difficulties in reognizing shape

# Factors that reduce visual range (TG2 Section 5.)

Basic information on the geographical importance of geometric centre of area of the structure, “daymark focal plane” height. With examples of buoy size.

Describe the effects of wave height in open water and the consequent reduction in range.

TG2 Section 5.

[Include Service factors]

Factors that reduce detection

Height of wave (increase and decrease)

Light situation (sunset or sundown)

Complex background

Visibility (fog)

Factors that reduce shape identification

Factor that reduce colour identification

Visibility of Daymark – height, size and shape aspects

There are three degrees of visibility of daymarks:

(a) When, because of the distance, the daymark is no more than a blotch on the background and neither shape nor colour can be distinguished (dotted outline) - the object is visible;

(b) When the Daymark is visible and its shape and outline can be seen, but the colour remains unclear – object is partly recognizable;

(c) When the shape and colour of the sign can be seen distinctly – the object is identified.

This gives two options to improve the conspicuity of an of a floating AtoN, changing the colours of the structure, or alter the size / shape.

Refer to IALA publications:

IALA Guidelines 1051 – Aids to Navigation in Built-up Areas Edition 1 - December 2005, page 5

The range of the light depends, among other factors, by the focal height of the light and its intensity. For a daymark his conspicuity or the distance that it can be seen, is determined by the height of the base of the daymark from the waterline and the size / shape of the daymark.

The purpose of this section is to present a method to predict the daylight range of a daymark, witch

- Evaluation of the geographical range using the equation presented in Nav guide.

Rg=2.03×ho×hb

Where:

Rg = geographical range (nautical miles)

ho = elevation of observer’s eye (metres)

Hb = elevation of the base of the daymark (metres)

We have to control that geographical range of lower part of the daymark is greater than the required range.

Table 2 - Geographical Range in Nautical Miles

| Observer eye height metres | Elevation of Mark / metres |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 10 | 50 | 100 | 200 | 300 |
| 1 | 2.0 | 4.1 | 4.9 | 5.5 | 6.1 | 6.6 | 8.5 | 16.4 | 22.3 | 30.8 | 37.2 |
| 2 | 2.9 | 4.9 | 5.7 | 6.4 | 6.9 | 7.4 | 9.3 | 17.2 | 23.2 | 31.6 | 38.1 |
| 5 | 4.5 | 6.6 | 7.4 | 8.1 | 8.6 | 9.1 | 11.0 | 18.9 | 26.9 | 33.3 | 39.7 |
| 10 | 6.4 | 8.5 | 9.3 | 9.9 | 10.5 | 11.0 | 12.8 | 20.8 | 26.7 | 35.1 | 41.6 |
| 20 | 9.1 | 11.1 | 12.0 | 12.6 | 13.1 | 13.6 | 15.5 | 23.4 | 29.4 | 37.8 | 44.2 |
| 30 | 11.1 | 13.2 | 14.0 | 14.6 | 15.2 | 15.7 | 17.5 | 25.5 | 31.4 | 39.8 | 46.3 |

Refer to IALA publications:

IALA NAvGuide 5, 2006 edition, page 36

- Evaluation of the size and shape of the daymark. This can be determined taking in account the studies used for the IALA Guideline 1023 for the size / shape of the dayboards of leading lines. The table 2 expresses dayboard size in terms of height in meters and range in kilometers based on the presumption that the dayboard height will be 1.5 times the width (shape restriction), a common international practice.

Table 2 - Operational Range in km for Specific Dayboard Lengths

| Vis, nmi | H (height will be 1.5 times the width |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.6m | 2.1m | 3.1m | 4.2m | 6.3m | 8.6m | 12.2m |
| 1 | 0.9 | 1.3 | 1.7 | 1.9 | 2.0 | 2.4 | 2.7 |
| 2 | 1.1 | 1.7 | 2.2 | 2.6 | 2.8 | 3.4 | 3.8 |
| 3 | 1.1 | 2.0 | 2.8 | 3.5 | 3.9 | 4.8 | 5.5 |
| 4 | 1.3 | 2.4 | 3.3 | 4.3 | 5.0 | 6.0 | 7.0 |
| 5 | 1.5 | 2.8 | 3.9 | 5.0 | 6.1 | 7.3 | 8.5 |
| 6 | 1.5 | 3.0 | 4.3 | 5.4 | 6.7 | 8.0 | 9.3 |
| 7 | 1.7 | 3.2 | 4.4 | 6.1 | 7.4 | 8.9 | 10.3 |
| 8 | 1.7 | 3.2 | 4.8 | 6.5 | 7.8 | 9.5 | 11.0 |
| 9 | 1.7 | 3.5 | 5.3 | 7.0 | 8.3 | 10.2 | 11.9 |
| 10 | 1.9 | 3.7 | 5.6 | 7.4 | 9.3 | 11.1 | 13.0 |

Refer to IALA publications:

IALA guideline 1023 for the design of leading lines december 2001, page 36

- For size of the topmarks the considerations are presented in NAvguide 2006 edition, at paragraph 3.1.13.6.2.

Refer to IALA publications:

IALA NAvGuide 5, 2006 edition, page 82

- For dimensioning the identification markings it is necessary to consider the requirements posed by the authority and the importance of it for the mariner.

Example:

  - An inland special mark signalling a danger, where his identification gives no extra information to the mariners does need large characters.

  - A special mark identifying a forbidden area, with a no entry sign, this marking should be visible in an adequate distance.

- The wave height / frequency in open water will have detrimental effects in range due to the following factors:

  - Variation in the visible area of the daymark, due to the pitching of the floating aid;

  - Changes in the shape due to the rolling effect;

  - Reduction in the exposure time of the floatings aids due to the relative position with the observer, witch can be obstructed by the crest of the wave.

# Service factors (TG3 Section 6)

Dirt/ fading of colour over time due to UV

TG3 Section 6

Service factors

Dirt in the form of green slime, bird droppings, green mould will make the visible identification of colour difficult or impossible. Where the top of the floatation bowl forms part of the daymark, sufficient freeboard should be provided to ensure that the top of the bowl is not regularly submerged encouraging development of green slime. In critical areas it may be necessary to frequently wash the buoy.

Fading is a result of the attack by ultra violet (UV) light present in sunlight and buoy daymarks are particularly exposed to this type of degradation, having no form of shade. Selection of the type of paint system or plastic material should be made for maximum UV stability. Service providers should carry out periodic tests to confirm that the daymark colour remains within IALA recommendation. It may be necessary to increase the frequency of painting in order to ensure that the daymark colour maintains its performance within IALA recommendation.

Theft, interference and tying up to buoys can have serious effects on the provision of buoy systems. The use of security locking systems can inhibit casual theft but absolute prevention of theft is impossible. Deliberate malicious interference can be dealt with through the legal system if the perpetrator can be identified. The use of buoys by fishing vessels as a temporary mooring should be discouraged through public information system and regular contact with the fishing community and local authorities. A remote monitoring system can provide early warning to the Service Provider in the event of damage to the electrical systems.

# Other considerations (TG3 Section 8.)

Other considerations.

The effects of aerial height on radar reflector, Racon, and AIS performance. Refer to other IALA documentation.

Consider removal but where else does this info fit???

TG3 Section 8.

Additional systems.

When considering the height of a daymark, allowance must be made for the effect of height on equipment mounted on the daymark such as radar reflector, Racon, AIS and meteorological equipment.

Radar reflector.

In order to maximise the range at which a buoy is visible on radar, the buoy should be fitted with a radar reflector. The optimum unit is normally a corner reflector as large as possible. The physical size of the radar reflector is limited by the size of the daymark and radar cross section (RCS) may be increased by the use of multiple reflectors mounted with the lower reflector at a height of 0.65 times the height of the upper reflector above water. Typically the divergence of corner radar reflectors is better than 20 deg but this varies considerable for different types of equipment. The luneberg type radar reflector provides excellent performance at wide divergence angles with consistent performance throughout the horizontal plane.

Radar target enhancers can also be used to increase the RCS of a buoy. However this requires a power supply with consequent effect on the size of the buoy power system.

Racon

IALA guidance for provision of racons is covered in Recommendation R101 and in the Navguide. In mounting racons on buoys, it is essential that the racon is mounted as high as possible and clear of obstructions that will affect radio propagation. Racons can be mounted inverted as well as upright so long as the antenna is vertical.

AIS

Guidance on provision of AIS AtoNs is given in other IALA Recommendations and Guidelines. When fitting AIS on buoys, it is necessary to mount the antennas at the highest point to ensure that there is no shadowing of the GPS or VHF antennas. Sensors for met/ hydro data to broadcast on AIS must similarly be mounted clear of obstructions and compete for space with other components. Performance of AIS is not significantly affected by buoy movement. Power consumption must be carefully planned when fitting AIS transponders.

# Use of Multiple Marks (TG2 Section 9.)

Use of multiple marks.

TG2 Section 9.

Multiple AtoN

When determining the dimension of the daymark it is necessary to evaluate the purpose of the floating aid and the relationship to other aids to navigation and observable features;

Examples:

- Marking restricted / dangerous areas (one large daymark or two or more smaller marks at the area limits)

- Using a daymark combined with other AtoN (ex: RACON);

# Climate effects (TG3 Section 10 )

Climate effects – designs for varying environmental conditions such as ice, floating islands, breaking waves.

TG3 Section 10

Environmental limitations

Local environmental conditions can give rise to the need for special design of daymarks which may not conform to the general guidance in this document. Such conditions include drifting ice, debris in rivers, buoys in breaking wave conditions.

Buoys located in areas exposed to annual drifting ice, possibly of considerable thickness, that can destroy equipment mounted on the buoy body. Buoys in such areas must generally be designed with a conical body with a minimum of projections and be completely sealed. Use of additional daymark boards or topmark may not be feasible. Consequently the use of can shape for lateral buoys is not always possible.

Debris in rivers, such as trees, logs and even whole floating islands can inflict damage to a buoy similar to that in drifting ice.

Where a buoy system is deployed in areas of breaking waves or drying out during low tide, the buoy design must be adapted to ensure survivability of the buoy and its equipment and the design may not conform to the general format.

Bird droppings can seriously affect the performance of buoy systems as well as obscuring the colour of the daymark. Droppings on lantern or solar panels will cause reduced performance or failure of the light. The corrosive effects of bird droppings or acid rain can cause damage leading to reduced life or premature failure. Daymark design must therefore be such as to avoid providing any natural bird resting sites. The use of long flexible spikes on all potential land sites can be helpful. However care must be taken to ensure that these do not create a hazard for people attending the buoy.

# References

[1] Blaise Pascal’s old article in the IALA Bulletin (Xavier to provide details) kindly translated by Xavier

[2] CIE110 Spatial distribution of daylight - Luminance distribution of various reference skies

ANNEX 1

Practical examples

Practical application (EXAMPLES one for cloudy sky and two for cloudy sky)

ANNEX 2

French recommendation about Daymark visibility

Contrast at short distance

At short distance, the effect of atmospheric extinction can be ignored, thus…

If we have O an object, F uniform background with a significant size compared to the object; Lo (object) and Lf  (Background) luminance .

Contrast at short distance C(o) is :

C(0)=Lo−LfLf

Contrast characterized the possibility of the eye to distinguish an object from a background where the object is projected.

Contrast with the sky as background

With Cc(o) contrast with the sky and Lc luminance of the sky.

Cc(0)=Lo−LcLc

Contrast with other backgrounds, but using the sky as reference

When the object is projected on a different background than the sky, we have to keep sky luminance for reference because eye of the observer is adapted to the sky.

With Ls the luminance of the ground, contrast  relative to the sky Cs(o) is :

Cs(0)=Lo−LsLc

Approximate values for contrast Cc(o) (sky as background ) and Cs(o) (other backgrounds)

Now we have to define the sky

We consider two different cases : a uniform cloudy sky and a clear sky. In most of cases, the reality is just between that 2 cases.

We consider that the daymark has a single colour and is in a vertical plane.

Case 1:  Uniform cloudy sky

Ls and Lo are defined empirically by :

Ls=2.34β1Lc

Lo=3β2Lc(0.308+0.389β)

With a ground in a horizontal plane.

| Lc | luminance just near horizon. In case of uniform cloudy sky, horizon luminance is 1/3 zenith luminance |
| --- | --- |
|  | is illuminance factor of the object (0> |
|  | illuminance factor of ground (0.6 for white sand, 1 for snow, 1? for water) |

Case 2: Clear sky

Luminance Lc just near horizon is very dependant on azimuth in relation to the sun, and height of the sun (zenith distance of sun).

Lc is between 180Cd/m2 (Az 100° when sun is near horizon the evening) and 19000Cd/m2 (Az 0°, zenith distance 20°). Medium value, in France for Lc is 5000Cd/m2.

For all contrast calculi we have Lo(0) object illuminated by the sky only, Ls(1) luminance of a white ground and ground luminance is Ls = 2 Ls(1).

For an object with the sky as background:

Lo=Lo(0)+12Ls

If the object is not white we have to multiply Lo by 

For an object with a background other than the sky

Lo−Ls=Lo(0)−12Ls

If object is not white we have

Lo−Ls=βLo(0)+β−22Ls

In France :

Ls is between 300Cd/m2 (dark ground) and 10000Cd/m2 (white ground with sun at zenith).

Lo is between 1Cd/m2 and 32000Cd/m2

Purpose of the above is to arrive at order of magnitude values for luminance values

So

Contrast with the sky as background:

Cc(o)=1−β1Lo(0)Lc+12LsLc

Contrast with background other than the sky, but using the sky as reference since the ambient luminance level is defined by the sky:

Cc(o)=β1Lo(0)Lc+1−2β1Ls2Lc

With CIE model of the sky we can obtain Lc and Lo(0) for a white object. Like that we can calculate Lo(0) for coloured object and Ls (CIE110 Spatial distribution of daylight - Luminance distribution of various reference skies).

IALA recommendation for surface colours (E-108) contains required values for reflection factor B

Contrast at distance

Atmospheric absorption, defined by meteorological visibility, has an action about conspicuity.

We admit that can be characterized by a modification of contrast of the object projected on background.

Contrast with the sky at distance X

Cc(X)=0.05XVCc(0)

Contrast relative to the sky at distance X (Background other than then sky)

Cs(X)=0.05XVCs(0)

Visibility law of Daymark

Visibility conditions

We define d as the characteristic length of daymark (size of square with same area of daymark).

Approximation of solid angle under daymark is seen.

The solid angle of subtence can be approximated as: d2X2

Experience says that for a daymark visible at distance X we have to satisfy 2 conditions :

- Contrast at distance X have to be greater than 0.05

So C(0)=0.05XVC(0)>0.05

- Solid angle under daymark is seen have to be more important than a value which depend on X

So d2X2>0.038C(X) (with d in meter and X in kilometer)

At short distances, the size of the object is not as important as the contrast.

At long distances, we have to take into account the solid angle of the object  to determine POD…..

General comment on limits of visual range

Since C(X) contrast has to be better than 0.05

So d2X2≥0.038C(X)=0.76

It’s case solid angle which daymark is seen is superior or equal to 1/1000 St

When C(0) is equal to 1

C(0)=0.05XVC(0)>0.05 => X<V

It’s why you can’t see a daymark a distance of observation more important of meteorological visibility distance.

Visibility distance

Viewing at long distances

If daymark is seen under a solide angle less than 1/1000 St (d/X<1) so

C(0)(0.05)XVd2X2=0.038 (with d in meter and X in kilometer)

Viewing at short distances

If daymark is seen under a solide angle more than 1/1000 St (d/X>1) so

C(0)(0.05)XV=0.05

Distance of visibility depends on contrast and not size of daymark.

Annex 3

German input to the workshop

1. General remarks about colours for buoys

There are a lot of different discussions about buoys in Germany. The discussion is mainly influenced by the aim to reduce the cost for the buoys and their maintenance.

To set back the focus on the main task of a buoy, it became inevitable to make a list of the features of a buoy and to make a valuation between the features.

As a result we got the distinction between the so called primary and secondary features:

Primary features give reasons for a buoy (or a light) itself. They explain why we use buoys for a waterway and are linked to the nautical requirements.

Secondary features are features to ensure that a buoy (or a light) will work properly. They are necessary to support the primary features.

Example:

Primary features of a buoy are:

- colour and shape

- radar echo

- position

- availability

Secondary features are:

- mooring

- maintenance

- floating in water

- lifetime

- costs

In general the secondary features should not limit the quality of the primary features. However the primary features should not cause secondary features that cannot be fulfilled (e.g. because they are too expensive).

In detail the primary features of a (lighted) buoy are:

- colour (daytime: surface, nighttime: lights)

- shape (topmarks and special arrangement of coloured stripes or areas)

- flash characters of the light (nighttime)

- radar reflector

- position

- availability

The surface colour belongs to the primary features of a buoy and there have to be very hard 'secondary' arguments to justify the limitation the quality of that colour.

2. Three steps of visual perception

When approaching an object from a far distance we move through three different steps of visual perception (I directly translate the German words, might be you have different definitions):

(1) Object is visible: You see something but you don't know what it is.

For example: The object is still too small to be recognized as a certain type of buoy.

(2) Object is recognizable (identified): You see the object and know what it is, because you identify the colour or shape.

For example: The object can be recognized to be a certain type of buoy of the IALA buoyage system.

(3) Object is conspicuous: You cannot put the eyes away from the object.

Although we try to get distant buoys to become conspicuous, an object is conspicuous for your eye, when you just stand in front of it.

For the visual features (shape and colour) it becomes evident to distinguish between a visible (1) and an identified (2) object.

3. About Threshold of colours

To quantify the visual performance it is helpful to define the Maximum Distance of Recognition MDR.

MDR is the maximum distance the shape or the colour of the object can be identified.

In Germany we have the convention that the shape of a simple object (cone, cylinder or ball) can be identified when the angle is at least 3' (0.05°). This leads to an equation for MDR.

Fig. 1

In addition it is necessary that the object has enough contrast to the background.

For large objects, a distance MDR can be defined by the luminance contrast

C ≥ 0.05 (that is the international accepted value).

From IALA dictionary:

C=L2−L1L1

L1	luminance of the background

L2	luminance of the object

There are formulae for the threshold of the luminance contrast for small objects. However the concept of contrast can only be applied in a grey world.

When colour comes into consideration the contrast is not the right quantity to describe the visual performance.

Example: A red area with luminance 1 against a background with luminance 1 has a contrast of zero, but can appear conspicuous.

The identification of a colour does not depend on the luminance contrast.

The identification of colour depends on the luminosity and the saturation of the colour (The background at sea is assumed to be grey.).

Example: Assume a coloured area against a grey background. When the coloured area is very dark, you cannot identify the colour. Because of the luminance contrast you will see the area very well. However when you do not have a specific shape of the area, it will not show any information to the observer (perception step 1) except the information 'there is something'. When the colour gets brighter you will get the colour information (perception step 2). For large objects you get the information even if contrast is negative, for small objects it might be necessary to have positive contrast to identify the colour.

The luminosity of the colour can be described by the ratio L2/L1.

The saturation can be described by DIN 6164 [1]. There are alternative ways.

Unfortunately we have no theory for the identification of colour, which combines the influence of the size, luminosity and saturation of the coloured object, and that can be used to calculate MDR.

In general the distance MDR increases with the luminosity and the saturation of the colour [2].

In consequence fluorescent colours have a greater MDR.

4. Colour versus shape (German buoys)

In the following figures the shape of two typical German buoys are shown. The shape of the German buoys do not represent any information in accordance with IALA buoyage system (cylinder, cone , ball). For some buoys there are top marks.

Fig. 2

Fig. 3

The top mark may be omitted for lateral buoys, so colour is the only visible (primary) feature at daytime.

From the geometrical convention we use in Germany we can derive three distances for this certain type of buoy (LT 81).

(a) buoy becomes visible	3 300 m

(b) identification of the stripes 'cardinal west'	1 000 m

(c) identification of the shape of the top marks	 	690 m

For all three aspects it is necessary to have a contrast or colour difference against the background. (a) and (b) require a suitable colour to ensure that the buoy is identified according to the IALA buoyage system.

When a very saturated and bright colour is applied (for lateral green and red only) the buoy can be identified in the same moment as it becomes visible. So the MDR is 3 300 m for good weather conditions for lateral buoys.

Conclusions (for German buoys only):

Considering the primary feature 'colour' of a buoy, the lateral marks with bright and saturated colours (fluorescent) have the longest MDR and are therefore the most effective way to mark a waterway.

The distances to identify a buoy with two colours or with top mark (single danger, cardinal) are always much shorter than for the lateral buoys. This is a disadvantage caused by the current IALA buoyage system.

5. Desaturation caused by the atmosphere

Although there is no theory about the maximum distance of recognition for coloured objects, which includes luminosity, saturation and size of the object, it is possible to compare the colours.

One way to evluate the performance of colours is done with the law of Koschmieder [3], which can be transformed into spectral equations (Hoffmann, [4]).

Fig. 4

Figure 4 shows the geometry to explain the influence of the atmosphere on the light that reaches the observer.

Light from the object is scattered away from the direction to the observer (1).

Light from the background is scattered into the direction to the observer (2).

Remark:

In detail the amount of light scattered into one direction depends on the position of the sun. When the sun is behind the object, the amount of light scattered into the direction of the observer is higher than predicted. When the sun is behind the observer, it is less. So Koschmieder's law gives the mean value of the scattered light.

In general Koschmieder's law is used to explain the decrease of the contrast between the object and the background.

Hoffmann extended the law to spectral formulae. This can be used to calculate the change of colour when the light moves through the atmosphere (see appendix A1).

To simplify the calculation we did not yet take into account the spectral dependence of the atmospheric transmission. We will do this in future.

The scattering of light in the atmosphere causes two effects on the light the observer sees.

The first effect is that the visible luminance of the object approaches the luminance value of the background when the distance between object and observer increases. This is shown for two green colours in figure 5 and a meteorological visibility of 10 M (Ordinary green is RAL 6024).

Fig. 5

The second effect is the desaturation. This is shown in figure 6 for fluorescent colours.

Fig. 6

Figure 6 is based on outdoor measurements of coloured samples and application of the theory 'Koschmieder / Hoffmann' as explained in appendix A1.

We still have no threshold to evaluate between the different colours. We will not get a threshold, which is based on size, luminosity and saturation of the coloured object, since there is no theory. However looking at the saturation alone, we can define a threshold (see figure 7).

We chose the saturation S = 6 as the threshold. This decision is arbitrary of course, but 'S = 6 ' corresponds to the border of the region of red and green towards white (for green S = 5..6, and for red S = 6..7). So it represents an existing convention.

Fig. 7

In figure 7 we compared a fluorescent green and an ordinary green RAL 6024. We get an intersection at S = 6 for:

fluorescent green: 	3 700 m

green RAL6024:	 400 m

At the distance of 3 700 m fluorescent green produces nearly the same colour stimulus as RAL 6024 at 400 m.

So we can say that - in this model - the fluorescent colour is about 9 times more effective than the ordinary green.

We use this procedure to compare different green and red colours only. This is because the green and red colour is the only visible feature for most lateral buoys at daytime.

We have investigated six different colours and the results are presented in table 1.

| Colour | Distance for S = 6 | L2/L1 |
| --- | --- | --- |
| Fluorescent Green | 3700 m | 1,07 |
| Preferred Ordinary Green | 2200 m | 0,89 |
| Green RAL 6024 | 400 m | 0,78 |
| Fluorescent Red | 3900 m | 1,36 |
| Preferred Ordinary Red | 1800 m | 0,91 |
| Red RAL 3020 | 1500 m | 0,74 |

Table 1

Figure 8 shows the colorimetric values of the six colours.

Fig. 8

It can be read from table 1 that the RAL colours, which are used for road signs, are not suitable for buoys. There are ordinary colours (preferred) that ensure a greater distance of recognition than the RAL colours.

The preferred colours benefit from their higher degree of saturation and their luminosity.

Scrapped Text

A daymark is an unlighted navigation mark (IALA Dictionary 2-6-030)

Buoy:     2-6-060

A floating object of defined size, shape and colour, usually made of metal or plastic, which is anchored at a given position and serves as an aid to navigation. It may carry a light or an acoustical or other device.

the daytime character of an aid to navigation

A solid structure, or a structure in the form of scaffolding, fixed in the ground and standing out sharply against the background, in order to be easily recognisable.

This guideline….Part A and Part B describe….

- Review guideline on daymarks

- E112 Rec on leading lines

- E108 Rec for surface colours

- 1023 Guideline for leading lights

- 1015 guideline on painting of buoys

- 1051 guidline on provision of Aton in built up areas.

- Navguide

- Code for internal water CEVNI from EU.

Annexe 4

Example of meteorological visibility determination

Include Brazil input paper

Give example of visibility for different countries

CIE 110  Ref…