### 5.1.1 A typical set of navigational needs and the resulting required buoy lantern performance (example)

It is useful to take the theoretical study above, and develop it further, to define the operational

need and the resulting required lantern performance. The following table is of course for

night time navigation without background illumination, and the ranges and transmittivity

figures are selected for a typical example.

| Operational need | Environmental conditions – sea state | Environmental conditions - visibility | Observer range from buoy | Comment |
| --- | --- | --- | --- | --- |
| Requirement 1, Picking up the start of a buoyed channel at long range | Heavy seas | Visibility likely to be encountered 90% of the time. | 4 M | The ship is approaching the start of the channel. It is acceptable for the visible range to be reduced below 4 M in poor visibility and or very heavy seas. |
| Requirement 2, Navigating in a buoyed channel | Very heavy seas | Visibility likely to be encountered 98% of the time. Includes heavy rain | 1 M | Mariner is now in the confined channel, and must see the light, as he approaches the buoy or pair of buoys. |
| Requirement 3, Passing a buoy or pair of buoys | Very heavy seas | Visibility likely to be encountered 99% of the time. Includes thick fog | 0.05 M | Mariner is now in the confined channel and must see the light to know his lateral position in the channel |

Using this set of operational needs, the parameters which will influence the lantern performance can be determined.

| Operational need | Sea state → buoy motion | Visibility → Transmissivity factor | Observer range from buoy | Comment |
| --- | --- | --- | --- | --- |
| Requirement 1, Picking up the start of a buoyed channel at long range | Heavy seas α+β = 10° | Visibility likely to be encountered 90% of the time. TM = 0.74 | 4 M |  |
| Requirement 2, Navigating in a buoyed channel | Very heavy seas α+β = 20° | Visibility likely to be encountered 98% of the time. Includes heavy rain TM = 0.223 | 1 M |  |
| Requirement 3, Passing a buoy or pair of buoys | Very heavy seas α+β = 20° | Visibility likely to be encountered 99% of the time. Includes thick fog TM = 0.001 | 0.05 M | Observer is close to the buoy, and well above. Bridge heights up to 40 metre |

The Tm factor used in Requirement 1 is derived from data published for Japan. Refer to publication “Basic Theory of Aids to Navigation”, 1988, issued by Tokokai organisation (Associate Member of IALA). Table 2.1-2 on page 79 “Ranges of Visibility in Japan” shows data for visibility derived from observations throughout the coastline of Japan, and indicates an average Tm of 10.2 nautical miles. A figure of Tm =0.74 (10 nautical miles visibility) is thus used in Requirement 1. The 0.223 figure is for  meteorological visibility of 2 km.

Now the performance required of the lantern can be computed.

| Operational need | Lantern vertical divergence range | Input data to intensity calculation | Computed intensity Icomputed | Photometric Intensity Iphoto |
| --- | --- | --- | --- | --- |
| Requirement 1, Picking up the start of a buoyed channel at long range | 0 to ±10° | TM = 0.74 D = 4 M | 36.6 cd | 51.6 cd |
| Requirement 2, Navigating in a buoyed channel | +10° to +20° & -10° to -20° | TM = 0.223 D = 1 M | 3.1 cd | 4.4 cd |
| Requirement 3, Passing a buoy or pair of buoys | +20° to +90° | TM = 0.001 D = 0.05 M | 0.0025 cd |  0.0034cd |

### 5.1.2 Significance of the example

The example given above is intended only to demonstrate the theory of this method of deriving lantern vertical divergence requirements. It should not be viewed as an IALA specification for buoy lanterns, but should be considered as indicating a design process. The process shown relates the operational performance requirements to the lantern intensity and vertical divergence needed to meet those requirements, via consideration of visibility conditions and buoy motion.

It is up to the designer of the AtoN to ensure that he is aware of the operational needs, and also to have available the meteorological and hydrological data, the visibility data, and the buoy motion data he needs to carry through the analysis and reach the correct conclusion.