Conspicuity of Green and White Flickering Light

CDR Hideki NOGUCHI

Japan Coast Guard

1. Abstract

Japan Coast Guard (JCG) has studied and developed flickering light but all experiments were carried out by red LED only.  Therefore JCG carried out another field experiment on green and white flickering light in order to confirm the conspicuity in 2008.  This paper outlines the experiment and the result.  The detail of the experiment will be reported on the 2010 IALA Conference Paper.

2. Outline of Field Experiment

The filed experiment was carried out as follows;

Date: 7th October 2008 (Tue.)

Place: Kujukuri beach, Sanmu city, CHIBA

Evaluation distance: 2km

Number of Evaluator: 52

Method (Both colors are same): One LED lantern produced conventional flashing light, Fl 3sec, first and then produced flickering flashing light of same character.  The both lights were adjusted to become the same luminous quantity.  The evaluator compared two lights and gave plus pint for better one and minus point for worse one.  In order to confirm the difference of the duty ratio and the flicker frequency, the first experiment was carried out by fixed frequency, 10Hz and changing duty ratio, 10 to 90%.  The second was by fixed the duty ratio, 30% and changing flicker frequency, 6 to 14Hz.

3. Result

The both green and white flickering light has conspicuity and the lower the flicker frequency or the duty ratio, the conspicuity becomes higher.  However the degrees of the conspicuities are different.  The following figure shows the comparison of red, green and white flickering light when flicker frequency is fixed as 10Hz.  The result of red are quoted from the field experiment of 2007.  Because of the difference of the number of the evaluator, the point is adjusted.

Figure: Comparison of Red, Green and White Flickering Light

4. Conclusion

At the duty ratio of 50% or less, the conspicuity of green flickering light is obviously lower than red and white flickering light.  It is seemed that the flicker range which is proposed by Mr. Xavier KERGADALLAN at the expert meeting in Japan, is also different by colors and therefore the design of flickering light as aids to navigation required careful consideration.