EEP14 Light Source Update – 1st October 2009

# LED Light Source Arrangement

Tests are ongoing at the General Lighthouse Authorities R&RNAV Directorate on an LED light source for omnidirectional optics. The assembly currently being tested is an array of four Citizen “CitiLED” CL-L223-13N LEDs. Each LED is itself a 13W array of LEDs. The LEDs are run at 700mA current with a total power of 4 x 13 = 52W.

Photometrically the light source performs very well in fixed and rotating lenses but, with the present heatsink, the light source can only sustain a 50% duty cycle without overheating. A larger, more efficient heatsink has been designed and is currently being manufactured.

Ian Tutt

GLA R&RNAV

Trinity House

Harwich UK

ian.tutt@gla-rrnav.org