Rapporteur Report

Visible Light Communication

CDR Hideki NOGUCHI

Japan Coast Guard

1. Abstract

The Lighthouse Sub Project of the Visible Light Communication Consortium (VLCC) carried out a long distance communication experiment in 2008 and it succeed in the world record of most long-distant, 2km at transmission rate of 1022bps and 1km at 1200bps, the twice of the previous communication rate, using image sensor technology.  The Lighthouse Sub Project is composed of Japan Coast Guard, Casio Computer Co., Ltd., NEC Corporation and Toshiba Corporation and studies possibility of the telecom by the visible light communication.

2. Visible Light Communication of Lighthouse

The following figure shows the image of lighthouse where visible light communication is applied.  The information of the lighthouse is transmitted using the LED flash light of the lighthouse to an image sensor onboard.

Image sensor

Information Display

LED light

LED    modulator

Information server

Long. and lat. of Lighthouse

3. Outline of Experiment

The experiment was carried out as follows;

Date: 8th October 2008 (Wed.)

Place: Kujukuri beach, Sanmu city, CHIBA

Evaluation distance: 1km, 2km, 4km

Lantern: LED lantern TypeⅡ(luminance range of 2n.m.)

Color: White, Red and Green

Lighting Character: Fl 4sec (duration of light is 0.4sec.)

LED Light

Photo 1: LED Lantern

Photo 2: LED light at 2km

4. Result

Visible light communication of 1km at communication rate of 1200bps and 2km at communication rate of 1022bps were succeeded but the communication of 4km was failed due to heavy rain.

5. Possible Application

The image sensor technology is able to receive more than one target at one time even though under conditions of moderate ship movement.  Therefore the possible application of visible light communication of lighthouse or lighted buoy is to send necessary information such as name, position, discrepancy of aid to ships in moderate sea areas such as inside of bay, harbor, port.  The visible light communication has possibility of becoming one of e-Navigation tool using conventional visual aids.

Note by the Secretary

The author has indicated that if this topic is of interest to the Committee a full report can be provided to EEP15.