| TECHNICAL PROCEDURE No. TP010 |  | # Maintenance of Buoy Solar Unit |  |
| --- | --- | --- | --- |
| 2 | 5 November 2004 |  |  |
| Issue | Date | Written By | Approved By |

1.	Scope

This procedure covers the maintenance or refurbishment of the standard buoy solar units following 5 years' service on station to bring them fit for a further 5 year service period.

The work is to be undertaken at Harwich and Swansea Depots under the direction of the Coast Managers (Field Operations).

2.	Inspection and cleaning following return from Vessel

The condition and function of the solar unit equipment should be thoroughly checked as soon as practicable after the solar unit is returned to the Depot.

All equipment should be checked as listed on the Solar Buoy Service Checklist, Table 3, Solar Buoy Quick Reference Guide, HB01-01.

The complete solar unit should be soaked with fresh water externally and cleaned of salt deposits and bird lime.

The condition of all electrical cables, glands and connections, including the battery ring terminals, should be checked.  If of doubtful condition the cable should be replaced, the Solar Modules and Regulator assemblies being returned to the supplier via stores, when the cable is part of these items.

The solar unit should be checked for compliance with the latest issue design drawings and specifications.

3.	Strip-down of Solar Unit

The following items should be removed from the solar unit:

Batteries

Lantern

Regulator, if this cannot be fully tested in-situ

4.	Batteries

Follow the instructions given in the THLS Technical Procedure No.  TP003, Maintenance of the Sonnenschein A512 C 80AH Batteries on Solar Buoys.

5.	Lantern

Carefully check the lantern for signs of water ingress, checking inside the base of the flasher unit without disturbing the base seal.

Check the lens and plastic base housing for opaqueness, which may be caused by prolonged exposure to ultraviolet radiation.  Replace if obviously "clouded"; "Clouding" of the base housing would effect the operation of the photocell.  Clean the lens and base housing with Loctite Natural Blue Cleaner and Degreaser (RS 693-804).

Clean any salt and corrosion deposits from the aluminium base plate and smear the face where it lies against the base housing, the 'O' ring and the fixings on the underside of the base plate with petroleum jelly.

Remove and dispose of the used lamp.

Replace the lamp and lens as required for use on the next station.

Fully test the lantern, following lantern test procedure.

6.	Regulator

Check the regulator housing for damage, excessive corrosion, or the possibility of water ingress, especially around the LEDs (Beco BR3A).

Check the connection between the battery lead and ring terminals.

Fully test the regulator, following the regulator test procedure.

7.	Solar Modules

Check the glass face of the solar modules for cracks or signs of water ingress.

Measure the output voltage and short circuit current, comparing with a module known to be in good condition exposed to the same irradiation, at the same temperature.

8.	Solar Unit Box

Visually check for cracks at welds, battery fixings and fixing down fasteners.  If suspect, remove fittings from the damaged area, clean and inspect using an inspection light and magnifying glass.  Undertake dye penetrant testing if necessary.

Check for excessive corrosion, especially at crevices and at contact surfaces with dissimilar metals.  If excessive corrosion exists, remove fittings from the corroded area and clean away salt and corrosion deposits.

Check the door quick release fastening studs in the solar module and springs in the box.  Lubricate the fasteners and lightly smear the box face in way of the door seal with petroleum jelly.

9.	Reassembly of Solar Unit

Where solar modules, a regulator assembly or other fittings are to be fitted to the solar unit, apply zinc chromate jointing compound between the contact surfaces between stainless steel and aluminium and assemble whilst the jointing compound is wet.

Fit the required number of batteries and solar modules, regulator (if this has been removed) and lantern to suit the buoy station.

Check all fixings are secure and fastenings tight.

Ensure all electrical cables are adequately supported and tied in place.

Check that dummy plugs and sockets are fitted.

Lightly smear all electrical plugs with silicone grease before making electrical connections.  Check that plugs are fully pushed home in sockets.

Check that battery connections are tight and smear with silicone grease.

10.	Final Checks

Check that the correct fasteners, insulation bushes and gaskets are used / available for fitting the unit to the buoy.

Finally, check that the navigation light flashes to the expected intensity and correct character.