# Technical Committee TC 4-47

Application of LEDs in Transport Signalling and Lighting

Road Lighting

Terms of Reference:

To review the application and methods of measurement of LED systems for transport lighting and signalling as far as they affect visual performance.

To prepare a Technical report which includes the findings of the review and recommendations for the characteristics and measurement of LED signals and lighting.

## 1.	Introduction

The scope of this Report is restricted to the application of LEDs in Road Lighting.  The use of LEDs in road lighting is just started to be implemented.  The implementation has been measured as there are many issues concerning LED luminaires in the road environment that have yet to be resolved.

The promise of LED-based luminaires is great with advantages in low energy consumption, long life, low maintenance, emitting white light and ease of being implemented into an adaptive road lighting scheme.

Some of the more major issues of concern are the lack of long term experience of LED luminaires in the road environment, the likely failure modes, the lighting quality of an LED road lighting scheme and how to specify it, how to evaluate the cost/benefit of an LED scheme, the test and measurement requirements for an LED luminaire and what monitoring is needed of the performance of an LED road lighting installation over time.

The efficacy of LEDs themselves is about 65lm/W which is similar to the efficacy of currently used discharge lamps.   However, this will improve markedly in the very near future. The highest efficacies are for the highest colour temperature sources and these will appear a bluish white.  The increase in the blue content of these high CCT LED sources over that of commonly used white light sources is of great concern to astronomers.  To alleviate this problem, LED sources with intermediate CCT could be used.

The LED-based luminaire has, to date, a flat panel or series of flat panels comprising an array of LEDs with electronic control gear, the LED driver.  There will be losses in the drivers which should be taken into account in any comparison with other lamp-based luminaries.  The flux output from LED luminaires decreases as their junction temperature increases and so efficient thermal management of the LED component is essential for maintaining maximum output and long life.

To date, LED luminaires have been of the cut-off type and, as a consequence, spacings have been less than the long spacings commonly used in road lighting.  The spacings may be increased by increasing the luminaire upcast but this might result in increased discomfort glare from the small, high luminance LED luminaires.

The replacement of LED luminaires is also problematic.  The development of LEDs and of LED luminaires is so rapid that a particular model will be obsolete at the end of its service life.  This may mean that the luminaire would have to be replaced with a newer equivalent model and this should be taken into account when arriving at a cost of the LED installation over its expected life of 20 years.

In order to achieve the widespread use of LEDs in road lighting and to gain the promised benefits, these issues must be addressed and resolved.  The purpose of this Report is to detail the issues of concern so that manufacturers can produce effective luminaires, and specifiers and users have the knowledge of the required attributes of an effective and cost beneficial road lighting design.

It is the stance of this Report that the widespread use of LED road lighting is to be vigorously pursued.  To help to make this happen, it is imperative to point out the hurdles that are currently preventing its widespread use.  By detailing the concerns of users, road lighting designers and visual ergonomists, these can be addressed urgently so that the potential of LED road lighting is realised in a short space of time.

The pace of LED development, both in the LED component itself and in the applications, is rapid and the conclusions of this Report may become obsolete very quickly.  Consequently, we put in a caveat that the content of this Report is based on the data, research and information that is current up to the date of publication.

## 2.	The Effect of Road Lighting on Astronomical Observations

3.	Optical Performance and Lighting Quality

LED Luminaires comprise of arrays of individual LEDs each of which have a very high luminance.  As a consequence disability glare may become a problem especially if the luminaire has a large degree of upcast to obtain longer spacings.  The currently used method of calculation for evaluating Threshold Increment needs to be confirmed as being appropriate for LED luminaires.

Luminous intensity emitted above the horizontal is wasted energy and some restriction on the upward light is necessary for a road lighting luminaire whatever its light source.  For astronomers light emitted close to the horizontal is a major problem and especially for light sources that have a significant proportion of blue light.  This problem is apparent over very large distances.  Many LED luminaires have high CCT LEDs as their source as these have a higher efficacy than the lower CCT LEDs.  In regions around astronomical observatories, LEDs with lower CCTs should be used or else some restriction on the power per unit wavelength over certain wavelengths.

The prevalent technology in order to create white light from LEDs is the phosphor conversion, using blue LEDs covered by a phosphorous coating. The blue light of the LED is partially converted to yellow light finally resulting in the emission of white light. The CCT is controlled by the type and the thickness of the phosphorous layer.

Even though LEDs with a high CCT (and a low conversion rate) are more efficient than warm white LEDs with a lower CCT the choice of the CCT is dominated by cultural and aesthetic preferences.

This degree of freedom offers the opportunity to use different colour temperatures in dedicated traffic areas in order to attract attention to sensitive areas like junctions or pedestrian crossings. On the other hand an inconsistent choice of CCTs could lead to irritations wherever roads with different CCTs join due to different light sources or different LED types. Lighting designers need to take this into consideration, lighting master plans should specify the CCT in order to ensure a consistent appearance.

## 4.	Road Lighting Installation Requirements

All the developed countries and many others have road lighting specifications that are thorough and well-proven over many decades of experience.  This section considers how these specifications apply to LED luminaires and whether there is any need for change or additional requirements so that LED luminaires can confidently be included as alternatives to the currently used light sources.

4.1	Luminous Intensity Distribution

Experience with LED luminaires has shown that their intensity distribution is more likened to a cut-off distribution with a hard edge.  The hard edge coming at about 60 degrees from the vertical.  While this can give extremely uniform horizontal illuminance at pavement level, the vertical illuminance at, say, one metre above the ground can be very non-uniform for realistic spacings.  This has significant shortcomings for object visibility both for Category P roads and for Category V roads.  The luminance distribution of Category V luminaires has it peak luminous intensity at around 70-75 degree for the downward vertical.  This provides economical spacing and adequate vertical illuminance.

It may be beneficial that, when specifying light technical parameters (LTPs) for road lighting, the uniformity of vertical illuminance and its minimum value at an appropriate height is included.

For Category V roads, the average luminance and luminance distribution are the major light technical parameters.  The failure mode in one type of design of LED luminaires can have a significant adverse affect on the luminance distribution.  In some designs, different arrays of LEDs illuminate different areas of the pavement surface, consequently a failure of one array in the luminaire will cause a marked change in the luminance uniformity.

Figure: left: free form lenses, one lens provides the complete distribution pattern; right: oriented LED optics, different LEDs illuminate different areas

4.2	Lifetime

The lifetime claims for LED luminaires are very long and around 50,000 hours.  This is somewhat longer than 10 years.  However, their failure is not usually catastrophic but a gradual degradation.  It then becomes an issue as to when the installation no longer complies with the relevant Standard or Regulation.  Either the responsible authority assumes the lifetime data given by the luminaire manufacturer is correct and institutes an appropriate maintenance cycle or the authority monitors the performance of the installation at regular intervals.  The performance measure is likely to be a relative one, in that horizontal illuminance over a grid could be measured at a convenient height at the time of installation and then the same grid measured and compared on an annual basis.

4.3	Maintenance

There are several possible maintenance issues which may not be identified yet as the likely failure modes of an LED luminaire over the long term are not known. .  If an LED luminaire has a failure in one panel of its array, then it may be possible to replace this.  If this replacement happens after several years, the new panel may have a noticeably higher luminance which could effect the uniformity of the luminance distribution.

The pace of LED luminaire development is rapid.  This is very encouraging in that early shortcomings are being designed out at a fast rate.  However, if the LED luminaires in an installation last the whole of their service life (10 years), the original model may not be available.  This would mean the complete replacement of the luminaire after only about half the currently expected lifetime of an installation.  This possibility must be taken into when making cost comparisons between discharge and LED based luminaires.

The use of standardized LED modules with specified interfaces may be one way in order to overcome this tradeoff. Such standardized modules could follow the evolution of LED technology by maintaining the initial interfaces, which determine the integration of such module into the luminaire. This would ensure the availability of such modules from different suppliers even after the end of the module’s service life.

The design of installations uses a maintenance factor such that the initial values of the LTPs are higher than that specified so the installation still complies at the end of the maintenance cycle.  The maintenance factor usually varies around 0.7-0.8.  The manufacturers of LED luminaires claim that the service life of the luminaire is reached when the output falls to 0.7 of its initial value, that is the maintenance cycle would be 10 years.  This may not be palatable to the road authorities who would prefer are more frequent monitoring of the installation with regard to spot failures and soiling etc.  Perhaps an enlarged definition of maintenance factor is needed to include LED luminaires.

4.4	Energy Efficacy

Certainly, one of the most attractive characteristics of LED luminaires is their low energy consumption and the efficacy of individual white LEDs is equivalent to the commonly used discharge sources in road lighting.  It is difficult to compare the efficacies of LED luminaires with standard discharge source luminaires.  The current way of comparing the optical efficiency of standard road lighting luminaires is through their Light Output Ratio (LOR) which is the ratio of the luminaire emitted flux to the light source emitted flux.  However, an LED luminaire does not have a single source whose luminous flux can be measured.  Consequently, the luminous intensity distribution of the LED luminaire must be measured absolutely and its emitted flux calculated as an absolute value.  The efficacy of the luminaire can then be found in lm/W.

However, what is of primary interest is the energy efficacy of the lighting installation as a whole and in particular the effective lumens/W.  Here, the effective lumens is the flux that falls on the area of interest the road surface and immediate surrounds for Category V roads or the property line to property line area for Category P roads.  Flux that falls outside these areas of calculation are wasted and should not be included when calculating the energy efficacy of the road lighting installation.

Another advantage of LED luminaires is their ease of being implemented in adaptive lighting schemes.  This has the benefit that lighting levels may be reduced or increased depending on traffic density, time of day, weather conditions etc.  This will have major benefits to the total energy consumed over the lifetime of the installation. There is a benefit also with adaptive lighting in that the installation can meet the LTP specifications at the start of the maintenance cycle at a lower input power (as opposed to having to start well above the specified LTPs to allow for degradation during the maintenance cycle) and then increase the input power as the LEDs degrade over the maintenance cycle.  This both saves energy and increases the lifetime of the LED luminaire.

4.5	Specification of Photometric Parameters

Most countries will have their own Standards and Regulations that specify what the levels of LTPs that make a complying road lighting installation.  These will need to be reviewed in the light of the new technology.  In particular, object visibility is determined primarily by the magnitude and uniformity of vertical illumination at heights from ground level up to about 1.5m.

One advantage of LED luminaires is that there is a choice of the colour of the white light.  This could be useful in providing greater conspicuity of intersections by using a high CCT LED luminaire to light the intersection within a HPS installation.

5.	Measurement of LED Luminaires

There are several differences in the technology of LED luminaires that require different measurement procedures from that of luminaires with discharge sources.  As mentioned above, the measurement has to be carried out as an absolute measurement.  This means the distribution goniophotometer has to be calibrated absolutely and the photodetector has to have a well-corrected V() response.

It is well-known that the luminous output of LEDs decreases as the junction temperature increases, consequently LED luminaires have substantial thermal management systems, which in some cases results in a heavy luminaire.  Existing lighting columns cannot tolerate luminaires greater than 25kg and there is a limitation in Belgium of 20kg as a safety requirement for the personnel who mount and de-mount the luminaires.  Because of the temperature sensitivity of LEDs, it is necessary to specify an ambient temperature for the measurements and this would usually be 25oC.

It is common for the LED drivers to power the LEDs with pulse-width modulation (PWM) at quite high frequencies.  Without specialised equipment it is not possible to determine the peak current and standard wattmeters will not give accurate r.m.s values for the input watts to the LEDs.  The relationship between the peak current of a PWM driver and LED lifetime is not definite at present.  More information here would be helpful.  The advantage of PWM is that it is a simple and linear method for dimming the LED luminaires.

The installation geometry should be mirrored in the test laboratory.  Differences in upcast angle will change the change the luminous intensity distribution not only because the maximum intensity is pointed differently but because the thermal equilibrium conditions will be different.

If the colour of the LED luminaire is needed or a measurement of its CRI, the spectral power distribution should be measured at intervals of no greater than 5nm and with a bandwidth no greater than 5nm.

There are some published test methods for LED luminaires in general published by the IESNA as LM79/80 and by the ECE as EN13032.

6.	Assessing the Cost of LED-based Road Lighting Installations

Two advantages of adopting LED-based luminaires for road lighting installations is that they promise a low energy consumption and have a long life.  When comparing different technologies to populate a road lighting installation, the benefits and costs should be evaluated over the whole of life of the installation that has normally been taken to be 20 years.

The costs should include installation, energy consumption, maintenance, replacement and disposal costs.  The costs for LED-based luminaires may need to include monitoring costs as the end of service life is not as definite as it is with the commonly used discharge lamps (HPS, MH and HPM).

7.	Environmental Effects

Long-term experience with LED road lighting luminaires is now only just becoming available and only for the first wave of LED luminaires.  The wide range of environmental effects are yet to be quantified.  The adverse effects of temperature are well-known but the effects of solar loading and low temperature effects such as freezing of the optical surfaces are yet to be fully experienced.

Other environmental effects that will need to be assessed are the tolerance of LED luminaires to vibration and humidity and whether the design of the luminaire will encourage or discourage soiling.

8.	Production Issues

The quality control of traditional road lighting luminaires is well-known and installations can be designed with a good deal of confidence that the installed design will closely match the planned design.  There is little experience of the manufacturing tolerances of LED luminaires, indeed the rapid development of different models means that what information is gleaned for one model may not transfer to the later models.  Until large numbers of a settled design come into use, these questions will remain.

As LEDs do not emit IR radiation, all heat losses (~75 % of the consumed electric power) need to be dissipated through the PCB supporting the LED. As LED lifetime and efficacy significantly depend on the temperature of the n-p junction, efficient cooling of the LEDs is requisite. Hence thermal design plays a crucial role for the design of efficient LED luminaires.

This limits the choice of materials for the housing. Injected plastic parts will probably disappear as the thermal conductivity is too low.

If they are well designed, exterior fins on the top of the luminaire are an important design element in order to ensure efficient heat dissipation from the luminaire to the ambiance. However the risk that this exchange can be limited by deposits caused by birds’ excrements, leaves or dust, which will not be rinsed by the rain. As this might lead to a lifetime diminution, standardized tests, for the design evaluation, which simulate these effects, need to be established.

At present, there is also the question of the availability of the element Indium which is crucial for the manufacture of LEDs appropriate for road lighting.  This scarcity brings to the fore whether LEDs can be recycled and what would be the method and cost of disposal.