IALA Guideline No. ####

On

Design of Daymarks for Aids to Navigation

Edition 0.1

December 2011

AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter10, rue Schnapperdes Gaudines, 78100

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  contact@iala-aism.org       Internet:  www.iala-aism.org

This document has been reformatted, to conform to the current IALA template

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

Document Revisions	1

Table of Contents	3

Index of Tables	5

Index of Figures	5

Daymarks for Aids to Navigation	6

1	Introduction	6

2	SCOPE	6

3	Definitions	6

4	Basic Principles	6

4.1	Degrees of Perception	7

4.2	Information Process	7

4.3	Daymark – the source of the information	7

4.3.1	Viewing Distance	7

4.3.2	Luminance	8

4.3.3	Surface Colour	8

4.3.4	Size	8

4.3.5	Profile	8

4.3.6	Lettering (only general comments)	9

4.3.7	Structures (solid vs. latticed or open – the “partially transparent” case)	9

4.3.8	Combination of Colours	9

4.4	Viewing Conditions – the medium of transmission	9

4.4.1	Visibility and Distance	9

4.4.2	Background	9

4.4.3	Daylight Incidence and diffusion (weather conditions and sun position; reflections on objects of different shapes)	9

4.5	Observer – the receiver of the information	9

4.5.1	Eye Resolution (subtense angles)	9

4.5.2	Observation Contrast	9

4.5.3	Colour Recognition	9

4.5.4	Knowledge and Experience	9

5	DESIGN METHODOLOGY	9

5.1	Required range	9

5.1.1	Height above sea level	10

5.1.2	Height of the mark	10

5.2	Orientation of Daymark	10

5.2.1	Directional vs. Omni directional Daymarks (consider flat panels)	10

5.3	Minimum Dimensions of Daymarks	10

5.3.1	Different shapes within the IALA MBS	10

5.3.2	Numbers and Letters (make reference here to appropriate standards)	10

5.4	Contrast	10

5.4.1	Luminance Factor	10

5.4.2	Photometric Contrast	10

5.4.3	Observation Contrast	10

5.5	Colours	10

5.5.1	Colour Contrast	10

5.5.2	Practical Knowledge	10

5.6	Combination of Colours (color vs. shape in multicoloured aids)	10

6	standard procedure (depending on data coming from item 6)	10

6.1	Steps in Designing a Daymark	10

6.2	Daymark Range Table	11

7	examples – case studies	11

8	ANNEXES	11

9	Limits of visual Perception of daymarks	13

9.1	General Calculation based on observation contrast	13

9.1.1	Contrast at short distance	13

9.1.2	Contrast with the sky as background	14

9.1.3	Contrast with other backgrounds, but using the sky as reference	14

9.1.4	Approximate values for contrast Cc(o) (sky as background ) and Cs(o) (other backgrounds)	14

9.1.5	Contrast at distance (Observation Contrast)	15

9.2	Visibility law of Daymark	15

9.2.1	Visibility conditions	16

9.2.2	General comment on limits of visual range	16

9.2.3	Visibility distance	16

9.3	Identification of colour	16

10	Factors that reduce visual range	17

10.1	Factors that reduce detection	17

10.2	Service factors	17

11	Use of Multiple Marks	18

12	Climate effects	18

12.1	Environmental limitations	18

13	References	20

ANNEX A	Practical examples	21

ANNEX B	German input to the workshop	22

Index of Tables

Table 1	Geographical Range in Nautical Miles	7

Table 2	Properties of colours	12

Table 3	Title required	30

Index of Figures

Figure 1	Conspicuity difference between a non transparent daymark and transparent daymark	13

Figure 2	Title requred	23

Figure 3	Title required	25

Figure 4	Title required	25

Figure 5	Title required	26

Figure 6	Title required	27

Figure 7	Title required	28

Figure 8	Title required	29

Figure 9	Title required	30

Daymarks for Aids to Navigation

# Introduction

Existing IALA documents provide information on the proportions of daymarks but only limited information is given on the range at which daymarks can be effectively detected, recognised and identification.

This guideline provides a general informative overview of the main factors affecting the visual performance of a daymark as an aid to navigation and how to determine the range of that daymark.

# SCOPE

The scope of this document will cover all those AtoN as defined in the IALA MBS [1]. This guideline does not cover the use of signs, lettering and lights as aids to navigation during daytime.  However, when designing visual AtoN consideration should be given to the negative impact of signs, lettering and other features of the overall AtoN design not covered by this document.

# Definitions

| Daymark | IALA Dictionary 2-6-030 For the purpose of this guideline, a daymark includes shape, colour and color combination (as defined in MBS) as well as any identifying markings. |
| --- | --- |
| Transparent daymark | #### See 4.2.2.3 (rev1 document) - Effect of partially transparent daymarks |
| Photometric contrast | Contrast of a daymark against its background without taking into account atmospheric extinction. |
| Observation contrast | Contrast of a daymark against its background taking into account atmospheric extinction. |
| Daymark Range | The maximum distance at which the visual task of a daymark can be accomplished (identification). |

# Basic Principles

The visual task is to provide a daymark, according to the IALA Maritime Buoyage System, that can be easily identified at a given distance, against a given or prevailing background. This means that the mariner identifies the shape, the colour and colour combination.

The range of the daymark should comparable to the night range of the light. However, in general, the range of the daymark is usually less than the light range.

A Daymark is defined by size, luminance, colour, shape and height above sea level.  To estimate the visual performance the background behind the daymark has to be taken into account.  Background luminance and colour depend on geographical and meteorological conditions.  Reference background depends on height of the vessel’s bridge and direction of observation.

These parameters depend on the following:

- geographical location

- atmospheric conditions (this is characterized by meteorological visibility)

- observer behaviour (physical and psychological factors)

## Degrees of Perception

There are three degrees of perception of daymarks:

- Detection: The observer is aware of an object.

- Recognition: The observer is aware that the object is an AtoN.

- Identification: The observer is aware which AtoN the object is.

## Information Process

When considering the perception process of daymarks, we have;

- Something which creates	the signal Source (interaction of sunlight with the
object);

- Something which conducts 	the signal Media (the atmosphere);

- Something which receives 	the signal Sensor (the observers eye).

To study this perception process is to define all the parameters which characterise these 3 factors. These definitions will be described in the following paragraphs.

## Daymark – the source of the information

### Viewing Distance

The minimum height hb,min of the lowest part of the daymark is given by the formula for geographical range Rg.

Rg=2.03×ho+hb,min		hb,min=Rg2.03−ho2

Where:

Rg = geographical range (nautical miles)

ho = elevation of observer’s eye (metres)

hb = elevation of the base of the daymark (metres)

The constant is to allow for the curvature of the Earth and Atmospheric factors [8]

If there are actual or potential obstructions between the observer and the daymark, then the height may have to be modified to allow for this obstruction.

Error: Reference source not found shows some examples of ranges that can be achieved given the elevation of the observer and the elevation of the base of the Daymark being observed.

- Geographical Range in Nautical Miles

| Observer’s eye height (metres) | Elevation of Daymark (metres) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 10 | 15 | 20 |
| 5 | 4.5 | 6.4 | 7.9 | 9.1 | 10.2 | 14.4 | 17.6 | 20.3 |
| 10 | 6.4 | 9.1 | 11.1 | 12.8 | 14.4 | 20.3 | 24.9 | 28.7 |
| 20 | 9.1 | 12.8 | 15.7 | 18.2 | 20.3 | 28.7 | 35.2 | 40.6 |
| 30 | 11.1 | 15.7 | 19.3 | 22.2 | 24.9 | 35.2 | 43.1 | 49.7 |
| 40 |  |  |  |  |  |  |  |  |
| 50 |  |  |  |  |  |  |  |  |
| 60 |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  |  |  |  |  |

### Luminance

The luminance (‘brightness’) of the daymark depends on its profile, its colour, the illumination of the object and the orientation of the surface elements. Parts of the daymark may show different luminance caused by different orientation or shadowing.

### Surface Colour

The Surface Colour should fulfil the specifications of IALA Recommendation On Surface Colours in order to be used according to IALA MBS.  These Specifications leave freedom to adjust the colour to a specific viewing condition.

Surface Colour can be produced by

- Films, sheeting

- Paint

- Plastic, coloured throughout

#### Aging (deterioration of colours of buoy floats due to marine growth; UV deterioration of pigments)

### Size

Formula and figure to calculate the angular subtense from size and distance.

- Old navguide 1993 2ed (860 * height)

- Discussion paper for IALA ENG15 USCG, 07/31/26

- Canadian Coast Guard Directive

- Leading light program (IALA xls-Chart)

- German Documents (1980-docs)

- Perimeter calculations for equivalent angular subtense

### Profile

The daymark has a 3-dimensional profile. When viewing the daymark, the user perceives a 2-dimensional shape, which is a projection of the 3-dimensional profile.

The 3-dimensional construction () of the daymark profile is of significant influence on the 2-dimensional shape seen from different directions.

For the design of the daymark it is important to know from which direction it shall be viewed.

Flat mark: view only in a small horizontal sector (e.g. leading marks)

A cylindrical, conical and spherical profile (rotational symmetry) is best option  for omnidirectional marks (360° in horizon)

Crossed plates may be less effective than profiles with symmetry (shadowing effects)

Provide figures and further explanations

### Lettering (only general comments)

### Structures (solid vs. latticed or open – the “partially transparent” case)

#### Structural obstructions (example: solar panels appear black, disturbing identification of aids, especially cardinal marks)

### Combination of Colours

## Viewing Conditions – the medium of transmission

### Visibility and Distance

### Background

### Daylight Incidence and diffusion (weather conditions and sun position; reflections on objects of different shapes)

## Observer – the receiver of the information

### Eye Resolution (subtense angles)

Task:

find suitable values for the eye resolution.

find suitable values for the required angular subtense for the recognition of shapes, for colour detection, for the separation of objects (e.g. excel chart of leading lines)

check the sources

### Observation Contrast

Task:

Define contrast in this section

Provide information on required contrast (V=0,05 is valid only for large objects)

### Colour Recognition

Recognition depends on colour difference

Define colour difference: Mc Adam’s ellipse

Required colour difference

Old remark:

Recognition (identification) depends on saturation of colour (less important fading effect), depends on luminance difference of light issued of daymark and background (observation contrast).

### Knowledge and Experience

# DESIGN METHODOLOGY

## Required range

### Height above sea level

### Height of the mark

## Orientation of Daymark

### Directional vs. Omni directional Daymarks (consider flat panels)

## Minimum Dimensions of Daymarks

### Different shapes within the IALA MBS

#### Conical, Cylindrical and Spherical

#### Cross and “X”

### Numbers and Letters (make reference here to appropriate standards)

## Contrast

### Luminance Factor

### Photometric Contrast

### Observation Contrast

## Colours

### Colour Contrast

### Practical Knowledge

## Combination of Colours (color vs. shape in multicoloured aids)

# standard procedure (depending on data coming from item 6)

## Steps in Designing a Daymark

STANDARD METHOD (PROPOSAL FROM XAVIER) (Formerly item 2.2 – to be revised).

When designing a daymark: Methodology for simple case (to be revised intersessionally)

- First determine the required range (service distance i.e. at what distance should daymark be detected);

- Type of mark (cardinal, lateral) and select the colour and shape of the daymark;

- Conditions of utilisation of daymark (height of vessel bridge, direction of observations);

- Calculate photometric contrast (see formula annexe 2);

- Determine the fraction of time, the service is to be provided (policy matter - 95% could be chosen);

- Determine the available Meteorological visibility that prevails (95%) of the time in the service area of the aid (remember that Sea state also affects service and is not taken into account in the equations.);

- Calculate contrast as a function of distance, using selected meteorological visibility (see formula ANNEX B);

- Calculate the required (equivalent area) size of daymark (see formula ANNEX B);

- Select the appropriate shape of the daymark while ensuring that the size requirements are met.

The results must be adjusted while taking into account the “other” factors mentioned later in this document.

## Daymark Range Table

# examples – case studies

# ANNEXES

Detection and identification of daymarks

- Properties of colours

| Colour | Detection | Shape identification | Colour identification |
| --- | --- | --- | --- |
| Black | Good (high contrast value) if viewed against the sky. Bad if viewed against a dark background such as a dark coastline. In case of foggy conditions, a dark background becomes gray at distance, hence a black object is seen better in foggy conditions. | Same properties than for visibility. | Good. The colour does not change with different illumination conditions. |
| White | Good if viewed in conditions with clear sky, illuminated directly or from the side against any type of background. Bad for cloudy sky when viewed against the sky or sea as background. In case of foggy conditions, background becomes gray, hence a white object presents a bad colour difference with background. | Same properties than for detection. | Good. |
| Ordinary Red | Good colour difference with sky and coast. | Medium shape identification. | Very good colour identification. |
| Fluorescent red | Very good colour difference with sky and coast. | Low shape identification. | Very good colour identification. |
| Green | Medium to low detection because of meteorological and physiological effect. | Same properties than for detection. | Low colour recognition Difficult identification of colour. Problem with vegetation on the coast. If object is viewed at large distance (appears as a point source), it can be difficult to distinguish between green and blue. |
| Fluorescent green | Good colour difference with background. | Low shape identification. | Good colour identification. |
| Yellow | Good if viewed in conditions with clear sky. Bad for cloudy sky when viewed against the sky or sea as background. In case of foggy conditions, background becomes grey, hence a white object presents a bad colour difference with background. | Same properties than for detection. | Good if viewed in conditions with clear sky. In cloudy sky or foggy conditions bad colour identification. Yellow seems to white. Good colour identification if just near dark colour |
| Blue | It seems to be not very good (meteorological and physiological effects) but need to be confirmed. | Same properties than for detection. | Risk of confusion with sea and sky in sunshine. |

For all colours except black: when viewed in the direction of the sun, the object may appear to be black.

Luminance of fluorescent colours is greater than that of ordinary colours since a part of the short wavelength incident light is transformed to lower wavelengths contributing to the intended colour (blue to red).  More important problem is lifetime of fluorescent colour in comparison of ordinary colour.

#### Effect of partially transparent daymarks

If parts of the daymark surface is transparent (holes or gaps), the contrast decreases, and thus the range reduced.

There is at present no verified method available for contrast correction due to partial transparency, but a rough estimate can be achieved by taking a medium contrast value.

- Conspicuity difference between a non transparent daymark and transparent daymark

# Limits of visual Perception of daymarks

Unfortunately there is no theory for the identification of colour, which combines the influence of the size, luminosity and saturation of the coloured object, and that can be used to calculate.

Calculation based on observation contrast can be used to define the minimum distance of recognition of a daymark. Choice of specific colours can be used to conspicuity of the daymark.

## General Calculation based on observation contrast

### Contrast at short distance

At short distance, the effect of atmospheric extinction can be ignored, thus…

If we have O an object, F uniform background with a significant size compared to the object; Lo (object) and Lf (Background) luminance.

Contrast at short distance C(o) is:

C(0)=Lo−LfLf

Contrast characterized the possibility of the eye to distinguish an object from a background where the object is projected.

### Contrast with the sky as background

With Cc(o) contrast with the sky and Lc luminance of the sky.

Cc(0)=Lo−LcLc

### Contrast with other backgrounds, but using the sky as reference

When the object is projected on a different background than the sky, we have to keep sky luminance for reference because eye of the observer is adapted to the sky.

With Ls the luminance of the ground, contrast relative to the sky Cs(o) is :

Cs(0)=Lo−LsLc

### Approximate values for contrast Cc(o) (sky as background ) and Cs(o) (other backgrounds)

Now we have to define the sky

We consider two different cases: a uniform cloudy sky and a clear sky.  In most of cases, the reality is just between those 2 cases.

We consider that the daymark has a single colour and is in a vertical plane.

#### Case 1:  Uniform cloudy sky

Ls and Lo are defined empirically by:

Ls=2.34β1Lc

Lo=3β2Lc(0.308+0.389β)

With a ground in a horizontal plane.

Where:

Lc =	luminance just near horizon.  In case of uniform cloudy sky, horizon luminance is 1/3 zenith luminance

 =	illuminance factor of the object (0>

 =	illuminance factor of ground (0.6 for white sand, 1 for snow)

#### Case 2: Clear sky

Luminance Lc just near horizon is very dependant on azimuth in relation to the sun, and height of the sun (zenith distance of sun).

Lc is between 180Cd/m2 (Az 100° when sun is near horizon the evening) and 19000Cd/m2 (Az 0°, zenith distance 20°).  Medium value, in France for Lc is 5000Cd/m2.

For all contrast calculi we have Lo(0) object illuminated by the sky only, Ls(1) luminance of a white ground and ground luminance is Ls = 2 Ls(1).

For an object with the sky as background:

Lo=Lo(0)+12Ls

If the object is not white we have to multiply Lo by 

For an object with a background other than the sky

Lo−Ls=Lo(0)−12Ls

If object is not white we have

Lo−Ls=βLo(0)+β−22Ls

In France :

Ls is between 300Cd/m2 (dark ground) and 10000Cd/m2 (white ground with sun at zenith).

Lo is between 1Cd/m2 and 32000Cd/m2

Purpose of the above is to arrive at order of magnitude values for luminance values

Thus

Contrast with the sky as background:

Cc(o)=1−β1Lo(0)Lc+12LsLc

Contrast with background other than the sky, but using the sky as reference since the ambient luminance level is defined by the sky:

Cc(o)=β1Lo(0)Lc+1−2β1Ls2Lc

With CIE model of the sky we can obtain Lc and Lo(0) for a white object. Like that we can calculate Lo(0) for coloured object and Ls (CIE110 Spatial distribution of daylight - Luminance distribution of various reference skies).

IALA Recommendation for surface colours (E-108) contains required values for reflection factor 

### Contrast at distance (Observation Contrast)

Atmospheric absorption, defined by meteorological visibility, has an action about conspicuity.

We admit that can be characterized by a modification of contrast of the object projected on background.

Contrast with the sky at distance X

Cc(X)=0.05XVCc(0)

Contrast relative to the sky at distance X (Background other than then sky)

## Visibility law of Daymark

### Visibility conditions

We define d as the characteristic length of daymark (size of a square with an equivalent area of the daymark).

Approximation of solid angle under daymark is seen.

The solid angle of subtence can be approximated as: d2X2

Experience says that for a daymark visible at distance X we have to satisfy 2 conditions:

- Contrast at distance X have to be greater than 0.05

So C(0)=0.05XVC(0)>0.05

- Solid angle under daymark is seen have to be more important than a value which depend on X

So d2X2>0.038C(X) (with d in meter and X in kilometre)

At short distances, the size of the object is not as important as the contrast.

At long distances, we have to take into account the solid angle of the object to determine POD

### General comment on limits of visual range

Since C(X) contrast has to be better than 0.05

So d2X2≥0.038C(X)=0.76

Its case solid angle which daymark is seen is superior or equal to 1/1000 St

When C(0) is equal to 1

C(0)=0.05XVC(0)>0.05 => X<V

It’s why you can’t see a daymark a distance of observation more important of meteorological visibility distance.

### Visibility distance

#### Viewing at long distances

If daymark is seen under a solide angle less than 1/1000 St (d/X<1) so

C(0)(0.05)XVd2X2=0.038 (with d in meter and X in kilometre)

#### Viewing at short distances

If daymark is seen under a solide angle more than 1/1000 St (d/X>1) so

C(0)(0.05)XV=0.05

Distance of visibility depends on contrast and not size of daymark.

## Identification of colour

There are formulae for the threshold of the luminance contrast for small objects.  However the concept of contrast can only be applied in a grey world.

When colour comes into consideration the contrast is not the right quantity to describe the visual performance.

Example: A red area with luminance 1 against a background with luminance 1 has a contrast of zero, but can appear conspicuous.

The identification of a colour does not depend on the luminance contrast.

The identification of colour depends on the luminosity and the saturation of the colour (The background at sea is assumed to be grey).

Example: Assume a coloured area against a grey background.  When the coloured area is very dark, you cannot identify the colour.  Because of the luminance contrast you will see the area very well.  However when you do not have a specific shape of the area, it will not show any information to the observer (perception step 1) except the information 'there is something'.  When the colour gets brighter you will get the colour information (perception step 2).  For large objects you get the information even if contrast is negative, for small objects it might be necessary to have positive contrast to identify the colour.

The luminosity of the colour can be described by the ratio L2/L1.

The saturation can be described by DIN 6164 .There are alternative ways.

Unfortunately we have no theory for the identification of colour, which combines the influence of the size, luminosity and saturation of the coloured object, and that can be used to calculate MDR.

In general the distance MDR increases with the luminosity and the saturation of the colour.

In consequence fluorescent colours have a greater MDR.

# Factors that reduce visual range

## Factors that reduce detection

- For size of the topmarks the considerations are presented in NAVGUIDE 2006 edition, at paragraph 3.1.13.6.2.

- Complex background and meteorological parameters (sunset or sundown, atmospheric transmissivity, ...) can modify detection with time

- For dimensioning the identification markings it is necessary to consider the requirements posed by the authority and the importance of it for the mariner.

Example:

  - An inland special mark signalling a danger, where his identification gives no extra information to the mariners does need large characters.

  - A special mark identifying a forbidden area, with a no entry sign, this marking should be visible in an adequate distance.

- The wave height / frequency in open water will have detrimental effects in range due to the following factors:

  - Variation in the visible area of the daymark, due to the pitching of the floating aid;

  - Changes in the shape due to the rolling effect;

  - Reduction in the exposure time of the floating aids due to the relative position with the observer, which can be obstructed by the crest of the wave.

## Service factors

- Dirt in the form of green slime, bird droppings, green mould will make the visible identification of colour difficult or impossible.  Where the top of the floatation bowl forms part of the daymark, sufficient freeboard should be provided to ensure that the top of the bowl is not regularly submerged encouraging development of green slime.  In critical areas it may be necessary to frequently wash the buoy.

- Fading is a result of the attack by ultra violet (UV) light present in sunlight and buoy daymarks are particularly exposed to this type of degradation, having no form of shade.  Selection of the type of paint system or plastic material should be made for maximum UV stability.  Service providers should carry out periodic tests to confirm that the daymark colour remains within IALA recommendation.  It may be necessary to increase the frequency of painting in order to ensure that the daymark colour maintains its performance within IALA recommendation.

- Theft, interference and tying up to buoys can have serious effects on the provision of buoy systems.  The use of security locking systems can inhibit casual theft but absolute prevention of theft is impossible.  Deliberate malicious interference can be dealt with through the legal system if the perpetrator can be identified.  The use of buoys by fishing vessels as a temporary mooring should be discouraged through public information system and regular contact with the fishing community and local authorities.  A remote monitoring system can provide early warning to the Service Provider in the event of damage to the electrical systems.

# Use of Multiple Marks

When determining the dimension of the daymark it is necessary to evaluate the purpose of the floating aid and the relationship to other aids to navigation and observable features;

Examples:

- Marking restricted / dangerous areas (one large daymark or two or more smaller marks at the area limits).

- Using a daymark combined with other AtoN (ex: RACON).

# Climate effects

## Environmental limitations

Local environmental conditions can give rise to the need for special design of daymarks which may not conform to the general guidance in this document.  Such conditions include drifting ice, debris in rivers, buoys in breaking wave conditions.

Buoys located in areas exposed to annual drifting ice, possibly of considerable thickness, that can destroy equipment mounted on the buoy body.  Buoys in such areas must generally be designed with a conical body with a minimum of projections and be completely sealed.  Use of additional daymark boards or topmark may not be feasible.  Consequently the use of can shape for lateral buoys is not always possible.

Debris in rivers, such as trees, logs and even whole floating islands can inflict damage to a buoy similar to that in drifting ice.

Where a buoy system is deployed in areas of breaking waves or drying out during low tide, the buoy design must be adapted to ensure survivability of the buoy and its equipment and the design may not conform to the general format.

Bird droppings can seriously affect the performance of buoy systems as well as obscuring the colour of the daymark.  Droppings on lantern or solar panels will cause reduced performance or failure of the light.  The corrosive effects of bird droppings or acid rain can cause damage leading to reduced life or premature failure.  Daymark design must therefore be such as to avoid providing any natural bird resting sites.  The use of long flexible spikes on all potential land sites can be helpful.  However care must be taken to ensure that these do not create a hazard for people attending the buoy.

# References

- IALA Maritime Buoyage System and other Aids to Navigation

- French lighthouse technical documentation

- CIE110 Spatial distribution of daylight - Luminance distribution of various reference skies

- IALA Conference Presentation 1990 about lettering and signs

- IALA Guideline 1023 The Design Of Leading Lines Edition 1.1 December 2005

- M. Blaise, IALA Bulletin No. 47 (April, 1971) article Titled “Daymarks as Aids to Navigation.”

- IALA Recommendation E-200 On Marine Signal Lights

- IALA NAVGUIDE

- IALA Recommendation E-108 on Surface Colour ….

- Practical examples

Practical application (EXAMPLES one for cloudy sky and two for cloudy sky)

- German input to the workshop

- General remarks about colours for buoys

There are a lot of different discussions about buoys in Germany.  The discussion is mainly influenced by the aim to reduce the cost for the buoys and their maintenance.

To set back the focus on the main task of a buoy, it became inevitable to make a list of the features of a buoy and to make a valuation between the features.

As a result we got the distinction between the so called primary and secondary features:

Primary features give reasons for a buoy (or a light) itself.  They explain why we use buoys for a waterway and are linked to the nautical requirements.

Secondary features are features to ensure that a buoy (or a light) will work properly.  They are necessary to support the primary features.

Example:

Primary features of a buoy are:

- colour and shape;

- radar echo;

- position;

- availability.

Secondary features are:

- mooring;

- maintenance;

- floating in water;

- lifetime;

- costs.

In general the secondary features should not limit the quality of the primary features.  However the primary features should not cause secondary features that cannot be fulfilled (e.g. because they are too expensive).

In detail the primary features of a (lighted) buoy are:

- colour (daytime: surface, night time: lights);

- shape (topmarks and special arrangement of coloured stripes or areas);

- flash characters of the light (night time);

- radar reflector;

- position;

- availability.

The surface colour belongs to the primary features of a buoy and there have to be very hard 'secondary' arguments to justify the limitation the quality of that colour.

- Three steps of visual perception

When approaching an object from a far distance we move through three different steps of visual perception (I directly translate the German words, might be you have different definitions):

- Object is visible: You see something but you don't know what it is.

For example: The object is still too small to be recognized as a certain type of buoy.

- Object is recognizable (identified): You see the object and know what it is, because you identify the colour or shape.

For example: The object can be recognized to be a certain type of buoy of the IALA buoyage system.

- Object is conspicuous: You cannot put the eyes away from the object.

Although we try to get distant buoys to become conspicuous, an object is conspicuous for your

For the visual features (shape and colour) it becomes evident to distinguish between a visible (1) and an identified (2) object.

- About Threshold of colours

To quantify the visual performance it is helpful to define the Maximum Distance of Recognition MDR.

MDR is the maximum distance the shape or the colour of the object can be identified.

In Germany we have the convention that the shape of a simple object (cone, cylinder or ball) can be identified when the angle is at least 3' (0.05°).  This leads to an equation for MDR.

- Title requred

In addition it is necessary that the object has enough contrast to the background.

For large objects, a distance MDR can be defined by the luminance contrast

C ≥ 0.05 (that is the international accepted value).

From IALA dictionary:

C=L2−L1L1

L1	luminance of the background

L2	luminance of the object

There are formulae for the threshold of the luminance contrast for small objects. However the concept of contrast can only be applied in a grey world.

When colour comes into consideration the contrast is not the right quantity to describe the visual performance.

Example: A red area with luminance 1 against a background with luminance 1 has a contrast of zero, but can appear conspicuous.

The identification of a colour does not depend on the luminance contrast.

The identification of colour depends on the luminosity and the saturation of the colour (The background at sea is assumed to be grey).

Example: Assume a coloured area against a grey background.  When the coloured area is very dark, you cannot identify the colour.  Because of the luminance contrast you will see the area very well.  However when you do not have a specific shape of the area, it will not show any information to the observer (perception step 1) except the information 'there is something'.  When the colour gets brighter you will get the colour information (perception step 2).  For large objects you get the information even if contrast is negative, for small objects it might be necessary to have positive contrast to identify the colour.

The luminosity of the colour can be described by the ratio L2/L1.

The saturation can be described by DIN 6164 [1].  There are alternative ways.

Unfortunately we have no theory for the identification of colour, which combines the influence of the size, luminosity and saturation of the coloured object, and that can be used to calculate MDR.

In general the distance MDR increases with the luminosity and the saturation of the colour [2].

In consequence fluorescent colours have a greater MDR.

- Colour versus shape (German buoys)

In the following figures the shape of two typical German buoys are shown.  The shape of the German buoys do not represent any information in accordance with IALA buoyage system (cylinder, cone, ball).  For some buoys there are top marks.

- Title required

- Title required

The top mark may be omitted for lateral buoys, so colour is the only visible (primary) feature at daytime.

From the geometrical convention we use in Germany we can derive three distances for this certain type of buoy (LT 81).

- buoy becomes visible	3 300 m

- identification of the stripes 'cardinal west'	1 000 m

- identification of the shape of the top marks	690 m

For all three aspects it is necessary to have a contrast or colour difference against the background.  (1) and (2) require a suitable colour to ensure that the buoy is identified according to the IALA Maritime Buoyage System.

When a very saturated and bright colour is applied (for lateral green and red only) the buoy can be identified in the same moment as it becomes visible.  So the MDR is 3 300 m for good weather conditions for lateral buoys.

Conclusions (for German buoys only):

Considering the primary feature 'colour' of a buoy, the lateral marks with bright and saturated colours (fluorescent) have the longest MDR and are therefore the most effective way to mark a waterway.

The distances to identify a buoy with two colours or with top mark (single danger, cardinal) are always much shorter than for the lateral buoys.  This is a disadvantage caused by the current IALA buoyage system.

- Desaturation caused by the atmosphere

Although there is no theory about the maximum distance of recognition for coloured objects, which includes luminosity, saturation and size of the object, it is possible to compare the colours.

One way to evaluate the performance of colours is done with the law of Koschmieder [3], which can be transformed into spectral equations (Hoffmann, [4]).

- Title required

Figure 5 shows the geometry to explain the influence of the atmosphere on the light that reaches the observer.

Light from the object is scattered away from the direction to the observer (1).

Light from the background is scattered into the direction to the observer (2).

Remark:

In detail the amount of light scattered into one direction depends on the position of the sun.  When the sun is behind the object, the amount of light scattered into the direction of the observer is higher than predicted.  When the sun is behind the observer, it is less.  So Koschmieder's law gives the mean value of the scattered light.

In general Koschmieder's law is used to explain the decrease of the contrast between the object and the background.

Hoffmann extended the law to spectral formulae.  This can be used to calculate the change of colour when the light moves through the atmosphere (see appendix A1).

To simplify the calculation we did not yet take into account the spectral dependence of the atmospheric transmission.  We will do this in future.

The scattering of light in the atmosphere causes two effects on the light the observer sees.

The first effect is that the visible luminance of the object approaches the luminance value of the background when the distance between object and observer increases.  This is shown for two green colours in figure 5 and a meteorological visibility of 10 M (Ordinary green is RAL 6024).

- Title required

The second effect is the desaturation.  This is shown in figure 6 for fluorescent colours.

- Title required

Figure 7 is based on outdoor measurements of coloured samples and application of the theory 'Koschmieder / Hoffmann' as explained in appendix A1.

We still have no threshold to evaluate between the different colours.  We will not get a threshold, which is based on size, luminosity and saturation of the coloured object, since there is no theory.  However looking at the saturation alone, we can define a threshold (see figure 7).

We chose the saturation S = 6 as the threshold.  This decision is arbitrary of course, but 'S = 6 ' corresponds to the border of the region of red and green towards white (for green S = 5..6, and for red S = 6..7).  So it represents an existing convention.

- Title required

In Figure 8 we compared a fluorescent green and an ordinary green RAL 6024. We get an intersection at S = 6 for:

fluorescent green: 	3 700 m

green RAL6024:	 400 m

At the distance of 3 700 m fluorescent green produces nearly the same colour stimulus as RAL 6024 at 400 m.

So we can say that - in this model - the fluorescent colour is about 9 times more effective than the ordinary green.

We use this procedure to compare different green and red colours only.  This is because the green and red colour is the only visible feature for most lateral buoys at daytime.

We have investigated six different colours and the results are presented in Table 3.

- Title required

| Colour | Distance for S = 6 | L2/L1 |
| --- | --- | --- |
| Fluorescent Green | 3700 m | 1,07 |
| Preferred Ordinary Green | 2200 m | 0,89 |
| Green RAL 6024 | 400 m | 0,78 |
| Fluorescent Red | 3900 m | 1,36 |
| Preferred Ordinary Red | 1800 m | 0,91 |
| Red RAL 3020 | 1500 m | 0,74 |

Figure 9 shows the colorimetric values of the six colours.

- Title required

It can be read from table 1 that the RAL colours, which are used for road signs, are not suitable for buoys.  There are ordinary colours (preferred) that ensure a greater distance of recognition than the RAL colours.

The preferred colours benefit from their higher degree of saturation and their luminosity.

For example, this could be flat, cylindrical, cone, crossed, etc.