16th IALA CONFERENCE

# Aids to Navigation in the Digital World

Shanghai, China, 22-26 May 2006

A risk management approach to defining remote monitoring and control policy

James Doyle

(Commissioners of Irish Lights, Ireland)

In the digital age, electronic systems are designed to be self-diagnostic and generate alarms in the event of malfunction. Electronic positioning systems such as DGPS and Loran C provide automatic warnings if the service is operating outside specification. This can lead to an expectation that all aids to navigation should alarm if malfunctioning.

Traditionally, many aids to navigation were either monitored locally by resident Lighthouse Keepers or faults were reported by passing Mariners. With the increasing use of automatic aids to navigation systems, coupled with the cost and environmental implications of the consequences of AtoN malfunction, there is a need to ensure that these aids are working correctly in a proactive rather than reactive manner.

This paper describes a risk management approach to defining the need for, and scope of, remote monitoring and control systems as a means of ensuring that failures of Aids to Navigation are notified in a timely and effective way.

- Introduction

It is a matter of definition that the running of any organisation is primarily concerned with the achievement of objectives. This achievement will face all manner of risks. The task of management is to respond to these risks so as to maximise the likelihood of achieving the purpose.

IALA has advocated the concept of risk assessment in the design of ports and waterways Aids to Navigation (AtoN) systems through two complimentary techniques, the PAWSA (Port and Waterways Safety Assessment) and IWRAPS (IALA Waterway and Port Assessment Programme) risk models. However risk management techniques can and should also be used in the design and management of AtoN system provision.

The PAWSA system uses a safety assessment model based on user consultation, while the IRAWPS uses risk assessment based on an empirical approach to waterway navigation system design and management. The objective of this paper is to use a qualitative risk analysis approach to determine what monitoring of Commissioners of Irish Lights AtoN is justified, bearing in mind that opinions on this topic vary widely and depend on a balance between cost, reliability of systems and legal concerns regarding “duty of care” issues.

- Risk

The essence of risk is the uncertainty of outcome (whether positive or negative). The term risk refers to the combination of the probability of these potential events and the magnitude of their consequences. Most people think of risk as the probability of an occurrence. When an event is considered “risky”, it usually means that there is a high likelihood that the event will occur. The EXXON VALDEZ oil spill had a very low probability of occurring but the consequences were so high that a lot of money was spent to make sure that adequate provisions will be in place to deal with the impacts if such an incident occurs again. Risk is therefore the product of probability and impact.

Risk = Probability x Impact

- Probability: the evaluated probability of a particular outcome actually happening (including a consideration of the frequency with which the outcome may arise)

- Impact: the evaluated effect or result of a particular outcome actually happening

The task of risk management is the management of this risk to an acceptable level, by taking action on probability or impact (or both). It therefore requires identification of the elements to be considered, not all of which may be controllable.

- Risk Management

The Commissioners of Irish Lights carry out a comprehensive risk management review every three years in accordance with their risk management strategy.

| ## Risk Details | ## Consequence | ### Likelihood | ### Impact | ### Response | ## Control |
| --- | --- | --- | --- | --- | --- |
| Hazard marked but out of service (casualty) | Potential marine accident | Low | High | Treat | IALA Standards Monitoring Insurance |
| AtoN out of position | Potential accidents | Low | High | Treat | Radio nav. warning Monitor Insurance |
| No radio nav. warning | Potential accidents | Low | High | Treat | Monitoring |
| AtoN not available | Potential accidents | Low | Medium | Treat | Monitoring |
| Failure of AtoN(s) AtoN(s) exhibit incorrect characteristics | Shipping casualty Collision, wreck, pollution Liability claim from owners Loss of reputation Increase in insurance premiums | Low | High | Treat | Monitoring Planning Preventative maintenance Training Insurance Radio Navigation Warnings Notice to Mariners |

Table 1. Corporate risks for which remote monitoring is a control measure

The main purpose of the review is to identify and measure the major business risks facing the Service, to consider any likely changes in the authorities’ risk exposure in the short to medium term and to make recommendations for responding to those risks through the use of loss prevention, control measures and insurance cover.

Risks are assessed under the headings of external threats, financial, health and safety, human resources, information technology, operations and strategic. Table 1 shows the corporate risks for which remote monitoring and control (RCMS) is considered to be a control measure. It is therefore essential that RCMS policy is sufficiently robust to meet this requirement.

- Remote Monitoring and Control Systems

Remote Control and Monitoring Systems (RCMS) are used to monitor and control AtoN systems, providing three fundamental functions:

- Confirmation that the AtoN is operating correctly and providing warning if the AtoN fails

- Information about the condition of the system and warning if failure is imminent

- Control to enable resetting of systems without visiting the station

Traditionally, many aids to navigation were either monitored locally by resident Lighthouse Keepers or faults were reported by passing Mariners. With the increasing use of automatic aids to navigation systems, coupled with the cost and environmental implications of the consequences of AtoN malfunction, there is a need to ensure that these aids are working correctly in a proactive rather than reactive manner. In addition, RCMS offers the opportunity to maintain systems in a more cost effective way through condition monitoring and remote reset of fail conditions.

In the digital age, electronic systems are designed to be self-diagnostic and generate alarms in the event of malfunction. Electronic positioning systems such as DGPS and Loran C provide automatic warnings if the service is operating outside specification. This can lead to an expectation that all aids to navigation should alarm if malfunctioning.

However, provision of RCMS equipment has cost implications as well as benefits. In addition to the capital cost of equipment, the cost of larger or additional power systems and maintenance of the RCMS equipment must be considered. Reliability of the RCMS must be at least equivalent to or better than the equipment being monitored. Consequently RCMS may not be the most cost effective solution in all cases and assessment through risk analysis can be used to optimise the use of RCMS.

- RCMS Risk Analysis

The strategic risk management cycle comprises the following steps

  - Identify the risks and define a framework

  - Evaluate the risks

  - Assess risk appetite

  - Identify suitable responses to risk

5.1	Identify the risks and define a framework.

Table 2 offers a summary of the most common categories of risk with examples of the nature of the source and effect issues.

| AtoN can fail to provide the required service in a number of ways. These failures can create a number of risks to mariners and affect all types of vessels. Once detected, an AtoN failure can be advised to mariners by issuing a Navigation Warning, allowing the mariner to take action to avoid the consequences of the AtoN failure. AtoN Failure modes | Risks arising from AtoN failure | Types of vessels |
| --- | --- | --- |
| AtoN failed AtoN false information – incorrect character AtoN false information – out of position AtoN failed and no Navigation Warning issued | Collision Grounding Deviation Nuisance Near miss Congestion | Passenger vessel High speed vessel Tanker Cargo vessel Fishing vessel Yacht |

Table 2. AtoN failure modes and effects

Monitoring is central to this process, through detecting the AtoN failure and ensuring that Navigation Warnings are issued. The hazards arising from monitoring include

- AtoN failure undetected

- Do not know status of AtoN

- False report of AtoN outage when AtoN operating correctly

- Incorrect use of AtoN status data arising from unreliable procedures

- Incorrect use of AtoN status data arising from human error

| Table 3 lists the consequences resulting from AtoN failures. These can vary from financial loss arising from loss of ship time to loss of life and major environmental pollution. The importance of the AtoN clearly has a major impact on the consequences of AtoN failure. Impact / Consequences arising from AtoN failure |  |  |
| --- | --- | --- |
| Minor | Moderate | Major |
| Minor injury Minor pollution Cost to coastguard Minor damage to vessel Minor damage to cargo Loss of ship time | Moderate injury Moderate pollution Cost to shipping owner Reputation damage Serious damage to vessel Serious damage to cargo Creating shipping hazard | Death Major pollution Cost Reputation loss Loss of vessel Loss of cargo Blocking port Creating shipping hazard |

Table 3. AtoN failure impact

5.2	Evaluate the risks.

Some types of risk lend themselves to a numerical diagnosis – particularly financial risk. For other risks – for example reputation risk  - a much more subjective view is all that is possible. It will be necessary to develop some framework for categorising risks as perhaps high / medium/ low.

| Table 4 shows a commonly used matrix of probability/ likelihood and impact/ consequence. The product of probability and impact provides a risk rating. Risk rating |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Impact / Consequence |  |  |
|  |  | Minor (1) | Moderate (2) | Major (3) |
| Likelihood | Unlikely (1) | 1 | 2 | 3 |
|  | Likely (2) | 2 | 3 | 6 |
|  | Very likely (3) | 3 | 6 | 9 |

Table 4. Risk rating matrix

| AtoN stations can be categorised in terms of their navigation importance, location, type of vessel and impact of a marine incident as considered in Table 2. The type of vessel using a waterway, the type of hazard marked and the impact of a marine incident are factors that govern the classification of a station as major or minor. In Table 5, lighthouses are divided into offshore, where access is difficult, and onshore where ease of access allows ready determination of the status of the AtoN. Major floating aids (MFA) such as lightships can be very difficult to access and may be difficult to see from shore. The navigation significance of buoys can vary greatly, depending on the location of the buoy, the hazard marked and the proximity of other AtoN. Station categorisation |  |  |
| --- | --- | --- |
| Category | Station type | Location type |
| A1 | Lighthouse | Major offshore |
| A2 | Lighthouse | Major onshore |
| A3 | Lighthouse | Minor offshore |
| A4 | Lighthouse | Minor onshore |
| B1 | MFA | ALF and Lanby |
| C1 | Buoys | Primary navigational significance |
| C2 | Buoys | Lighted more significant |
| C3 | Buoys | Lighted less significant |
| C5 | Beacons | Lighted |

Table 5. Categorisation of stations

5.3	Assess risk appetite.

An important issue in considering response to risk is the identification of the risk appetite of the organisation. Risk appetite is the amount of risk to which the organisation is prepared to be exposed before it judges action to be necessary. The fact that the resources available to control risks are likely to be limited means that value for money decisions have to be made. Apart from the most extreme circumstances it is unusual for good value for money to be obtained from any particular risk being completely obviated with total certainty.

Because of the cost of provision and maintenance of RCMS, it may not be cost effective to provide RCMS for all AtoN.

5.4	Identify suitable responses to risk.

The type of monitoring used at each category of station is based on the risk rating identified in Section 5. In the present digital age where malfunctions of systems are proactively brought to the attention of users through automatic alarms, the concept of User reporting outages must be considered suspect. Some form of proactive monitoring must be employed to ensure that the AtoN service provided meets advertised parameters.

| Responses to risk can be divided into four response categories – Transfer / Tolerate / Treat / Terminate. The available methods of monitoring are listed in Table 6. Monitor categorisation |  |
| --- | --- |
| User monitoring | User reports outages |
| Watcher | Aid is checked once per night, eg buoys, non resident Attendant |
| Attendant monitoring | Station is connected to Attendants residence by some form of alarm system, including resident Attendants |
| Central RCMS | Outages reported to CIL Dun Laoghaire monitor centre automatically via RCMS |

Table 6. Categorisation of monitoring

| Actions |  |  |  |
| --- | --- | --- | --- |
| Risk rating | Assessment | Priority | Action |
| 1 | Trivial risk | Non-urgent | No monitoring |
| 2 | Acceptable risk | Non-urgent | Watcher |
| 3-4 | Moderate risk | Action required | Full RCMS |
| 6-9 | Substantial risk | Urgent action required | Full RCMS |

Having identified the risk rating, the assessment and action taken is identified in Table 7.

Table 7. Categorisation of monitoring

- Analysis of Remote Monitoring and Control Risk

- Table 8 applies the above to determine the monitoring that should be used at each category of station.

| ## Station category | ## Station type | ## Significance | ## Likelihood of undetected AtoN failure | ### Impact of undetected AtoN failure | ### Risk rating | ## Control |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | Lighthouse | Major offshore | 3 | 3 | 9 | Full RCMS |
| A2 | Lighthouse | Major onshore | 2 | 3 | 6 | Full RCMS |
| A3 | Lighthouse | Minor offshore | 3 | 2 | 6 | Full RCMS |
| A4 | Lighthouse | Minor onshore | 2 | 2 | 4 | Full RCMS |
| B1 | MFA | ALF and Lanby | 3 | 3 | 9 | Full RCMS |
| C1 | Buoys | Primary navigational significance | 3 | 3 | 9 | Full RCMS |
| C2 | Buoys | Lighted more significant | 3 | 2 | 6 | Full RCMS |
| C3 | Buoys | Lighted less significant | 3 | 1 | 3 | Watcher |
| C5 | Beacons | Lighted | 3 | 2 | 6 | Full RCMS |

Table 8. RCMS Risk Analysis

- In determining the likelihood of a AtoN failure going undetected, the location of the AtoN is clearly critical. The type of vessel affected by each category of AtoN is implicit in the category of AtoN. Consequently, the impact of an AtoN outage not being detected is determined by the navigation significance of the AtoN.

- The risk rating indicates that most AtoN stations should be monitored on a continuous automatic basis. Watcher monitoring, where the AtoN is only checked at intervals no longer meets the requirements of Users in an age where vessel speeds are increasing, where vessel traffic density is increasing and where there is ever increasing reliance on integrated digital systems.

- Gain assurance about the effectiveness.

It is essential that assurance about the effectiveness of the actions is obtained, through a reporting system and through internal audit, where appropriate. Organisational procedures are in place to ensure that Monitoring is effective in identifying and reporting on the status of AtoN systems. Monthly reporting identifies both the ability of the RCMS to identify and report AtoN outages and also the availability of the RCMS.

- Embed and review.

In an organisation the objectives of directorates, divisions, teams and individuals is related to the organisations top-level objectives. Risk management demands that appropriate awareness of, and responsibility for, risk issues lies in parallel with the structure of objectives. At every level of objectives there should be a parallel delegation of responsibility for the associated risk issues. This is critical to embedding risk management into the structure of the organisation.  The process of the three year review of risk management strategy allied to a risk analysis approach to defining and managing systems ensures that risk management should become an intrinsic part of the way the organisation works.

- Conclusion

In the present digital age where malfunctions of systems are proactively brought to the attention of Users through automatic alarms, the expectation of Users is that equipment is assumed to be working correctly unless advised otherwise. Thus proactive monitoring should be employed to ensure that the AtoN service provided meets advertised parameters and the AtoN outages are quickly identified and Users notified. Using a relatively simple approach, risk analysis techniques can be used to draw up a policy for the provision and maintenance of AtoN systems. The example of monitoring of AtoN systems discussed can be applied to a wide range of equipment and systems.

The simple analysis described concludes that full Remote Monitoring and Control (RCMS) should be employed at most AtoN stations. The advent of AIS with its immediate failure detection and direct User notification capability will greatly enhance the ability of AtoN service providers to meet this requirement.