This questionnaire is designed to gather information from IALA members on their experiences with the use of plastic buoys. This will then be collated and used to enable members to make informed choices on the selection of the most appropriate material for their specific buoy application.

480

- How many buoys do you have in your current buoy fleet ? ………………………………..…

Regarding the materials of construction, how many are:

400

0

Steel …………………………………………………………………………..……………….

Composite metal & plastic ………………………………………………………..…………

80

Plastic ……………………………………………………………………………………..…..

Other (please specify) ……………………………………………………………………….

- Buoy fleet by buoy body diameter and material, please complete the table below:

| Material | 1m dia or less | 1 to 2.2 m dia | Greater than 2.3m dia |
| --- | --- | --- | --- |
| Steel | 0 | 10 | 390 |
| Composite metal & plastic | 0 | 40 | 0 |
| Plastic | 0 | 40 | 0 |
| Other (please specify) | 0 | 0 | 0 |

- For your plastic buoy fleet, what type of plastic material is used ?

various

Polypropelene

- Are the plastic buoy bodies filled or empty ?

9

91

Filled	30		Empty

Please specify filler type    Foam and polystyrene

6

- How many years have you been using plastic buoys ? ……

- ……………………………...….

- Regarding the location / conditions of service for your plastic buoys, check the boxes that apply:

| Inshore | X |
| --- | --- |
| Offshore |  |
| Exposed |  |
| Sheltered | X |
| Breaking waves |  |
| Dries at low tide | x |
| Severe UV exposure | NA |
| Severe sub zero temperature | NA |

- Are your plastic buoys a commercial design or your own authority’s design ?

X

Commercial Design

- How often do you visit your buoys at sea ?

| Material | 1m dia or less | 1 to 2.2 m dia | Greater than 2.3m dia |
| --- | --- | --- | --- |
| Steel |  |  | ANNUAL |
| Composite metal & plastic |  | ANNUAL |  |
| Plastic |  | ANNUAL |  |
| Other (please specify) |  |  |  |

- For your plastic buoys, what is the main reason for inspection / maintenance at sea ?

General condition and measurement of mooring system.

Check the moorings, check the light, lift the sinker to stop it getting “sanded”, check the position, general check for damage and condition.

- How often do you return your buoys to shore for maintenance ?

| Material | 1m dia or less | 1 to 2.2 m dia | Greater than 2.3m dia |
| --- | --- | --- | --- |
| Steel |  |  | 6 years |
| Composite metal & plastic |  | At end of life only |  |
| Plastic |  | At end of life only |  |
| Other (please specify) |  |  |  |

- For your plastic buoys, what is the main reason for inspection / maintenance ashore ?

They are not refurbished ashore, but we have had limited experience of commercial buoys with various mooring issues.

Damage due to collision

Damage due to wear & tear

Colour fade beyond agreed limits

- What experience do you have in modifying plastic buoys to take new navigation aids, such as AIS ?

None

- Have you experienced any in service defects with plastic buoys (such as colour retention, mechanical failure, mechanical damage, mooring lug wear) ?

Yes, pigment colour retention is a major issue for us.

Yes, Colour retention, particularly on Red

Mechanical failure with the superstructure parting from the buoy body

- Please detail any experience you have of repairing or re-painting plastic buoys ?

No, although I know of some companies that are happy to undertake re coating of the same base colour.

Only replacing damaged parts with new items & repair to cracked metal framework

- Can you provide any cost comparisons between plastic buoys and traditional steel buoys (this may include initial cost, use of a smaller ship to support the buoy fleet, reduction in buoy maintenance facilities, reduction in overall maintenance costs)

- Are you planning to purchase more plastic buoys in the near future ?

X

Yes

10 years

- What is the anticipated life of your plastic buoys ? ………………………………………

- What is your disposal policy for plastic buoys at the end of their life ?

within current legistation

Chop up for regrind plastic.

Steel for re cycling

- With your experience, what is your general opinion of the use of plastic buoys ?

Ideal for sheltered inland waters, limitations in flexibility and general quality is poor.

OK for small buoys less than 2.0 m dia and OK for sheltered waters.

Small one piece plastic buoys seem robust but composite plastic buoys with larger superstructures we have found to be fragile and not durable in exposed or short sea conditions. The steel buoy, whilst needing ashore maintenance every 6  to 8 years is durable and repairable.

- Please provide some photographs of your plastic buoys with sizes & weights (please resize and compress any photographs before insertion):

Type 3 Mobilis Jet1800

Weight 780kgs, Diameter 2.2m,

Height 3.8m excl topmark

Type 4 Tideland SB98

Weight 261kgs, Diameter 1.5m,

Height 1.9m excl topmark