Comment on Draft revised Guideline 1058, June 2011

Use of Simulation as a Tool for Waterway Design and AtoN Planning

German Traffic Technologies Centre, Frank Hermann, 2011-09-16

Relevant Papers:

- Draft IALA Guideline No. 1058 Edition 2, June 2011

- EEP Liaison Note Simulation in the Design of AtoN / EEP-16/output/7, 1 April 2011

Comment:

The Liaison note shows the physical limitations of the optical equipment (projectors and screens) used as the front end of a simulation process.

Examples for the physical limitations are (extended compared to liaison note)

- The resolution of projectors and screens is insufficient to simulate the resolution of the human eye: There is a gap between eye resolution (1') and shown resolution (>10').

- The maximum illuminence of the projectors is about 100-300 Candelas per metre; maximum real-world Illuminence is around 10,000 candelas per metre

- The background illumination in the natural world varies from bright sun light to new moon in the interval 0.01 lx to 100 000 lx. Projectors and screens can be used in an interval from 10 to 1000 lx only.

- Contrast is not comparable either; contrast of computers, projectors and screens is around 1:1x104, actual real-world contrasts are around 1:1x107.

- The Colour space of screens and projectors is only a small fraction of the true colour spaces in the natural world.

Whereas the position and the arrangement of daymarks or lights and can be an important part of a simulation process, in most situations the simulation cannot be used to estimate the visual features and design of a daymark and a light.

In detail a simulation does not give realistic scenarios for:

- intensity and colour of a light

- colour of a daymark

- size and shape of a daymark at far observation distances

- background lighting

- meteorological visibility

- atmospheric influence as clouds and rain

- direct and indirect sun illumination, back light

- light reflection at the water surface

The current draft of Guideline 1058 June 2011 does not provide a hard conflict with these limitations. However it would be helpful to give some information about these limitations.

It could be stated that in general visual trials should not be performed with projectors or computer screens, but with well designed lab (indoor) or outdoor experiments instead.