EEP17	Input paper

Agenda item	EEP17 / 8.1 – Action Item 33

Task Number	Task 2

Author(s)	Mariano Luis Marpegan – Raúl Escalante

Operational aspects of installing AIS transponders on floating AtoN - DRAFT

# Summary

The objective of this paper is to enumerate the different operational aspects and problems of installing AIS transponders on floating AtoN.  This paper should be considered as a first draft

GPS Antenna

VHF Antenna

IP Box 69K

Led Lantern

Solar panel 50 W

## SUPPLIERS

Suppliers whose AIS- AtoN have been tested are listed:

A fluent relationship with the supplier is fundamental for the success of the project. It should be kept in mind that AIS-AtoN is not plug and play equipment. A lot of work has to be done both at the laboratory and in the field to obtain a proper and continuous functioning of the equipment.

It is important to verify that the AIS-AtoN transponder is manufactured by the supplier or if it is manufactured by another supplier and incorporated in third parties equipments

## Important aspects to consider when selecting the supplier are:

### Type of equipment is required

When the device is in process of specification, it must be consider the possibilities. There are equipments type I working in Fixed Access Time Division Multiple Access (FATDMA) with a low power consumption associated and there are type III working in Random Access Time Division Multiple Access (RATDMA) where the power consumption can be critical on AIS fitted on floating Aids to navigation.

The range is between 2 watts and 12 watts with the corresponding effect on the design of the power supply.

### Experience

It is important to verify that the supplier has experience in delivering the type of product that is being ordered. That is, that you are not buying a prototype.

### Capacity of giving after sales service

It is always necessary to make all type of inquiries to the supplier. Therefore it is necessary that the supplier has a good after sales service.

Availability of suppliers qualified staff to investigate operational problems

## Field installation

### Laboratory tests

It is important to set the measurement protocol in the tests that are performed on new equipment to be incorporated into stock and to those devices returning from the buoy station after a replacement for preventive maintenance. These measurements include operating current and voltage and stability to acquire GPS signals and subsequent transmission.

See Annex A for laboratory procedures.

### Authorizations

Not until 2008 the International Telecommunications Union (ITU) established the standards for the codification of Maritime Mobile Service Identification (MMSI) for AIS- AtoN. Usually the Organization with responsibility at National level gives a preliminary authorization which after time becomes definitive.

When the Service have to obtain the MMSI for the first time in their country an interaction with the corresponding agency to define them is usually required.

### Capabilities required for lanternists. (Maintainers)

The technical staff associated with the maintenance and operation of AIS- Aton should have specific training that allows them to work independently in the resolution of faults that occur in AIS-AtoN. These people are not the usual lanternist.

### Transmission Range

The transmission range is directly related to the height of the VHF antenna and transmission power. When defining the equipment to be installed it is essential to have accurate knowledge of the Wave Ratio Stationary (SWR). This is a value equal to one (1) when using calibrated antennas in the frequency of AIS. If the VHF antenna is not suitable for AIS, the SWR will be locate near to 10 and the transmission range will be less than 5 kilometers.

The usual marine VHF antenna type has its center frequency between 156 -157 MHz and have a bandwidth of between 6 and 7 MHz But the frequency channels used in AIS are located at 161.975 MHz and 162.025 MHz, it is necessary to use antennas specifically tuned for the AIS frequencies.

This concept is critical at the AIS-AtoN is designed to operate with low power. Therefore the transmission power must be optimized.

### Importance of water tightness

For AIS Aton equipments that are not water tight when supplied the following remarks are of the upmost importance.

The current installation is performed under normal temperature. When installed under the conditions of the installation place it is exposed to sudden temperature changes during the night. This generates water vapor present in the IP box which condenses forming drops of water.
The cool air and released water vapor in the form of droplets generates a reduction in volume of air which results in a vacuum inside the IP box. If the tightness of the box does not prevent moist air entering the interior of the box, temperature changes between day / night generates a constant condensation of water vapor.

One possibilityof assembly for AIS-AtoN installations is to elevate the IP box location over the water level and reducing the three critical points ​​that produce condensation.

- Temperature gradient (difference temperature between air inside and outside the container).

- Water vapor (air temperature higher load to more water vapor).

- Air circulation

Canceling one of these three points will reduce the continuous condensation. Therefore different solutions can been adopted.

- Reducing the size of the IP box minimize the air volume inside

- Filling the stainless steel IP 69 box with polyethylene foam generates a reduction in the temperature gradient.

- The polyethylene foam also reduces the amount of existing air and consequently reduces water vapor.

- Using high quality IP boxes contribute in avoiding vacuum within the IP box and the entry of air from the outside.

- To optimize the tightness of the device cable inputs are sealed, cable glands, encapsulation equipment and double boxing.

### Protection for atmospheric discharge and grounding

The implementation of a protection gaseous to ensure the equipment against an atmospheric discharge is essential in any installation of electronic equipment in the river.
These atmospheric discharges affects stability of the equipment for the operation in the GPS signal acquisition and transmission.

The implementation of a secure protection ends in the reliability of ground.

### Compatibility between equipment and peripherals

When the AIS transponder is not provided by the supplier with AIS or GPS antennae special care is to be taken due to possible it can cause an electrical imbalances that can affect the reliability of operation of the equipment as a whole.

### Viability of transmitting the MESSAGE 8

Message 8 is a very important one. It allows transmitting additional information like meteorological information using the AIS system. For AtoN Services that have meteorological networks this capability offers the possibility of transmitting data with very low cost of transmission. One actual problem to use this AIS capability is that AIS transponders as usually supplied by providers do not interface easily with meteoroligal transducers.

Different suppliers present alternatives to capture the meteorological information.

- one AIS AtoN receives signals only from sensors supplied by the firm. This is very inconvenient for already installed networks due to the difficulty of changing sensors

- another AIS AtoN requires additional software.

- in another case it requires that the interphase is designed and built by the user.

This means that in all cases an important development of software and in some cases hardware has to be carried out in-house.

As a general conclusion it can be said that despite a lot of effort it is not easy to transmit message 8 and that it should not be taken for granted that this will always be possible.

## Reception stations and Processing of AIS information

The first objective of installing AIS on AtoN is to give ships the possibility of detecting AtoN by receiving message 21. Ships can do this by having the proper technology on board.

For the AtoN Service that has to verify the proper functioning of installed AIS it is also important to have the capability of receiving this message. Some VTS Services don´t have the operational capability of receiving Message 21. In some cases where VTS Services are not under the same Authority as to AtoN Services it is not easy to establish a fluent access to the AIS information. An alternative to this situation could be to install an “own network” of receiving stations associated to the dimensions of the area to be covered several receiving stations had to be used to collect data from AtoN and passing ships to retransmit it to central office.

The receiving stations are a joint of electronic equipment used to capture the AIS frame and webcast to an IP address where a server is located that stores and graphics information broadcasted by retransmitting stations.
The real time incoming data on the server is plotted on a chart of the waterway and in turn it´s stored in a database for later analysis.
Currently there are two types of retransmitting stations: those that are on land and those that are installed on fixed beacons in water.
On land retransmitting stations are located  in port or coastal areas near the coast above ground level in order to achieve large coverage and can be equipped with 220V power and internet service. This simplifies the design of the station and minimizes maintenance for power supply purposes.

Retransmitting stations installed on fixed beacons in water give less coverage but permit cover areas where there are no possibilities of using land stations.

Retransmitting stations should be arranged strategically with an optimal distance between them. At least a minimum overlap should exists between adjacent stations. This kind of installation consists of a VHF antenna located around 50m above the water level. The antenna is calibrated at 162 MHz and connected to an AIS receiver. It captures all the information contained in the AIS frame, then the receiver delivers data format (RS232) port of a converter RS232 to TCP / IP. This converter connects to the server or a fixed IP and a specific port by sending data over the WAN.

## Spare transponders

It is necessary to have spare transponders to replace those that fail, are damaged by passing ships, vandalized or out of service for any other reason. The best way to maintain the service is to replace a malfunctioning transponder and to proceed to investigate the failure and eventually proceed to it´s reparation at the workshop

The spare number of transponder required depends on the installed number of them and an elevate compromised of level service. Assuming a high level of service the following list gives an indication

| Installed number of transponders | Recommended number of spare transponders |
| --- | --- |
| 1 | 1 |
| 2 to 3 | 1 |
| 4 to 6 | 2 |
| 7 to 10 | 3 |
| 11 to 16 | 4 |
| 17 to 50 | 25 % |

## Purpose of the document

The purpose of this document is to serve as a basis for discussion for a Guideline on the subject

## Related documents

Body text.

# Background

Give the background to the paper.  (Body Text)

# Discussion

Develop as required.  (Body Text)