BUOY DESIGN HYDRODYNAMICS

INTRODUCTION

This guideline provides information on those aspects of buoy design that effect the buoyancy and stability of the buoy.

The Guideline 1066 ----------- should be consulted when loads due to wind and tidal effects are being considered.

It is recommended that a standard textbook on naval architecture should be consulted if the reader is not familiar with basic ship stability calculations.

The aspects of buoy design discussed in this guideline provide the basis for designing a buoy from first principles but can also be used when considering mounting different superstructures and new equipment on existing buoys or operating existing buoys in different mooring environments.

BUOYANCY

This vital aspect ensures that the buoy will remain afloat and that the AtoNs will remain effective in all weather conditions.

There must be sufficient buoyancy to support the entire weight of the buoy and the moorings that the buoy supports.

The size and hence the weight of the buoy will largely depend on the navigational requirements of the AtoN.

The size of the daymark will depend on the daytime range at which the buoy must be visible and the height of the daymark or tower will depend on the focal plane height that is required for the light or for aerial systems.

The wave regime at the operating site may further effect the daymark size and tower height if the buoy is to be seen in rough waters.

In still water situations the buoy must have sufficient buoyancy to support its own weight plus the weight of the mooring (usually chain) between the buoy and the sea bed  (at the highest water level).

When there are wind and/or tidal flow loads acting on the buoy then the buoy will increase the loads in the mooring and more buoyancy will be required to support these added mooring loads. See IALA Guideline 1066 ------------

Waves will have some effects on the mooring loads and hence buoyancy requirements. Wave effects are extremely complex. However if the waves are not breaking then the buoy will follow the water surface and there will be little increase in the buoyancy requirements in order to cope with the waves.

STABILITY

Stability measures the ability of the buoy to remain upright and for the AtoN to remain effective in all operating conditions.

There are two aspects of stability that will be incorporated in the buoy design.  These are form stability and ballast stability.

Form stability is the stability provided by the shape of the buoy. In a normal cylindrical buoy the greater the diameter of the buoy the more stable it will be. An extreme example of this is the LANBY buoy which has a shallow 'dish' like body. Due to its large diameter this provides stability in most situations, however in extreme conditions this can capsize and will be more stable in an inverted position

Ballast stability is provided by the centre of mass of the buoy being in a position such that it tends to right the buoy.  The deep tailtube buoy provides an example of this type. The ballast weights on the base of the tailtube provide righting forces that operate in a similar way to the mass of a pendulum returning the pendulum to the vertical position.

Stability calculations evaluate the combined effects of form stability and ballast stability.

CALCULATIONS

The starting point for the calculations will be a detailed drawing of the buoy along with details of all the equipments to be mounted on it.

The all up weight of the buoy will then be a summation of all these components. Weights of steel work,  aluminium fabrications mouldings etc can be calculated but it is recommended that checks should be made by weighing components, where this is practical.

The centre of mass, usually referred to as the centre of gravity, can then be established by summing the moments of each component about a chosen datum. As buoys are usually largely symmetrical about their vertical axis it is only necessary to take the moments in the vertical (X) axis. The datum is usually taken as the base line of the keel or skirt of the buoy. The moments are then always positive.

The immersed volumes of the components (skirt, tail tube, mooring eyes etc) of the buoy must be calculated and the volume of the buoy body established that will provide sufficient buoyancy to float the total weight of the assembled buoy.

At this stage the maximum mooring loads can be established from Guideline 1066 -----   these can be added to the weight of the buoy to establish the resulting line of floatation (water line)  and reserve buoyancy (freeboard).

The moments of the immersed components about the datum can be summed and the centre of buoyancy established.

For calculation of stability it is usual to consider the buoy as a free floating body, ie without the moorings. Some authorities include a small part of the moorings as a mass attached to the mooring eye(s). In the case of buoys with twin mooring eyes the bridle part of the mooring is included as part of the buoy mass.

With the total weight, centre of buoyancy and centre of gravity being established then the metacentric height can be calculated. If this is positive, then if the buoy is displaced from the upright position it will return back to its upright position. If the answer is negative then any displacement of the buoy will result in it tending to capsize. The body diameter will then have to be increased or, more usually the centre of gravity will have to be lowered.

The metacentric height provides a measurement of the righting moment of the centre of mass of the buoy relative to the centre of buoyancy.

A large value for the metacentric height will result in the buoy returning rapidly to the upright position and have a quick or 'jerking' motion. A small metacentric height will result in a slow rolling motion, possibly with excessive displacement from the vertical.

The effectiveness of the visual AtoNs on the buoy will depend on these being clearly visible. If the buoy has a rapid or jerky motion then it will be difficult to clearly recognise the daymark or the light character. A slow and steady rolling motion is therefore preferred, however the extent of the rolling must not be excessive.

The ideal value for the metacentric height is difficult to define. A guide of 1/10 of the body diameter was quoted for standard steel buoys but this is thought to be rather small for lighter plastic buoys.

ANGLE OF HEEL

The angle of heel that the buoy may assume in wind and tide conditions can be calculated from the  wind and tide loading information provided in Guideline 1066 ----   Moments can then be taken and the resulting heel angle calculated. The worst case is usually assumed when both tide and wind forces are trying to heel the buoy in the same direction.

If it is anticipated that servicing personnel will work on board the buoy then calculations should be made with the estimated weight of the servicing personnel on the buoy to ensure that a reasonable angle of heel is maintained.

PRACTICAL MEASUREMENTS

The weights of all the buoy components can be measured to confirm calculations. This is particularly important in the case of moulded plastic components where the wall thickness may be difficult to measure.

If suitable calm water facilities are available , with a crane, then floatation trials can be carried out and weights used to heel the buoy so that the metacentric height can be measured.

When the buoy is to be used in fast tide or river situations towing trials can be carried out to evaluate the buoy's performance. The buoy can be towed from its mooring point with a ballast weight in the tow line that is sufficient to maintain a downward slope in the line from the buoy to the weight.