EEP17	Input paper

Agenda item	11.2

Task Number

Author	Puertos del Estado. Spain

# BASIC GUIDELINES FOR THE DESIGN OF VISUAL AID TO NAVIGATION DAYMARKS

# Summary

This document contents examples and practical solutions and recommendations for the design of visual aids to navigation daymarks.

# Background

EEP Committee is drafting a guideline “On the Design of Daymarks in AtoN” Collecting the existing information, both in the IALA as in other countries, Puertos del Estado has developed a basic guideline for the practical design of daymarks that you can find attached..

# References

Draft Guideline “On the Design of Daymarks in AtoN”..........

# Action requested of the Committee

The EEP Committee is requested to consider if this document could be added as an appendix at the end of the draft of the guideline “On the Design of Daymarks in AtoN”, or if their most important points could be insert in the appropriate places in the new version of the draft.

|  | BASIC GUIDELINES FOR THE DESIGN OF VISUAL MARINE AID TO NAVIGATION DAYMARKS |
| --- | --- |

MARINE AIDS TO NAVIGATION AREA

1st Edition – December 2010

TABLE OF CONTENTS

1.  INTRODUCTION	- 3 -

2.  RECOGNITION DISTANCE OF A DAYMARK	- 3 -

2.1  Geographic range	- 5 -

2.2 Meteorological visibility	- 5 -

3.  COLOUR OF A MARITIME SIGNAL'S SURFACE	- 6 -

4.  SHAPE	- 10 -

5.  ASPECTS TO TAKE INTO ACCOUNT IN THE DESIGN OF STRUCTURES	- 10 -

6.  DIMENSIONS	- 14 -

6.1  Dimensions of a daymark with a complex message	- 15 -

7.  PROCESS OF IDENTIFYING THE MESSAGE ON A DAYMARK	- 15 -

8.  TOP-MARKS	- 17 -

8.1  Colour	- 17 -

8.2  Shape	- 17 -

8.3  Dimensions	- 17 -

9.  OTHER DAYMARKS	- 20 -

9.1  Marking of bridges	- 20 -

9.2 Marking of quays, dolphins, floating structures, etc.	- 20 -

10.  CONCLUSIONS AND RECOMMENDATIONS	- 21 -

11.  REFERENCES:	- 23 -

ANNEX I: Summary of recommendations on daymarks	- 24 -

ANNEX II: Sizing of a daymark (simple shapes)	- 25 -

BASIC GUIDELINES FOR THE DESIGN OF VISUAL

AID TO NAVIGATION DAYMARKS

# 1.  INTRODUCTION

Sight is the main instrument used by sailors when navigating along coastal areas. Both daytime and night-time visual aids are therefore essential. Though a lot of attention has been placed on night-time marks or light signals, the same cannot be said for daymarks, which are used during the day.

It is often considered that the installation of a maritime signal consists of installing a light on a post or on any other kind of support without conducting any kind of preliminary study on the recognition distance, colour and other features of the signal's daymark, thereby ignoring its great importance. It should not be forgotten that most of the traffic occurs during the day. Hence, a signal should have a sufficient size and be in an optimal state of upkeep to comply with the necessary recognition distance.

A visual marine aid to navigation may be equipped with a light or not (in which case it is known as "unlighted signal"), but it will always have a use as a daymark. Should the signal belong to IALA the Maritime Bouyage System (IALA-MBS), its characteristics, colours, shape and top-mark will be defined by such system. On the other hand, if it does not belong to the IALA-MBS – as is the case of lighthouses, main beacons, leading lines and sector lights – there are no specific rules. However, their daymark (the lighthouse tower, the leading line's dayboard, etc.) should always comply with some characteristics regarding size, colour, contrast, etc., so that it may be easily identified against the background.

This document is not a calculation model, but rather a practical guide which puts forward a series of recommendations to help designers of maritime signal installations to ensure that daymarks can be identified by navigators at a reasonable distance most of the time and without the possibility of causing confusion.

# 2.  RECOGNITION DISTANCE OF A DAYMARK

The theoretical calculation of a daymark's recognition distance is a much more complex matter than that of a light's night-time range. Hence, there are very few studies on this issue and their results are not very conclusive and are always based on empirical field studies. The recognition distance of a single signal can vary greatly depending on environmental conditions, the time of day, etc.

The main aspects which exert an influence on a daymark's recognition distance include the following:

- Environmental conditions:

The area's usual weather conditions, clear sky, cloudy sky, etc.

Atmospheric visibility

Position of the sun (which varies throughout the day and season of the year)

Angle of observation

Background upon which it is projected or contrast

- The observer's visual capacity

Visual acuity, sight defects, etc.

- The mark's parameters

Dimensions

Shape

Colour

Top-mark

"Recognition distance" is defined as the distance at which a signal should be sighted by a navigator, so that he has sufficient manoeuvre time to perform a complete change of course of 360º.

Evidently, every kind of vessel will require a different margin of manoeuvre. Hence, the daymark should be designed to have a sufficient recognition distance for the vessels which usually navigate through the area.

The book "Normas Técnicas sobre Obras e Instalaciones de Ayudas a la Navegación" (Technical Standards Book on Aid to Navigation Works and Installations) published in 1986 considers this distance to be of ten times a vessel's length. Other works, however, like the Normas Técnicas del Proyecto de Mejora de las Señales Marítimas de España (Technical Standards on the Spanish Maritime Signal Improvement Project) of 1967, make reference to 7.5 times a vessel's length.

As a general recommendation, daymarks should preferably be designed to have a recognition distance of ten times the length of the vessels which usually navigate through the area.

As is to be supposed, in restricted water areas, such as the inside of ports, channels, etc., it will not be necessary to size the signal for such distance of ten lengths, as the vessels will never be at a great distance from the signal due to the area's physical limitations. In this case, the daymark should be sized so that it is sufficiently visible from any inner area from where it could be sighted by a vessel irrespective of its size.

On the other hand, signals with a recognition distance of less than 500 m should never be installed on aids with a nominal luminous range equivalent to or less than three nautical miles, nor with a recognition distance of less than 1,000 m for nominal luminous ranges exceeding such value.

The recognition distance of a daymark can be improved through the use of binoculars. These can be of great help, especially in recognising a sign's top-marks and letters, as these elements will not have a sufficient size to be recognised at the same distance as the rest of the mark due to construction reasons. These elements, as we will see further below, will serve to confirm the type of signal.

In the case of the leading lines, the recognition distance should cover at least the leading line's entire area of use.

# 2.1  Geographic range

This is the maximum distance at which an object can be seen due to the earth's curvature. As most daymarks will be seen at relatively short distances, the earth's curvature will hardly ever limit the daytime range of a signal.

Hm = Height of the mark in metres with regard to sea level

ho = Height of observation in metres with regard to sea level

D   = Geographic range in nautical miles

2.08 = Atmospheric refraction (in the geographical area)

# 2.2 Meteorological visibility

The atmosphere is the transmission channel used to perceive the signal. Hence, the distance at which the signal will be seen is conditioned by the meteorological visibility existing at that moment, which can be highly variable.

# 3.  COLOUR OF A MARITIME SIGNAL'S SURFACE

IALA Recommendation E-108 deals with the issue of colours and their use on marine aids to navigation. This recommendation sets forth the ordinary colours (white, black, green, red, blue and yellow) as well as the fluorescent colours (orange, green, red and yellow) which should be used. Each colour is identified by its chromaticity coordinates (x, y and z) in the CIE chromaticity diagram.  This diagram sets out the chromaticity regions recommended for use on marine aids to navigation.

| Chromaticity regions allowed for ordinary colours | Chromaticity regions allowed for fluorescent colours. |  |
| --- | --- | --- |

Additionally, the visibility characteristics of the colours used on marine aids to navigation are described in this IALA Recommendation, a summary of which is set out in the following table:

Properties of the different colours

| Colour | Visibility | Identification of the colour |
| --- | --- | --- |
| White | Very good (high contrast) under clear sky conditions with direct or lateral lighting against all kinds of onshore backgrounds. Poor (low contrast) when seen against a clear sky or against the sea. | Poor when backlit, the object can be seen as black. |
| Black | Good when seen against the sky. Poor when seen against a background of dark-coloured or shaded land. | Good. The colour does not change under different levels of lighting. |
| Red | Very good. Good contrast when seen against the sky and against the coast | Poor when backlit. The object's colour is confused with black |
| Green | Not good. Fluorescent colours are better, but these degrade more easily and have a shorter life span of < 1 year. | Poor colour recognition. Easily confused with vegetation. |
| Yellow | Not very good, especially against pale-coloured backgrounds. | Difficult colour recognition, is confused with white. Good identification if combined with black. |
| Blue | Difficult to recognise at a distance due to its high atmospheric absorption. | Risk of being confused with the sea and a blue sky. |

Colours are identified in the market by their RAL number (international standard which identifies a colour's chromatic characteristics). The catalogue of RAL numbers is regularly updated with the inclusion of new colours. There are currently 213 classic RAL numbers.

The latest IALA Recommendation, Recommendation E-108, recommends using the following RAL numbers:

Ordinary colours recommended by IALA

| NUMBER | NAME | LUMINANCE (β) |
| --- | --- | --- |
| RAL 3028 | Pure red | > 13% |
| RAL 6037 | Pure Green | > 15% |
| RAL 1023 | Traffic Yellow | > 50% |
| RAL 2008 | Bright Orange Red | > 25% |
| RAL 5019 | Capri Blue | > 7% |
| RAL 9016 | Traffic White | > 80% |
| RAL 9017 | Traffic Black | < 1% |

Fluorescent colours recommended by IALA

| NUMBER | NAME | LUMINANCE (β) |
| --- | --- | --- |
| RAL 3024 | Luminous Red | > 25% |
| RAL 6038 | Luminous Green | > 25% |

Taking into account that the daymark should have as much contrast as possible against the background, the RAL number of the colours described above may be changed to improve their visibility, as long as their chromaticity coordinates remain within the regions recommended by IALA. Depending on the surrounding's conditions, the colour of the seawater, the most usual atmospheric conditions, the area's vegetation, etc., the same colour hue may be more visible in one place than in another.

In order to facilitate colour recognition, mainly in the case of lateral marks, it is recommended that the lower part of the signal should be painted white. Contrast with the surroundings will thus be enhanced. Likewise, the letters or numbers (obligatory in the case of channel buoys) which identify the signal should be painted in colours which contrast with the signal's colour such as, for instance, black on white, white on red, etc.

Only the dimensions of the part painted in the colour corresponding to the type of signal should be used to calculate a daymark's useful surface area. In the case of the figure below, this area would be Hc.

Examples of how contrast can be enhanced

The use of fluorescent or high-visibility colours greatly enhances the daytime recognition of a signal, but care should be taken to ensure that these colours lie within the chromaticity regions recommended by IALA.

The installation of a top-mark corresponding to the signal's type greatly enhances the recognition of signals which navigators tend to see with a backlight at certain times of the day. This is especially advisable in case of lateral signals, which usually do not have a top-mark, apart from buoys.

How the colours of a same mark are seen differently depending of the viewing point with respect of the sun

The use of reflecting materials is also advisable in approach channels and even illuminating the mark at night to enhance the mark's recognition under low-light conditions. In this regard, it should be noted that Recommendation E-106 deals with the use of retroreflecting materials and IALA Guideline 1061 with the illumination of structures.

Examples of daymark illumination at night

# 4.  SHAPE

Although the IALA Maritime Bouyage System does not specify a specific "shape" for daymarks, in the case of floating aids it is advisable to use the specific "shape" corresponding to each type of signal (in other words, cylinders, cones, spheres, etc.) in order to facilitate their identification by the navigator. This is particularly advisable for lateral marks or safe water marks.

In any event, the shape of the signal being incompatible with its colour should be avoided. This means, for example, that a conical buoy painted in red may never be used as a port-side lateral mark (region A).

Buoys with fins, forming the silhouette corresponding to the type of mark

In general terms, it should be taken into consideration that svelte shapes are recognised better and provide the following advantages:

-  They have greater geographic range.

-  They offer greater contrast against ground-level objects.

-  They are differentiated better when backlit.

# 5.  ASPECTS TO TAKE INTO ACCOUNT IN THE DESIGN OF STRUCTURES

The dimensions needed for the daytime part of the signal will condition the structure's design and affect the following aspects:

- Special care should be taken when dealing with the daymarks of signals marking out harbour mouths. Their size should be suitable to provide a sufficient recognition distance for the size of the ships to which they are meant to provide service.

- Where the signal is comprised of a post supporting equipment, it should be of a sufficient diameter (never less than 400 mm), so that it has a suitable recognition distance once it has been painted in the colour of the corresponding mark.

- The mark's height should be enough for it to have a sufficient recognition distance in addition to reaching the necessary geographic range. It should be noted that the height of where the mark is situated and not the structure's height should be taken into consideration. Additionally, any possible obstacles that may hinder the mark's visibility – like vehicles parked in front of the signal, road traffic, vegetation, vessels, etc. – and reduce the mark's effective size should be foreseen.

- Where it is not possible to achieve a sufficient diameter in the post or spar, a board or boards should be affixed thereto to make them look like a solid body at a certain distance. Structures to support equipment may also be designed with handrails or guardrails made up of panels or boards to increase the visible surface area.

Examples of how the visible surface area of structures can be increased.

- Rectangular panels painted in the signal's specific colour should be affixed onto latticework structures with small cross-section profiles which cannot achieve a sufficient painted surface area, so that they appear like a solid body at a distance.

- In the case of marks painted in several colours, the calculation should be made by taking each colour individually and the surface area's recognition distance will be that of the most restrictive colour. This means that marks having several colours (cardinal marks, modified lateral marks, etc.) should be larger for the same recognition distance.

- Where the message is comprised of several colours, an effort should be made to ensure each colour's visible surface area has a similar size in keeping with recommended proportions. Due to this, an effort should be made not to use buoy hulls as part of the message. The colours forming the message should preferably be painted solely on the pillar or spar. Should it be necessary to use the buoy's hull as part of the message, its painted surface area should be of a similar size as the other surface areas comprising the message.

This only applies to signals whose message is made of horizontal bands of different colours, as is the case of cardinal marks, modified lateral marks and isolated danger marks. This will not apply to all other cases (marks having a single colour or vertical stripes, such as special lateral marks, safe water marks and wreck marks).

- Where the signal will only be used in a single direction (as in the case of leading lines), the installation of dayboards may be sufficient. However, where it will be seen from any direction, the mark should project the same surface area in all directions.

- Wind force should be taken into account for dayboard structures having large surface areas. In order to reduce wind resistance, panels made up of boards having gaps between them can be built to let the wind through. Additionally, if these are slanted as per the drawing below, their wind resistance will be reduced.

| Figure 1 | Figure 2 |
| --- | --- |

- Over the last few years, there has been a trend of replacing traditional leading lines with directional lights that provide daytime service. Wherever possible, it is advisable to maintain the old leading line's front and rear daymarks for this kind of signal even when the light is used during the day. Where this is not possible, the daymark on which the directional light is installed should be of a sufficient size to ensure the navigator can easily identify the place where the light is located at daytime.

- IALA Guideline 1023 on the Design of Leading Lines. The dayboard should be painted with a white area or another colour which offers an adequate contrast, so that an appropriate alignment between the two marks is ensured.

- There are no standards defining the colours of these boards, but it should be ensured that they offer the maximum possible contrast with the background. Draughtboard patterns and many stripes and bands are not advisable, as these reduce each colour's individual surface area and therefore its recognition distance. The upper part of the dayboard should be slanted by around 4º or 5º in order to avoid bird depositions falling on the dayboard's surface. Where possible, this practice should also be extended to include other kinds of daymarks.

| Recognition distance (Nautical miles); Minimum width, W (in metres). 0.25 0.5 0.75 1 2 3 4 5; 0.2 0.4 0.6 0.8 1.5 2.3 2.9 3.7. |  |
| --- | --- |

Dimensions of leading light dayboards according to U.S. Coast Guard Manuals (CIM 16500).

# 6.  DIMENSIONS

Before discussing the dimensions a daymark should have, what was mentioned above concerning the little research conducted on this issue and the large variations there may be depending on observation conditions should be recalled. Due to this fact, it is difficult to find studies that coincide and equivalent multiplication factors on the basis of the documents consulted. The method proposed herein is a simple model to apply and we feel it will meet the requirements of most cases, though the final dimensions may be changed on the basis of the results of any field observations conducted locally on the signal.

The second edition of the IALA NAVGUIDE Manual (1993) indicates that the recognition distance will be 860 times the height of the mark. In addition, the German Marine Aids to Navigation Service has recently verified through different field studies that objects having simple shapes can be seen at up to a distance of 1,000 times the mark's height under favourable conditions. The German Marine Aids to Navigation Service uses the term MDR, or maximum recognition distance, and has established that a mark can be seen if there is an angle of 3' or 0.05º between it and the observer. Hence, a simple shape measuring 1 metre in height can be seen under favourable conditions at a distance 1,000 times its height, that is to say at a distance of 1 Km.

Simple shape

Eye

Ф = 3' (= 3°/60)

Distance

Height



As in most cases the surface area or silhouette will not be a simple shape (a square, a circle, etc.), a good approach would be to take the equivalent square side as the value for the mark's height for the purposes of calculation. In other words, this would be the side of an imaginary square having the same surface area as the mark under consideration.

In order to calculate this equivalent square, a surface area having a height of twice its width with a maximum height limit of seven times its width should preferably be used for large structures like lighthouses, etc. The rest of the mark's surface area which exceeds this proportion should not be used in the calculation.

For instance, in the case of a dayboard measuring 1 x 2:

Surface area the equivalent square side is:

Recognition distance:

# 6.1  Dimensions of a daymark with a complex message

What has been described up to this point applies to daymarks having simple coloured shapes like a rectangle, a square, a circle, etc. Where the message is more complex, as in the case of a signal's numbers, letters, etc., the distances at which these messages can be seen will be less.

| In order to know the distance at which a message made up of numbers or letters can be seen, a simple rule used in many forms of advertising is applied, which consists of multiplying the character's height by 500 where a character's width is one-fifth its height. Furthermore, sufficiently large margins should also be sized so that the number is not confused with the rest of the mark. |  |
| --- | --- |

As regards colours, these should have sufficient contrast to be distinguished at a distance, such as black on white, white on dark blue, etc. It should also be taken into account that small cap letters are seen better than capitalised letters.

# 7.  PROCESS OF IDENTIFYING THE MESSAGE ON A DAYMARK

When a navigator approaches a signal, for instance a buoy, the first thing he will recognise is its silhouette or shape and then its colour. He will subsequently recognise the top-mark and finally its numbers or letters. Thus, the process of identifying a signal's complete message goes through different stages or, in other words, different elements of the signal are recognised depending on the distance. The stages of this process are as follows:

Detection distance: The navigator sees an object, but he will usually not be able to deduce its shape or colour and will not know what kind of signal it is. This distance is usually above 1,000 times the object's height in the case of simple shapes (with approximately a height double its width).

Recognition distance: At this distance, the navigator can perfectly discern the type of mark it is. He can distinguish its colours well and will be able to differentiate its top-mark. This distance will be between 1,000 and 850 times the equivalent square side, a value which can vary depending on the shape. In the case of top-marks at the same distance, a square can be confused with a cylinder of a similar size, but not with a triangle, which can be distinguished better at a greater distance. If it has several colours, each will have to be dealt with individually.

Identification distance: Numbers or letters which serve to identify the signal should be distinguishable without a shadow of doubt at this distance like, for example, the number of a channel buoy. In the case of numbers or letters, the navigator will be approximately at a distance of 500 times their height.

These values are evidently approximate values. In practice, they may vary greatly, depending on atmospheric transparency, the observer's visual acuity, the sun's position, etc. In addition, it should also be added that the use of binoculars facilitates other signal recognition and identification processes once the signal has been detected.

Dimensions of standard buoys

For instance, two buoys like the ones in the drawing above of an identical size will have the following approximate characteristics regarding their detection, recognition and identification distances on the basis of whether factors of 1,000 or 500 are applied:

| ELEMENTS | DETECTION DISTANCE | RECOGNITION DISTANCE | IDENTIFICATION DISTANCE |
| --- | --- | --- | --- |
| Lateral Mark | >1800 m. | 1800 m. | 160 m. |
| Cardinal Mark | >1800 m. | 1100 m. | 160 m. |
| Conical Top-Mark |  | >500 m. |  |

# 8.  TOP-MARKS

When discussing the recognition distance of a daymark, we are referring to the signal as a whole. The top-mark is an element which serves to help in confirming the signal's message through the use of binoculars or when one is at a distance below the recognition distance of the signal as a whole.

Top-marks are mandatory on all kinds of buoys, except for some lateral or safe water marks, whose specific shape is perfectly defined in their design.

The use of top-marks is also obligatory on cardinal and isolated danger marks situated on land or on fixed structures.

# 8.1  Colour

The same as for daymarks.

# 8.2  Shape

The shapes used in the IALA-MBS include cones, spheres, cylinders, X shapes (cross of Saint Andrew) and crosses (wreck or emergency buoys).

The shapes must be solid or built with several flat plates assembled at right angles, so that they have the appearance of a solid body at any point from which they may be seen.

Where top-marks are used with buoys, it should be ensured that the buoy keeps all of its characteristics regarding floatability, centre of gravity, oscillation period, etc. when a top-mark is installed.

# 8.3  Dimensions

The shapes should be as large as possible, taking into account the limitations on the element's stability and wind resistance, as it is situated at a great distance from the flotation line. The IALA Maritime Buoyage System Guidelines and the Libro de Normas Técnicas of 1986 describe the relationship between the base of the mark and the diameter of the buoy's hull on the flotation line (between 15% and 25% depending on the type of mark).

It is advisable that the X value of the shape's base should never be less than 50 cm.

| CARDINAL MARKS |  |
| --- | --- |
| SPECIAL MARKS | PORT STARBOARD |
| SPECIAL AND WRECK MARKS | WRECK SPECIAL MARK |
| SAFE WATER And ISOLATED DANGER MARKS | SAFE WATER ISOLATED DANGER |

Top-mark proportions

"x" should never be less than 50 cm.

Daymark recognition distances according to the Manual de Sinalização Náutica of the Brazilian lighthouse service.

| GEOMETRIC SHAPE | RECOGNITION DISTANCE |
| --- | --- |
|  | DR =1250 h |
|  | DR =833 d |
|  | DR =833 L |
|  | DR =500 h |

# 9.  OTHER DAYMARKS

# 9.1  Marking of bridges

The daytime marking of bridges is covered by IALA Recommendation O-113. According to this recommendation, the limits of the navigable area are defined by:

To starboard, a green triangle on a white square

To port, a red rectangle on a white square

Best point of passage, a white circle with red vertical stripes

Where possible, the starboard and port marks should be painted on the bridge's pillars or alternatively dayboards depicting their shape should be affixed.

# 9.2 Marking of quays, dolphins, floating structures, etc.

In some cases, it may be worthwhile painting the shape of lateral daymarks at the end of quays. In such cases, the background should be painted white, as in the case of bridge marks, if there is not enough contrast with the quay's colour.

| Other kinds of structures, like some jetties or dolphins, should have slanted bands painted on them in colours offering sufficient contrast between them and which avoid the possibility of being confused with a lateral mark such as, for instance, black-yellow, orange-black, etc. In general terms, any fixed or floating offshore structure, whether it has its own signal or not, should be painted in colours offering a good contrast against the background. |  |
| --- | --- |

# 10.  CONCLUSIONS AND RECOMMENDATIONS

As has been seen, many factors are involved when calculating a daymark which will exert an influence on the signal's recognition distance. The following aspects should therefore be taken into consideration:

- When sizing a signal, it should be required that the minimum dimensions of the recognition distance thus set at least comply with the rule of being visible at an angle of 1 millisteradian or 0.05º (around 1,000 times the equivalent square side). Special care should be taken that these dimensions are complied with and increased, where possible, for harbour mouth and leading line marks, even where they are equipped with a day light.

- In order to calculate the surface area of a signal, a height of up to twice the width and with a maximum limit of a height seven times the width should be used as proportions of the equivalent square side. The rest of the surface area exceeding this proportion should not be taken into account for the purposes of calculation.

- The visibility study should always be conducted on the direction from which the signal is most likely to be seen by navigators.

- Onshore fixed signals which have the possibility of being seen against the light by navigators due to their position should be equipped with a top-mark to improve their identification.

- Calculations should be made for each colour on an individual basis. In other words, for marks having two colours (lateral marks painted with bands, cardinal marks, modified lateral marks, etc.), the recognition distance shall be that of the most restrictive colour's surface area.

- Where bands or stripes are used, these should be as wide as possible to increase the visible surface area of each one of them.

- It is advisable to paint an area of lateral marks in white, preferably one-third of the signal's lower part, in order to enhance contrast and thus their identification. If the lower part is painted in white, the mark's useful surface area is reduced. However, in general terms, this reduction in surface area is offset by an increase in contrast which, as can be seen in the theoretical study below, greatly increases the recognition distance.

- Any possible bird depositions should be avoided as much as possible, as stained surfaces areas may lead to confusion regarding the signal's message. This can be achieved through various methods: by slanting vertical surfaces in the upper part, by making the structure form a salient element in the upper part, by placing wire rods to scare off birds or prevent them from perching, etc.

- Special attention should be placed on offshore floating aids and structures in the freeboard area and areas affected by the tidal range. Neutral-coloured anti-algae paints – grey for instance – should be used and marks should be kept free from incrustations and algae by means of an appropriate maintenance programme, so as not to give rise to confusion by the algae or incrustations looking like a black band. This problem can be especially serious for cardinal marks.

- The surface area needed to attain the range required by the signals should be ensured in the design stage of signal structures. Where the signal is comprised of a post supporting equipment, it should be of a sufficient diameter, so that it has a suitable recognition distance once it has been painted in the colour of the corresponding mark.

- Where it is not possible to achieve a sufficient diameter in the post or spar, boards or fins should be affixed thereto to make it look like a solid body at a certain distance. Equipment support platforms may also be designed with handrails or guardrails made up of panels or boards to increase the visible surface area.

- In the case of latticework structures, it is advisable to affix boards in the signal's specific colour so that they appear like a solid body at a distance.

- Top-marks and numbers painted on signals serve to confirm the type of signal and the message they bear. Obviously the recognition distance of these elements will be less due to construction reasons, though an effort should be made to ensure they are as large as possible.

- The mark's height should be suitable to attain the visual range set for it. We are referring here to the mark's height and not that of the structure. Thus, any possible obstacles which may hinder its visibility should be foreseen, such as: vehicles parked in front of the signal, buildings, road traffic, vegetation, moored vessels, etc. which reduce the effective surface area and consequently the required range.

- As regards shape, using the shape corresponding to each type of signal (cone, rectangle, etc.) will facilitate the signal's identification by the navigator. This is particularly true for navigation channels marked out with floating lateral marks.

- Signal numbers are optional. Buoy numbers in approach channels are obligatory (the buoy number, not the national or international number)

- Top-marks are mandatory in buoys which do not have the specific shape defined for the type of mark.

- Care should be taken to ensure that top-mark supports cast as small a shadow as possible onto lantern solar panels, especially autofeed lanterns equipped with solar panels fitted on the upper part.

- It is advisable to illuminate daymarks and bridge pillars on bridges crossing navigable channels under low-light conditions.

# 11.  REFERENCES:

■ IALA Aids to Navigation Guide (NAVGUIDE) 2nd and 4th editions

■ IALA Maritime Bouyage System Guidelines

■ U.S. Coast Guard Manuals (CIM 16500)

■ Normas Técnicas del Proyecto de Mejora de las Señales Marítimas de España, 1967

■ Libro de Normas Técnicas sobre Obras e Instalaciones de Ayudas a la Navegación, 1986

■ IALA Recommendation E-108 Surface Colours Used as Visual Signals on Aids to Navigation

■ IALA Recommendation E-106 The Use of Retroreflecting Material on Aids to Navigation Marks

■ IALA Guideline No. 1061 on Light Applications for the Illumination of Structures

■ IALA Guideline No. 1023 on The Design of Leading Lines

■ Manual de Sinalizaçāo Náutica, Brazilian Lighthouse Service

■ Marques et alignements de jour, Service des Phares et Balises-Centre d’Études Techniques Maritimes et Fluviales

# ANNEX I: Summary of recommendations on daymarks

| # SUMMARY OF RECOMMENDATIONS ON DAYMARKS |  |
| --- | --- |
| DESIGN | 1,000 times the equivalent square side. |
|  | If necessary, add fins, panels or boards to increase the surface area. |
|  | Design from the viewing angle normally used. |
|  | Calculate colours on an individual basis. |
|  | Where stripes or bands are used, increase their surface area and reduce their number to increase range. |
|  | Sufficient height to remain clear of any obstacles. |
| ENHANCING THE SIGNAL'S RECOGNITION | White area in the lower part of lateral marks to increase contrast. |
|  | Where possible, the signal should have the specific shape of the mark. |
|  | Top-marks and numbers facilitate the signal's recognition. |
|  | Equip onshore signals seen against the light with top-marks. Obligatory for cardinal and onshore isolated danger marks. |
|  | In some cases, it may be advisable maintain the light lit during the day, particularly for leading lines in areas of frequent mist. IALA Guideline No. 1023 recommends it even where the necessity of attaining daytime range means that the dayboards are too large, more than 3 m high. |
|  | Clean up algae on floating aids to avoid them being confused with a black band and keep marks clear of bird depositions. |
| BUOYS | Where the buoy does not have the specific shape corresponding to the type of mark, a top-mark is obligatory. |
|  | Buoy number obligatory in channels (order number in the channel from offshore to onshore, even numbers to port and odd numbers to starboard). |
|  | The specific shape of the mark on the buoy is advisable. |
|  | Care should be taken to ensure the top-mark does not cast a shadow onto lantern panels (also applies to onshore beacons). |

# ANNEX II: Sizing of a daymark (simple shapes)

# Sizing of a daymark (simple shapes)

# This section reflects a theoretical study on the sizing of daymarks drawn up by the French Maritime Signalling Service. This document served as a basis to draw up the standards on this matter set forth in the book "Normas Técnicas" (Technical Standards Book) of 1986, which has been referred to elsewhere herein.

As will be seen below, it is necessary in many cases to have field measurements available on the luminance of objects, the background, the sun and its variations depending on its position, as well as values on atmospheric transparency, etc. in order to calculate contrast. This complicates the calculations and places them beyond our scope in the majority of cases.

# Short-distance contrast

Contrast is the eye's capacity to distinguish an object from the background upon which it is projected. This, along with atmospheric visibility (at long distances), will exert a decisive influence on a daymark's recognition distance, as will be seen further below.

Short-distance contrast C(0) is given by the following relationship:

C(0) = I Lo – LF I

LF

Where:

LO is the object's luminance

LF is the background's luminance

If it is projected against the sky:

CC(0) = I Lo – LC I

LC

Where:

LO is the object's luminance

LC is the sky's luminance

Contrast against land relative to the sky (the signal is projected against land, but the sky above the land is also seen, an area having a greater level of luminance, which will exert an influence on the eye's adaptation and consequently condition the value for contrast):

CS(0) = I LO – LS I

LC

Where:

LO is the object's luminance

LS is the land's luminance

LC is the sky's luminance

Approximate contrast values for CC(0) (contrast against the sky) and CS(0) (contrast against land relative to the sky)

Only the two extreme cases will be taken into consideration, namely a uniformly cloudy sky (described in the book "Normas Técnicas 1986" with tables of luminance values) and a clear sky.

It is considered that the daymark is a flat dayboard of a single colour placed in a vertical position.

Uniformly cloudy sky*:

Where:

Lc= Is the luminance of a uniformly cloudy sky

1= Is the land's luminance factor

2= Is the dayboard's luminance factor depending on its colour

*(Described with tables in the book "Normas Técnicas”, 1986)

Clear sky:

The large number of possible cases depending on the sun's relative position does not allow for tables of direct CC(0) or CS(0) contrast readings to be drawn up. Hence, the values of Lc, Ls and Lo will have to be determined separately.

The sky's luminance Lc in the area of the horizon will depend on the sun's azimuth depending on the time of day and the zenith distance depending on the latitude and season of the year with respect of the mark's point of observation.

The value of Lc will be between the 180 Cd/m2 (at sunset with the sun near the horizon) and 19,000 Cd/m2 with the sun at 0º at midday. The average value for luminance Lc on the horizon in clear weather at these latitudes can be established to be 5,000 or 6,000 Cd/m2.

In order to calculate contrasts, it should be pointed out that if we call the luminance of an object solely due to the sun’s light and the sky Lo(0) (if the object is white, β2=1); we call the luminance of the land Ls(1), supposing it is white and horizontal; and we call the luminance of the land (proportional to β1 and irrespective of the sun’s azimuth) Ls=β1 Ls(1), the mark’s effective luminance would be:

For a white object with the sky as the background:

If the object is not white, we multiply Lo by β2

If the white dayboard is projected against land, the contrast relative to the sky would be:

If the object is not white:

The value of Ls is between 300 Cd/m2 (dark land) and 10,000 Cd/m2  (pale-coloured land with the sun at the zenith) .

Lo is between 1 Cd/m2  (shaded side of the dayboard surrounded by dark land and the sun's azimuth at 140º with respect to the dayboard's normal) and 32,000 Cd/m2  (white dayboard surrounded by white land and the sun at 50º above the horizon beating on the dayboard).

If the mark is projected against the sky as the background, the contrast would be:

If the mark is projected against a background of land, its contrast relative to the sky (due to fact that the level of surrounding illumination will be defined by the sky's illumination) would be:

- Long-distance contrast

Under these conditions, the state of meteorological visibility will exert an influence on the object's appreciation.

If the beacon or signal is projected against the sky, CC(0) the contrast at a distance X is given by:

V= is meteorological visibility in the same units as X (distance)

The contrast against land relative to the sky Cs(0) at a distance X is given by:

Recognition distance of a daymark

Experience shows that two conditions need to be met for a signal to be visible at a distance "x":

1.- The contrast at that distance "x" should be greater than 0.05

Then:

2.- The solid angle at which the signal is observed should be above a limit value, which depends on the distance "x":

Long-distance recognition distance. The angle of vision is less than 1 millisteradian:

K= 0.038. d in metres, X and V in Km.

Short-distance recognition distance. The angle of vision is greater than 1 millisteradian:

X and V in the same units

In these cases, the distance at which it is seen depends on the contrast and not on the size of the mark.

We may also come across the following equations, both of which are the result of simplifying X:

or

Observation

According to this, we can deduce that if an object is seen at an angle greater than or equivalent to one millisteradian:

Ф>1 milliradian = 0,057º and the conditions of contrast and visibility are favourable, a simple shape like, for example, a square measuring 1 metre in height will be seen at 1,000 metres.

Hence, an object can be seen under favourable conditions at a distance of up to around 1,000 times the size of the equivalent square side. The German Marine Aids to Navigation Service uses the term MDR, or maximum recognition distance, and has established that a mark can be seen if there is an angle of 3' or 0.05º between it and the observer. Hence, a simple shape measuring 1 metre in height can be seen under favourable conditions at a distance 1,000 times its height.

Simple shape

Eye

Ф = 3' (= 3°/60)

Distance

Height



| Contrast C(0) | Observation distance (Miles) | Observation distance (Km) | Meteorological visibility (Miles) | Meteorological visibility (Km) | Object's dimensions (Surface area m2) | Object's dimensions (Equivalent square side) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.2 | 0.27 | 0.5 | 10 | 18.52 | 0.052 | 0.227 |
| 0.2 | 0.54 | 1 | 10 | 18.52 | 0.223 | 0.473 |
| 0.2 | 0.81 | 1.5 | 10 | 18.52 | 0.545 | 0.738 |
| 0.2 | 1.08 | 2 | 10 | 18.52 | 1.050 | 1.025 |
| 0.2 | 1.35 | 2.5 | 10 | 18.52 | 1.779 | 1.334 |
| 0.4 | 0.27 | 0.5 | 10 | 18.52 | 0.026 | 0.160 |
| 0.4 | 0.54 | 1 | 10 | 18.52 | 0.112 | 0.334 |
| 0.4 | 0.81 | 1.5 | 10 | 18.52 | 0.272 | 0.522 |
| 0.4 | 1.08 | 2 | 10 | 18.52 | 0.525 | 0.725 |
| 0.4 | 1.35 | 2.5 | 10 | 18.52 | 0.890 | 0.943 |
| 0.6 | 0.27 | 0.5 | 10 | 18.52 | 0.017 | 0.131 |
| 0.6 | 0.54 | 1 | 10 | 18.52 | 0.074 | 0.273 |
| 0.6 | 0.81 | 1.5 | 10 | 18.52 | 0.182 | 0.426 |
| 0.6 | 1.08 | 2 | 10 | 18.52 | 0.350 | 0.592 |
| 0.6 | 1.35 | 2.5 | 10 | 18.52 | 0.593 | 0.770 |
| 0.8 | 0.27 | 0.5 | 10 | 18.52 | 0.013 | 0.113 |
| 0.8 | 0.54 | 1 | 10 | 18.52 | 0.056 | 0.236 |
| 0.8 | 0.81 | 1.5 | 10 | 18.52 | 0.136 | 0.369 |
| 0.8 | 1.08 | 2 | 10 | 18.52 | 0.263 | 0.512 |
| 0.8 | 1.35 | 2.5 | 10 | 18.52 | 0.445 | 0.667 |

0.000

0.200

0.400

0.600

0.800

1.000

1.200

1.400

1.600

1.800

2.000

0.5

1

1.5

2

2.5

Range  (Km)

Surface Area (m2)

Contrast 0.2

Contrast 0.4

Contrast 0.6

Contrast 0.8

Daytime visibility of an object seen at an angle of less than one millisteradian (d/x<1).

NOTE: C(0) = 1 would correspond to the contrast of a white dayboard against a black background

# Support tools

- The spreadsheet drawn up by IALA employing the method described in Guideline No. 1023 may be used to calculate leading line dayboards and similar applications.

IALA: International Association of Marine Aids to Navigation and Lighthouse Authorities

The IALA NAVGUIDE gives a value of 2.03 for this factor.

CIE: International Commission on Illumination

Some of these colours may not appear in classical RAL charts if they are not recent. This is due to the fact that these colours were recently included (the number of colours was increased from 210 to 213 at the end of 2009).

According to data obtained by field measurements and provided by the French Lighthouse and Beacon Service