AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline ####

On

Standard Format for Electronic Exchange of AtoN Product Information

Edition 1

June 2012

10 rue des Gaudines, 78100

Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  iala-aism@wanadoo.fr       Internet:  www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

Document Revisions	1

Table of Contents	3

Standard Format for Electronic Exchange of AtoN Product Information	5

1	Introduction	5

2	Purpose	6

3	ABBREVIATIONS AND Definitions	6

4	Scope	6

5	General concepts	7

5.1	Product Information File (PIF)	7

5.2	Individual AtoN PIF	9

5.3	AtoN product family PIF	10

5.4	Injection of PIF contents into an asset management system	10

5.5	Extraction of updated PIF contents from an asset management system	11

5.6	PIF structure	11

5.7	PIF validation	11

5.8	Use of PIF contents for modelling of the light signal	12

6	SFEEAPI Specification	12

6.1	AtoN Product Information File XML encapsulation	12

6.2	PIF header tags	12

6.3	The ‘Product’ section	13

6.4	The ‘Manufacturer’ section	15

6.5	The ‘ContactInfo’ section	16

6.6	The ‘OperationalParameters’ section	17

6.7	The ‘PhysicalParameters’ section	18

6.8	The ‘OpticalParameters’ section	19

6.9	The ‘ElectricalParameters’ section	27

6.10	The ‘ConfigurationLog’ section	33

6.11	The ‘LifetimeEventLog’ section	36

6.12	The ‘FailureRepairLog’ section	37

6.13	The ‘CustomContent’ section	38

7	References	38

ANNEX A	SFEEAPI Product Information File TEMPLATE	39

ANNEX B	SFEEAPI product Information File (PIF) DTD for validation	50

ANNEX C	SFEEAPI Product Information File content example	51

Index of Tables

Table 1	PIF Header tags	12

Table 2	Title required	13

Table 3	Title required	15

Table 4	Title required	16

Table 5	Title required	17

Table 6	Title required	18

Table 7	Title required	19

Table 8	Title required	27

Table 9	Title required	33

Table 10	Title required	36

Table 11	Title required	37

Standard Format for Electronic Exchange of AtoN Product Information

# Introduction

In the past, electronic equipment units purchased for building up marine Aid to Navigation (AtoN) site systems were supplied by the manufacturers with accompanying printed instruction manuals where typical functional parameters were stated using manufacturer specific paragraph structure, detail level, and data formats.  At present time, the only difference is that many if not all suppliers provide the same documentation in electronic portable document format (PDF); while this allows transferring the key parameters into other computer software applications by manual copy-and-paste operation, both the way of presentation and detail level remain diverse between different manufacturers of functionally similar equipment.

Various database centric online asset management systems are becoming widely used for keeping the inventory on purchased equipment, operated by the authorities responsible for the provision of marine navigation safety infrastructure (the AtoN and lighthouse administrations).  Once the information on a particular product is entered into such database, it can be used efficiently by all relevant structural units within an organisation; the ‘bottleneck’ is still the time consuming and error prone data input process, requiring careful and tedious processing of manufacturer supplied documentation by human operators.  It is only a matter of time when the AtoN administrations start requiring submission of product data by the manufacturers in electronic formats compatible with the input requirements of the software solutions operated by a particular AtoN administrator to reduce their administrative burden, and to shift the full responsibility for validity of submitted data to the manufacturers.

Both the AtoN administrations and AtoN equipment manufacturers will benefit from establishing a common specification for such information exchange processes - a Standard Format for Electronic Exchange of AtoN Product Information (SFEEAPI).  Considering the widely varying complexity of information processing systems available to different AtoN administrations and equipment manufacturers, it is reasonable to define such standard as voluntary and open, foreseeing the possibility of both manual and automated creation and utilisation of the product information exchanged in SFEEAPI compliant data files – AtoN Product Information Files (PIF).  The Extensible Markup Language (XML; [[2]]), a subset of the Standard Generalized Markup Language ([1]) is identified as a best suitable carrier to serve the above purpose: it provides a simple tagged information exchange format, resulting in small files with clear ASCII text and no embedded harmful content.  A PIF is not a replacement for regular Instructions for Use documents and other manuals possibly provided with an AtoN product; it is rather a concentrate of product parameters that can be utilised in automated information processing systems.  A PIF can point to a user manual, facilitating automated uploading of such and other relevant electronic documents into the information system of any user.

Simple rules and formats for information exchange established by SFEEAPI create preconditions for future interoperability between information systems of AtoN equipment suppliers and users that will benefit both the users and manufacturers, allowing to maintain high quality of product configuration and performance history throughout its usable lifetime.  With the advent of information systems enabling automated AtoN system quality assurance procedures like evaluation of technical suitability of products, failure and availability statistics, automated AtoN site power consumption budget analysis etc., the need for creating a single set of industry wide formatting rules for electronic product documentation is evident.  Once such format is established, development of information systems at the organisations of both the users and the manufacturers can proceed with guaranteed global interoperability.

The SFEEAPI has no direct links with e-Navigation related activities, therefore it does not need to conform to more complicated formatting requirements of the latter as outlined in the IALA Guideline 1072 on ‘AtoN Information Exchange & Presentation’ and in the IHO publication S-100 ‘IHO Universal Hydrographic Data Model’.  Nevertheless, once the product information transfer into an operational database of AtoN equipment is accomplished, such data can be utilized at provision of e-Navigation services as far as applicable.

# Purpose

This Guideline describes an electronic data format intended for preparation of a standardized technical data file with comprehensive AtoN product information, and a method for using such files primarily for passing AtoN product information from equipment suppliers (manufacturers) to equipment users (AtoN administrations) for injection into information systems in a semi-automated process.

The Guideline is intended for AtoN equipment manufacturers and users, helping to set the course on waters most likely unavoidable in the near future.  Implementation of the Guideline by any equipment manufacturer is voluntary, with detail level of usage decided in accordance with the specifics of particular products and feasibility.  Implementation of the Guideline by both the AtoN equipment manufacturers and AtoN administrations will guarantee interoperability of corresponding information systems at exchanging AtoN product information.

# ABBREVIATIONS AND Definitions

| AtoN equipment | Electronic products used within the AtoN infrastructure to establish aids to marine navigation |  |
| --- | --- | --- |
| ASCII DTD | American Standard Code for Information Interchange Document Type Definition; a set of markup declarations that define a document type for SGML family of markup languages |  |
| PIF | Product Information File; an AtoN product data file with the content encoded in UTF-8 (ASCII) text using XML in accordance with the current Guideline |  |
| SFEEAPI | Standard Format for Electronic Exchange of AtoN Product Information as defined within this Guideline |  |
| SGML UTF-8 XML | Standard Generalized Markup Language (ISO 8879:1986) Universal Character Set (UCS) Transformation Format — 8-bit; (ISO/IEC 10646) Extensible Markup Language, a subset of ([2]) | [2] |

# Scope

The scope of this Guideline covers single AtoN signal light products and complete electronic light systems that can be described within a single document with a single set of technical parameters.  An AtoN Product Information File (PIF) utilizing content formatting with XML tags is intended either for a single product or system with a unique serial number, a batch of similar AtoN products, or as a generalization, for a whole clearly defined family of simple products.

The Guideline does not consider other AtoN products in detail at this time, allowing identifying such system components with a limited data set.  The structure of a PIF is open for addition of new equipment categories (product classes), thus the scope may be extended in the future.

# General concepts

## Product Information File (PIF)

A Product Information File (PIF) is a human-legible text file created by the manufacturer of an AtoN product (signal light system), containing technical and administrative information on the subject product and constituting a central vehicle of SFEEAPI.  It contains the information either on a particular product or a product family that is extracted from the manufacturer’s information system within the extent deemed necessary by the manufacturer for the purpose of transferring the information to the purchaser during the sales process.  In addition to technical information, it may contain extended information on the manufacturer and distributors.  It may also contain relevant measurement results, configuration settings and changes, references to other electronic documents like instruction manuals, test protocols, failure reports, etc. Early adopters are expected to fill in a PIF template manually, until the capability to create PIF files in a semi-automated process is implemented in manufacturer’s product information systems.

While it is expected that a PIF is submitted by an equipment manufacturer to the purchaser in the process of sale as a document containing proprietary information to be treated in confidence by the purchaser, it is manufacturer’s choice whether to make any PIF public.  An XML tag ContentsClassification is foreseen for determining the classification status of a particular PIF either as Proprietary or Public.

To enable automated processing, the contents of a PIF is formatted using strictly defined XML tags and rules in accordance with section 6, with each PIF validated by the AtoN equipment manufacturer prior to submission to equipment purchaser (user).  Nevertheless, extent and detail level of the information provided with any product is up to the manufacturer. The only strictly defined data item that must be specified in each PIF is classification of the product, defined by the XML tag ProductClass.  Naturally, a PIF would be useless without the product model number, key performance parameters and at least basic information on manufacturer.

XML files contain textual information in a simple open tagged format which can be displayed as a normal document using web browsers, and processed by several commercial and open source software applications like Microsoft Word and Excel 2007 (not to mention the Notepad).

- Fragment of a PIF template opened in MS Word 2007 for editing with the XML Structure box opened by right clicking on an XML tag and selecting ‘View XML Structure’.

Once created by an AtoN product manufacturer, a PIF is not intended to remain a static dataset for the lifetime of the product, although it may become a one-time product information delivery vehicle in some cases.  Such a file is merely a temporary form of existence for structured product information items assembled for the process of transition from one information system to another, valid at the time of creation.  Certain time-stamped sections allow for updating the PIF with new information accumulated in the course of a product’s life cycle in the user’s information system.  Updating a PIF with performance history of a product, such as measurements of luminous intensity or power consumption, significant events, performance warnings and failures is foreseen for a scenario where a user returns a product to a manufacturer for repair or re-adjustment, accompanied by an up-to-date PIF.

All data inside PIFs are expected to be provided in the English language, with real numeric values provided using only decimal notation with the period (‘.’) as a decimal separator.  All parameter values are provided using a single set of measurement units that are shown at the end of the corresponding XML tags separated by underline to guarantee unambiguous understanding.

A comprehensive PIF template for an AtoN light system is provided in ANNEX A.  While an equipment manufacturer may choose to use only some of the data fields of this template when preparing PIFs for its AtoN products, it is recommended to leave the full set of tags in the file in order to make the future updating simpler.  In the future, ability to generate a PIF is expected to be built into the database centric product/asset information management systems at both the manufacturers as well as on the user’s side.

## Individual AtoN PIF

A manufacturer may decide to supply a dedicated PIF with a complex AtoN product or system.  In such case, the file usually contains individual product settings and results of the measurements conducted on the particular product for which such PIF was created.

In addition to detailed information on the primary AtoN product, a PIF can carry administrative information and documentation on secondary AtoN products – components of an AtoN system built around the primary product.  It is meaningful to join up in one PIF file the information on AtoN equipment units that would work together as a clearly defined AtoN site system with a single total power consumption figure.

It is recommended to create the name of an individual AtoN PIF file in the following format with components separated by ‘_’:

C_MMMMMM_SSSSSS_PIF_YYYYMMDD.XML

The characters have the following meaning with names abbreviated by the manufacturer if needed:

C – company name of the manufacturer (maximum of 20 characters);

MMMMMM – AtoN product model name or number (maximum of 20 characters);

SSSSSS - AtoN product serial number (maximum of 20 characters);

PIF – indication of a Product Information File;

YYYYMMDD – full date of file creation/change in accordance with ISO 8601:2004 ([3]);

.XML – file name extension enabling ease of processing by XML capable software applications.

When the contents of a PIF are changed due to updating of administrative or technical information, the date in the file name shall be changed to reflect the date of last changes made.

## AtoN product family PIF

In case of simple AtoN products, it may not be feasible to prepare an individual PIF for supply with each product sold.  For such product families, a single PIF can be created by the product manufacturer, containing only a general description and typical or MIN/MAX parameter values.

It is recommended to create the name of an AtoN product family PIF file in the following format with components separated by ‘_’:

C_MMMMMM_ SSSSSS-SSSSSS_PIF_YYYYMMDD.XML

The characters have the following meaning:

C – company name of the manufacturer (maximum of 20 characters);

MMMMMM – AtoN product model name or number (maximum of 20 characters);

SSSSSS-SSSSSS – optional for supply of a batch of AtoN products: range of serial numbers separated by ‘-’ (maximum of 41 characters);

PIF – indication of a Product Information File;

YYYYMMDD – full date of file creation/change in accordance with ISO 8601:2004 ([3]);

.XML – file name extension enabling ease of processing by XML capable software applications.

Once an AtoN product supplied with a family PIF requires manufacturer’s attention again due to a failure or a need for re-adjustment/configuration, the user can create an individual PIF based on the existing family PIF, updating it with subject product serial number and a time-stamped entry describing the cause of returning the product to the manufacturer.

## Injection of PIF contents into an asset management system

Once a manufacturer has created a PIF for an AtoN product sold, and has transferred it to the purchaser either by e-mail, on a CD, a flash disk or using some other suitable media, the purchaser can inject the product information into the asset management systems database of the organisation that purchased the AtoN product.  It is important to transfer all relevant files referenced inside a PIF into same directory prior to injection into an asset management system.

The injection process is expected to proceed in semi-automated manner in the future:

- The asset management system operator activates the feature of the information system for AtoN PIF injection and points the system to the PIF;

- The system opens the PIF, scans the contents, performs validity checking of the known fields, and presents the operator with key AtoN product information, requesting a confirmation to proceed with injection of product data and accompanying files;

- Upon receiving a confirmation from the operator, the system creates necessary database records for the purchased equipment, distributing the product parameters and other information in the PIF for which processing capability is built into the system in accordance with existing rules (storing the whole PIF as well is expected).

The asset management software systems handling AtoN product information are expected to operate using relational databases that can link the information from PIF’s with any other kind of relevant AtoN related data as necessary, allowing the product information to ‘live’ inside the system.  Entries accumulated over time for each product may consist of measurement results, re-location and re-configuration information, observations and failure reports.

When the existing asset management system database of a user provides the necessary space for data items available from PIFs, it should not be a prohibitively costly task to arrange for additional functionality for PIF contents injection to be implemented.  Until the asset management systems do not provide a facility for such semi-automated PIF processing, operators can access the PIF contents using XML capable web browser, editor or spreadsheet software to simply copy and paste the necessary information.

## Extraction of updated PIF contents from an asset management system

When a product that has been purchased with accompanying PIF requires manufacturer’s attention, it can be returned to the manufacturer with the updated PIF contents extracted from the user’s asset management system, provided that such advanced functionality is available.

Once such PIF is returned to the manufacturer, it can be updated again when the product has been processed and returned to the user.  In such cases, it is important to avoid duplicate injection of product information into databases.  The system must be capable of recognizing a familiar product based on the serial number, complementing its records on the system only with information items that have changed (for instance, new user manual, additional test or failure analysis reports).

## PIF structure

An AtoN PIF consists of a header declaring compliance to a particular SFEEAPI version, classification of the information and creator information, followed by the information sections:

- The ‘Product’ section provides main product information: model, description, serial number, class, manufacturing date, warranty term etc.;

- The ‘Manufacturer’ section provides manufacturer specific information: company name, location, contact information, etc, but also purchase contract and purchaser references;

- The ‘ContactInfo’ section provides contact information of other relevant parties: distributors, technical support, maintenance and repair organizations, etc.;

- The ‘OperationalParameters’ section provides product life cycle related information;

- The ‘PhysicalParameters’ section provides static product information;

- The ‘OpticalParameters’ section provides light source information;

- The ‘ElectricalParameters’ section provides electrical information;

- The ‘ConfigurationLog’ section provides information on product or system settings and configuration management including data on relevant configuration items;

- The ‘LifeTimeEventLog’ section provides lifelong significant event information;

- The ‘FailureRepairLog’ section provides product failure and repairs information;

- The ‘CustomContent’ section allows providing additional information using custom-defined data items.

Last three sections are optional at the time of product delivery. Each section is described in detail in section 6.

## PIF validation

In order to enable validation of the PIF content for correctness of all data items filled in by the manufacturer by automated information processing systems, a separate Document Type Definition (DTD) text file containing all data type definitions for the XML schema used in the PIF documents is provided as Annex 2 of this Guideline.

This external DTD will be associated with the PIFs and must be available to the information processing systems at least at the time of injecting a PIF.  It can be used as a specification for creating the software modules for amending the existing asset management systems with PIF content injection capability.  Nevertheless, filling in the PIFs manually is expected to dominate during a time period of considerable length after publication of this Guideline.

## Use of PIF contents for modelling of the light signal

If a manufacturer has provided all the necessary information in sections OpticalParameters, ElectricalParameters and Configuration, it becomes possible to use the contents of a PIF for modelling of the behaviour of the light signal generated by subject AtoN product or system.  With tools expected to emerge in the future, such modelling would allow to check the visibility range of the light signal overlaid on a chart throughout the product lifetime, with variations in temperature and power supply voltage conditions.

# SFEEAPI Specification

## AtoN Product Information File XML encapsulation

All data items in an AtoN Product Information File (PIF) are encapsulated using a set of XML tags defined for the SFEEAPI purposes.  Such tags are strictly defined and may not be changed in the process of preparation and processing of PIFs, except for incrementing the tag numbers separated by a dot and adding full groups where applicable.  In general, only the data item contents surrounded by tags may be modified. Filling in certain tags is a strict requirement while most of data items are intended for voluntary utilization.  While most of the data items accept contents of characters at any reasonable volume, certain data items require strict compliance with data range, formatting, or pre-determined words for specifying equipment classes, etc.

The file starts with standard XML format specifier line:

<?xml version=‘1.0’ encoding=‘UTF-8’ standalone=‘yes’?>

The actual data file contents are contained between the following root tags:

<ATON_PRODUCT_INFORMATION_FILE> .... </ATON_PRODUCT_INFORMATION_FILE>

The following paragraphs explain the use of PIF XML tags in detail, section by section.  Blocks of tags forming a subsection are shown on light pink background.

## PIF header tags

- PIF Header tags

| Tag | Type | Req? | Range | Description |
| --- | --- | --- | --- | --- |
| SFEEAPI_Compliance | Char | Y |  | Identification of the SFEEAPI version to which the contents of the PIF complies. |
| ContentsClassification | Char | Y | strict | Determines whether the PIF is intended for publication or aimed strictly at the addressee (customer). Two classes are valid values: either Public or Proprietary. |
| ValidSinceDate | Char | Y |  | Provides a start date of validity of specifications in the PIF (YYYYMMDD). |
| PIFCreator | Char |  |  | Name (and version) of the software system that generated the PIF. |

## The ‘Product’ section

- Title required

| Tag | Type | Req? | Range | Description |
| --- | --- | --- | --- | --- |
| Product |  | Y |  | The section of data items presenting the administrative information on the AtoN product or system. A single Product section must exist inside any PIF. |
| ModelName | Char | Y |  | AtoN product’s model name as designated by equipment manufacturer. |
| ModelModification | Char |  |  | AtoN product’s model name extension (where applicable). |
| PartOrderingNumber | Char |  |  | A manufacturer assigned number that is expected to be used at the time of ordering identical products (where applicable). |
| ModelDescription | Char | Y |  | Short description of the AtoN product. |
| SerialNumber | Char | Y |  | Serial number of the AtoN product (used in case of individual PIF), or an array of serial numbers separated by ‘;’ in case of several products supplied with a single ‘batch PIF’. |
| ManufacturingDate | Char |  |  | Date of manufacturing of the primary AtoN product (YYYYMMDD). |
| ShippingDate | Char |  |  | Date of shipping to the customer of the primary AtoN product (YYYYMMDD). |
| WarrantyTermMonths | Integer | Y |  | Length of the AtoN products warranty period in full months. |
| CountryOfOrigin | Char |  |  | Country of origin (preferential origin) of the product. If this data item is empty, the primary product described by a PIF is expected to have the preferential origin of the manufacturer’s country of residence. |
| ProductClass | Char | Y | strict | Defines the class of the primary product for entry into asset management systems database. May be extended in the future. One of the following classes must be used: BuoyLight, CompactLight, Floodlight, OmnidirectionalBeacon, RangeLight, RotatingBeacon, SectorLight, Other |
| LightSourceClass | Char | Y | strict | Defines the light source technology of the product for entry into asset management systems database. One of the following classes must be used: Discharge, Incandescent, LED, Other |
| ProductPicture |  |  |  | A block of tags encapsulating reference to product pictures accompanying the PIF or available from the web URL. Several ProductPicture blocks may be used in a PIF. |
| PictureURL | Char |  |  | Either a file name of a product picture accompanying the PIF, or a web URL. |
| Caption | Char |  |  | Caption for the picture file. |
| UserManualURL | Char |  |  | Either a file name of a the user manual for the primary AtoN product accompanying the PIF, or a web URL (PDF expected). |
| ProductDocument |  |  |  | This subsection (block of tags) encapsulates reference to product related documents accompanying the PIF or available from the web URL. Several ProductDocument blocks may be used in a PIF. |
| DocClass | Char |  | strict | Specifies the class (type) of the document to be uploaded into the asset management system. One of the following classes must be used: Drawing, Instructions, Protocol, Specification, Template, Other. |
| DocName | Char |  |  | Title of the document to be uploaded. |
| DocURL | Char |  |  | Name of the document file accompanying the PIF, or web URL. |
| DocSize_kB | Integer |  |  | Size of the document. |
| Comment | Char |  |  | General comment on AtoN product / system. |

## The ‘Manufacturer’ section

- Title required

| Tag | Type | Req? | Range | Description |
| --- | --- | --- | --- | --- |
| Manufacturer | Char | Y |  | This section provides information on the manufacturer supplying the product described in the PIF. |
| Name | Char | Y |  | Name of the manufacturer |
| RegistrationNumber | Char | Y |  | Company registration number of the manufacturer |
| VATNumber | Char |  |  | VAT number of the manufacturer |
| StreetAddress | Char |  |  | Street address of the manufacturer |
| ZipCode | Char |  |  | Postal code of the manufacturer |
| City | Char |  |  | City of residence of the manufacturer |
| Country | Char | Y |  | Country of residence of the manufacturer. Unless the CountryOfOrigin tag is filled, this will also be considered a country of origin of the primary product. |
| Website | Char |  |  | Website address of the manufacturer. |
| Email | Char |  |  | Email address of the manufacturer. |
| Phone | Char |  |  | Phone number of the manufacturer. |
| ContactPerson | Char |  |  | Designated contact person of the manufacturer. |
| Comment | Char |  |  | General comment entered by the manufacturer. |
| ContractReference | Char | Y |  | Reference to a purchase order number or a contract between the Manufacturer and purchasing organisation. |
| PurchaserReference | Char | Y |  | Reference to a purchaser or designated contact person for a relevant contract. |

## The ‘ContactInfo’ section

- Title required

| Tag | Type | Req? | Range | Description |
| --- | --- | --- | --- | --- |
| ContactInfo |  |  |  | This section can provide information on several entities (companies) related to the AtoN product described in the PIF in several functions. |
| Contact |  |  |  | This subsection encapsulates contact information of an entity (company) related to the AtoN product in a function defined in ContactClass; repeated for as many times as necessary (for different distributors, service providers, etc.). |
| ContactClass | Char |  | strict | Specifies the class (type) of the contact in relation to the AtoN product. One of the following classes must be used: Distributor, Maintenance, Repair, Techsupport, Other. |
| Name | Char |  |  | Contents of the tags is similar to the section Manufacturer. |
| RegistrationNumber | Char |  |  |  |
| VATNumber | Char |  |  |  |
| StreetAddress | Char |  |  |  |
| ZipCode | Char |  |  |  |
| City | Char |  |  |  |
| Country | Char |  |  |  |
| Website | Char |  |  |  |
| Email | Char |  |  |  |
| Phone | Char |  |  |  |
| ContactPerson | Char |  |  |  |
| Comment | Char |  |  |  |

## The ‘OperationalParameters’ section

- Title required

| Tag | Type | Req? | Range | Description |
| --- | --- | --- | --- | --- |
| OperationalParameters |  | Y |  | This section describes the operational parameters of the AtoN product. It can have repeated tag blocks describing maintenance. |
| ServiceLife_Y | Integer |  |  | Service life estimate. |
| MTBF | Integer |  |  | Mean Time Between Failures as estimated by the manufacturer, [h]. |
| EnvironmentTemperatureMin_DegC | Integer | Y |  | Lowest temperature of the operating environment. |
| EnvironmentTemperatureMAX_DegC | Integer | Y |  | Highest temperature of the operating environment. |
| VibrationTolerance_g | Real |  |  | Vibration tolerance level. |
| Maintenance |  |  |  | Time-directed maintenance related requirement blocks may be repeated as many times as required to convey a full maintenance schedule. Information on situational maintenance is not conveyed in PIF. |
| Task |  |  |  | Nested block for description of specific maintenance tasks. |
| OperationalHours | Real |  |  | Provided when the task needs to be performed after certain amount of operational hours have been reached. |
| CalendarHours | Real |  |  | Provided when the task needs to be performed after certain amount of calendar hours have been reached. |
| Description | Char |  |  | Short description of the maintenance task. |
| InstructionsURL | Char |  |  | File name (if supplied on the same media with the PIF) or web URL of the maintenance task instructions. |
| DocSize_kB | Integer |  |  | Size of the document. |
| Comment | Char |  |  | Any clarification of physical parameters of the product. |

## The ‘PhysicalParameters’ section

- Title required

| Tag | Type | Req? | Range | Description |
| --- | --- | --- | --- | --- |
| PhysicalParameters |  | Y |  | This section describes the physical parameters of the AtoN product. It has no repeated tag blocks; this information is expected to remain static for the lifetime of the product. |
| Height_mm | Integer |  |  | Maximum vertical dimension of the product. |
| Width_mm | Integer |  |  | Maximum width (horizontal dimension) of the product. |
| Depth_mm | Integer |  |  | Maximum depth (length) of the product. |
| Diameter_mm | Integer |  |  | Maximum diameter of the product (usually same as width). |
| MountDescription | Char |  |  | Short description of the mounting arrangement of the product. |
| BodyMaterial | Char |  |  | Dominating material(s) used for making the primary AtoN products body (enclosure). |
| Weight_g | Integer | Y |  | Weight of the AtoN product. |
| Comment | Char |  |  | Any clarification of physical parameters of the product. |

## The ‘OpticalParameters’ section

- Title required

| Tag | Type | Req? | Range | Description |
| --- | --- | --- | --- | --- |
| OpticalParameters | Char | Y |  | This section provides detailed information on optical parameters of the AtoN product. It encapsulates sub-subsections providing typical values of light signal generation related parameters claimed by the manufacturer, and consecutive measurement values taken at different times of the AtoN products life cycle. |
| LensDiameter_mm | Integer |  |  | Diameter of the lens or width of the light emitting aperture of the light source. |
| LensHeight_mm | Integer |  |  | Height of the lens or light emitting aperture of the light source. |
| LensTiers | Integer |  |  | Number of tiers (vertical levels) used for generating the light signal; number of lens blocks. |
| FocalHeight_mm | Integer |  |  | Focal height of the AtoN light measured from the bottom surface of the product. |
| LensMaterial | Char |  |  | Material used for making the lenses. |
| Typical |  | Y |  | This subsection provides typical values for light signal generation related parameters claimed by the manufacturer. Due to the significant differences in light generating components, individual products may deviate from the typical parameters. In case when same AtoN product is used for provision of daytime and night time light signals, two Typical subsections should be used to describe both modes of operation, explained under Comment. |
| Sector |  | Y |  | This sub-subsection provides typical values for light signal generation related parameters for each sector of an AtoN light (a single sector in case of omnidirectional light distribution). It is repeated as many times as needed to describe all sectors. |
| SectorWidth_deg | Real | Y |  | Typical width of a particular light signal sector [º]. |
| SectorStart_deg | Real |  | 0 360 | Beginning marker of the light signal sector [º]. |
| SectorEnd_deg | Real |  | 0 360 | End marker of the light signal sector [º] |
| SectorUncertainty_deg | Real |  |  | Typical uncertainty limits of the horizontal sector specification, ± [º]. |
| BoundaryResolution_min | Real |  |  | Full sector boundary resolution specification for the sector lights, minutes of arc [‘]. |
| TypicalIntensity_cd | Real | Y |  | Typical luminous intensity of the light signal in the current sector. |
| IntensityUncertainty_cd | Real |  |  | Typical uncertainty limits of the luminous intensity specification, ± [cd]. |
| IntensitySpecTemperature_deg | Real |  |  | Temperature for which the luminous intensity specification is given, [ºC]. |
| IntensityDegradation_h | Integer |  |  | Estimation of operational hours at a given maximum ambient temperature until degradation of light output to 70% of the initial luminous intensity, [h]. |
| DegradationTempLimit_degC | Integer |  |  | Maximum ambient temperature for which the degradation of light output to 70% of the initial luminous intensity is provided, [ºC]. |
| SectorUniformity | Real |  |  | Uniformity specification for the current sector, ± [%]. |
| VerticalDivergenceFWHM_deg | Real | Y |  | Typical vertical divergence of the light signal at 50% level of typical intensity in the current sector, [º]. |
| VerticalDivergenceFWTM_deg | Real |  |  | Typical vertical divergence of the light signal at 10% level of typical intensity in the current sector, [º]. |
| VerticalDivergenceUncertainty | Real |  |  | Typical uncertainty limits of the vertical divergence angle specification, ± [º]. |
| SectorColour | Char | Y |  | Colour of the light signal emitted in the current sector. |
| Wavelength_nm | Real |  |  | Typical dominating wavelength of the light signal emitted in the current sector, [nm]. |
| WavelengthUncertainty_nm | Real |  |  | Uncertainty of the wavelength specification for the current sector, [nm]. |
| SpectrumColourCoordinates |  |  |  | This is a tag block specifying typical colour coordinates in CIE 1931 colour space of the light signal emitted in the current sector, and colour temperature [K] in case of white light. |
| ColourCoordinate_x | Real |  |  | Colour Coordinate x. |
| ColourCoordinate_x_Uncertainty | Real |  |  | Uncertainty of Colour Coordinate x specification, ± [ ]. |
| ColourCoordinate_y | Real |  |  | Colour Coordinate x. |
| ColourCoordinate_y_Uncertainty | Real |  |  | Uncertainty of Colour Coordinate x specification, ± [ ]. |
| ColourTemperature_K | Real |  |  | Colour temperature [K] in the sector (in case of white light). |
| ColourTemperatureUncertainty_K | Real |  |  | Colour temperature uncertainty, ±[K]. |
| SpectrumProtocolReference | Char |  |  | This block of tags refers to a spectrum measurement protocol file that a manufacturer has based its specification on. |
| DescriptionNR | Char |  |  | Name and number of the spectrum measurement protocol. |
| ProtocolURL | Char |  |  | File name (if supplied on the same media with the PIF) or web URL of the manufacturers’ spectrum measurement protocol. |
| HorIntensityDistribution_degCd | Char |  |  | An array of sequential values of angular positions in degrees [º] and luminous intensity values in [cd] that represent a typical horizontal distribution of the luminous intensity of the light source. Value separator is semicolon ‘;’. |
| VertIntensityDistribution_degCd | Char |  |  | An array of sequential values of angular positions in degrees [º] and luminous intensity values in [cd] that represent a typical vertical distribution of the luminous intensity of the light source. Value separator is semicolon ‘;’. |
| IntensityTempCurve_DegCd | Char |  |  | An array of sequential values of temperatures in degrees [ºC] and luminous intensity values in [cd] that represent a typical temperature dependence curve of the luminous intensity of the AtoN product or system. Value separator is semicolon ‘;’. |
| IntensityProtocolReference | Char |  |  | This block of tags refers to a luminous intensity measurement protocol file that a manufacturer has based its specification on. |
| DescriptionNR | Char |  |  | Name and number of the luminous intensity measurement protocol. |
| ProtocolURL | Char |  |  | File name (if supplied on the same media with the PIF) or web URL of the manufacturers luminous intensity measurement protocol. |
| DocSize_kB | Integer |  |  | Size of the document. |
| Comment | Char |  |  | Any clarification of typical optical parameters of the AtoN product. |
| MeasDateTime | Char |  |  | Time stamp of the latest measurements that the typical optical parameter specification is based upon: YYYYMMDDTHHMM (hours and minutes may be skipped). |
| LightMeasurement |  |  |  | This subsection provides actual measurement values of light signal generation related parameters, either provided by the manufacturer or conducted by other parties for different modes of operation, or consecutive measurements during the products lifetime. |
| Sector |  |  |  | This sub-subsection provides measurement values for light signal generation related parameters for each sector of an AtoN light (a single sector in case of omnidirectional light distribution). It is repeated as many times as needed to describe all sectors. |
| SectorWidth_deg | Real |  |  | Measured width of a particular light signal sector [º]. |
| SectorStart_deg | Real |  | 0 360 | Measured beginning of the light signal sector [º]. |
| SectorEnd_deg | Real |  | 0 360 | Measured end of the light signal sector [º]. |
| SectorUncertainty_deg | Real |  |  | Uncertainty of the horizontal sector measurement, ± [º]. |
| BoundaryResolution_min | Real |  |  | Full sector boundary resolution of a sector lights measured, minutes of arc [‘]. |
| AverageIntensity_cd | Real |  |  | Measured average luminous intensity of the light signal in the current sector, [cd]. |
| IntensityUncertainty_cd | Real |  |  | Uncertainty of the luminous intensity measurement, ± [cd]. |
| IntensityMeasTemperature_deg | Real |  |  | Temperature at which the luminous intensity was measured, [ºC]. |
| SectorUniformity | Real |  |  | Determined light signal uniformity in the current sector, ± [%]. |
| VerticalDivergenceFWHM_deg | Real |  |  | Measured vertical divergence of the light signal at 50% level of average intensity in the current sector, [º]. |
| VerticalDivergenceFWTM_deg | Real |  |  | Measured vertical divergence of the light signal at 10% level of average intensity in the current sector, [º]. |
| VerticalDivergenceUncertainty | Real |  |  | Uncertainty of the vertical divergence angle measurement, ± [º]. |
| SectorColour | Char |  |  | Colour of the light signal emitted in the current sector. |
| Wavelength_nm | Real |  |  | Measured dominating wavelength of the light signal emitted in the current sector, [nm]. |
| WavelengthUncertainty_nm | Real |  |  | Uncertainty of the wavelength measurement in the current sector, [nm]. |
| SpectrumColourCoordinates |  |  |  | This is a tag block specifying measured colour coordinates in CIE 1931 colour space of the light signal emitted in the current sector, and colour temperature [K] in case of white light. |
| ColourCoordinate_x | Real |  |  | Measured Colour Co-ordinate x. |
| ColourCoordinate_x_Uncertainty | Real |  |  | Uncertainty of Colour Co-ordinate x measurement, ±[ ]. |
| ColourCoordinate_y | Real |  |  | Measured Colour Coordinate x |
| ColourCoordinate_y_Uncertainty | Real |  |  | Uncertainty of Colour Co-ordinate x measurement, ±[ ]. |
| ColourTemperature_K | Real |  |  | Measured colour temperature [K] in the sector (in case of white light). |
| ColourTemperatureUncertainty_K | Real |  |  | Colour temperature measurement uncertainty, ±[K]. |
| SpectrumProtocolReference | Char |  |  | This block of tags refers to a spectrum measurement protocol file that is a source of values for data items in this section. |
| DescriptionNR | Char |  |  | Name and number of the spectrum measurement protocol. |
| ProtocolURL | Char |  |  | File name (if supplied on the same media with the PIF) or web URL of the spectrum measurement protocol. |
| DocSize_kB | Integer |  |  | Size of the document. |
| HorIntensityDistribution_degCd | Char |  |  | An array of sequential measurement values of angular positions in degrees and luminous intensity values in cd that represent the horizontal distribution of the luminous intensity of the light source. Value separator is semicolon ‘;’. |
| VertIntensityDistribution_degCd | Char |  |  | An array of sequential measurement values of angular positions in degrees [º] and luminous intensity values in [cd] that represent the vertical distribution of the luminous intensity of the light source. Value separator is semicolon ‘;’. |
| IntensityTempCurve_DegCd | Char |  |  | An array of sequential measurement values of temperatures in degrees [ºC] and luminous intensity values in [cd] that represent a typical temperature dependence curve of the luminous intensity of the AtoN product or system. Value separator is semicolon ‘;’. |
| IntensityProtocolReference | Char |  |  | This block of tags refers to a luminous intensity measurement protocol file that is a source of values for data items in this section. |
| DescriptionNR | Char |  |  | Name and number of the luminous intensity measurement protocol. |
| ProtocolURL | Char |  |  | File name (if supplied on the same media with the PIF) or web URL of the luminous intensity measurement protocol. |
| DocSize_kB | Integer |  |  | Size of the document. |
| Comment | Char |  |  | Any clarification of measured optical parameter values. |
| MeasDateTime | Char |  |  | Time stamp of the latest measurements in this subsection: YYYYMMDDTHHMM (hours and minutes may be skipped). |

## The ‘ElectricalParameters’ section

- Title required

| Tag | Type | Req? | Range | Description |
| --- | --- | --- | --- | --- |
| ElectricalParameters |  | Y |  | This section provides detailed information on electrical parameters of the AtoN product. It encapsulates sub-subsections providing typical values of power consumption related parameters claimed by the manufacturer, and consecutive measurement values taken at different times of the AtoN products life cycle. |
| Typical |  | Y |  | This section provides typical electrical parameters of the AtoN product or system. |
| CurrentType | Char | Y | strict | Type of the current for which the primary AtoN product is intended: AC, DC, Universal. |
| PowerSupplyType | Char |  |  | Type of the power supply: Linear, step up converter, step down converter. |
| ReversePolarityProtection | Char |  |  | Description of reverse polarity protection employed in the AtoN product or system. |
| SurgeProtection | Char |  |  | Description of surge protection employed in the AtoN product or system. |
| PowerSupplyInrushCurrent_A | Real |  |  | Description of the inrush current level pulled from the power supply during cold starts; value in [A] if given. |
| VoltageNominal_V | Real | Y |  | Nominal power supply voltage of the primary AtoN product or system, [V]. |
| VoltageMinimum_V | Real |  |  | Minimum power supply voltage of the primary AtoN product or system, [V]. |
| VoltageMaximum_V | Real |  |  | Maximum power supply voltage of the primary AtoN product or system, [V]. |
| LightSourceRegulation | Char |  |  | Description of the methods employed by the AtoN product or system for regulation of the light intensity: temperature or aging dependent control of LED current, etc. |
| FlashSynchronisation | Char |  |  | Description of the capability of the AtoN product or system to synchronize the flashing with other AtoN lights. |
| ExtControlOfFlashing | Char |  |  | Description of the means necessary for applying external control of flashing the light signal of the AtoN product or system. |
| MonitoringCapability | Char |  |  | Description of the means necessary for applying remote monitoring to the AtoN product or system. |
| ControlCapability | Char |  |  | Description of the capability of the AtoN product to accept remote control. |
| SolarCellType | Char |  |  | Type of the solar cells if used. |
| SolarCellPower_W | Real |  |  | Power output capability of solar cells if used, [W]. |
| SolarCellNumber | Integer |  |  | Provides the number of (external) solar cells used by the product/system. |
| BatteryType | Char |  |  | Provides the model number of batteries if used in the system (each can be described in more detail at the ConfigurationItems section if necessary). |
| BatteryCapacity | Real |  |  | Capacity of the battery (if used), [Ah]. |
| BatteryTechnology | Char |  |  | Desribes battery technology. |
| BatteryVoltage_V | Real |  |  | Specifies voltage of a single battery used, [V]. |
| BatteryNumber | Integer |  |  | Provides the number of batteries used by the product/system. |
| FlasherType | Char |  |  | Type of a flasher if present. |
| LightSensorType | Char |  |  | Type of a light sensor if present. |
| PowerConsumption |  | Y |  | This subsection provides a specification for power consumption related parameters for an AtoN light product or system. In case when same AtoN product is used for provision of daytime and night time light signals, two subsections should be provided to describe both modes of operation. |
| PowerConsumptionFlashDay_W | Real | Y |  | Typical (nominal) power consumption in flash in day time operating mode, [W]. |
| PowerConsumptionFlashNight_W | Real | Y |  | Typical (nominal) power consumption in flash in night time operating mode, [W]. |
| PowerConsumptionEclipseDay_W | Real |  |  | Typical (nominal) power consumption in eclipse in day time operating mode, [W] |
| PowerConsumptionEclipseNight_W | Real |  |  | Typical (nominal) power consumption in eclipse in night time operating mode, [W]. |
| PowerConsumptionStandby_W | Real |  |  | Typical (nominal) power consumption during standby state, [W]. |
| PowerConsumption_VW | Real |  |  | An array of sequential measurement values of power supply voltages in [V] and corresponding power consumption in [W] that represent the consumption behaviour in flash at the temperature specified below throughout the range allowed by the manufacturer. Value separator is semicolon ‘;’. |
| PowerSpecTemperature_deg | Real |  |  | Temperature for which the power consumption specification is given, [ºC]. |
| VoltageNomCurrentConsumption_A | Real | Y |  | Typical current consumption at the nominal power supply voltage, [A]. |
| VoltageMinCurrentConsumption_A | Real |  |  | Typical current consumption at power supply voltage minimum, [A]. |
| VoltageMAXCurrentConsumption_A | Real |  |  | Typical current consumption at power supply voltage maximum, [A]. |
| PowerFactorAC | Real |  |  | Power factor specification for an AtoN product operating from AC utility power. |
| CurrentUncertainty_A | Real |  |  | Typical limits for current consumption specification, [A]. |
| VoltageUncertainty_V | Real |  |  | Typical limits for power supply voltage specification, [V]. |
| ProtocolReference |  |  |  | This block of tags refers to a power consumption measurement protocol file that a manufacturer has based its specification on. |
| DescriptionNR | Char |  |  | Name and number of the power consumption measurement protocol. |
| ProtocolURL | Char |  |  | File name (if supplied on the same media with the PIF) or web URL of the manufacturers power consumption measurement protocol. |
| DocSize_kB | Integer |  |  | Size of the document |
| Comment | Char |  |  | Clarifications of power consumption specifications. |
| MeasDateTime | Char |  |  | Time stamp of the latest measurements in this subsection: YYYYMMDDTHHMM (hours and minutes may be skipped). |
| PowerMeasurement |  |  |  | This subsection provides measurement values for power consumption related parameters of an AtoN light. It is repeated as many times as needed to describe all operating modes or consecutive measurements during products life cycle. It can be linked by comment to a particular Configuration. |
| PowerConsumptionFlashDay_W | Real |  |  | Measured nominal power consumption in flash, [W]. |
| PowerConsumptionFlashNight_W | Real |  |  | Measured nominal power consumption in flash in night time operating mode, [W]. |
| PowerConsumptionEclipseDay_W | Real |  |  | Measured nominal power consumption in eclipse in day time operating mode, [W]. |
| PowerConsumptionEclipseNight_W | Real |  |  | Measured nominal power consumption in eclipse in night time operating mode, [W]. |
| PowerMeasTemperature_deg | Real |  |  | Temperature at which the power consumption was measured, [ºC]. |
| VoltageNomCurrentConsumption_A | Real |  |  | Measured current consumption at the nominal power supply voltage, [A]. |
| VoltageMinCurrentConsumption_A | Real |  |  | Measured current consumption at power supply voltage minimum, [A]. |
| VoltageMAXCurrentConsumption_A | Real |  |  | Measured current consumption at power supply voltage maximum, [A]. |
| PowerFactorAC | Real |  |  | Measured power factor of an AtoN product operating from AC utility power. |
| CurrentUncertainty_A | Real |  |  | Uncertainty of current consumption measurement, [A]. |
| VoltageUncertainty_V | Real |  |  | Uncertainty of power supply voltage measurement, [V]. |
| ProtocolReference | Char |  |  | This block of tags refers to a power consumption measurement protocol file that a manufacturer has based its specification on. |
| DescriptionNR | Char |  |  | Name and number of the power consumption measurement protocol. |
| ProtocolURL | Char |  |  | File name (if supplied on the same media with the PIF) or web URL of the manufacturers’ power consumption measurement protocol. |
| DocSize_kB | Integer |  |  | Size of the document. |
| Comment | Char |  |  | Clarifications of power consumption measurements. |
| MeasDateTime | Char |  |  | Time stamp of the latest measurements in this subsection: YYYYMMDDTHHMM (hours and minutes may be skipped). |

## The ‘ConfigurationLog’ section

- Title required

| Tag | Type | Req? | Range | Description |
| --- | --- | --- | --- | --- |
| ConfigurationLog |  |  |  | This section provides information on the configuration and settings of an AtoN product or system at the time of leaving the manufacturers factory, and after re-configurations in the course of its usable lifetime (if recorded). It allows to convey basic information on related AtoN system configuration items using a recurring tag block. |
| Configuration |  |  |  | This subsection is a core element of the ConfigurationLog, offering a way to record time stamped AtoN system configuration changes. It includes a nested tag block for AtoN system configuration items. The first occurrence of this subsection corresponds to factory configuration and settings of an AtoN product or system. |
| Name | Char |  |  | Name of a specific configuration of an AtoN product or system. The first configuration at the time of supply to the user should be named ‘Factory’. The following re-configurations in the course of products usable lifetime should be given short names not exceeding 20 characters – for instance, DateTime strings (YYYYMMDDTHHMM), if no suitable names can be found. |
| FirmwareVersion | Char |  |  | Firmware version number of the key configuration item of the AtoN system (where applicable). |
| FlashCharacter | Char |  |  | Flashing character applied to the AtoN product or system. |
| PWMDutyCycle | Real |  |  | Pulse Width Modulation (PWM) duty cycle applied to the AtoN product, [%]. |
| NominalRangeNight_NM | Real |  |  | Night time nominal range of the AtoN product resulting from luminous intensity, flash character and PWM duty cycle applied. |
| NominalRangeDay_NM | Real |  |  | Daytime nominal range of the AtoN product resulting from luminous intensity, flash character and PWM duty cycle applied. |
| PowerAutonomy_h | Integer |  |  | Provides the extent of power supply autonomy for products or systems utilizing battery backup (both solar and utility power systems), [h]. |
| PowerCableLength_m | Real |  |  | Length of the power cable supplied with the AtoN product or system. |
| PowerCableType | Integer |  |  | Description of the power cable supplied with the AtoN product or system. |
| LightSwitchSetting | Char |  |  | Description of the setting of the AtoN products light switch. |
| DayNightSwitchingLevel_lx | Real |  |  | Configured ambient light threshold for day/night intensity switching, [lx]. |
| LightSwitchingLevelON_lx | Real |  |  | Configured lower ambient light threshold for switching the AtoN light ON, [lx]. |
| LightSwitchingLevelOFF_lx | Real |  |  | Configured higher ambient light threshold for switching the AtoN light OFF, [lx]. |
| DayNightSwitchingDelay_s | Real |  |  | Time delay for Day/Night mode switching (also for night time only flashing), [s]. |
| IntensityDegradation |  |  |  | Block of tags providing information on yearly light source intensity degradation depending on configuration settings. |
| Colour | Char |  |  | Affected colour of the AtoN product. |
| YearlyPercentage | Real |  |  | Percentage of yearly luminous intensity degradation of the light source when operated at current configuration settingsat at +20ºC, [%]. |
| SourceTemperature_degC | Real |  |  | Estimated operating temperature of the light source (semiconductor junction of an LED) at current configuration settings at +20ºC, [ºC]. |
| ConfigurationItem |  |  |  | Block of tags providing information on configuration items forming and AtoN system in cooperation with the primary AtoN product. |
| ConfAction | Char |  | strict | Configuration management related action other than maintenance or repair: Factory / Added / Removed / Other. |
| ModelName | Char |  |  | System configuration items model name or number. |
| ModelModification | Char |  |  | System configuration items model modification. |
| PartOrderingNumber | Char |  |  | Manufacturers (spare) part ordering number for a particular configuration item. |
| ModelDescription | Char |  |  | Short description of the configuration item. |
| SerialNumber | Char |  |  | Serial number of the configuration item. |
| Document |  |  |  | Block of tags for inclusion of relevant documents for AtoN system configuration items. |
| DocClass | Char |  | Strict | Class (type) of a particular document: Drawing / Instructions / Protocol / Specification / Template / Other. |
| DocName | Char |  |  | Name of a particular document. |
| DocURL | Char |  |  | Name of the document file accompanying the PIF, or web URL. |
| DocSize_kB | Integer |  |  | Size of the document. |
| Comment | Char |  |  | Description of a particular configuration. |
| ChangeDateTime | Char |  |  | Time stamp of the latest configuration change: YYYYMMDDTHHMM. |

## The ‘LifetimeEventLog’ section

- Title required

| Tag | Type | Req? | Range | Description |
| --- | --- | --- | --- | --- |
| LifetimeEventLog |  |  |  | This section allows providing information on the significant events occurring in the life cycle of an AtoN product or system which is not directly related to failures or configuration changes. It can be used for conveying maintenance and observation reports. All events, configuration changes and measurements can be interrelated within the system by nearly identical time stamps. |
| Event |  |  |  | This recurring subsection contains tags for description of events and inclusion of related documents. |
| EventType | Char |  |  | Partly strict event types are introduced to define most of common events that could be recognized by any SFEEAPI compliant asset management system. Any significant event type that needs to be described using the Event section can be introduced by a user, providing relevant type and documentation: Observation / Maintenance / Relocation / EndofLife. |
| EventIdentNo | Char |  |  | Identification number of an event in users information system. |
| EventTimeStamp | Char |  |  | Time stamp of the latest event: YYYYMMDDTHHMM. |
| EventInformation | Char |  |  | Detailed information on the event. |
| Document |  |  |  | A block of tags for inclusion of documents related to a specific event. |
| DocName | Char |  |  | Name of a particular document. |
| DocURL | Char |  |  | Name of the document file accompanying the PIF, or web URL. |
| DocSize_kB | Integer |  |  | Size of the document. |

## The ‘FailureRepairLog’ section

- Title required

| Tag | Type | Req? | Range | Description |
| --- | --- | --- | --- | --- |
| FailureRepairLog | Integer |  |  | This section allows communicating information on product failures between the user and the manufacturer. |
| Failure |  |  |  | This recurring subsection contains tags for description of failure cases and repairs and inclusion of related documents. |
| FailureClass | Char |  | strict | Critical, NonCritical. |
| FailureIdentNo | Char |  |  | Identification number of the failure case in the users information system. |
| FailureTimeStamp | Char |  |  | Time stamp of the current failure: YYYYMMDDTHHMM. |
| FailureDescription | Char |  |  | Detailed information on the product failure. |
| RepairIdentNo | Char |  |  | Identification number of the failure case in the manufacturers information system. |
| RepairInformation | Char |  |  | Detailed failure analysis information. |
| RestoreTimeStamp | Char |  |  | Time stamp of the products operation restored: YYYYMMDDTHHMM. |
| Document |  |  |  | A block of tags for inclusion of documents related to the current failure case. |
| DocName | Char |  |  | Name of a particular document. |
| DocURL | Char |  |  | Name of the document file accompanying the PIF or web URL. |
| DocSize_kB | Integer |  |  | Size of the document. |
| Comment | Char |  |  | Users’ final comments on a particular failure case. |

## The ‘CustomContent’ section

| Tag | Type | Req? | Range | Description |
| --- | --- | --- | --- | --- |
| CustomContent |  |  |  | This section allows define new data items for provision of information which cannot be communicated by existing XML tags. |
| CustomData |  |  |  | This recurring subsection contains tags for description of custom data items and provision of corresponding values. A reasonable amount of custom data items may be used in a PIF file. |
| CustomDataItemName | Char |  |  | Name of a custom data item. |
| Value | Char |  |  | Value of a custom data item. |
| Description | Char |  |  | Description and purpose of a custom data item. |
| DateTime | Char |  |  | Time stamp related to a custom data item: YYYYMMDDTHHMM. |

# References

- ISO 8879:1986, Standard Generalized Markup Language.

- Extensible Markup Language (XML) Specification: http://www.w3.org/XML/

- ISO 8601:2004(E), Data elements and interchange formats — Information interchange — Representation of dates and times

- SFEEAPI Product Information File TEMPLATE

File: AtoN_PIF_TEMPLATE_20111013.XML

To fill in a Product Information File (PIF) template manually using MS Word (version 2007 or above), proceed as follows:

- Copy the template and rename it for use with a particular AtoN product, entering the product model number (NNNNNN), serial number (SSSSSS), and date of creation (YYYYMMDD):

NNNNNN_SSSSSS_YYYYMMDD.XML

- Move the mouse cursor on the new PIF file icon, open the right button menu, scroll down to the option ‘Open With’ and pick ‘Microsoft Office Word’.

- Once the document has been opened, move the mouse cursor to the data item input fields between XML tags (shown on a grey background and encapsulated in curved red frames) and enter the necessary information.  In case of tags with pre-filled selection of strict classification strings, leave only a single relevant class (like BuoyLight) and delete the rest together with separators (‘/’).

Note: To get a better overview of the document, it is recommended to move the mouse cursor on any of the XML tags, open the right button menu, and select the option ‘View XML structure’; this action results in the XML structure navigation window opening on the right side of the screen.

- When the necessary information is entered, check it for correctness and save the file.  Ignore the warning informing about the XML file in namespace {Unknown} being overwritten and proceed with overwriting the existing file by activating ‘Yes’.

- Browse to the icon of the PIF that was just created and double click on it to open the file in default XML viewer which is most likely a web browser.

Note: When it becomes necessary to duplicate some of the tag blocks like sector descriptions or references to documents, this can be accomplished directly inside MS Word environment by selecting and copying such tag blocks – just pay attention that a full block is copied and inserted into correct section, just below the preceding similar tag block.  Entering proper date (and time where applicable) information helps to keep the entries in order.

Contents of the PIF template start on the next page.

<?xml version=‘1.0’ encoding=‘UTF-8’ standalone=‘yes’?>

<ATON_PRODUCT_INFORMATION_FILE>

<SFEEAPI_Compliance>0.1</SFEEAPI_Compliance>

<ContentsClassification>Public/Proprietary</ContentsClassification>

<ValidSinceDate> </ValidSinceDate>

<PIFCreator> </PIFCreator>

<Product>

<ModelName> </ModelName>

<ModelModification>  </ModelModification>

<PartOrderingNumber> </PartOrderingNumber>

<ModelDescription> </ModelDescription>

<SerialNumber> </SerialNumber>

<ManufacturingDate> </ManufacturingDate>

<ShippingDate> </ShippingDate>

<WarrantyTermMonths> </WarrantyTermMonths>

<CountryOfOrigin> </CountryOfOrigin>

<ProductClass>BuoyLight/Floodlight/OmnidirectionalBeacon/RangeLight/RotatingBeacon/SectorLight/Other</ProductClass>

<LightSourceClass>Discharge/LED/Incandescent/Other</LightSourceClass>

<ProductPicture>

<PictureURL> </PictureURL>

<Caption></Caption>

</ProductPicture>

<UserManualURL> </UserManualURL>

<ProductDocument>

<DocClass>Drawing/Instructions/Protocol/Specification/Template/Other </DocClass>

<DocName> </DocName>

<DocURL> </DocURL>

<DocSize_kB> </DocSize_kB>

</ProductDocument>

<Comment> </Comment>

</Product>

<Manufacturer>

<Name> </Name>

<RegistrationNumber> </RegistrationNumber>

<VATNumber> </VATNumber>

<StreetAddress> </StreetAddress>

<ZipCode> </ZipCode>

<City> </City>

<Country> </Country>

<Website> </Website>

<Email> </Email>

<Phone> </Phone>

<ContactPerson> </ContactPerson>

<Comment> </Comment>

<ContractReference> </ContractReference>

<PurchaserReference> </PurchaserReference>

</Manufacturer>

<ContactInfo>

<ContactClass>Distributor/Maintenance/Repair/TechSupport/Other </ContactClass>

<Name> </Name>

<RegistrationNumber> </RegistrationNumber>

<VATNumber> </VATNumber>

<StreetAddress> </StreetAddress>

<ZipCode> </ZipCode>

<City> </City>

<Country> </Country>

<Website> </Website>

<Email> </Email>

<Phone> </Phone>

<ContactPerson> </ContactPerson>

<Comment> </Comment>

</ContactInfo>

<OperationalParameters>

<ServiceLife_Y> </ServiceLife_Y>

<MTBF> </MTBF>

<IngressProtectionClass> </IngressProtectionClass>

<EnvironmentTemperatureMin_DegC> </EnvironmentTemperatureMin_DegC>

<EnvironmentTemperatureMAX_DegC> </EnvironmentTemperatureMAX_DegC>

<VibrationTolerance_g> </VibrationTolerance_g>

<Maintenance>

<Task>

<OperationalHours> </OperationalHours>

<CalendarHours> </CalendarHours>

<Description> </Description>

<InstructionsURL> </InstructionsURL>

<DocSize_kB> </DocSize_kB>

</Task>

</Maintenance>

<Comment> </Comment>

</OperationalParameters>

<PhysicalParameters>

<Height_mm> </Height_mm>

<Width_mm> </Width_mm>

<Depth_mm> </Depth_mm>

<Diameter_mm> </Diameter_mm>

<MountDescription> </MountDescription>

<LensMaterial> </LensMaterial>

<BodyMaterial> </BodyMaterial>

<Weight_g> </Weight_g>

<Comment> </Comment>

</PhysicalParameters>

<OpticalParameters>

<LensDiameter_mm> </LensDiameter_mm>

<LensHeight_mm> </LensHeight_mm>

<LensTiers> </LensTiers>

<FocalHeight_mm> </FocalHeight_mm>

<LensMaterial> </LensMaterial>

<Typical>

<Sector>

<SectorWidth_deg> </SectorWidth_deg>

<SectorStart_deg> </SectorStart_deg>

<SectorEnd_deg> </SectorEnd_deg>

<SectorUncertainty_deg> </SectorUncertainty_deg>

<BoundaryResolution_min> </BoundaryResolution_min>

<TypicalIntensity_cd> </TypicalIntensity_cd>

<IntensityUncertainty_cd> </IntensityUncertainty_cd>

<IntensitySpecTemperature_deg> </IntensitySpecTemperature_deg>

<IntensityDegradation_h> </IntensityDegradation_h>

<DegradationTempLimit_degC> </DegradationTempLimit_degC>

<SectorUniformity> </SectorUniformity>

<VerticalDivergenceFWHM_deg> </VerticalDivergenceFWHM_deg>

<VerticalDivergenceFWTM_deg> </VerticalDivergenceFWTM_deg>

<VerticalDivergenceUncertainty> </VerticalDivergenceUncertainty>

<SectorColour> </SectorColour>

<Wavelength_nm> </Wavelength_nm>

<WavelengthUncertainty_nm> </WavelengthUncertainty_nm>

<SpectrumColourCoordinates>

<ColourCoordinate_x> </ColourCoordinate_x>

<ColourCoordinate_x_Uncertainty> </ColourCoordinate_x_Uncertainty>

<ColourCoordinate_y> </ColourCoordinate_y>

<ColourCoordinate_y_Uncertainty> </ColourCoordinate_y_Uncertainty>

<ColourTemperature_K> </ColourTemperature_K>

<ColourTemperatureUncertainty_K> </ColourTemperatureUncertainty_K>

<SpectrumProtocolReference>

<DescriptionNR> </DescriptionNR>

<ProtocolURL> </ProtocolURL>

<DocSize_kB> </DocSize_kB>

</SpectrumProtocolReference>

</SpectrumColourCoordinates>

</Sector>

<HorIntensityDistribution_degCd> </HorIntensityDistribution_degCd>

<VertIntensityDistribution_degCd> </VertIntensityDistribution_degCd>

<IntensityTempCurve_DegCd> </IntensityTempCurve_DegCd>

<IntensityProtocolReference>

<DescriptionNR> </DescriptionNR>

<ProtocolURL> </ProtocolURL>

<DocSize_kB> </DocSize_kB>

</IntensityProtocolReference>

<Comment> </Comment>

<MeasDateTime> </MeasDateTime>

</Typical>

<LightMeasurement>

<Sector>

<SectorWidth_deg> </SectorWidth_deg>

<SectorStart_deg> </SectorStart_deg>

<SectorEnd_deg> </SectorEnd_deg>

<SectorUncertainty_deg> </SectorUncertainty_deg>

<BoundaryResolution_min> </BoundaryResolution_min>

<AverageIntensity_cd> </AverageIntensity_cd>

<IntensityUncertainty_cd> </IntensityUncertainty_cd>

<IntensityMeasTemperature_deg> </IntensityMeasTemperature_deg>

<SectorUniformity> </SectorUniformity>

<VerticalDivergenceFWHM_deg> </VerticalDivergenceFWHM_deg>

<VerticalDivergenceFWTM_deg> </VerticalDivergenceFWTM_deg>

<VerticalDivergenceUncertainty> </VerticalDivergenceUncertainty>

<SectorColour> </SectorColour>

<Wavelength_nm> </Wavelength_nm>

<WavelengthUncertainty_nm> </WavelengthUncertainty_nm>

<SpectrumColourCoordinates>

<ColourCoordinate_x> </ColourCoordinate_x>

<ColourCoordinate_x_Uncertainty> </ColourCoordinate_x_Uncertainty>

<ColourCoordinate_y> </ColourCoordinate_y>

<ColourCoordinate_y_Uncertainty> </ColourCoordinate_y_Uncertainty>

<ColourTemperature_K> </ColourTemperature_K>

<ColourTemperatureMeasurementUncertainty_K> </ColourTemperatureMeasurementUncertainty_K>

<SpectrumProtocolReference>

<DescriptionNR> </DescriptionNR>

<ProtocolURL> </ProtocolURL>

<DocSize_kB> </DocSize_kB>

</SpectrumProtocolReference>

</SpectrumColourCoordinates>

</Sector>

<HorIntensityDistribution_degCd> </HorIntensityDistribution_degCd>

<VertIntensityDistribution_degCd> </VertIntensityDistribution_degCd>

<IntensityTempCurve_DegCd> </IntensityTempCurve_DegCd>

<IntensityProtocolReference>

<DescriptionNR> </DescriptionNR>

<ProtocolURL> </ProtocolURL>

<DocSize_kB> </DocSize_kB>

</IntensityProtocolReference>

<Comment></Comment>

<MeasDateTime> </MeasDateTime>

</LightMeasurement>

</OpticalParameters>

<ElectricalParameters>

<Typical>

<CurrentType>AC/DC/Universal</CurrentType>

<PowerSupplyType> </PowerSupplyType>

<ReversePolarityProtection> </ReversePolarityProtection>

<SurgeProtection> </SurgeProtection>

<PowerSupplyInrushCurrent_A> </PowerSupplyInrushCurrent_A>

<VoltageNominal_V> </VoltageNominal_V>

<VoltageMinimum_V> </VoltageMinimum_V>

<VoltageMaximum_V> </VoltageMaximum_V>

<LightSourceRegulation> </LightSourceRegulation>

<FlashSynchronisation> </FlashSynchronisation>

<ExtControlOfFlashing> </ExtControlOfFlashing>

<MonitoringCapability> </MonitoringCapability>

<ControlCapability> </ControlCapability>

<SolarCellType> </SolarCellType>

<SolarCellPower_W> </SolarCellPower_W>

<SolarCellNumber> </SolarCellNumber>

<BatteryType> </BatteryType>

<BatteryCapacity> </BatteryCapacity>

<BatteryVoltage_V> </BatteryVoltage_V>

<BatteryTechnology> </BatteryTechnology>

<BatteryNumber> </BatteryNumber>

<FlasherType> </FlasherType>

<LightSensorType> </LightSensorType>

<PowerConsumption>

<PowerConsumptionFlashDay_W> </PowerConsumptionFlashDay_W>

<PowerConsumptionFlashNight_W> </PowerConsumptionFlashNight_W>

<PowerConsumptionEclipseDay_W> </PowerConsumptionEclipseDay_W>

<PowerConsumptionEclipseNight_W> </PowerConsumptionEclipseNight_W>

<PowerConsumptionStandby_W> </PowerConsumptionStandby_W>

<PowerConsumption_VW> </PowerConsumption_VW>

<PowerSpecTemperature_deg> </PowerSpecTemperature_deg>

<VoltageNomCurrentConsumption_A> </VoltageNomCurrentConsumption_A>

<VoltageMinCurrentConsumption_A> </VoltageMinCurrentConsumption_A>

<VoltageMAXCurrentConsumption_A> </VoltageMAXCurrentConsumption_A>

<PowerFactorAC> </PowerFactorAC>

<CurrentUncertainty_A> </CurrentUncertainty_A>

<VoltageUncertainty_V> </VoltageUncertainty_V>

<ProtocolReference>

<DescriptionNR> </DescriptionNR>

<ProtocolURL> </ProtocolURL>

<DocSize_kB> </DocSize_kB>

</ProtocolReference>

<Comment></Comment>

<MeasDateTime> </MeasDateTime>

</PowerConsumption>

</Typical>

<PowerMeasurement>

<PowerConsumptionFlashDay_W> </PowerConsumptionFlashDay_W>

<PowerConsumptionFlashNight_W> </PowerConsumptionFlashNight_W>

<PowerConsumptionEclipseDay_W> </PowerConsumptionEclipseDay_W>

<PowerConsumptionEclipseNight_W> </PowerConsumptionEclipseNight_W>

<PowerMeasTemperature_deg> </PowerMeasTemperature_deg>

<VoltageNomCurrentConsumption_A> </VoltageNomCurrentConsumption_A>

<VoltageMinCurrentConsumption_A> </VoltageMinCurrentConsumption_A>

<VoltageMAXCurrentConsumption_A> </VoltageMAXCurrentConsumption_A>

<PowerFactorAC> </PowerFactorAC>

<CurrentUncertainty_A> </CurrentUncertainty_A>

<VoltageUncertainty_V> </VoltageUncertainty_V>

<ProtocolReference>

<DescriptionNR> </DescriptionNR>

<ProtocolURL> </ProtocolURL>

<DocSize_kB> </DocSize_kB>

</ProtocolReference>

<Comment></Comment>

<MeasDateTime> </MeasDateTime>

</PowerMeasurement>

</ElectricalParameters>

<ConfigurationLog>

<Configuration>

<Name> </Name>

<FirmwareVersion> </FirmwareVersion>

<FlashCharacter> </FlashCharacter>

<PWMDutyCycle> </PWMDutyCycle>

<NominalRangeNight_NM> </NominalRangeNight_NM>

<NominalRangeDay_NM> </NominalRangeDay_NM>

<PowerAutonomy_h> </PowerAutonomy_h>

<PowerCableLength_m> </PowerCableLength_m>

<PowerCableType> </PowerCableType>

<LightSwitchSetting> </LightSwitchSetting>

<DayNightSwitchingLevel_lx> </DayNightSwitchingLevel_lx>

<LightSwitchingLevelON_lx>  </LightSwitchingLevelON_lx>

<LightSwitchingLevelOFF_lx> </LightSwitchingLevelOFF_lx>

<DayNightSwitchingDelay_s> </DayNightSwitchingDelay_s>

<IntensityDegradation>

<Colour> </Colour>

<YearlyPercentage> </YearlyPercentage>

<SourceTemperature_degC> </SourceTemperature_degC>

</IntensityDegradation>

<ConfigurationItem>

<ConfAction> </ConfAction>

<ModelName> </ModelName>

<ModelModification>  </ModelModification>

<PartOrderingNumber> </PartOrderingNumber>

<ModelDescription> </ModelDescription>

<SerialNumber> </SerialNumber>

<Document>     <DocClass>Drawing/Instructions/Protocol/Specification/Template/Other </DocClass>

<DocName> </DocName>

<DocURL> </DocURL>

<DocSize_kB> </DocSize_kB>

</Document>

<Comment> </Comment>

</ConfigurationItem>

<ChangeDateTime> </ChangeDateTime>

<Comment> </Comment>

</Configuration>

</ConfigurationLog>

<LifetimeEventLog>

<Event>

<EventType> </EventType>

<EventIdentNo> </EventIdentNo>

<EventTimeStamp> </EventTimeStamp>

<EventInformation> </EventInformation>

<Document>

<DocName> </DocName>

<DocURL> </DocURL>

<DocSize_kB> </DocSize_kB>

</Document>

</Event>

</LifetimeEventLog>

<FailureRepairLog>

<Failure>

<FailureClass>Critical/NonCritical</FailureClass>

<FailureIdentNo> </FailureIdentNo>

<FailureTimeStamp> </FailureTimeStamp>

<FailureDescription> </FailureDescription>

<RepairIdentNo> </RepairIdentNo>

<RepairInformation> </RepairInformation>

<RestoreTimeStamp> </RestoreTimeStamp>

<Document>

<DocName> </DocName>

<DocURL> </DocURL>

<DocSize_kB> </DocSize_kB>

</Document>

<Comment> </Comment>

</Failure>

</FailureRepairLog>

<CustomContent>

<CustomData>

<CustomDataItemName> </CustomDataItemName>

<Value> </Value>

<Description> </Description>

<DateTime> </DateTime>

</CustomData>

</CustomContent>

</ATON_PRODUCT_INFORMATION_FILE>

- SFEEAPI product Information File (PIF) DTD for validation

The DTD will be finalized and published when the scope of the PIF is fixed.

- SFEEAPI Product Information File content example

File: SFEEAPI_PIF_Example_20111013.XML

Contents start on the next page. ?????

<?xml version=‘1.0’ encoding=‘UTF-8’ standalone=‘yes’?>

<ATON_PRODUCT_INFORMATION_FILE>

<SFEEAPI_Compliance>0.1</SFEEAPI_Compliance>

<ContentsClassification>Public</ContentsClassification>

<ValidSinceDate>20111013</ValidSinceDate>

<PIFCreator>MS Notepad</PIFCreator>

<Product>

<ModelName>AtoN Lantern No1 </ModelName>

<ModelModification>R</ModelModification>

<PartOrderingNumber>112233-44</PartOrderingNumber>

<ModelDescription>1NM Red AtoN Lantern (requires external flasher)</ModelDescription>

<SerialNumber>110123456</SerialNumber>

<ManufacturingDate>20110901</ManufacturingDate>

<ShippingDate>20111013</ShippingDate>

<WarrantyTermMonths>24</WarrantyTermMonths>

<CountryOfOrigin>France</CountryOfOrigin>

<ProductClass>BuoyLight</ProductClass>

<LightSourceClass>LED</LightSourceClass>

<ProductPicture>

<PictureURL> </PictureURL>

<Caption></Caption>

</ProductPicture>

<UserManualURL> </UserManualURL>

<ProductDocument>

<DocClass>Instructions</DocClass>

<DocName>AtoN Lantern No1. Instructions for Use</DocName>

<DocURL>No1_instructions.pdf</DocURL>

<DocSize_kB>100</DocSize_kB>

</ProductDocument>

<Comment> </Comment>

</Product>

<Manufacturer>

<Name>AtoN Lantern manufacturer Ltd</Name>

<RegistrationNumber>77665544</RegistrationNumber>

<VATNumber>FR-9876543</VATNumber>

<StreetAddress>Rue de Dumas</StreetAddress>

<ZipCode>112233</ZipCode>

<City>Marseille</City>

<Country>France</Country>

<Website>http://www.buoy1.com/</Website>

<Email> mail@buoycompany.com</Email>

<Phone> </Phone>

<ContactPerson> </ContactPerson>

<Comment> </Comment>

<ContractReference>9977553311</ContractReference>

<PurchaserReference>Mr. Alexander Buyer</PurchaserReference>

</Manufacturer>

<ContactInfo>

<ContactClass>Distributor/Maintenance/Repair/TechSupport/Other </ContactClass>

<Name> </Name>

<RegistrationNumber> </RegistrationNumber>

<VATNumber> </VATNumber>

<StreetAddress> </StreetAddress>

<ZipCode> </ZipCode>

<City> </City>

<Country> </Country>

<Website> </Website>

<Email> </Email>

<Phone> </Phone>

<ContactPerson> </ContactPerson>

<Comment> </Comment>

</ContactInfo>

<OperationalParameters>

<ServiceLife_Y> </ServiceLife_Y>

<MTBF> </MTBF>

<IngressProtectionClass> </IngressProtectionClass>

<EnvironmentTemperatureMin_DegC>-10</EnvironmentTemperatureMin_DegC>

<EnvironmentTemperatureMAX_DegC>50</EnvironmentTemperatureMAX_DegC>

<VibrationTolerance_g> </VibrationTolerance_g>

<Maintenance>

<Task>

<OperationalHours> </OperationalHours>

<CalendarHours> </CalendarHours>

<Description> </Description>

<InstructionsURL> </InstructionsURL>

<DocSize_kB> </DocSize_kB>

</Task>

</Maintenance>

<Comment> </Comment>

</OperationalParameters>

<PhysicalParameters>

<Height_mm> </Height_mm>

<Width_mm> </Width_mm>

<Depth_mm> </Depth_mm>

<Diameter_mm> </Diameter_mm>

<MountDescription> </MountDescription>

<LensMaterial> </LensMaterial>

<BodyMaterial> </BodyMaterial>

<Weight_g>1200</Weight_g>

<Comment> </Comment>

</PhysicalParameters>

<OpticalParameters>

<LensDiameter_mm> </LensDiameter_mm>

<LensHeight_mm> </LensHeight_mm>

<LensTiers> </LensTiers>

<FocalHeight_mm> </FocalHeight_mm>

<LensMaterial> </LensMaterial>

<Typical>

<Sector>

<SectorWidth_deg>360</SectorWidth_deg>

<SectorStart_deg> </SectorStart_deg>

<SectorEnd_deg> </SectorEnd_deg>

<SectorUncertainty_deg> </SectorUncertainty_deg>

<BoundaryResolution_min> </BoundaryResolution_min>

<TypicalIntensity_cd>30</TypicalIntensity_cd>

<IntensityUncertainty_cd> </IntensityUncertainty_cd>

<IntensitySpecTemperature_deg> </IntensitySpecTemperature_deg>

<IntensityDegradation_h> </IntensityDegradation_h>

<DegradationTempLimit_degC> </DegradationTempLimit_degC>

<SectorUniformity> </SectorUniformity>

<VerticalDivergenceFWHM_deg>12</VerticalDivergenceFWHM_deg>

<VerticalDivergenceFWTM_deg> </VerticalDivergenceFWTM_deg>

<VerticalDivergenceUncertainty> </VerticalDivergenceUncertainty>

<SectorColour>RED</SectorColour>

<Wavelength_nm> </Wavelength_nm>

<WavelengthUncertainty_nm> </WavelengthUncertainty_nm>

<SpectrumColourCoordinates>

<ColourCoordinate_x> </ColourCoordinate_x>

<ColourCoordinate_x_Uncertainty> </ColourCoordinate_x_Uncertainty>

<ColourCoordinate_y> </ColourCoordinate_y>

<ColourCoordinate_y_Uncertainty> </ColourCoordinate_y_Uncertainty>

<ColourTemperature_K> </ColourTemperature_K>

<ColourTemperatureUncertainty_K> </ColourTemperatureUncertainty_K>

<SpectrumProtocolReference>

<DescriptionNR> </DescriptionNR>

<ProtocolURL> </ProtocolURL>

<DocSize_kB> </DocSize_kB>

</SpectrumProtocolReference>

</SpectrumColourCoordinates>

</Sector>

<HorIntensityDistribution_degCd> </HorIntensityDistribution_degCd>

<VertIntensityDistribution_degCd> </VertIntensityDistribution_degCd>

<IntensityTempCurve_DegCd> </IntensityTempCurve_DegCd>

<IntensityProtocolReference>

<DescriptionNR> </DescriptionNR>

<ProtocolURL> </ProtocolURL>

<DocSize_kB> </DocSize_kB>

</IntensityProtocolReference>

<Comment> </Comment>

<MeasDateTime> </MeasDateTime>

</Typical>

</OpticalParameters>

<ElectricalParameters>

<Typical>

<CurrentType>DC</CurrentType>

<PowerSupplyType>Linear</PowerSupplyType>

<ReversePolarityProtection> </ReversePolarityProtection>

<SurgeProtection> </SurgeProtection>

<PowerSupplyInrushCurrent_A> </PowerSupplyInrushCurrent_A>

<VoltageNominal_V>12</VoltageNominal_V>

<VoltageMinimum_V> </VoltageMinimum_V>

<VoltageMaximum_V> </VoltageMaximum_V>

<LightSourceRegulation> </LightSourceRegulation>

<FlashSynchronisation> </FlashSynchronisation>

<ExtControlOfFlashing> </ExtControlOfFlashing>

<MonitoringCapability> </MonitoringCapability>

<ControlCapability> </ControlCapability>

<SolarCellType> </SolarCellType>

<SolarCellPower_W> </SolarCellPower_W>

<SolarCellNumber> </SolarCellNumber>

<BatteryType> </BatteryType>

<BatteryCapacity> </BatteryCapacity>

<BatteryVoltage_V> </BatteryVoltage_V>

<BatteryTechnology> </BatteryTechnology>

<BatteryNumber> </BatteryNumber>

<FlasherType> </FlasherType>

<LightSensorType> </LightSensorType>

<PowerConsumption>

<PowerConsumptionFlashDay_W>0</PowerConsumptionFlashDay_W>

<PowerConsumptionFlashNight_W>2</PowerConsumptionFlashNight_W>

<PowerConsumptionEclipseDay_W> </PowerConsumptionEclipseDay_W>

<PowerConsumptionEclipseNight_W> </PowerConsumptionEclipseNight_W>

<PowerConsumptionStandby_W> </PowerConsumptionStandby_W>

<PowerConsumption_VW> </PowerConsumption_VW>

<PowerSpecTemperature_deg> </PowerSpecTemperature_deg>

<VoltageNomCurrentConsumption_A>12</VoltageNomCurrentConsumption_A>

<VoltageMinCurrentConsumption_A> </VoltageMinCurrentConsumption_A>

<VoltageMAXCurrentConsumption_A> </VoltageMAXCurrentConsumption_A>

<PowerFactorAC> </PowerFactorAC>

<CurrentUncertainty_A> </CurrentUncertainty_A>

<VoltageUncertainty_V> </VoltageUncertainty_V>

<ProtocolReference>

<DescriptionNR> </DescriptionNR>

<ProtocolURL> </ProtocolURL>

<DocSize_kB> </DocSize_kB>

</ProtocolReference>

<Comment></Comment>

<MeasDateTime> </MeasDateTime>

</PowerConsumption>

</Typical>

</ElectricalParameters>

</ATON_PRODUCT_INFORMATION_FILE>