This questionnaire is designed to gather information from IALA members on their experiences with the use of plastic buoys. This will then be collated and used to enable members to make informed choices on the selection of the most appropriate material for their specific buoy application.

~15,000

- How many buoys do you have in your current buoy fleet ? ………………..……………

Regarding the materials of construction, how many are:

~2,700

Steel …………………………………………………………………………..……………….

~100

Composite metal & plastic ………………………………………………………..…………

~9,000

Plastic ……………………………………………………………………………………..…..

~100

Aluminum …………………………………………………………………………………….

~70

Cast Iron …………………………………………………………………….………………….

- Buoy fleet by buoy body diameter and material, please complete the table below:

Buoys in active operation

| Material | 1m dia or less | 1 to 2.2 m dia | Greater than 2.3m dia |
| --- | --- | --- | --- |
| Steel | 1235 | 806 | 613 |
| Composite metal & plastic | 15 | 83 | 10 |
| Plastic | 8312 | 452 | 3 |
| Aluminum | 13 | 73 | - |
| Cast Iron | 9 | 60 | - |

- For your plastic buoy fleet, what type of plastic material is used ?

The external shell material is mainly Roto-moulded polyethylene plastic with some extruded High Density Polyethylene extrusions.

- Are the plastic buoy bodies filled or empty ?

Most

Few

Filled			Empty

Please specify filler type    ……the material used is usually expanded polystyrene foam.

15

- How many years have you been using plastic buoys ? …………………………………...….

- Regarding the location / conditions of service for your plastic buoys, check the boxes that apply:

| Inshore | X |
| --- | --- |
| Offshore |  |
| Exposed | X |
| Sheltered | X |
| Breaking waves | X e.g. Jeddore Harbour, Nova Scotia |
| Dries at low tide | X e.g. Bay of Fundy |
| Severe UV exposure | X e.g. Southern Ontario conditions |
| Severe sub zero temperature | X e..g Arctic waters |

- Are your plastic buoys a commercial design or your own authority’s design ?

√

Commercial Design			Own Design

- How often do you visit your buoys at sea ?

Size is not the primary factor. The Canadian Coast Guard (CCG) utilizes many buoys as part of its Floating Aids to Navigation system to provide safe and navigable waters to mariners. Most of these buoys fall into three operational system categories:

- Year Round Buoy system: A buoy that is used all year round to provide a certain Level of Service (LOS) to Mariners. It is visited every two to four years.

  - In the east the buoys are visited every 3 years for a mooring replacement and cleaning. The buoy is exchanged after six years.

  - In the west, the complete buoy systems are exchanged every four years. Very active or problem sites are visited every 2 years for a mooring check and cleaning. In this case the mooring would be replaced based on inspection.

- Seasonal-Year Round Buoy system: A buoy is used in the spring, summer and autumn months to provide a certain LOS. As this LOS is no longer required during the winter months the buoy remains in the water. A spring mooring positioning check is often completed, therefore this is generally an annual  occurrence.

- Seasonal Buoy system: A buoy that is used in the spring, summer and autumn months to deliver a certain Level of Service (LOS). That buoy and its mooring is then retrieved in the autumn and returned in the spring. In some cases the summer buoy is replaced with a buoy that is used as mooring marker to keep the mooring accessible for eventual buoy redeployment in the Spring.

| Material | 1m dia or less | 1 to 2.2 m dia | Greater than 2.3m dia |
| --- | --- | --- | --- |
| Steel | n/a | n/a | n/a |
| Composite metal & plastic | n/a | n/a | n/a |
| Plastic | n/a | n/a | n/a |
| Other (please specify) | n/a | n/a | n/a |

- For your plastic buoys, what is the main reason for inspection / maintenance at sea ?

- Position verification following the winter ice season for our seasonal-year round buoys.

- Mooring replacement (corrosion, wear)

- Marine growth accumulation

- Lantern visibility issue reported

- Buoy loss due to storm event

There is no real difference between the steel and plastic system at sea, since the issues are not related to buoy material, but rather the supporting components.

We have not modified any plastic buoys to accept the AIS system.

- How often do you return your buoys to shore for maintenance ?

Size is not the primary factor (See question 9).

  - Year round and seasonal year-round buoys every 4 years to 6 years

  - Seasonal buoys every 6 -8 months

| Material | 1m dia or less | 1 to 2.2 m dia | Greater than 2.3m dia |
| --- | --- | --- | --- |
| Steel | n/a | n/a | n/a |
| Composite metal & plastic | n/a | n/a | n/a |
| Plastic | n/a | n/a | n/a |
| Other (please specify) | n/a | n/a | n/a |

- For your plastic buoys, what is the main reason for inspection / maintenance ashore ?

1. Reason for inspections are safety and preventive maintenance. The buoys are visually inspected for general damage, wear (hardware, mooring, lifting attachments), hull integrity, and discolouration.

2. Repairs (e.g. hardware replacement) are completed based on inspection results

3. Cleaning is to allow for inspection, storage, and improve visual perception prior to re-deployment (e.g. removal of bird droppings).

- What experience do you have in modifying plastic buoys to take new navigation aids, such as AIS ?

Nil

- Have you experienced any in service defects with plastic buoys (such as colour retention, mechanical failure, mechanical damage, mooring lug wear) ?

Yes. Colour retention is a problem due to UV exposure. It is difficult to stay within the IALA colour areas for the expected life of the plastics. Mechanical failures have occurred due to overload impacts from ice, vessel or debris, and improper lifting or handling events. Welded plastic joints have also been a source of hull breach – the cause may be temperature changes or plastic degradation or other.

- Please detail any experience you have of repairing or re-painting plastic buoys?

The CCG does have some personnel trained in the repair of plastic buoys which is limited to fixing small holes and cracks with polyethylene filler materials. The repainting of plastic buoys was tried many years ago with mixed success, and is not practiced in Canada.

- Can you provide any cost comparisons between plastic buoys and traditional steel buoys (this may include initial cost, use of a smaller ship to support the buoy fleet, reduction in buoy maintenance facilities, reduction in overall maintenance costs)

Maintenance Costs (MC) over 20 years of a Small steel vs comparable plastic spar.*

| Item (Life) | 20 year period |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0.3 m Steel ORT |  | 0.3 m Plastic ORT |  |
|  | No. of occurances | Total Cost | No. of replacements | Total Cost |
| Paint system** (8 yrs) | 2@ $133 | $ 266 | - | - |
| Repair Costs*** (6 years) | - | - |  |  |
| Total MC | - | $266 | - | - |
| Initial Purchase Cost | $ 250 x 3* |  | $ 315 x 1.6* |  |
| Estimated Disposal Costs | - |  | $60 |  |
| Total Life Cycle Cost | $ 1016 |  | $ 564 |  |

Maintenance Costs (MC) over 20 years of a Medium steel vs comparable plastic buoy.*

| Item (Life) | 20 year period |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.4 m Steel |  | 1.5 m Plastic |  |
|  | No. of occurances | Total Cost | No. of replacements | Total Cost |
| Paint system** (8 yrs) | 2@$1,500 | $ 3,000 | - | - |
| Repair Costs*** (6 years) | 3@($ 413) | $1238 |  |  |
| Total MC | - | $4,238 | - | - |
| Initial Purchase Cost | $ 14,000 |  | $ 5,000 x 1.6* |  |
| Estimated Disposal Costs | - |  | $150 |  |
| Total Life Cycle Cost | $ 18,238 |  | $ 8,150 |  |

The lifespan of the 1.4 meter steel buoy is estimated at 40 years. The lifespan of the plastic buoy is

estimated at 12 years.

- *- Assumption: Average lifespan for steel ORT is 7 years and plastic is 13 years.

- **- The paint system is assumed to last 8 years for this analysis.

- ***- Steel ORT’s are not repaired when damaged and are dosposed. 1.4 steel buoys are repaired every 6 years. The cost is an estimated average.

- The cost estimates use 2006 values. Deployment and retrieval costs are excluded.

- Disposal costs do not include transportation costs from CCG facilities to scrap yard or recycle depot.

- Are you planning to purchase more plastic buoys in the near future ?

√

Yes			No

10 -15 years

- What is the anticipated life of your plastic buoys ? ………………………………………

- What is your disposal policy for plastic buoys at the end of their life ?

The CCG’s current disposal policy on plastic buoys is to employ a third party to mechanically separate the different plastics and metals used present in the buoys and then to send the separated plastics to a plastics recycler for reuse.

We are also trying to include recycling into our procurement contracts at the time of purchase. This practice can be costly, and is still not fully implemented due to the vastness of the country.

- With your experience, what is your general opinion of the use of plastic buoys ?

Plastic buoys offer many advantages over traditionally steel buys as well as disadvantages.  Generally plastic buoys are lighter then traditionally steel buoys, do not require painting and can be handled using smaller boats. The disadvantage is that their colour fastness over time as well as their ease of repair and final disposal costs.

We generally like the smaller marker buoys (less than 1 meter dia.) as they are much lighter then their steel counterparts and can be easily deployed and retrieved by local contractors using smaller boats.

If they are lost or damaged they are easily replaced because of their economical low costs. Larger plastic buoys ( > 1 meter dia.) are more problematic if they are damaged or lose their colour and are more expensive repair or to replace, and the initial cost savings is quickly reduced.

- Please provide some photographs of your plastic buoys with sizes & weights (please resize and compress any photographs before insertion):

CCG PLASTIC BUOYS

| Buoy Picture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CCG Buoy Type | SMALL SPAR | SMALL MARKER | MEDIUM MARKER | FAST WATER | LARGE MARKER | MEDIUM SPAR | LARGE SPAR | MEDIUM PILLAR | LARGE PILLAR |
| Buoy Material | Polyethylene | Polyethylene | Polyethylene | Polyethylene | Polyethylene | Polyethylene | HDPE | Polyethylene | Polyethylene |
| Overall Height (m) | 3.0 | 1.5 | 1.8 | 2.2 | 2.3 | 5.5 | 6.3 | 2.8 | 12.0 |
| Diameter (mm) | 184 / 368 | 400 | 762 | 757 | 1,2 | 600 | 500 | 600 / 1500 | 2400 |
| Buoy Weight (kg) | 34 - 42 | 9 - 12 | 30 - 37 | A) 41 – 50 B) 77 - 94 | 120 - 130 | 81-106 | 430 | 235 - 287 | 1500 |
| Minimum Focal Height (m) | 1.0 | 0.75 | Max. 1.5 | 0.5 | Max. 1.6 | 1.20 | 2.3 | 1.30 | 4.50 |