AISMAssociation Internationale de Signalisation Maritime IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline No.####

On

Technical Features and Technology Relevant for Simulation of AtoN

Edition 0.5

Date issued]

10, rue des Gaudines

78100Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01 Fax:  +33 1 34 51 82 05

e-mail:  contact@iala-aism.org     Internet: www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

Document Revisions	1

Table of Contents	3

Index of Tables	4

Index of Figures	4

1	Introduction	5

2	Scope	5

3	Definitions	6

4	User needs and requirements	7

5	Modelling and Simulation of AtoN	8

5.1	Introduction	8

5.2	Relevant IALA Recommendations and Guidelines	8

5.2.1	IALA Recommendations	8

5.2.2	IALA Guidelines	8

6	Visual System	10

6.1	Presentation Technology	10

6.1.1	Introduction	10

6.1.2	Display systems	10

6.1.3	Projection Theatres	12

6.1.4	Warping and blending	13

6.1.5	Video walls	13

6.1.6	IR and Night Vision	13

6.2	Modelling	13

6.2.1	The human vision	13

6.2.2	Conceptual model	14

6.2.3	Compensation for Presentation System Limitations	18

6.2.4	Software implementation	18

7	Simulation of Radars	19

7.1	Presentation	19

7.2	Modelling	19

8	Simulation of sound	20

8.1	Presentation	20

8.1.1	Speaker systems	20

8.1.2	Elephant ears and advanced directional sound reproducing systems	20

8.2	Modelling	20

9	Simulation of Shipborne navigation systems	21

9.1	ECDIS	21

9.2	PPU	21

9.3	AIS as Virtual AtoN	21

9.4	e-Navigation systems	21

10	Conclusions	22

Index of Tables

Table 1	Title required	11

Table 2	Title required	11

Table 3	Title required	12

Index of Figures

Figure 1	AtoN modelling and presentation	6

Figure 2	Visual Simulation of AtoN	14

Figure 3	Conceptual model of the fixed AtoN	14

Figure 4	Conceptual model of the floating AtoN	15

Figure 5	Conceptual model of the environment	15

# Introduction

IALA’s Guideline 1058 ‘On the use of simulation as a tool for waterway design and AtoN planning’ is intended to be a high level, strategic document to assist AtoN authorities in understanding how simulation tools can assist them in planning and implementing AtoN.  This guideline shall be seen as a more technical guideline supplementing Guideline 1058.

Users of this guideline are mainly intended to be AtoN designers, developers, researchers and testers including mariners.  This group wish to know to what extend a given simulator system is providing the required quality and fidelity to suit the purpose of studies and investigations.

Users will with the present guideline be provided with a guideline generating an overview of current status of simulation technology with focus on capabilities and limitations of simulation software, visual systems, visualization media and other relevant systems and is identifying a collection of features that are important to consider when specifying objectives of a simulator based study for planning, research and test of AtoN.

Today’s modern technology and the pace at which the technology developed for computer based simulation systems are progressing are providing users with a high level of fidelity and realism.  On the other hand this technology has its limitations.  This guideline will investigate the capabilities of relevant software, modelling techniques and systems but also illustrate the limitations.

The guideline is representing the technological status at the time of publication.  As marine simulation tools include several elements of particular types of technology that is under constant and fast development readers shall be aware that certain parts of this guideline very well may have advanced further and it is recommended that users consult simulator system producers for information on latest developments.

# Scope

This guideline covers:

- Modelling and simulation of AtoN with a view to software implementation;

- Visual presentation technology such as:

  - Projectors, monitors, screens, hand-held displays LED technology, virtual reality techniques etc.;

  - Projection theatres;

  - Video walls;

  - IR and night Vision.

- Radar simulation;

- Sound simulation;

- Simulation of Ship Navigation systems including ECDIS, PPU, virtual AtoN and e-Navigation systems.

Reference is made to relevant IALA Recommendations and Guidelines where phenomena relevant for simulation of AtoN are addressed.

# Definitions

Within simulation real world systems may be used as a partial model encompassing the total system.

- Stimulation

A real world system, such as for example a radar, need to believe that it lives in a real world.  Stimulation is used when a real radar video signal is produced in a simulator model and feed as in real life into a real radar display identical to those used onboard real ships.

- Emulation

Emulation is applied if the whole radar sub-system, both antenna and display is modelled and not being identical to a real onboard system.

- Modelling

A model is a simplified representation of a system at some particular point in time or space intended to promote understanding of the real system.

- Simulation

A simulation is the manipulation of a model in such a way that it operates on time or space to compress it, thus enabling one to perceive the interactions that would not otherwise be apparent because of their separation in time or space.

In IALA Guideline 1058 the following definition has been used:

Simulation is the process of designing a model of a real system and conducting experiments with this model for the purpose either of understanding the behaviour of the system or of evaluating various strategies (within the limits imposed by a criterion or set of criteria) for the operation of the system.  (R.E. Shannon, 1975)

- Presentation

The continuous state of the simulation model needs to be presented to the system operator, i.e. the navigator, using realistic and relevant stimuli as used real world operations.

- AtoN modelling and presentation

# User needs and requirements

This guideline is based on user needs and requirements expressed by members of the ANM and EEP committees.

Simulation is in the context of the present guideline focusing on the needs and requirement used in conjunction with:

- Research;

- Development;

- Design; and

- Test.

of individual or combinations of AtoN.

Simulation is providing a cost efficient flexible tool that can support the abovementioned activities, but the users shall take due care to understand the capabilities and limitations of the individual elements of a simulation system.

As for all simulation based studies it is also very important to carefully consider when and to what extend the end-users/mariners shall be involved in the simulation for planning, execution and evaluation of the results of a simulation study.

Ideally the ‘Probability of Detection’ of an AtoN shall be identical or similar in a simulated and real environment.  End users have a natural desire to achieve this level.  It is not possible to generate a one-to-one situation but this guideline is intended to provide an insight that should assist the users in understanding the gap but still be able to use the new technology to its best.

# Modelling and Simulation of AtoN

## Introduction

The present guideline covers the four sub-domains of ‘receiving’ AtoN. Each method requires individual means of modelling and presentation as discussed in the following sections.

Important and relevant knowledge for the real and practical aspect of AtoN useful when devising a model is found in the comprehensive set IALA Recommendations and Guidelines as listed in Section 5.2 and Appendix A.

## Relevant IALA Recommendations and Guidelines

The following recommendations and guidelines have been reviewed and identified as relevant for the simulation of AtoN.

### IALA Recommendations

R-101	On Maritime Radar Beacons (racons) (January 1995 – Revised 2004)

O-104	On ‘Off Station’ Signals for Major Floating Aids

E-106	For the Use of Retroflecting Material on Aids to Navigation Marks within the IALA Maritime Buoyage System

E-107	On Moorings for Floating Aids to Navigation

E-108	For the Surface Colours used as Visual Signals on Aids to Navigation (specifications for ordinary and fluorescent colours)

E-109	For the Calculation of the Range of a Sound Signal

E-110 	For the Rhythmic Characters of Lights on Aids to Navigation (Edition 2.0)

E-111	For Port Traffic Signals

E-112	For Leading Lights

A-123	On the Provision of Shore Based AIS

O-133	On Emergency Wreck Marking Buoy (for use on trial basis)

O-138	On the Use of GIS and Simulation by Aids to Navigation Authorities

E-141	On Aids to Navigation Training

E-143	On Virtual Aids to Navigation Edition 1

E-200	On Marine Signal Light, Part 0 - Overview

E-200-1	On Marine Signal Light, Part 1 - Colours

E-200-2	On Marine Signal Light, Part 2 - Calculation, definition and notation of luminous range

E-200-3	On Marine Signal Light, Part 3 - Measurements

E-200-4	On Marine Signal Light, Part 4 - Determination and Calculation of Effective Intensity

E-200-5	On Marine Signal Light, Part 5 - Estimation of the Performance of Optical Apparatus

### IALA Guidelines

1023	For the Design of Leading Lines

1038	On Ambient Light Levels at which Aids to Navigation should Switch On and Off

1041	On Sector Lights

1043	On Light Sources Used in Visual Aids to Navigation

1047	On Cost Comparison of Buoy Technologies

1048	On LED Technologies and their use in Signal Lights

1049	On the Use of Modern Light Sources in Traditional Lighthouse Optics

1051 	On the Provision of Aids to Navigation in Built-up Areas

1058 	On the Use of Simulation as a Tool for Waterway Design and Aids to Navigation Planning

1061	On Light Applications Illumination of Structures

1065	On Vertical Divergence

1066	On the Design of Floating Aid to Navigation Moorings

1069 	On the Synchronization of Lights

APPENDIX 1, provides a summary list of phenomena discussed in the IALA Recommendations and Guidelines of relevance for modelling and simulation of AtoN.

# Visual System

## Presentation Technology

### Introduction

The Presentation Technology is the system that is responsible for the transmission of the visual cues to the mariner on the simulator bridge.

A number of presentation technologies are available, ranging from a single monitor display to a LED video wall.  All have their specific strengths and limitations and some are much more expensive than others.

For a simulation to be effective the mariner must be able to derive all relevant information from the visual stimuli – thus the resolution of the display may be more important that its luminance, although there may be some interaction due to the physical characteristics of the human eye.

The first question to be answered before choosing a presentation technology is what the aforementioned ‘relevant information’ will be. Then, from the available technologies the most cost-effective alternative that is able to transfer this information may be chosen.  In the simulation models the limitations of the presentation technology can be compensated to some extent.  If the information cannot be sufficiently transferred by any of the available technologies, an additional not necessarily visual feedback to the mariner could be provided – of course, only to the extent to compensate for the limitation of the visual presentation.

Of course, the final testing may include laboratory and in-site measurements to overcome the limitations of digital simulation but in view of the costs involved this will be limited to the best options as found from the simulation.

### Display systems

#### Present limitations

The following sections will go further into different technical possibilities for presentation of images for simulation.  In general, the limitations refer to the following aspects:

- Resolution: The resolution of the eye is 1’ whereas simulated images reach about 10’;

- Illuminance of objects: in the real world up to 10,000 cd/m2, simulated images up to 300 cd/m2 . The full range of background illumination in real world conditions extends from 10-3 cd/m2 on an overcast, moonless night to 105 cd/m2 in direct sunlight;

- Contrast: real-world contrast ratios are up to 1:107, simulator images reach 1:104;

- Colour space: the real-world, continuous spectrum is digitized as a limited number of colours.

Although technical developments are continuing and systems may exist that perform better, one must bear in mind the nature of these limitations when designing a simulation study.

#### Projectors

A projection system involves a light source from which an image is projected onto a screen.  The mariner observes the screen and as such the image transfer is indirectly.  The projection screen is usually at a distance of several metres from the observer position which may enhance the sense of reality.  A drawback is that the radiation losses and reflection losses limit the luminance of the image such that the simulated conditions resemble at best dusk lighting with accordingly small contrast.

The projector techniques used determine the light production at the source, contrast and colour resemblance, response speed, etc.  Continuing developments are stimulated mainly by the gaming industry and digital cinema and supported by the constant increase of computer power (‘Moore’s law’).  Probably the most important parameter for simulation of visual AtoN is the resolution – and as all this is about digital imagery, this is determined by the number of pixels per minute of arc seen from the mariner’s position.

For the generation of the image the available techniques are CRT, LCD, DLP (digital light processor) and  LCoS (liquid crystal on silicon).  All have their pros and cons and prices differ widely.

For digital cinema and gaming the resolution is usually not the prime issue whereas the costs per pixel rise dramatically with increasing image resolution.  Normal image resolutions for this application are UXGA (1600 x 1200) and at the high end QXGA (2048 x 1536).  Most recent development at this moment is the so-called 4K technology, indicating a 4096 pixels wide image.

Some configurations available today:

- Title required

| Monitors Projector type | Resolution | Contrast ratio 1 : | Max light output Ansi lumen |
| --- | --- | --- | --- |
| 3 CRT | Max 2500 x 2000 |  | 800 |
| 3 chip DLP | 2048 x 1080 | 2000 | 30000 |
| 3 chip DLP | 1920 x 1200 | 2000 | 12000 |
| 1 chip DLP | 1920 x 1200 | 2000 10,000 with night goggle |  |
| LCoS | 1536 x 2048 | 6,000,000 |  |
| LCoS (low-cost) | 1920 x 1080 | 50,000 | 1300 |
| LCD (low-cost) | 1400 x 1050 | 7,500 | 7500 |

When the simulated image is presented on one or more monitors, the radiation and reflection losses attached to projection are avoided.  The monitors can be in place of the bridge windows, the window sills giving a natural separation of the adjacent monitors.  The sense of reality could be somewhat less. The mariner’s position on the bridge should be fixed; otherwise the absence of parallax would be apparent from the window sills.

An advantage of monitors over a projection system is that pixels are not distorted by optical aberrations, so that the resolution is not reduced by blurring of the pixel images.  Also the usually very laborious adjustment process to get the best possible projected image is avoided.

Some examples of available monitors are:

- Title required

| Display type | Dimensions | Resolution; Pixel pitch | Contrast ratio 1 :?? | Max luminance Cd / m2 |
| --- | --- | --- | --- | --- |
| LED 28’ | 500 x 500 mm | 2048 x 2048; 0.25 mm | 1000 | 300 |
| LCD 30’ | 640 x 400 mm | 2560 x 1600; 0.25 mm | 1000 | 250 |
| LPD (Laser Phosphor Display) | 3048 x 1524 mm | 1920 x 960; 1.6 mm | 100000 | 800 |

#### LED screens

A large number of LED displays may be tiled to form a video wall.  Each tile consists of a relatively small number of LEDs, each LED responsible for a single pixel of the image.  The image has relatively high luminance and contrast figures.  If the resolution of the image has to be 1’, then the distance between the mariner and the wall must be nearly 14 m when using 4 mm LEDs.  It is clear that this would lead to a huge amount of tiles for a 360 degree view.

Examples of tiles are given.

- Title required

| Display type | Dimensions | Resolution; Pixel pitch | Contrast ratio 1 : | Max luminance Cd / m2 |
| --- | --- | --- | --- | --- |
| 4 mm pitch 3-in-1 | 448 x 504 mm | 96 x 108; 4 mm | 4000 | 2000 |
| 16 mm pitch 3 led | 448 x 504 mm | 60 x 60; 16 mm | 7000 | 6000 |

#### Head-mounted and Hand-held Displays

If a mariner would use binoculars on the simulator bridge he would probably be able to distinguish the image pixels more clearly.  But that’s not what we want: the binoculars should improve the image resolution.  Of course, this may be simulated also.  In that case the binoculars would contain electronic displays and a sensor to determine the target being aimed at.  The displays are fed with the calculated, ‘enlarged’ image.

Somewhat along the same line the Virtual Reality techniques can be seen.  Instead of creating a very large projection wall for the image, we might provide the mariner with a small display showing everything he should be able to see at that moment.  This display is then built into a VR helmet which the mariner wears on his head, blocking the view on his real surroundings.  Like the detection of the target of the binoculars, the helmet must sense the position of the mariners head and the direction of his eyesight.  This information must lead to the adaptation of the image very fast, as the latency (the time the image lags behind) must be less then 50 ms for the mariner to keep the illusion that he is really looking around.

This technique is used also in flight simulators.

#### Combination of techniques

A simulation may make use of a combination of the techniques described above.  Examples are the binoculars as mentioned in the previous paragraph, but other combinations may also be applied to solve specific lacks of information.  As an example, a separate monitor could be used to fill in the rear view on a projected 270 degree view bridge.  Possibly the lights of AtoN could be superimposed on a projected outside view with separate laser beams, allowing for high contrast and high resolution values.

### Projection Theatres

On a full-mission bridge simulator a large viewing angle, preferably round vision, is desirable.  A frequently applied setup is a circular projection screen with a number of projectors underneath or on top of the simulator bridge, each one being responsible for a sector of the outside view.  Besides the characteristics of the projectors, as discussed earlier, some specific issues relate to the composition of the image in such a theatre.  Although there are specialised companies that provide solutions, some aspects are discussed briefly here.

### Warping and blending

The projected image of each projector should ideally be a perfect cylinder section, all colours sharp and aligned throughout the entire image.  As the projectors cannot all be exactly in the centre of the cylinder section distortions must be compensated.  Also, projection of a flat image source would lead to a flat projected image, so the focus must be corrected for the cylindrical image.  These corrections are fairly standard for the solution providers, although days of adjustment may be involved and at regular intervals to maintain an optimal image.

#### Dynamic perspective

Detection of the actual viewing point of the pilot/master and changing the perspective of the projected image accordingly (example: when the pilot enters the bridge wing, the viewing point shifts to that side).

#### Colour matching

#### Stitching and transitions

### Video walls

A video wall consists of a number of video monitors.  Additional aspects compared to those mentioned under monitors are the edges between adjacent monitors,…

### IR and Night Vision

Mainly intended for simulation of military operations, the visualisation of IR and night vision is available from some manufacturers.

## Modelling

In the following sections relevant phenomena’s are presented in a conceptual model hierarchy.

This guideline recommends considering these phenomena when using simulation systems as an assessment tool of AtoN, to the least.

As also stated later, a model of phenomena may contain elements that compensate for the deficits in the visual presentation system.

### The human vision

#### Luminance

Minimum 0.2 m lux

Dynamic contrast 1.000.000:1

Static contrast 10.000:1

#### Colour resolution

#### Spatial resolution

Field of view

Resolution

Glare

Lens flare

Blooming

### Conceptual model

- Visual Simulation

  - AtoN

    - Floating

    - Fixed

  - Environment

- Visual Simulation of AtoN

The conceptual model can be detailed further as indicated in the following figures 3, 4 and 5.

- Fixed AtoN

  - Lights

    - Point

    - Line

    - Surface

  - Construction

    - Geometry

    - Surface materials

    - Local Illumination

- Conceptual model of the fixed AtoN

- Floating AtoN

  - Mooring system

  - Environmental forces

  - Floating body

    - Construction

      - Geometry

      - Surface materials

  - Lights

    - Point

    - Line

    - Surface

- Conceptual model of the floating AtoN

- Environment

  - Terrain and man-made constructions

    - Geometry

    - Surface material

    - Lights

  - Ships

    - Geometry

    - Surface material

    - Lights

  - Ocean

  - Weather

  - Atmosphere

  - Sun and moon

- Conceptual model of the environment

For each of the conceptual models the following phenomena should be considered

#### Navigation Lights

A navigation light are lights, typical considered as point lights either fitted on floating aids, beacons, light houses and on ships that to be model to the highest possible level.  The model of a navigation light shall include the possibility for specifying the

- Colour;

- Nominal range/ or luminance;

- The vertical divergence;

- The sectors and their cut-off precision High precision cut-off must be possible PEL lights;

- Oscillating lights;

- Flashing characteristics, including flickering, synchronised and sequential lights;

The maximum flickering frequency that a system can represent relates to the so-called Frame Rate (FPS).  Table X in annex Y can be used to estimate the maximum flicker frequency for a individual frame rates taking into account the Nyquist sampling theorem up to the Flicker-fusion frequency;

- Appearance at close and long range A suitable model may compensate for the way a point light is perceived long range and low intensities.  For accessing the feasibility of AtoN the focus shall be on the representation on the long range.  Observed at close range a high intensity navigation light may illuminate nearby structures.

#### Floating AtoN

A floating AtoN shall be influenced by the:

- Wind;

- Waves;

- Current;

- Mooring system;

The motions of a floating aid influenced by the above mentioned effects may modulate the flashing characteristics influencing the identification of the aid.

A real time simulator is most likely not used for designing specific buoys and the required mooring system.

However, generic or specific motion response data models, such as RAO’s may be imported into a real time simulator in order to get rather precise cues for the suitability of the floating platform as a stable carrier of and a navigation light and as a daymark in the required environmental conditions.  Making detailed motions calculations for a large number of floating aids over a large area in various wave climates may be very computational challenging.

#### Surface Material

The surface colours of AtoN shall be modelled as closely as possible to the real world colours and in accordance with IALA Recommendations E-106, E-108 including fluorescent paint.

Bands of reflective material assisting in the identification of the floating aids when illuminated at close range are not considered of major importance.

Reflection from glossy surfaces such as large glass surfaces may have significant influence on the overall contrast and therefor of major importance for the conspicuity both at day and night.

The surface material shall also reflect indirect lighting from spot lights providing flood lighting,

Indirect illumination of daymarks during night time may be pre-calculated achieving good realisim at low computational cost.

#### Geometry

The physical appearance of daymarks shall be model as realistically as possible, including overall size of mark and the topmark.  For many studies, generic buoys scaled to the correct size may be feasible.

#### Atmosphere

Scattering of light

Diffraction

Dissipation

Glare above cities

#### Terrain and Man-made constructions

Cultural lights may be considered as rival to the navigation lights.  Cultural lights shall be included as appropriate.  Near shore for inbound studies they can most like not be neglected.  The cultural lights may appear form:

- Houses;

- Large flashing screens and advertise signs;

- Moving cars, aircrafts, cranes, etc.;

- Offshore structures;

- Illumination onboard ships, such as in the accommodation and deck-lighting.

Objects shall appear with the correct perspective.  So-called billboards may be used objects in the far distance for which the perspective doesn’t change significantly.

Wind turbines shall have rotating blades when considered relevant for the study.

Shadows?

#### Ocean

The model for the ocean shall be included when assessing floating AtoN in realistic conditions.

The model shall include the effect of:

- Tide;

The tide may have a significant influence on relative height of fixed AtoN.

- Sea state;

The sea influences how floating aids are observed both at day and night, as also described in section xxx for floating aids

- Current;

The current may influence the heel of floating aids thereby changing the relative vertical divergence of lights, including characteristics and nominal range.

- Reflection in calm water surface.`

#### Weather

Wind influencing the floating aids is considered having a minor effect compared to that of current and sea state.

Clouds scatters light from the sun and the moon influencing the global lighting in the environment.

Precipitation and fog scatters light as clouds do but also and not least significantly the man-made lighting and reflected light from objects in the environment, including navigation and cultural lights.

Feasibility studies of AtoN may be made for various weather conditions.

#### Astronomical objects

The Sun and the Moon are important light sources during day and night illuminating the scene.
The scattering of the light through the atmosphere is of paramount importance for these to ‘lights’ and cannot be separated.

The simulation shall have a correct model of the azimuth and inclination based on time and day for these objects that feed into the scattering equation of light through the atmosphere.

#### Atmosphere

Whereas weather phenomena such as precipitation and fog may be considered as low altitude atmospheric phenomena, the light from the sub and the moon is also scattered by phenomena’s through the entire atmosphere.  This may include the following effects:

Humidity

Temperature

Pollutants

Frequency dependent scattering of light

### Compensation for Presentation System Limitations

As the total system for the visual presentation of AtoN to an observer, including the model and presentation is not capable of meeting the specification as found for the real system; new research and development is developing methods that aim of compensating for such gaps.

One such technology is called Tone Mapping.  Tone mapping technology adjust the rendered image such that the human brain is lured to believe that the image observed on a Low Dynamic Range display is perceived as if it was displayed on a system having a much Higher Dynamic Range High Dynamic Range for colour and contrast.

The tone mapping can be explained as making a model of the human eye that will adjust LDR images as if they were HDR images.

As described in the section on software implementation limitations in software and supporting graphics hardware are becoming fewer and fewer while still being able to meet real timer performance criterion.

### Software implementation

#### Hierarchical implementation

- Scene Graph Manager:

- Low Level API:

  - Current standard graphics API (OpenGL, DirectX) using 3x8 bit colour LDR (Low Dynamic Range):

  - Modern graphics API using 3x? bit colour space HDR (High Dynamic Range).

- Graphics Driver.

# Simulation of Radars

Not complete

## Presentation

## Modelling

Modelling of intrinsic radar flaws and limitations

Noise, clutter, blind distance, beam width, etc.

Modelling of the scenery

AtoN echoes

Racons

Wind farm echoes

# Simulation of sound

This section is limited to the possibilities of simulating the sound of AtoN – audible signals.

## Presentation

### Speaker systems

Modern full mission bridge simulator systems are normally fitted with quadraphonic speaker systems of a high quality providing good realism in the representation of the sound of AtoN and supporting the perception of sound level and direction.

### Elephant ears and advanced directional sound reproducing systems

On vessels with enclosed bridges, typical cruise vessels, a system called ‘Elephant Ears’ are used to attenuate the sounds encountered by a vessel. A sophisticated microphone system mounted outside the bridge on e.g. the forecastle will send the registered sounds to speakers on the bridge.

The elephant ear system has been integrated to simulators using a system of diodes on a panel indicating the direction of the sound and providing the attenuated sound through speakers.

Advanced systems adopted from military aircrafts can via speakers or advanced headsets give the mariner an indication of the direction and distance to a sound emitting device at a level that the mariner will not have by just using his own senses.

## Modelling

Gongs, bells, horns and sirens are typical audible signals emitted by AtoN. The mariner’s perception of sound will depend on several factors:

- the sound level emitted from an AtoN;

- the distance and direction to the sound emitting AtoN;

- the ambient noise level at the position where the mariner is listening to the sound – typically on the ship’s bridge.  Elements such as engine noise, weather noise, radio noise and other noise from devices positioned on the bridge or close to the mariner’s position on the bridge will affect the reception of AtoN sound signals.  The reception is also affected if the mariner is located inside a fully enclosed bridge or on an open bridge or in the open bridge wings.

- Environmental conditions affecting the speed of sound such as air humidity, presence of snow, rain or fog.  Ice bergs with snow, cliffs covered with ?????

- Physical environment in case sound dampening elements are present close to the vessel (e.g. snow covered icebergs, stand covered formations etc.).  Steep formations and other elements can give reflections (echo).

A given sound from an AtoN is recoded digitally and can be reproduced with the correct time pattern through speaker systems in the simulator.  The sound level, the degradation of the sound level due to the factors described above can be implemented in a simulator and if, for instance, quadraphonic speakers are used the direction of the sound can be reproduced in the simulator.

Various algorithms are used to ensure a high level of realism including the effect of environmental conditions.  Obviously, if such algorithms are not used or if the speaker system is not providing sufficient quality supporting the input or if only one speaker or stereo speakers are provided the directional element will be missing.  Likewise, if the initial production or recording of a given sound is of low quality any given high quality sound and speaker system cannot remedy this condition.

Please refer to IALA Guideline XXXX on the use of Audible Signals as Aids to Navigation for details that should be considered when simulating sound signals and IALA Recommendation E-109 for the calculation of the range of sound signals.

# Simulation of Shipborne navigation systems

## ECDIS

Features like AIS labels of other vessels and AtoN.

## PPU

(like ECDIS but relatively accurate position of the own ship shown on the intended track, prediction of the track based on current field, rudder and rpm, etc.)

## AIS as Virtual AtoN

## e-Navigation systems

IMO has developed a strategic vision for e-navigation, to integrate existing and new navigational tools, in particular electronic tools, in an all-embracing system that will contribute to enhanced navigational safety while simultaneously reducing the burden on the navigator.  As the basic technology for such an innovative step is already available, the challenge lies in ensuring the availability of all the other components of the system, including electronic navigational charts, and in using it effectively in order to simplify, to the benefit of the mariner, the display of the occasional local navigational environment.  e-Navigation would thus incorporate new technologies in a structured way and ensure that their use is compliant with the various navigational communication technologies and services that are already available, providing an overarching, accurate, secure and cost-effective system with the potential to provide global coverage for ships of all sizes.

Future e-navigation systems will include the representation of AtoN and such systems will be possible to integrate to marine bridge simulators like other navigational instruments both for test of such systems including the test of proper representation of AtoN and eventually as part of the total package of instruments representing the instrumentation of a ship’s bridge.

A few prototypes of e-navigation systems are currently being produced and could be integrated to simulators.  The systems are by nature complex and involve the interaction between several systems including data transfer systems.  The integration of a fully operational and working e-navigation system is depending on the simulators ability to provide input from all sensors and systems connected to the e-navigation system.  A simulator should normally be able to simulate errors and malfunctioning systems.  It is not fully understood at this point to what extend this is possible to replicate in a simulator.

# Conclusions

- LIST OF PHENOMENA COVERED IN RELEVANT IALA RECOMMENDATIONS AND GUIDELINES

Preliminary list of phenomena covered in relevant IALA Recommendations and Guidelines. Keywords need to be elaborated.

| References - IALA Recommendations and Guidelines |  |  |
| --- | --- | --- |
| 1023 | Leading Lines |  |
|  | Lights | Beam width |
|  |  | Beam fading |
|  |  | Range fading |
|  |  | Intensity |
|  |  | Characteristics |
|  |  | Daytime |
|  |  | Night time |
|  | Marks | Range fading Calibrate with table E6-1 |
| 1038 | Ambient Light swith on/off |  |
|  | Clouds |  |
|  | Fog, snow, etc. |  |
|  | Switch to control one or several lights |  |
|  | Shadows from eg passing ships |  |
|  | Sensor direction |  |
| 1041 | Sector Lights |  |
|  | Type of light |  |
|  |  | Point source |
|  |  | Projected |
|  |  | Slot |
|  |  | Range |
|  |  | Diverged Beam (Laser) |
|  | Sector boundaries | Width |
|  | Oscillating boundaries |  |
|  | Intensity variation between sectors |  |
|  | Daytime/night time intensity levels |  |
|  | Vertical divergence |  |
|  | Character |  |
|  | Weather | Ice on lens |
| 1043 | Light Sources used in Visual Aids to Navigation |  |
|  | Spectral content |  |
|  | Scattering in atmosphere |  |
|  | Light Pipes |  |
|  | Point Lights |  |
| 1047 | Cost Comparison methodology of Buoy Technologies |  |
|  | Radar reflector |  |
|  | Vertical divergence |  |
|  | Day mark quality |  |
|  | Anchor system |  |
| 1048 | LED Technologies and their use in Signal Lights |  |
|  | Should colours be different to identify LED light sources? |  |
|  | Colour variation due to temperature |  |
|  | With or without service allowance |  |
|  | Simulate beyond FFF? |  |
| 1049 | The Use of Modern Light Sources in Traditional Light Optics |  |
|  | Correct CIE colour for each | Lamp |
|  |  | Lens |
| 1051 | Provision and Identification of AIA to Navigation in Built-up Areas |  |
|  | background lighting | Flashing, cars, street lights, commercial, houses |
|  | Shadowing |  |
|  | Obstruction |  |
|  | atmospheric pollution |  |
| 1061 | Illumination of structures |  |
|  | Direct |  |
|  | Indirect |  |
|  | Auto switch on/off (light level) |  |
| 1065 | Aids to Navigation Signal Light Beam Vertical Divergence |  |
|  | geographical range for fixed platforms |  |
|  | geographical range for floating platforms |  |
|  | Dynamic effects for buoys | geometry, mass and mooring system |
|  | wind, waves, current |  |
|  | Vertical intensity profile |  |
|  | Spherical model of the Earth |  |
| 1066 | Design of Floating Aid to Navigation Moorings |  |
|  | Chains | Trash, rider, ground, bridle, etc. |
|  | Elastic |  |
|  | Sinker |  |
|  | Anchor |  |
|  | Environmental loads | wind, current, waves |
| 1069 | Synchronization of Lights |  |
|  | Synchronized |  |
|  | Sequential |  |
|  | Synchronization methodology | Master-slave, GPS, etc. |
|  | Failure modes |  |
| R-101 | Marine Radar Beacons (racons) |  |
|  | Polarization |  |
|  | Frequency agility and swept |  |
|  | Character |  |
|  | Duration and periodicity |  |
|  | Range |  |
| O-104 | ‘Off Station’ Signals for Major Floating Aids |  |
|  | Relevance of provision of such signals | Spheres |
|  |  | Red lights |
|  |  | Flags |
|  |  | Flares |
| E-106 | The Use of Retroflecting material on Aids to navigation Marks |  |
|  | Shall buoy models include the retroflective material codes (stripes) |  |
| E-107 | Moorings for Floating Aids to Navigation |  |
|  | Referring to guideline 1066 and 1024 |  |
| E-108 | The Surface Colours used as Visual Signals on Aids to Navigation |  |
|  | Glossiness |  |
|  | Colour type | fluorescent, ordinary |
|  | Symbols and characters |  |
|  | CIE chart matching |  |
| E-109 | Recommendation on the calculation of the range of sound signal |  |
|  | Frequencies | Harmonics |
|  | Intensity |  |
|  | Dispersion |  |
|  | Transmitivity |  |
|  | Obstacles |  |
|  | platform motions |  |
| E-110 | Rhythmic Characters of Lights on Aids to Navigation |  |
|  | temporal variations | rotation ON/OFF |
|  | Model | incandescence and nigrescence |
|  | frequency limits | Flickering |
| E-111 | Recommendation on Port Traffic Signals |  |
|  | 3 lights on top of each other | colours, characters |
| E-112 | On Leading Lights |  |
|  | Light point model |  |
|  | Day mark with illumination |  |
| O-133 | Emergency Wreck Marking Buoy |  |
|  | Lights, buoys, AIS, racons, sound |  |
| O-138 | The Use of GIS and Simulation by AIDS to Navigation Authorities |  |
|  | No technical input |  |
| E-141 | Standards for Training and Certification of AtoN Personnel |  |
| O-143 | Virtual Aids to navigation |  |
|  | Real AIS |  |
|  | Synthetic AIS |  |
|  | Virtual AIS |  |
| E-200 | Marine Signal Light |  |
| E-200-0 |  | Overview |
| E-200-1 |  | Colours |
|  | Chromaticity coordinates of light source and filter |  |
| E-200-2 |  | Luminous Range |
|  | Nominal range |  |
|  | Luminance as function of | Range |
|  |  | meteorological visibility |
|  | Service allowance |  |
| E-200-3 |  | Measurement |
| E-200-4 |  | Effective Range |
|  | Effective range | Function of frequency and spectral content |
| E-200-5 |  | Optical Apparatus |
|  | Horizontal and vertical beam intensity variation |  |
|  | Rotations performing character generation |  |
|  | Filters (boundaries) |  |