AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline No. ####

On

HYDROSTATIC DESIGN OF BUOYS

Edition 1

[Date issued]

[Previous Edition; Date issued]

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  contact@iala-aism.org       Internet:  www.iala-aism.org

Document Revisions (Title style)

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

Document Revisions (Title style)	1

Document Revisions (Title style)	2

Table of Contents (Title style)	3

Index of Tables	4

Index of Figures	4

IALA Guideline No. #### on BUOY DESIGN HYDRODYNAMICS	5

1	INTRODUCTION	5

2	BUOYANCY	5

3	STABILITY	5

4	CALCULATIONS	6

5	ANGLE OF HEEL	7

6	PRACTICAL MEASUREMENTS	7

7	Definition - Units - Notation	8

7.1	Float	8

7.2	Waterline	8

7.3	Diameter of the waterline	8

7.4	Flotation surface Sf	8

7.5	Inertia of flotation surface (water plane)	9

7.6	Flotation of a single symmetrical hull	9

7.7	Center of flotation  O	9

7.8	Weight	9

7.9	Displacement Δ	9

7.10	Displaced volume	9

7.11	Volume of reserve buoyancy	10

7.12	Center of Buoyancy	10

7.13	Buoyant force	10

7.14	Density of water	11

8	Calculation of  stability:	11

8.1	Displaced volume	11

9	Stability at small angles of heel:	13

10	Study of buoy stability :	14

10.1	Heading 2 again	15

	Bullet 1	15

10.1.1	Heading 3 again	15

11	Heading 1 again [Scope / Purpose (may be called Objectives)]	15

12	Heading 1 again [Definitions / Acronyms, as required]	16

13	Heading 1 again [as many as required]	16

14	Heading 1 again [Conclusions]	16

ANNEX A	Annex Title	17

	Office 2003, go to Format / Bullets and Numbering / Restart numbering (lower left in the box)	17

	Office 2007, go to down arrow next to Numbering icon and select Set Numbering Value	17

Index of Tables

Table 1	Title required	3

Index of Figures

Figure 1	Title required	3

IALA Guideline No. #### on BUOY DESIGN HYDRODYNAMICS

# INTRODUCTION

This guideline provides information on those aspects of buoy design that effect the buoyancy and stability of the buoy.  The Guideline 1066 should be consulted when loads due to wind and tidal effects are being considered.  It is recommended that a standard textbook on naval architecture should be consulted if the reader is not familiar with basic ship stability calculations.

The aspects of buoy design discussed in this guideline provide the basis for designing a buoy from first principles but can also be used when considering mounting different superstructures and new equipment on existing buoys or operating existing buoys in different mooring environments.

# BUOYANCY

This vital aspect ensures that the buoy will remain afloat and that the AtoNs will remain effective in all weather conditions.  There must be sufficient buoyancy to support the entire weight of the buoy and the moorings that the buoy supports.  The size and hence the weight of the buoy will largely depend on the navigational requirements of the AtoN.

The size of the daymark will depend on the daytime range at which the buoy must be visible and the height of the daymark or tower will depend on the focal plane height that is required for the light or for aerial systems. (see new guideline on daymarks)

The wave regime at the operating site may further effect the daymark size and tower height if the buoy is to be seen in rough waters.  The frequency of the waves and the roll frequency of the buoy must not be close together. Otherwise the buoy can roll in resonance with the wave.

In still water situations the buoy must have sufficient buoyancy to support its own weight plus the weight of the mooring (usually chain) between the buoy and the sea bed (at the highest water level).  A proper reserve of buoyancy must also exist to account for weather conditions and servicing requirements.

When there are wind and/or tidal flow loads acting on the buoy then the buoy will increase the loads in the mooring and more buoyancy will be required to support these added mooring loads. See IALA Guideline 1066 ------------

Waves will have some effects on the mooring loads and hence buoyancy requirements. Wave effects are extremely complex.  However if the waves are not breaking then the buoy will follow the water surface and there will be little increase in the buoyancy requirements in order to cope with the waves.

# STABILITY

Stability measures the ability of the buoy to remain upright and for the AtoN to remain effective in all operating conditions.  There are two aspects of stability that will be incorporated in the buoy design.  These are form stability and ballast stability.

Form stability is the stability provided by the shape of the buoy.  In a normal cylindrical buoy the greater the diameter of the buoy the more stable it will be.  An extreme example of this is the LANBY buoy which has a shallow 30 meter diameter 'dish' like body.  Due to its large diameter this provides stability in most situations, however in extreme conditions this can capsize and will then be stable in an inverted position.

Ballast (mass) stability is provided by the centre of mass of the buoy being in a position such that it tends to right the buoy.  The deep tailtube buoy provides an example of this type.  The ballast on the base of the tailtube provides righting forces that operate in a similar way to the mass of a pendulum returning the pendulum to the vertical position.

Stability calculations evaluate the combined effects of form stability and ballast stability.

# CALCULATIONS

The starting point for the calculations will be a detailed drawing of the buoy along with details of all the equipments to be mounted on it.  The all up weight of the buoy will then be a summation of all these components. Weights of steel work, aluminium fabrications, plastic mouldings etc can be calculated but it is recommended that checks should be made by weighing components, where this is practical.

The centre of mass, usually referred to as the centre of gravity, can then be established by summing the moments of each component about a chosen datum.  As buoys are usually largely symmetrical about their vertical axis it is only necessary to take the moments in the vertical (X) axis.  The datum is usually taken as the base line of the keel or skirt of the buoy.  The moments are then always positive.

(Insert a figure or picture  Philippe)

The immersed volumes of the components (skirt, tail tube, mooring eyes etc.) of the buoy must be calculated.  At this stage the mooring load must be decided and added to the weight of the buoy so that the line of floatation (water line) and reserve buoyancy (freeboard) can be calculated.

The moments of the immersed components about the datum can be summed and the centre of buoyancy calculated.

For calculation of stability it is usual to consider the buoy as a free floating body, ie without the moorings.  Some authorities include a small part of the moorings as a mass attached to the mooring eye(s). In the case of buoys with twin mooring eyes the bridle part of the mooring is usually included as part of the buoy mass.

With the total weight, centre of buoyancy and centre of gravity being established then the metacentric height can be calculated. The metacentric height provides a measurement of the righting moment of the centre of mass of the buoy relative to the centre of buoyancy.

- Metacentric height

description of abbreviations

If this is positive, then if the buoy is displaced from the upright position it will return back to its upright position.  If the answer is negative then any displacement of the buoy will result in it tending to capsize.  The body diameter will then have to be increased or, more usually the centre of gravity will have to be lowered.

A large value for the metacentric height will result in the buoy returning rapidly to the upright position and have a quick or 'jerking' motion.  A small metacentric height will result in a slow rolling motion, possibly with excessive displacement from the vertical.

The effectiveness of the visual AtoNs on the buoy will depend on these being clearly visible.  If the buoy has a rapid or jerky motion then it will be difficult to clearly recognise the daymark or the light character.  A slow and steady rolling motion is therefore preferred, however the extent of the rolling must not be excessive.

The ideal value for the metacentric height is difficult to define.  A guide of 10% of the body diameter was quoted for standard steel buoys (2-3 meter diameter) but this is thought to be rather small for lighter plastic buoys.

(Sipke will supply information on plastic buoys)

# ANGLE OF HEEL

The angle of heel that the buoy may assume in wind and tide conditions can be calculated from the wind and tide loading information provided in Guideline 1066.

Adrian: Insert an example with a drawing (skirt buoy)

Moments can then be taken and the resulting heel angle calculated.  The worst case is usually assumed when tide and wind forces are in opposite direction and trying to heel the buoy.

If it is anticipated that servicing personnel will work on board the buoy then calculations should be made with the estimated weight of the servicing personnel on the buoy to ensure that a reasonable angle of heel is maintained.

# PRACTICAL MEASUREMENTS

All the buoy components should be weight to confirm calculations.  The centre of gravity position can be confirmed by lifting the buoy by an off centre lifting eye.

(Insert diagramm: drawing or picture, Philippe)

The natural frequency can be estimated by displacing the buoy and timing the roll period.

If suitable calm water facilities are available, with a crane, then floatation trials can be carried out.  You can measure the draft and establish the line of floatation.  Weights at the outboard edge of the buoy can be used to heel the buoy.  The angle of heel can then be measured and used to calculate the metacentric height.

(Insert diagram and more explanation)

When the buoy is to be used in fast tide or river situations towing trials can be carried out to evaluate the buoy's performance.  The buoy can be towed from its mooring point with a ballast weight in the tow line that is sufficient to maintain a downward slope in the line from the buoy to the weight.

(Insert diagram)

# Definition - Units - Notation

## Float

The body of the buoy forms a closed volume that provides the buoyancy of the buoy when immersed in water.  This is usually a cylinder and is treated as such in the following calculations.

The float carries the superstructure/daymark, the tail tube or skirt and external ballast weights.

## Waterline

The waterline FLi is the line between the submerged part of the float and the part of the float out of the water.

## Diameter of the waterline

Diameter of the waterline D (m) is the diameter measured at the waterline.

## Flotation surface Sf

Flotation surface Sf (m2) is the area of the plane formed by the waterline of the buoy.  Usually 'water plane ' in English.

NOTE

The area of any tube passing through the float and open to the surrounding water must be subtracted.

## Inertia of flotation surface (water plane)

Inertia of flotation surface (water plane) If (m4) is the moment of inertia of the flotation surface. It characterizes the energy needed to rotate the surface around an axis.

For a circle If = 1/64 *  * D4

D : diametre (m)

In the following study, we consider that the axis pass through the centre of the flotation surface.

## Flotation of a single symmetrical hull

When the float of the buoy is inclined at an angle α, the volumes of the submerged and the emerged segments are equal.  The displacement of the buoy does not change.  The intersection of the submerged and emerged segments passes through the centre of the flotation surface.

## Centre of flotation O

Centre of flotation O is the centre of the plane of floatation.

## Weight

Weight P (N) is the vertical force of gravity downwards, applied on the buoy. This is the summation of the weights (m) of the individual components.

P= M *g =  mi *g

XCDG =  xi * dmi

M

## Displacement Δ

Displacement Δ (in kg) is the multiplication of the total volume displaced (Vd) by the buoy by the specific weight of water.

Δ = Vd * 

: density of water

## Displaced volume

Displaced volume Vd (m3) is the submerged part of the buoy limited by the water line. The displaced volume contains the skirt, mooring eyes, tail tube, external ballast etc.

The  total sum of all forces is zero:  Fext = 0

So the weight of the buoy minus the displacement is zero: P - Δ = 0

from [M * g] – [ Vd *  * g] = 0

we obtain Vd = m



where

m : total mass of the buoy

: density of water: 1 025 kg/m³

## Volume of reserve buoyancy

Volume of reserve buoyancy Rf  is the volume of the float that is above the water line.  This will prevent the buoy sinking if additional equipment is added or there is an increase in mooring loads.

Volume of the buoy = Volume of reserve buoyancy + displaced volume

V = Rf + Vd

then

Rf = V – Vd

## Centre of Buoyancy

Centre of Buoyancy B is the geometric centre of the displaced volume of water.  It is the application point of the buoyancy forces.

The position of the centre varies with loads, roll and other movements of the buoy.

XCDC =  xi * dvi

V

(discribe the formula: Philippe)

## Buoyant force

The buoyant force D is the force applied to the portion of the submerged float, due to the displacement of water. This force comes from the increase of the fluid pressure.

As pressure is greater on the lower part of a submerged object than on its top, the buoyant force is a vertical thrust directed upwards.

Definition: "An immersed body in a fluid undergoes a vertical force directed upwards and opposite to the displaced fluid weight.

D = Vd * * g

D = m* g

## Density of water

Density of water  depends on:

- Temperature

- Salinity

- Pressure

Typically, we retain the average value of density of seawater

- north Atlantic seas at 1025 kg / m³.

- Mediterranean sea at 1028 kg/ m³.

- estuary waters from 1000 kg/ m³ to 1015 kg/ m³.

# Calculation of  stability:

Mass balance and position of the centre of gravity :

Before starting the calculation of stability a mass balance should be made by summing all the masses of the component parts and equipment on the buoy and similarly their moments.  The position of the centre of gravity can then be determined.

Then, we obtain the total mass M of the buoy and the vertical position of the centre of gravity G (CDG noted).

In general, we use the orthonormal reference at the baseline of the buoy, noted point K. The moments are then always positive.

insert a diagramm

## Displaced volume

The displaced volume is calculated by dividing the total mass of the buoy by the density of water.

Thus:

Ainsi :

Displaced volume = 	 total mass of the buoy     (kg)

density of water (kg/m3)

Vd = m



Verification of the buoyancy:

This calculation ensures that the buoy doesn't sink. The displaced volume must be less than the total volume of the float.

Vd < V

Thus, we deduce the volume of reserve buoyancy, which allows us to estimate the additional load that the buoy can support.

- If the buoyancy reserve volume is positive (Rf > 0), then the buoy floats.

- If the buoyancy reserve volume is negative (Rf  <0), then the buoy sinks.

## Position of the centre of buoyancy

From the displaced volume and geometric characteristics of the float, we deduce the position in X, of the centre of buoyancy B.

Note:

CAD drawing systems provide centres of volume

(illustration or reference to figures before)

Righting arm:

At  equilibrium, the initial flotation is FL0. The centre of buoyancy is C0.

The buoy is displaced to an angle α relative to its equilibrium position and a new waterline FL1 appears. The centre of buoyancy  moves from C0 to C1.

M1 is the intersection point with the line (C0 G) and the vertical line passing by C1. The point M1 is called: metacenter.

The height C0 M1, corresponding to the angle of heel is called metacentric height.  It is noted "h".

The distance (C0 G) is designated "a " and is counted positive (> 0) when G is above C0.

For the angle of heel a righting moment noted m, occurs, such as:

m = P * GZ

GZ is the righting arm.

number and description of the figure

In the right angle of the triangle in Z

GZ = GM1 * sin(

then

GZ = ( h - a) * sin(

therefore

m = M*g * ( h - a) * sin(

We deduce :

- If M1 is above G, then (GM - a ) > 0 and the righting moment m straightens the buoy. (upright position)

- If M1 is below G, then (GM - a) <0 and the buoy overturns.

Stability Condition:

The equilibrium is stable when the centre of gravity  G is below the metacentre.

# Stability at small angles of heel:

When the angle moves from α to 0, then the point M1 tends to M, called the metacentre at the initial flotation FL0.

The "metacentric height" noted "GM" tends toward "ρ" called metacentric radius.

The righting moment becomes zero : m  = 0

Then, the distance GM is:

GM = ( - a ) is the modulus of initial stability.

and the metacentric radius "ρ" is written as:

ρ =  If

Vd

Weight stability and form stability :

To improve the righting moment, we can alter either the weight stability or the form stability.

The expression of the righting moment is only valid for small angles, approximately 10°.

Form stability: M*g * ρ : is the modulus of the form stability. It depends on the geometric shapes of the float, in the case of a cylindrical float on the diameter.

 = If   . in effect, If  depend on the diameter of the float.

Vd

Weight stability: M * a * g: is the modulus of the weight stability. It depends on the relative position of the G and the B.

It depends on the mass distribution on board the buoy.

By adding ballast, the weight increases and the G will be lowered. If the length of the tail tube is increased then the G may be lowered with little increase in weight. However, the displaced volume and center of buoyancy will change.

(add a rule of thumb at roll frequency calculation)

# Study of buoy stability :

Stability is the ability of the buoy to return to an upright position when external actions such as wind, waves and tide current have displaced it.

Stability is considered adequate if the buoy retains this capacity in all situations.

We can study the stablity via the curve of righting arm at small angles of heel.

Factors that may affect the stability are:

•	The loading of the buoy,

•	The presence of overturning moments due to external forces.

However, the curve of righting arm does not take into account the righting moment generated by the mooring line when the buoy is heeled.

However, not taking account this moment increases the safety margin of   the buoys stability.

Method: At each angle of heel , we determine the value of the righting arm GZ.

●	Analysis of the curve:

The total area under the curve of stability is the stability reserve.

Slope of the curve: the slope of the curve is the value of the modulus GM of the initial stability.

Limit angle of dynamic stability (θd): Angle for which the surfaces A1 and A2 are equal.

There is danger of capsizing beyond this angle for external loads acting on the buoy.

The area A1 is the energy stored by the float to reach the angle (θd).

It is the work necessary to tilt the vessel θ = 0 to θd.

The area A2 is the dynamic reserve that is enable to dissipate the energy stored in area A1.

Dynamic stability requires: A1 <A2.

Limit angle of static stability (θmax): The angle (θmax) is the peak of the curve. It is the maximum angle.

Limit angle of static capsizing (θs) : the angle (θs) is the static capsizing angle.

Beyond this angle, the righting moment becomes negative: a prompt capsizing happens , without the action of a heeling moment.

add some information on roll frequency

Followed by body text

Background would be a section of the introduction, if required.  It could refer to previous editions or other IALA documents that have been used / are superseded by this document.

## Heading 2 again

- List 1

Can be followed by List 1 text

  - List 1 indent

Can be followed by List 1 indent text

    - List 1 indent 2

Can be followed by List 1 indent 2 text

Followed by body text

- Bullet 1

Can be followed by Bullet 1 text

- Bullet 2

Can be followed by Bullet 2 text

- Bullet 3

Can be followed by Bullet 3 text

### Heading 3 again

Can be followed by followed by Body Text Indent 2

#### Heading 4 again

Can be followed by followed by Body Text Indent 3

#### Heading 4 again

Followed by Body Text Indent 3

# Heading 1 again [Scope / Purpose (may be called Objectives)]

Body Text

- Title required

Note.  Titles of Tables go above them.  This example has font size 10 with 3 pt above and below text and with the text centred vertically

# Heading 1 again [Definitions / Acronyms, as required]

Body Text

- Title required

Figure titles come after the figures.  Graphics should, preferably be inserted at a text point and then centred.

# Heading 1 again [as many as required]

Body Text

# Heading 1 again [Conclusions]

Body text

- Annex Title

Guidelines should have Annexes.  Appendices are attached to Annexes.

ANNEX HEAD1

Body Text

To restart the Annex Heading numbering at 1:

- Office 2003, go to Format / Bullets and Numbering / Restart numbering (lower left in the box)

- Office 2007, go to down arrow next to Numbering icon and select Set Numbering Value

Annex Heading 2

Body text

Annex Heading 3

Body Text Indent

Annex Heading 4

Body Text Indent 2

- Appendix title

- APPENDIX Heading 1

Followed by Body Text

  - Appendix Heading 2

Followed by Body Text

    - Appendix Heading 3

Followed by Body Text Indent