Buoy History

Visit to Trinity House in 1873

I have found the text of a visit to Trinity House in 1873 by col. George Elliot of the United States Lighthouse Board.  There are comments on various ligthouses and lightvessels and the following points noted about buoys and their moorings.

New buoys were manufactured from iron although the majority of buoys in service were still wooden.  Many iron buoys were water ballasted, that is they had a compartmented section at the bottom of the buoy that was allowed to fill with sea water through 25mm holes.  When these buoys were used on deep water stations the weight of the mooring chain was sufficient to stabilise the buoy and the ballasting holes were plugged with hardwood bungs.  This also had the effect of maintaining similar freeboards for buoys on shallow stations and those on deep stations.

'Egg Bottom' and 'Herbert's' pattern buoys were used offshore.(The Paris exhibition Trinity House drawing illustrates these buoy types). The Herbert's buoy has a hollow bottom with a recessed mooring eye and these were considered to ride better and have less mooring eye wear than the flat bottom and conical buoy types.

Small buoys were taken onshore every six months while large buoys would remain one or two years on station and be re-painted on station.

Trinity house had detailed specifications for their mooring chains.  All chain, shackles and swivels were proof loaded and samples were subject to breaking loads and inspection of welds.  Moorings were usually 3 x water depth.  Details are provided for particularly exposed moorings where 38mm chain is used with a 1.5 tonne sinker with a further 50m length of 38mm chain connecting to a second 1.5 tonne sinker.  Worn lightvessel chain was used for some buoy moorings and some of this was stud link.

Early fast water buoys.

The following detail from a marine painting shows a conical wooden buoy with a vertical staff and spherical, cage topmark on the Margate Hook station. This was painted in 1828. These conical wooden buoys would have floated on their side, as this picture illustrates, as soon as there was any tidal flow or current.

The line drawing  (Dated 1880) from the Dutch book 'Als 't diep verloopt verset men de bakens' shows the construction of a similar buoy and shows that an iron keel was fitted. The keel would keep the buoy aligned with the tidal flow and its weight would provide ballast to keep the daymark upright.

Early Lampchangers

The photograph shows early 'Wallace and Tiernan, lampchangers that are in the USCG training school museum.