IALA Guideline No. ####

On

Daymarks for Aids to Navigation

Edition 0.6

April 2012

AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  iala-aism@wanadoo.fr       Internet:  www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
|  | all | Developing guideline in EEP 17 |
|  |  | Intersessional working group for EEP18 |
|  |  | Working paper from EEP 18 |

Table of Contents

1	Introduction	5

2	SCOPE	5

3	Definitions	5

4	Basic Principles	6

4.1	Degrees of Perception	6

4.2	Information Process	6

4.3	Purpose	7

5	Daymark – the source of the information	8

5.1	Profile	8

5.1.1	Flat daymark	8

5.1.2	Compact daymark	9

5.1.3	Crossed plates	9

5.1.4	Lattice construction	10

5.2	Luminance of the object	11

5.3	Surface Colour	11

5.3.1	Aging of colour	13

5.3.2	Retroreflective sheeting	13

5.4	Size	13

5.5	Combination of colours	14

6	Viewing Conditions	15

6.1	Viewing Distance	15

6.2	Meteorological Visibilty	16

6.3	Koschmieder's law	16

6.4	Background	18

6.4.1	Sky as the background	19

6.4.2	Watersurface as background	20

6.4.3	Landscape as background	20

7	Observer – the receiver of the information	21

7.1	Simple model of the human eye	21

7.2	Eye resolution and subtense angles	22

7.3	Non-uniform daymarks	22

7.4	Shape recognition	23

7.5	Required contrast	25

7.6	Blaise Theory	25

7.7	Colour recognition	26

8	DESIGN OF DAymarKS	26

8.1	Floating Aids	26

8.1.1	3-dimensional Profile	27

8.1.2	Colour	27

8.1.3	Size of single coloured buoys	27

8.1.4	Cardinal-buoys	27

8.1.5	Safe water and new-danger marks	29

8.1.6	Structural obstructions	29

8.1.7	Buoy shape	31

8.1.8	Topmarks	32

8.2	Fixed aids	33

8.2.1	Traditional light houses	34

8.2.2	Additional tools for buildings used as daymarks	36

8.2.3	Dayboard of leading lines	37

8.2.4	Marking of bridges	38

8.2.5	Design of signs	38

8.2.6	Canadian daymarks	39

8.3	General Design Methodology	40

8.4	Additional information and advices (Super Structures)	42

9	Factors that reduce visual range	44

9.1	Factors that reduce detection	44

9.2	Service factors	45

9.3	Environmental limitations	45

Index of Tables

Index of Figures

Daymarks for Aids to Navigation

# Introduction

It is often considered that the installation of a maritime signal consists of installing a light on a support without conducting any kind of preliminary study on the daymark range, colour and other features of the daymark. But it should not be forgotten that most of the traffic occurs during the day. So the daymark should be identified by the mariners at a reasonable distance without possibility of confusion.

Existing IALA documents provide information on the proportions of daymarks but only limited information is given on the range at which daymarks can be effectively detected, recognised and identified.

This guideline provides a general informative overview of the main factors affecting the visual performance of a daymark as an aid to navigation and how to determine the useful design of a daymark.

It points out the aspect of visual perception and how to optimize the visual identification of a daymark. However the construction is limited by financial and technical aspects. So the guideline describes an aim for the construction of a daymark, which may not be reached due to the limitations in practice.

HINT:

The guideline includes many illustrations and fotos. Due to the limitations of the reproduction of pictures by computer monitors and printers and the different illumination in practice these pictures do not represent the real situation. It is advised not to use the pictures for visual trials or for the estimation of the visible performanve of daymarks.

# SCOPE

The scope of this document will cover all those AtoN as defined in the IALA MBS [1]. This guideline does not cover the use of signs and lights as aids to navigation during daytime.

For information about daytime lights refer to IALA-Rec. E-200 series [7].

For information about signs refer to IALA Conference Presentation 1990 about lettering and signs [4].

The competent authority should ensure that structures and objects in the vicinity of fairways and waterways do not conflict with marks according to the IALA Maritime Buoyage System.

If such structures include daymarks, these will be designed and maintained in accordance with the IALA Maritime Buoyage System and this guideline.

If other structures or objects are designed and built in the vicinity of the waterway, conflict between these and the daymarks (according to the MBS and this guideline) should be avoided.

# Definitions

| Daymark | IALA Dictionary 2-6-030 For the purpose of this guideline, a daymark includes shape, colour and colour combination (as defined in MBS) as well as any identifying markings. |
| --- | --- |
| Transparent daymark | #### Effect of partially transparent daymarks (see 5.1.4) |
| Photometric contrast | Contrast of a daymark against its background without taking into account atmospheric extinction. (intrinsic contrast ?) |
| Observation contrast | Contrast of a daymark against its background taking into account atmospheric extinction. |
| Daymark Range | The maximum distance at which the daymark can be identified according to the third degree of perception as stated in 4.1) |

# Basic Principles

The visual task is to provide a daymark, according to the IALA Maritime Buoyage System, that can be easily identified at a given distance, against a given or prevailing background. This means that the mariner identifies the shape, the colour and colour combination.

The visible range of a daymark will be determined by the Competent Authority in an overall assessment of navigational requirement for the waterway or particular location.  A daymark’s visible range will not necessarily be linked to the range of the associated light.

The recognition range (see 4.1, second degree of perception ) of an AtoN’s daymark should be comparable to the range of its light.  Once the AtoN’s is recognized, binoculars can be used to achieve identification.

However, if the recognition range of the daymark is less than the light, the following alternatives may be employed:

- Ensure that the distance between lighted AtoN are consistent with their visual daytime range;

- Place unlighted AtoN between lighted AtoN;

In cases where the daymark is deemed not necessary to safe navigation it may not be necessary to consider the range of the daymark.’

A Daymark is defined by size, luminance, colour, shape and height above sea level.  To estimate the visual performance the background behind the daymark and the illumination by the sun have to be taken into account.  Background and illumination depend on temporal, geographical and meteorological conditions.  Reference background depends on height of the vessel’s bridge and direction of observation.

## Degrees of Perception

When a navigator approaches a visual AtoN, for instance a buoy, the first thing he will recognise is the shape or colour of the buoy depending on the viewing conditions .

He will subsequently recognise the top-mark and finally its numbers or letters. Thus, the process of identifying a visual AtoNs complete message goes through different stages of perception.

There are three degrees of perception of daymarks

- Detection: - The observer is aware of an object. He sees an object, but he will usually not be able to deduce its shape or colour and will not know that it is an AtoN.

- Recognition: The observer is aware that the object is an AtoN.

- Identification: The observer is aware which AtoN the object is. At this distance, the navigator can perfectly discern the type of mark it is.

## Information Process

When considering the perception process of daymarks, we have;

- Something which creates 	the signal Source (interaction of sunlight with the
object)

- Something which conducts 	the signal Media (the atmosphere)

- Something which receives 	the signal Sensor (the observers eye)

In detail the information process for daytime observation has a lot of parameters which influences the perception (see figure 1).

Object:

The object as the signal source can show different shapes and can be illuminated by direct sunlight or diffus or combination of both. The background may be different depending on  locations and may vary with time (weather, sun position).

Media / Viewing Conditions:

The atmosphere as the media has a certain value for meteorological visibilty (e.g. haze, fog), which causes the light from the object to be scattered partially away from the line between the object and the observer. On the other hand direct and indirect light is scattered partially into the observation line towards the observer.

Observer:

In general the observer is limited by physiology. There are some agreements or conventions about the maritime observer - especially for commercial shipping. However there are no exact values for visual perception in physiology and the convention may be different in each country.

figure 1: information process

## Purpose

There are two different tasks when designing a daymark.

(1) The daymark has to be identified by a distant observer.The quality of the daymark can be described by the daymark range: the maximum distance where the daymark can be identified.

(2) The daymark has to be conspicuous in a complex background scene.

# Daymark – the source of the information

## Profile

The daymark has a 3-dimensional profile. When viewing the daymark, the user perceives a 2-dimensional shape, which is a projection of the 3-dimensional profile.

figure 2: Perception of an extended object

The 3-dimensional construction of the daymark profile is of significant influence on the 2-dimensional shape seen from different directions.

For AtoN daymarks the following constructions are in use.

### Flat daymark

A flat daymark can be used when the observer's position nearly perpendicular to the surface. Because there is only one surface element, which is orientated in a single direction, the surface looks uniform in all situations.

figure 3: flat daymark and line between the vertical of the surface and the observer's direction

The projected shape into the oberserver's direction changes with the angle between the vertical and the oberserv as shown in the following figure. Even for angle 15° and 30° the shape can be identified. A flat daymark is the recommended construction for leading or range lines (range dayboards).

figure 4: projected (visible) shape

### Compact daymark

#### A daymark which shows the same shape in all horizontal directions must have rotational symmetry for the vertical axis. The 3 shapes of IALA MBS will lead to cylinders, cones and spheres. Because there are surface elements which have a different orientation the projected surface may not look uniform.

figure 5: profiles with rotational symmetry

#### Compact omnidirectional daymarks are used on buoys and for low range. They can be manufactured in plastic (rotated polyethylen). For higher ranges the size and the weight of a compact daymark limits its use. A compromise may be the use of crossed plates or a lattice construction.

Some competent authorities are investigating alternative materials for compact daymarks; for example, the Swedish Maritime Administration (SMA) is looking into using rubber or flexible plastic bristles to create daymarks that will be more resilient to severe ice conditions.

### Crossed plates

Crossed plates are used for top marks and to enhance the buoy shape. They need less material, have a lower weight and are easy to manufacture. When the material is metal the crossed plates can act as a radar reflector as well.

figure 6: profiles with crossed plates

### Lattice construction

#### A lattice construction is similar to the three shapes above, but the structure is broken It is generally used for large daymarks to reduce weight and wind load.

figure 7: lattice construction derived from crossed plates

The gaps reduce the contrast of the daymark when it is looked at from a far distance (gap near or lower the eye's resolution). figure 7 shows crossed plates only, however a lattice construction may be used for flat daymarks and for daymarks with rotational symmetry (figure 8).

figure 8: lattice construction of a special mark buoy

## Luminance of the object

The luminance is the brightness of an object and is defined as the luminous intensity per unit projected area of any surface, as measured from a specific direction (see IALA-Recommendation E-200 On Marine Signal Lights, Part 3, Measurement).

It depends on its profile, its colour, the illumination of the object and the orientation of the surface elements. Parts of the daymark may show different luminance caused by different orientation or shadowing.

The luminance may change due to aging of the surface. In most cases non-fluorescent colours tend to get unsaturated resulting in a higher luminance. For fluorescent colours the luminance may become higher or lower depending on the specific material.

## Surface Colour

The Surface Colour is described by a luminance factor β and two chromaticity coordinates x, y (see IALA-Recommendation E-108 On Surface Colours).

The IALA MBS uses 5 Colours: Red, Yellow, White, Green, Blue and Black. The following figure shows the CIE standard chromaticity diagram and the location of the five colours (black and white are at the achromatic point).

figure 9: CIE standard chromaticity diagramm

The luminous factor and the chromaticity coordinates should fulfil the specifications of IALA Recommendation E-108 in order to be used according to IALA MBS. These Specifications leave freedom to adjust the colour to a specific viewing condition.

A non-fluorescent colour (ordinary colour) can only reflect light which is incident on the surface. To produce a specific colour the other spectral part of the incident light must be absorbed. To achieve a very saturated colour, the amount of spectrum absorbed becomes large and the reflected part very small. So a high saturation is only available for colours with low luminance. For ordinary colours a compromise between the levels for saturation and luminance has to be found.

For fluorescent colours a higher level for the luminance and saturation can be achieved.

| Colour | IALA Rec. |  |  | Maximum (fluorescent) |  |
| --- | --- | --- | --- | --- | --- |
|  | ß | NCS | RAL | ß | RAL |
| Red | > 0.07 | S 1085-Y80R | 3028 | ≈ 0.50 | 3024 |
| Yellow | > 0.50 | S 1080-Y | 1023 | ≈1.00 | - |
| Green | > 0.10 | S 2070-G10Y | 6037 | ≈ 0.50 | 6038 |
| Blue | > 0.07 | S 4050-R90B | 5017 | - | - |
| White | > 0.75 | S 0500-N | 9016 | - | - |
| Black | < 0.03 | S 9000-N | 9017 | - | - |

Table 1 - Values for the luminance factor β and Numbers of colour collections

For practical considerations it is better to describe the colour by colour collections. The IALA currently supports two colour collections which are known worldwide.

- Swedish Natural Colour System NCS

- German RAL Colour

The table shows the recommended colours according to the two collections. For further information see IALA Re. E-108.

Surface Colour can be produced by

- Films, sheeting

- Paint

- Plastic, coloured throughout

### Aging of colour

There are various aspects of colour aging. The deterioration of the colour may be caused by the following influences:

- fading of the colour pigment

- mechanical abrasion of the surface

- fouling (the colour is hidden under a natural cover)

The fading of the colour pigment often results in a desaturation of the colour, that means it moves towards the achromatic point.

### Retroreflective sheeting

A retroreflective surface should not be used for daymarks throughout. The retroreflective principle works only when the surface is illuminated at night with a searchlight near the observer. At daytime the retroreflective colours show low luminance and are not very conspicuious.

For this reason the industry has introduced retroreflective-fluorescent films. However the colours available are made for road traffic and there is not a complete set of films for the IALA MBS colours.

Retroreflective films can be used for some small parts of an unlighted buoy, so the mariner can detect the position and colour at night by use of a search light.

figure 10: typical application for retroreflective sheets on a buoy

## Size

The size of a daymark is directly linked to its range. As stated in 5.1 the observer sees a projection of the real object and the lateral dimensions of this projection are relevant for the estimation of the daymark range.

figure 11: lateral extension and range of a daymark

The angular subtense α (viewing angle) of the daymark can be calculated by :

- α [radiant] ≈ height / distance (for the vertical angle)

- α [radiant] ≈ width / distance (for the horizontal angle)

With these formulae it can be seen that the range of the daymark depends on the minimum useful viewing angle αmin assigned to the daymark.

Range ≈ (lateral dimension) / αmin [radiant]

With: lateral dimension: height or width

The minimum useful viewing angle for a specific daymark depends on all the parameters mentioned in 4.2. There are different conventions for this angle.

For symbols and letters an IALA publication was presented at IALA-conference 1990, which includes a convention about the minimum useful viewing angle for many CEVNI and SIGNI signs [4].

## Combination of colours

Many daymarks of the IALA Maritime Buoyage System consist of a combination of colours e.g. cardinal or isolated danger marks.

For the identification of the daymark the whole arrangement of stripes must be recognized.

In the example below (figure 12) the vertical size of the object is seen as the limiting factor for the range of the daymark. At the range d the single coloured object appears under a viewing angle of α for the observer. When the same object is coloured to show an isolated danger mark, the observer has to recognize each stripe in the same manner as the single coloured object.

So a single stripe has to appear under a vertical angle α. This leads to a range of the danger mark that is one third of the single coloured lateral mark.

figure 12: single and multi coloured daymarks

Allthough this calculation is not exact because the horizontal dimension was not regarded, it can be stated generally:

The range of a multi coloured daymark is significant lower than for

a single coloured daymark of the same size.

# Viewing Conditions

## Viewing Distance

The first aspect of viewing conditions is the distance between the daymark and the observer and their vertical position. Because of the curvature of the Earth the daymark must have a certain height to be seen above the horizon.

The minimum height hb,min of the lowest part of the daymark is given by the formula for geographical range Rg.

Rg=2.03×ho+hb,min

Where:

Rg = geographical range (nautical miles)

ho = elevation of observer’s eye (metres)

hb = elevation of the base of the daymark (metres)

The constant is to allow for the curvature of the Earth and Atmospheric factors [8]

If there are actual or potential obstructions between the observer and the daymark, then the height may have to be modified to allow for this obstruction.

Table 2 shows some examples of ranges that can be achieved given the elevation of the observer and the elevation of the base of the Daymark being observed.

Table 2 - Geographical Range in Nautical Miles (M)

| Observer’s eye height (metres) | Elevation of Daymark (metres) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 10 | 15 | 20 |
| 5 | 6.6 | 7.4 | 8.1 | 8.6 | 9.1 | 11.0 | 12.4 | 13.6 |
| 10 | 8.4 | 9.3 | 9.9 | 10.5 | 11.0 | 12.8 | 14.3 | 15.5 |
| 20 | 11.1 | 11.9 | 12.6 | 13.1 | 13.6 | 15.5 | 16.9 | 18.2 |
| 30 | 13.1 | 14.0 | 14.6 | 15.2 | 15.7 | 17.5 | 19.0 | 20.2 |
| 40 | 14.9 | 15.7 | 16.4 | 16.9 | 17.4 | 19.3 | 20.7 | 21.9 |
| 50 | 16.4 | 17.2 | 17.9 | 18.4 | 18.9 | 20.8 | 22.2 | 23.4 |

However the daymark range is in most situations smaller than the luminous range at night and many daymarks are for short range navigation only. So the geographical range for a daymark is often much longer than the daymarks range.

## Meteorological Visibilty

The meteorological visibilty is a way to describe the atmospheric extinction of light by the atmosphere.

Definition (IALA dictionary):

The greatest distance at which a black object of suitable dimensions can be seen and recognised by day against the horizon sky, or, in the case of night observations, could be seen and recognised if the general illumination were raised to the normal daylight level. The term may express the visibility in a single direction or the prevailing visibility in all directions.

The meteorological Visibilty is not the daymark range. The defintion describes a method of how to estimate the visibilty of the atmosphere. The object must be black to ensure maximum contrast aganist the sky at day and it must be very large to provide a sufficient viewing angle for the observer. In practice a mountain with forest or a dark building can be used.

An AtoN daymark has a colour according to the IALA MBS and therefore a smaller contrast than black. The size of the daymark is limited as well. From this point of view it can be stated generally:

The range of an unlighted daymark is always

less than the meteorological visibilty.

## Koschmieder's law

One parameter relevant for the visibilty of an object is the apparent contrast of an object at the position of the observer (observer contrast).

The contrast C is defined by (see IALA dictionary):

C=Lo−LbLb

Lo	Luminance of the object

Lb	Luminance of the background

In figure 13 the scattering of light in the atmosphere is illustrated. Some amount of the (coloured

) light which moves in the direction of the observer is scattered by the atmosphere into a different direction and gets lost (1). Some light (from background or sun light) is scattered into the direction of the observer (2).

figure 13: simplified illustration of light scattering in atmosphere

The photometric contrast C(0) can be measured directly in front of the daymark.

Koschmieder's law says that the visible contrast at the observer's eye at a distance d is:

C(d)=C(0)∗e−z∗d

Where:

z is the exponential factor (total attenuation includes absorption and scattering)

d is the distance

According to IALA-Rec E-200-2 the formula can be expressed by using the atmospheric transmissivity TM.

C(d)=C(0)∗TMd

Where:

TM is the transmissivity for 1 nautical mile

d is the distance in nautical miles

When the meteorological visibilty V is used the formula becomes:

C(d)=C(0)∗0.05dV

Where:

V is the meteorological visibility in nautical mile

d is the distance in nautical miles

Although Koschmieder's law is accepted as the basis for many calculations it must be stated that it is based on a very simple physical model. The atmosphere must be homogenous and the illumination must be uniform and diffuse.

Especially for direct sunlight illumination Koschmieder's law is not very exact because the amount of light scattered into the observer's direction depends on the sun position.

In the physical model of Koschmieder the visible luminance of the object at the Observer's eye can be calculated as well:

Lod=Lo∗e−z∗d+Lb∗(1−e−z∗d)

Where:

Lo is the luminance of the object

Lb is the luminance of the background

z is the exponential factor (total attenuation includes absorption and scattering)

d is the distance

The main conclusion of Koschmieder's law is that an object seen from a far distance will fade and contrast becomes zero.

The original formula of Koschmieder does not include colour. This was done by Hoffmann in 1977 [7]. He calculated the movement of chromaticity X, Y, Z depending on the distance, where Y is identical with L.

Xod=Xo∗e−z∗d+Xb∗(1−e−z∗d)

Yod=Yo∗e−z∗d+Yb∗(1−e−z∗d)

Zod=Zo∗e−z∗d+Zb∗(1−e−z∗d)

With these calculations it can be shown that the colour of the object will move to the colour of the background when seen from far distance.

Although the formulae above include colour it does not include the spectral dependency of the atmospheric light scattering.This could be done with the spectral distribution of the light radiance Sλ .

Sλ,od=Sλ,o∗e−zλ∗d+Zλ,b(1−e−zλ∗d)

This calculation requires the spectral distribution of the background Sλ,b , the object Sλ,o and the exponential factor zλ .

## Background

The contrast C depends on the Luminance of the daymark and the background in its vicinity, so the visual detection of the daymark is strongly influenced by the background.

The background can be divided into 3 main groups.

- water surface near the horizon

- sky near the horizon

- shore with different landscapes or buildings

All the backgrounds may change with weather conditions and season.

In detail we have:

Water surface:

- changes with wind and waves

- Ice in winter

- Sun light reflections

Sky:

- Blue sky, red sky

- Clouds

- sun behind daymark

Shore:

- Vegatation in summer and winter

- Snow

As the daymark cannot change there have to be made some conventions about which background will be dominant through time.

### Sky as the background

For small ships the daymark may appear before the horizon sky.

figure 14: Sky as Background

Although all topmarks are limited in size, it is found that the black topmarks of the IALA cardinal and isolated danger marks have best contrast.

The luminance of the sky varies at day between 100 and 50 000 cd/m² (see IALA-Rec. E-200 part 2). As an example the statistical distribution of the sky luminance near the horizon is shown in the next figure. The values are based on measurements at the baltic sea from January to December 2003.

figure 15: Statistical distribution of sky luminance, baltic sea 2003

Sun behind a daymark

The recognition of a daymark when the sun is visible in the vicinity at sunset or sunrise is poor. In many situations colours and symbols cannot be recognized. However the shape (outline) of the daymark can be detected.

### Watersurface as background

For large vessels and floating aids the vertical position of the observer is much higher than the position of the daymark so the background for observation is the watersurface.

figure 16: Watersurface as background

The watersurface is illuminated by the sky near horizon. A smooth watersurface acts as a perfect mirror for flat incident angles. However because of the roughness of the sea due to wind and waves the luminance of the watersurface is lower than that of the sky near horizon.

The measurements 2003 at the baltic sea showed that the luminance of the watersurface is approx. 50% of the sky luminance at the horizon.

The statistical distribution is shown in figure 16.

figure 17: Statistical distribution of watersurface luminance, baltic sea 2003

### Landscape as background

When the landscaoe or shore is the background of the daymark, the contrast depends on the 'albedo' or reflectance of the landscape. As for the water surface the landscape appears darker than the sky in most situations. In most cases the albedo value is identical to the luminance factor (see [14]).

Table 3 - Albedos for differents surfaces [15]

| Surface | Typical albedo |
| --- | --- |
| Asphalt | 0.04 - 0.12 |
| Forest | 0.08 - 0.15 |
| Bare soil | 0.17 |
| Green gras | 0.25 |
| Desert | 0.40 |
| New concrete | 0.55 |
| Ocean ice | 0.5 - 0.7 |
| Fresh snow | 0.9 - 0.9 |

# Observer – the receiver of the information

## Simple model of the human eye

Both physical and psychological aspects of the observer have influence on the visual perception. Allthough the human eye acts as the physical sensor, the picture the observers sees is the result of an image processing by the neuronal parts of the eye and the brain.

A simplified model of the human eye is presented in figure 18

figure 18: Simple model of the eye

The simple optical model of the human eye consists of a focusable lens, a pupil, which defines the open diameter of the lens and the fovea (sensor) near the optical axis.

The eye has the following properties:

- focal length: approx 16 mm when focused at infinity

- pupil diameter: 2 - 8 mm

- sensor density at fovea: cones 142 000 cells/mm² ( 377 cells/mm)

- visual sensitivity: approx. 10-5 cd/m² - 10+5 cd/m²

Remarks

The eye has two different sensor types: cones and rods. The cones are for daytime and the rods are for night time perception. For daymarks the cones are relevant. It can be assumed  that the identification of a daymark requires that the observer looks straight at it. That means the daymark and its picture are at the optical axis of the eye. In this region the cone density is at its maximum, so the eye resolution has its maximum too.

## Eye resolution and subtense angles

The eye's lateral resolution is approx. 1' (arc minutes).

The fovea cones have a distance of about 1/ 377 mm ≈ 2.65 μm and this distance corresponds to an angular separation of

α≈arctan2.65 μm16 mm≈0.57'.

For very good vision the observer might resolve objects that appear smaller than 1', but

when identification is required, the minimum useful viewing angle should be larger than 1'.

This means:

1 arc minutes is the lower limit for any useful viewing angle of a daymark.

## Non-uniform daymarks

One consequence of the limited eye resolution is the visual perception of a lattice construction or a 2-colour daymark.

As already stated the quality of a daymark for marine AtoNs depends on how it can be detected from a far distance. The daymark will appear smaller from far distance.

figure 19: Influence of viewing angle on non-uniform daymarks

The consequence is shown in the figure aboce for 3 different daymarks: a solid green square, a red lattice construction and a 2-colour daymark with red and green.

From a far distance the stripes of the lattice construction will melt together with the grey background. For the 2-colour daymark the red and green part will melt together as well.

In both cases the observed contrast at far distance will become the mean value over the whole daymark projection. For the lattice construction this will immediately reduce the contrast, e.g. if the lattice and the gap are of same size the contrast is half the value of a single lattice.

The colour of the daymark seen from far will be the mixture of the individual colour of the elements. For the lattice construction red and grey are mixed to an unsaturated red. Green and red produce a brown colour (dark yellow).

Concerning eye resolution the solid daymark will appear the same until it vanishes.

From this point of view a solid and single coloured daymark has the best visual performance (see also 5.5).

When illuminated by direct sunlight the 3-dimensional the surface elements of a daymark can show different luminance (figure 20).

figure 20: direct illumination of a daymark (flat, cone, cossed plates)

Whereas the flat daymark shows a uniform surface to the observer, the projected surfaces of the other designs break down into different regions with lower and higher luminance. Because the eye tends to prefer the brighter surfaces, the effective projeted surface is smaller for the cone.

## Shape recognition

A picture of the object (daymark) is projected at the fovea. In a simple model the sensors at the fovea (cones) have a distance of approx. 2,65 µm which leads to a resolution of 0.57' = 0.166 mrad (figure 21).

figure 21: simple fovea model

When the observer sees a simple daymark shape (cylinder, cone, sphere) the sensor elements are stimulated as shown in figure 22.

figure 22: stimulated cones for simple shapes

figure 22 illustrates the influence of the sensor density at fovea on shape recognition. The three simple IALA shapes can be safely identified when the shapes is projected into an array of 5 x 5 sensor elements.

So the minimum outer dimension of a simple shape projected at the fovea should have approx. 13.3 µm. Combined with the focal lenght of the eye (approx. 16 mm) the minimum value for shape identification is:

αshape≈arctan13.3 µm16 mm=0.0476°=2.86'≈3'

figure 23: recognition of simple shapes

As a rule of thumb the projection of a simple shape should appear at a minimum subtense angle of about 3' (= 0.873 mrad) to provide identification. For safety reason this value may be increased to 4 or 5'.

Some conventions about the daymark range DR of simple shapes are shown in the table below.

Table 4: Conventions on range and minimum viewing angle

| Object | Dimension | 1 (Brazil) | 2 (IALA 1983) | 3 (Germany) |
| --- | --- | --- | --- | --- |
| Cylindrical |  | 4,1' DR = 833 *B | 6,9' DR = 500*B | 3' DR = 1146*B |
| Spherical |  | 4,1' DR = 833 *B | 6,9' DR = 500*B | 3' DR = 1146*B |
| Conical |  | 2,75' DR = 1250 * B | 6,9' DR = 500*B | 3' DR = 1146*B |

1: Manual de Sinalização Náutica of the Brazilian lighthouse service [17]

2: IALA Maritime Buoyage system guidelines 1983 and IALA Navguide [10]

3: German paper 1987, [18]

## Required contrast

For a single object the contrast is defined by (see 6.3)

C=Lo−LbLb

Lo	Luminance of the object

Lb	Luminance of the background

Under ideal conditions a contrast of 0.02 is sufficient for the detection of a large black object before horizon sky (see IALA dictionary). For practical observation it was agreed to put the required minimum contrast to a value of 0.05.

For a large observation distance and for coloured daymarks not seen before the sky the required contrast needs to be much higher than 0.05.

## Blaise Theory

M. P. Blaise from the french administration published a theory in IALA bulletin April 1971. He stated that there are two different regions for the visibilty.

Angular subtense angle larger than 3' (0.8725 mrad): The visibility depends on the contrast only

Angular subtense angle smaller than 3' (0.8725 mrad):The visibilty depends on the product of the contrast and the apparent solid angle of the object

figure 24: Blaise theory

For a large distance d the solid angle is approximated by: Ω≈a∗bd2 .

To simplify calculation Blaise assumed a≈b so the solid angle is: Ω≈a∗b/d2 .

The viewing angle of the object is: α≈a/d (in radiant).

So Blaise theory can be expressed in the following way:

for α<0.8725 mrad	required contrast at the observers eye C>0.038∗10−6∗d2a∗b

for α>0.8725 mrad	required contrast at the observers eye C>0.05

Remark:

In the publication Blaise sets the limit for α to 1 mrad. However the formula for C is continous only for α = 0.8725 mrad = 3', so this value is chosen here.

## Colour recognition

The colour recognition depends on:

- the brightness of the colour

- the hue

- the contrast to the background

- the colour difference to the background

In most situations the background appears nearly grey, so the colour difference depends on the saturation of the daymark only.

Colour recognition is better when background is dark.

When the background is very bright (e.g. Sunset - sunrise) colour recognition is very poor.

The required hue and saturation are defined by the regions of the IALA-Recomendation E-108. These regions are based on considerations about the physiological features of the eye.

For large viewing distance and martime viewing condition it is found (see Blaise's article) that the Colours behave different. For example the most effective colours are red and orange whereas green and blue do not provide good colour recognition.

# DESIGN OF DAymarKS

## Floating Aids

For floating aids (mainly buoys) the following aspects have to be considered:

- arbitrary background (water, sky, landscape)

- standardization required (the buoy is not designed for a specific position)

- omnidirectional

- shape and colour recognition requiredSnow and ice accumulation.

### 3-dimensional Profile

The preferred 3-dimensional profile has rotational symmetry along the vertical axis (see 9), so the projected shape appears equal for nearly all psotitions around the buoy. Crossed plates and lattice construction can be used to reduce weight and cost, but perform less effective for visual perception.

### Colour

For buoys the background is arbitrary: it may be the water surface, the horizon sky or the landscape. It does not make sense to adjust the colour's brightness. Instead the buoys colours should be standardized where high saturated and bright colours should be preferred.

The standard may include two qualities for example ordinary colours for average areas, fluorescent colours to provide high conspicuity for difficult areas.

Standardized colour collections for buoy can be found in IALA - Rec. E-108.

### Size of single coloured buoys

As the contrast cannot be controlled there is no need to compensate contrast by size. According to Blaise theory the buoy should be made larger when contrast is not sufficient.

However Blaise theory does not deal with colour recognition. A coloured buoy may be seen even when it has contrast zero to its background.

For both aspects the minimum angular subtense of a single coloured buoy should be 3'. For safty reason a value of 4' or 5' may be chosen to define the daytime range and to calculate the required size of a buoy. It is accepted that the width of a daymark may be smaller than the height (see [13][18]).

In the next table some typical values for the size of the outline of buoys are shown. The operational range is only valid, if there is enough contrast or colour difference to the background

Table 5 - Typical values for the size of the visible part of a buoy

| Operational Range [m] | hight [m] |  |  | width [m] |  |
| --- | --- | --- | --- | --- | --- |
|  | 3' (0.873 m rad) | 4' (1.16 m rad) | 5' (1.45 m rad) | 1' (0.291 m rad) | 2' (0.582 m rad) |
| 1 000 | 0.87 | 1.16 | 1.45 | 0.29 | 0.58 |
| 2 000 | 1.75 | 2.32 | 2.91 | 0.58 | 1.16 |
| 3 000 | 2.62 | 3.48 | 4.36 | 0.87 | 1.75 |
| 4 000 | 3.49 | 4.64 | 5.82 | 1.16 | 2.33 |
| 5 000 | 4.37 | 5.80 | 7.27 | 1.45 | 2.91 |
| 10 000 | 8.73 | 11.64 | 14.54 | 2.91 | 5.82 |
| 15 000 | 13.09 | 17.45 | 21.82 | 4.36 | 8.72 |

### Cardinal-buoys

The recognition of a cardinal buoy is based on either the recognition of the yellow-black stripes or the top mark (two cones). In most cases the body of the buoy is larger than the topmark, so the range for the 'stripes' is longer than for the top marks.

As shown in 5.5 the daymark range of a 2-colour buoy is significally shorter than of a single colour buoy.

A typical design of cardinal buoy is shown in the next figure. The three stripes appear with different sizes according to the different diameters of the buoy.

figure 25: traditional cardinal buoys

When the observer moves away from the buoy the stripes become smaller and will disappear at the eye's resolution. Because they are different in size, each stripe has its own range. At these distances the buoy cannot be identified.

To prevent this situation the preferred cardinal buoy presents each stripe in the same size as shown in figure 26.

figure 26: preferred buoy design to optimize visual performance

Although the preferred design has many disadvantages considering mechanical and floating aspects, it has been established by some manufacturers.

The superstructure, which is the daymark of the buoy, should sit on the base body, which should show a light grey to avoid confusion with any colour.

figure 27: superstructure and base body

The influence on the buoy shape for cardinal marks was investigated by the french administration when they switched from a classical buoy shape to cylindrical shape (figure 28).

figure 28: Comparison of traditional and new generation buoy

It was confirmed by visual trials that the cylindrical shaped cardinal buoys increased the daymark range of the buoys significantly.

### Safe water and new-danger marks

In the IALA MBS these marks show vertical stripes. The number of stripes ist not fixed and there have been used 3 main arrangemants for the stripes.

figure 29: Vertical stripe arrangement on a safe water buoy

Whereas the arrangement '8 x 45°' is very conspicuous for short range, the width of the stripe should become as large as possible to enhance daymark range.

For the arrangement '4 x 90°' only one stripe is completely visible into one direction. For '6 x 60°' there is always a red and a white part visible. Regarding the daymark range the optimum arrangement seems to be in between the '4 x 90°' and '6 x 60°' solutions.

### Structural obstructions

A real buoy needs more equipment which might cause conflict with its daymark function for example an eyelet, screws, bolts, a ladder, a lantern or solar panels.

The task is to minimize the negative effect of all these additional parts.

This can be achieved with the following measures:

- As far as possible the additional parts should show the same colour as the buoy.

- When the position of the part may be moved, it should be moved to the top or to the lower end of the buoy. In general technical equipment will move to the top of the buoy.

- If the colour of the additional equipment can not be choosen to be the same as for the buoy it should be a light grey (e.g. RAL 7042).

figure 30: addional parts on a buoy

Solar panels are the most important structural obstruction on a light buoy. The colour of the solar panel is often a dark blue, which appears black from a far distance. There have been made investigation with coloured solar panels, but the efficiency of coloured solar panel is very low. If the low efficiency can be accepted, it is more attractive to accept smaller and high efficient solar panels in dark blue.

However the dark blue panels may corrupt the visual message from a cardinal buoy. For example if an arrangement of panels is set on the top of a cardinal south buoy, it may be appear as an east cardinal.

To avoid this the solar panels should be lowered to the black part of the cardinal buoy.

figure 31: Conflict between solar panels and cardinal marks

The next figure shows an example for a well designed cardinal buoy. The shape has been made cylindrical and the black and yellowstrips are of the same size. The solar panels are located in the black part of the buoy's daymark. The lower part is grey and is therefore neutral. To minimize the wind force a lattice construction is used.

figure 32: Optimized Cardinal buoy

Solar panels should not be distributed uniformly over the whole visible surface of a buoy, because the whole daymark is deteriorated. It is better to concentrate the panels at a minimum surface and provide as much compact and unhidden surface to achieve maximum daymark range.

figure 33: Distribution of solar panels on a safe water mark

### Buoy shape

Although the IALA Maritime Bouyage System does not give strict recommendations about the shape or profile of daymarks for floating aids, it is advisable to define specific shapes.

This will improve identification by the navigator as all buoys with the same meaning will show the same shape.

Since the establishment of the IALA buoyage system in 1980 there have been made many developments on buoys. For example, to improve the visual identification of lateral buoys, fins have been introduced to provide shape recognition. These panels should always show the same shape as that the topmarks provided by the IALA MBS.

This means, for example, that a conical buoy painted in red may never be used as a port-side lateral mark in region A.

figure 34: Buoys with fins, forming the silhouette corresponding to the type of mark

On the other hand, from observer eperience it is found out that svelte (tall, slim) buoy shapes may be more conspicuous in many cases.

### Topmarks

In general topmarks of buoys are sigificantly smaller than the buoy itself. So the daymark range of the topmark is shorter as well. This has led to the use of fins as discribed above.

However topmarks play an important role in confirming the AtoN’s message through the use of binoculars or when one is at a distance below the recognition distance of the AtoN as a whole.

Topmarks are optional on all kinds of buoys, but for cardinal and isolated danger marks IALA MBS strongly recommends the use of topmarks.

Buoys can be used in locations exposed to annual drifting ice, possibly of considerable thickness, that can destroy equipment mounted on the buoy body.  Use of additional daymark topmarks may not be feasible in these situations.

#### Shape

The shapes used in the IALA-MBS include cones, spheres, cylinders, X shapes and crosses (new danger mark). They can be made with the profiles shown in 5.1.

Where top-marks are used on buoys, it should be ensured that the topmarks do not deteriorate the floatability of the buoy.

#### Dimensions

The shapes should be as large as possible, taking into account the limitations on the element's stability and wind resistance, as it is situated at a great distance from the flotation line. The IALA Maritime Buoyage System Guidelines [11] describe the relationship between the base of the mark and the diameter of the buoy's hull on the flotation line (between 15% and 25% depending on the type of mark).

| CARDINAL MARKS | Cardinal topmarks N / E / S / W |
| --- | --- |
| LATERAL | Lateral topmarks (Region A) |
| SPECIAL AND NEW DANGER MARKS |  |
| SAFE WATER AND ISOLATED DANGER MARKS |  |

figure 35: Topmark proportions

## Fixed aids

Nearly all markings defined by IALA MBS for buoys, are used for fixed aids. So the information given above for buoys may be used for fixed aids.

However for floating aids there are a lot of design limitations caused by the floating features and maintenance aspects.

For fixed aids there is more freedom in the design of daymarks.

For example:

- A fixed daymark may be much larger and higher providing a long daymark range.

- It may be flat.

- It may be designed for a specific background.

- It may have a background panel to show more contrast.

- The supporting structure of a daymark may become part of the daymark or be used to enhance the conspicuity (figure 36).

figure 36: Supporting structure used as a daymark

### Traditional light houses

The use of red and white stripes is a traditional method to increase the conspicuity of light houses as a daymark. Regarding the daymark range the buildings should have a single colour to make the range as large as possible. The colour combination red-white is used to enhance conspicuity while accepting a reduced daymark range.

Because of its high conspicuity the colour combination red-white is widely used for aeronautical daymarks and road traffic signs as well.

figure 37: Red-white horizontal stripes

A colour combination can also be used to identify a daymark. For this reason special geometrical design (e.g. rhombus) or a different colour combination (green-white) can be used.

figure 38: Rhombus design for the daymark of a light house

Some colour combinations should not be used arbitrarily. This is because the IALA MBS has already defined some colour combinations for a specific nautical information.

These are:

- Green-red horizontal bands for channel division

- Black-white horizontal bands for isolated danger marks

- Black-yellow horizontal bands for cardinal marks

- White-Red vertical stripes for safe water marks

- Blue-yellow vertical stripes for new-danger marks

These colour combination should be used for fixed aids only if the daymark has the same nautical meaning as the corresponding buoy.

### Additional tools for buildings used as daymarks

In order to facilitate colour recognition, mainly in the case of lateral marks, it can be useful that the lower part of the building is painted white. The contrast with the surrounding and the upper part of the building, which shows the colour, can be enhanced.

figure 39: White contrast painting for fixed lateral daymark buildings

Only the dimensions of the part painted in the colour corresponding to the type of signal should be used to calculate a daymark's useful surface area.

The installation of a topmark or a specific shape for the building corresponding to the IALA MBS greatly enhances the recognition of the daymark. Shape recognition is very important at backlight situations at certain times of the day, because in backlight situations the colour recognition is poor (see next figure).

figure 40: influence of the sun position on colour and shape recognition

### Dayboard of leading lines

The daymarks of leading lines are described in IALA Guideline 1023 [5]. Because leading lines are used in a small horizontal sector only, the daymark should be flat.The guideline proposes a dayboard with white and red vertical stripes (W = L/2).

figure 41: Geometry of a leading line dayboard

Depending on the vertical height (Length) of the daymarks the guideline defines an operational range. The operational range is therefore the daymarks range for this specific daymark layout. For good viewing conditions (meteorological visibility 10 M) the operational range depends on the size (L, length) of the dayboard only.

Table 6: Dayboard size and range

| Dayboard Size L x W [m x m] | Operation Range [km] | Viewing angle for length L | Viewing angle for width W (= L / 2) |
| --- | --- | --- | --- |
| 1.6 x 0.8 | 1.9 | 2,9' | 1,5' |
| 2.1 x 1.05 | 3.7 | 2,0' | 1,0' |
| 3.1 x 1.55 | 5.6 | 1,9' | 1,0' |
| 4.2 x 2.1 | 7.4 | 2,0' | 1,0' |
| 6.3 x 3.15 | 9.3 | 2,3' | 1,2' |
| 8.6 x 4.3 | 11.1 | 2,7' | 1,3' |
| 12.2 x 6.1 | 13.0 | 3,2' | 1,6' |

The viewing angles of the vertical and horizontal size of the dayboard at maximum range can be compared to the values in Table 4. Because of the long range required and the resulting large dayboards the accepted minimum viewing angles are smaller than for buoy topmarks.

However the dayboard of a leading line does not need shape recognition. At great distance the observe will see the dayboard as a small red vertical bar. The white stripe may disappear because it is already smaller than the eye resolution.

### Marking of bridges

Another example of a flat daymark is the marking of bridges as defined in IALA Recommendation O-113. To increase the contrast between the sign and the background a white background panel is recommended.

figure 42: Daymarks defined by IALA-Rec. O-113

From visual perception a white background panel does not increase the daymark range but makes the signs more conspicuous.

figure 43: Preferred daymark design

### Design of signs

The aspect of the limited eye resolution should be taken into account when designing a sign.

Example 1:

The IALA recommendation O-113 on the marking of bridges recommends a circle with white and red vertical stripes. It does not define the exact picture of the daymark and the number of stripes, so both signs shown in the next figure are valid.

figure 44: Sign: best point of passage

Regarding the eye resolution the sign on the right shows very small stripes so the daymark range will be significantly shorte than for the left one. It is recommended to use the left design. If a small range can be accepted the left sign can be made smaller than the right sign.

Example 2:

A direction arrow is used at a channel division, when one channel is closed, to show the mandatory direction. figure 45 shows two designs of a direction arrow. The arrowhead of the left design is very small and will become invisible from a short viewing distance. The information (direction) is lost because the observer will see a black horizontal bar at far distance.

The right design has an arrowhead which is more than two times larger. This will increase the daymark range by a factor more than two, so this design should be preferred.

figure 45: Direction arrow

### Canadian daymarks

The Canadian coast guard uses flat day marks on fixed aids to navigation. These flat day marks are defined in an operational directive. For the lateral marks the following daymarks are specified.

figure 46: Canadian standard daybeacon

The shapes of the daymarks are in accordance with the IALA MBS. The colour in the centre is fluorescent red or green. For the port hand day beacon a black square may be used as an alternative.

The signs have a white contrast panel, which is surrounded by a small tape of retroreflective material in red or green.

The Canadian gives information about the the useful range (design distance) for a specific size of the central shape.

figure 47: Relevant dimensions of the daymark

Table 7: Design distance for Canadian daybeacons

| Type | Dimension | Design distance | Viewing angle at design distance |
| --- | --- | --- | --- |
| Port | SG = 0.3 m | 1000 m | 1' |
|  | SG = 0.45 m | 1500 m | 1' |
|  | SG = 0.9 m | 3000 m | 1' |
| Starboard | SR = 0.4 m | 1000 m | 1.4' |
|  | SR = 0.6 m | 1500 m | 1.4' |
|  | SR = 1.2 m | 3000 m | 1.4' |

From the table above it can be derived that there will not be shape recognition of the central central shape at design distance, because the viewing angle is near the eye resolution (see 7.2). If the dayboard size were choosen to provide a viewing angle of 3', they should be 3 times larger than described. This would cause very high costs especially for the supporting structures and the measures taken against the wind force.

However the Canadian directive is a very good example how to handle this handicap.

The recognition is improved by:

- using a white contrast panel

- using fluorescent colour

- using black instead of green for port sign.

Therefore, at large observation distances the mariner will identify the beacon by colour and not by shape. Because green cannot be identified as good as red colour from far distance, the use of a black square instead may be more effective.

## General Design Methodology

In the following a procedure is presented for finding an appropriate design for the required daymark.

The first step is to establish the parameters to use as input. In several steps the design (size, profile, colour) can be derived.

Input:

- the type of daymark (MBS, buoy, fixed, …)

- Purpose and required range

- for fixed AtoNs: arc (horizontal sector of utilisation: Omnidirectional / unidirectional)

- background: shore (albedo), sky, watersurface, snow, ice

Step 1:

- Position and Height above sea level (see section 6.1)

- Remark: For some daymarks the position cannot be changed.

Step 2:

- Minimum dimension for the different shapes (Buoy size, top mark, boards)

- Remark: A convention about the size must exists according to section 8.1.3

Step 3:

- Choose a profile according to Section 5.1

- Flat, rotational symmetry, crossed plates, lattice construction

Step 4:

- Choose a colour according to existing recommendations, guidelines and standards (maritime buoyage system, leading lines, dayboards, ordinary colour, fluorescent colour, combination of colours)

- Establish the required contrast and size of the coloured stripes of a two coloured daymark

- Establish the required contrast of a daymark to the background

Step 5:

- Consider further design aspects (e.g. hydrostatic design of buoys, safe access for maintenance, cost, supporting structure, wind load, wave force)

- Check for Practical Knowledge

As a result of step 5 it might be established that the design of the daymark is too expensive or is not suitable considering wind load, wave force or similar aspects. In these situations the input parameters should be checked and changed. With the new input the steps should be repeated. In general if the required range cannot be achieved it is recommended to use smaller daymarks at shorter distances.

EXAMPLE 1:

Standard lateral buoy with a daymark range of 2 M

Step 1:

Position is given by nautical requirements only

Height (geographical range) can be neglected from section  6.1

Step 2:

Size of the buoy for a convention 3’ vertical and 1’ horizontal

Size: H x W = 3.2 m x 1.07 m

H = 2 x 1852m x tan (3’)

W = 2 x 1852m x tan(1’)

Required horizontal sector 360° (omnidirectional)

Step 3:→

Preferred 3-dimensional profile: compact shape with rotational symmetry (see 5.1.2)

Step 4:

Ordinary Red or Green

Coloured plastic (non-fluorescent)

Step 5:

Manufacturing: roto-moulding polyethylene

Confirm the acceptability of dimensions with other tools (e.g. hydrostatic or wind load

calculations)

EXAMPLE 2:

Lateral daymarks for port entrance, fixed, with a daymark range of 5 M

Minimum observers height above sealevel: 1 m

Colour recognition only

Step 1:

Fixed Position is given by nautical requirements only (geographical coordinate)

Height (geographical range) can be neglected from section 6.1

Rg=2.03×ho+hb,min h0=Rg2.03−hb,min2=2.1m

(lowest part of the daymark above sealevel)

Step 2:

Size of the daymark for a convention 3’ vertical and 1’ horizontal

Size: H x W = 8.1 m x 2.7 m

H = 5 x 1852m x tan (3’)

W = 5 x 1852m x tan(1’)

Required horizontal sector 360° (omnidirectional)

Step 3:→

Preferred 3-dimensional profile: compact cylinder (see 5.1.2)

Step 4:

Ordinary Red or Green

Painting on steel construction

Painting of the lower part of the daymark inwhite to increase conspicuity

Step 5:

Confirm the acceptability of dimensions with other tools (e.g. wind load

calculations)

## Additional information and advices (Super Structures)

The dimensions needed for the daytime part of the signal will condition the structure's design and affect the following aspects:

- Snow and ice accumulation of snow and ice on buoys should be taken into consideration when designing buoys for areas where such conditions are common. Using steep angles, smooth surfaces and few projecting objects such as top-marks can reduce the effects.

When passing short range daymarks: minimize or avoid damage on ships when collision occurs.

- Special care should be taken when dealing with the daymarks of signals marking out harbour mouths. Their size should be suitable to provide a sufficient recognition distance for the size of the ships to which they are meant to provide service.

- Where the signal is comprised of a post supporting equipment, it should be of a sufficient diameter (never less than 400 mm), so that it has a suitable recognition distance once it has been painted in the colour of the corresponding mark.

- The mark's height should be enough for it to have a sufficient recognition distance in addition to reaching the necessary geographic range. It should be noted that the height of where the mark is situated and not the structure's height should be taken into consideration. Additionally, any possible obstacles that may hinder the mark's visibility – like vehicles parked in front of the signal, road traffic, vegetation, vessels, etc. – and reduce the mark's effective size should be foreseen.

- Where it is not possible to achieve a sufficient diameter in the post or spar, a board or boards should be affixed thereto to make them look like a solid body at a certain distance. Structures to support equipment may also be designed with handrails or guardrails made up of panels or boards to increase the visible surface area.

- Rectangular panels painted in the signal's specific colour should be affixed onto latticework structures with small cross-section profiles which cannot achieve a sufficient painted surface area, so that they appear like a solid body at a distance.

- In the case of marks painted in several colours, the calculation should be made by taking each colour individually and the surface area's recognition distance will be that of the most restrictive colour. This means that marks having several colours (cardinal marks, modified lateral marks, etc.) should be larger for the same recognition distance.

- Where the message is comprised of several colours, an effort should be made to ensure each colour's visible surface area has a similar size in keeping with recommended proportions. Due to this, an effort should be made not to use buoy hulls as part of the message. The colours forming the message should preferably be painted solely on the pillar or spar. Should it be necessary to use the buoy's hull as part of the message, its painted surface area should be of a similar size as the other surface areas comprising the message.

This only applies to signals whose message is made of horizontal bands of different colours, as is the case of cardinal marks, modified lateral marks and isolated danger marks. This will not apply to all other cases (marks having a single colour or vertical stripes, such as special lateral marks, safe water marks and wreck marks).

- Where the signal will only be used in a single direction (as in the case of leading lines), the installation of dayboards may be sufficient. However, where it will be seen from any direction, the mark should project the same surface area in all directions.

- Wind force should be taken into account for dayboard structures having large surface areas. In order to reduce wind resistance, panels made up of boards having gaps between them can be built to let the wind through. Additionally, if these are slanted as per the drawing below, their wind resistance will be reduced.

| Figure 1 | Figure 2 |
| --- | --- |

- Over the last few years, there has been a trend of replacing traditional leading lines with directional lights that provide daytime service. Wherever possible, it is advisable to maintain the old leading line's front and rear daymarks for this kind of signal even when the light is used during the day. Where this is not possible, the daymark on which the directional light is installed should be of a sufficient size to ensure the navigator can easily identify the place where the light is located at daytime.

- IALA Guideline 1023 on the Design of Leading Lines. The dayboard should be painted with a white area or another colour which offers an adequate contrast, so that an appropriate alignment between the two marks is ensured.

- There are no standards defining the colours of these boards, but it should be ensured that they offer the maximum possible contrast with the background. Draughtboard patterns and many stripes and bands are not advisable, as these reduce each colour's individual surface area and therefore its recognition distance. The upper part of the dayboard should be slanted by around 4º or 5º in order to avoid bird depositions falling on the dayboard's surface. Where possible, this practice should also be extended to include other kinds of daymarks.

# Factors that reduce visual range

## Factors that reduce detection

- For size of the topmarks the considerations are presented in NAVGUIDE 2006 edition, at paragraph 3.1.13.6.2.

- Complex background and meteorological parameters (sunset or sundown, atmospheric transmissivity, ...) can modify detection with time

- For dimensioning the identification markings it is necessary to consider the requirements posed by the authority and the importance of it for the mariner.

Example:

  - An inland special mark signalling a danger, where his identification gives no extra information to the mariners does need large characters.

  - A special mark identifying a forbidden area, with a no entry sign, this marking should be visible in an adequate distance.

- The wave height / frequency in open water will have detrimental effects in range due to the following factors:

  - Variation in the visible area of the daymark, due to the pitching of the floating aid;

  - Changes in the shape due to the rolling effect;

  - Reduction in the exposure time of the floating aids due to the relative position with the observer, which can be obstructed by the crest of the wave.

## Service factors

- Dirt in the form of green slime, bird droppings, green mould will make the visible identification of colour difficult or impossible.  Where the top of the floatation bowl forms part of the daymark, sufficient freeboard should be provided to ensure that the top of the bowl is not regularly submerged encouraging development of green slime.  In critical areas it may be necessary to frequently wash the buoy.

- Fading is a result of the attack by ultra violet (UV) light present in sunlight and buoy daymarks are particularly exposed to this type of degradation, having no form of shade.  Selection of the type of paint system or plastic material should be made for maximum UV stability.  Service providers should carry out periodic tests to confirm that the daymark colour remains within IALA recommendation.  It may be necessary to increase the frequency of painting in order to ensure that the daymark colour maintains its performance within IALA recommendation.

- Theft, interference and tying up to buoys can have serious effects on the provision of buoy systems.  The use of security locking systems can inhibit casual theft but absolute prevention of theft is impossible.  Deliberate malicious interference can be dealt with through the legal system if the perpetrator can be identified.  The use of buoys by fishing vessels as a temporary mooring should be discouraged through public information system and regular contact with the fishing community and local authorities.  A remote monitoring system can provide early warning to the Service Provider in the event of damage to the electrical systems.

## Environmental limitations

Local environmental conditions can give rise to the need for special design of daymarks which may not conform to the general guidance in this document.  Such conditions include drifting ice, debris in rivers, buoys in breaking wave conditions.

Buoys located in areas exposed to annual drifting ice, possibly of considerable thickness, that can destroy equipment mounted on the buoy body.  Buoys in such areas must generally be designed with a conical body with a minimum of projections and be completely sealed.  Use of additional daymark boards or topmark may not be feasible.  Consequently the use of can shape for lateral buoys is not always possible.

Debris in rivers, such as trees, logs and even whole floating islands can inflict damage to a buoy similar to that in drifting ice.

Where a buoy system is deployed in areas of breaking waves or drying out during low tide, the buoy design must be adapted to ensure survivability of the buoy and its equipment and the design may not conform to the general format.

Bird droppings can seriously affect the performance of buoy systems as well as obscuring the colour of the daymark.  Droppings on lantern or solar panels will cause reduced performance or failure of the light.  The corrosive effects of bird droppings or acid rain can cause damage leading to reduced life or premature failure.  Daymark design must therefore be such as to avoid providing any natural bird resting sites.  The use of long flexible spikes on all potential land sites can be helpful.  However care must be taken to ensure that these do not create a hazard for people attending the buoy.

- References

- IALA Maritime Buoyage System and other Aids to Navigation

- Signalisation maritime Documentation technique, French lighthouse technical documentation: CETMEF

- CIE110 Spatial distribution of daylight - Luminance distribution of various reference skies

- IALA Conference Presentation 1990 about lettering and signs, 'Studies on the recognizability of symbols and letterings on aids to navigation, Dr. Gerdes, Federal Rebublic of Germany.

- IALA Guideline 1023 The Design Of Leading Lines Edition 1.1 December 2005

- Theorie der horizontalen Sichtweite, Theory of horizontal visual range, Koschmieder, Meteorologische Zeitschrift 11/1926

- Farbe und Sichtbarkeit, Colour and visibility Hoffmann, DFVLR, 1977, Germany

- M. Blaise, IALA Bulletin No. 47 (April, 1971) article Titled “Daymarks as Aids to Navigation.”

- IALA Recommendation E-200 On Marine Signal Lights

- IALA NAVGUIDE 1993, Annex 2

- IALA Maritime Buoyage System Guidelines 1983

- IALA Recommendation E-108 on Surface Colour

- Standardization of U.S. coast Guard Leading Lines, IALA Conference 1998, Engineering

- Grundlagen der Lichttechnik, Fundamentals of light technology: Dietrich Gall, Germany

- Wikipedia article 'Albedo', 2011-08-16

- Libro de Normas Técnicas of 1986 . Ministerio de Obras Públicas y Urbanismo. Spain

- Manual de Sinalização Náutica of the Brazilian lighthouse service.

- "Über den Einfluss der Komponenten des Wasserverkehrssystem auf die Sicherheit des Seeschiffsverkehrs", University of Hanover, Germany

- IALA Recommendation On The Marking of Fixed Bridges over Navigable Waters, 1998

- Basic Guidelines for the design of visual marine aid to navigation daymarks, Puertos del Estado, Spain, 2010