EEP18	working paper

Agenda item	10

Task Number	102

Author(s)	J. Lindberg

Review of sections 3.1 and 3.2 of the NAVGUIDE

The Navguide have gone through a revision in 2010 and the overall impression is that the document is up to date. Some minor recommended changes are listed below.

3.1.2 Visibility of a Mark

Reference to the following IALA documents could be included:

E-106 ON the use of Retro reflector material…

E-108 On the Surface Colour….

3.1.4 Atmospheric  Transmissivity

The transmissivity factor typically used in regions where the atmosphere is very clear is T = 0.86, not T= 0.84.

3.2.2 Electric light

The LED chapter is updated with latest figures and attached as ANNEX A in this document. All changes are highlighted.

3.2.3 Photometry of Marine Aids to Navigation Signal Lights

In section 3.1.1 Signal Colours there is a figure of the IALA surface colours but the corresponding figure with the IALA colour boundaries for emitted light is not found anywhere in the document. It will be included in section 3.2.3.

3.2.6 Floating Aids To Navigation

Reference to IALA Guideline “1056  On Aids to Navigation Signal Light Beam Vertical Divergence” could be included in the end of the section.

ANNEX A

3.2.2 Electric Lights

Light Emitting Diode (LED)

Description of operation:

Coloured LEDs

• LEDs are electronic semiconductor devices that produce near monochromatic light. The semiconductor junction is encapsulated in clear plastic housing that usually incorporates a lens. Several LEDs may be grouped together in a cluster or an array to provide a light source of the required size and intensity with lamp redundancy. New high power LEDs allow short range lanterns using a single LED.

• LEDs operate from a low voltage DC supply. Correct operation depends on accurate control of the supply current.

White LEDs

• A semiconductor junction emitting blue/violet light is encapsulated with an integral phosphor such that both blue and broad band yellow light are emitted together to form a near white light. Research is in progress on combining red and green LED lights to produce a white light within the IALA chromaticity specification.

• LED marine lanterns are sometimes reported as having intense colours and ranges longer than the current IALA calculation method would suggest. Current work by IALA is investigating this.

Typical use:

• Lighted beacons on buoys and other short and medium range AtoN, but also long range LED lanterns are increasingly available in the market.

• Range lights consisting of flat arrays of LEDs or single high power LEDs.

• Signs and signals formed by arrays of LEDs in the shape of letters, numerals, signs etc.

Technical data:

• Power: Single LED 1mW to over 32W

LED cluster in lantern 1W to 60W or even higher.

• Luminous efficacy (without power supply) Efficiency:

This depends on colour: Red and green – 25 to 60 lumen/W

Yellow – 20 to 70 lumen/W

White – 20 to 90 lumen/W

Blue – 10 to 20 lumen/W

• Operating life: Depends on the LED junction operating temperature and operating environment

Possible values in excess of 100,000 hours for a single diode

Advantages:

• Very long life (so long as input power and temperature is carefully controlled) and hence low whole life costs.

• Life is so long that lampchangers are not considered necessary.

• High luminous efficiency in red and green.

• Light produced in saturated signal colours therefore coloured filters not needed.

• Mechanically robust when compared with conventional lamps.

• Rise and fall times are very fast.

• Relatively cool operation.

• Ease of ability to cluster LEDs

Disadvantages:

• Complex electronic control needed to achieve long life and high performance.

• Generally difficult to match to existing optics.

• Luminous efficiency decreases slowly with life.

• White LEDs will be outside the new CIE (2001) white colour region.

• White LEDs will be very inefficient with red and green filters.

• Lamp life can be severely reduced if input power and temperature are not carefully controlled.

• LED technology is not yet suitable for long range lights.

Safety:

• No special hazard.

Disposal:

• Consult National and local regulations regarding disposal of semiconductors.

Note: Extensive research in LED technology is continually providing more LED efficient products