EEP18	Input paper

Agenda item	8.3

Task Number	11

Author(s)	Aivar Usk

Review of US Military Standard MIL-STD-3034 “RELIABILITY-CENTERED MAINTENANCE (RCM) PROCESS”

# Summary

At EEP17 the author was given action 45 to review the US military standard MIL-STD-3034 “RELIABILITY-CENTERED MAINTENANCE (RCM) PROCESS” inter-sessionally for relevance to IALA requirements for Risk Assessment Techniques in AtoN Design and maintenance (Activity 11.1.1, Task 11*) and to prepare an Input Paper for EEP18. This paper provides a short overview of MIL-STD-3034 from the aspect of identifying content relevant to maintenance planning.

## Purpose of the document

This input paper is intended for informing the EEP in regard to existing guidelines for risk assessment techniques in equipment maintenance contained in the MIL-STF-3034 that could possibly be included either directly or by reference when preparing a corresponding IALA Guideline for Risk Assessment Techniques in AtoN Design and maintenance.

## Related documents

United States military standard MIL-STD-3034 “RELIABILITY-CENTERED MAINTENANCE (RCM) PROCESS”, 21 January 2011, 64 pages; approved for use by the Department of the Navy.

# Background

It is a good practice to peek into existing relevant standards of a domain before drafting a new IALA Guideline in order to utilize the prior knowledge as far as possible. The US military standards have been prepared by domain experts with long and deep theoretical and practical experience, presenting an excellent publicly available free source of structured information on various aspects of engineering, operation, maintenance, etc of mission critical equipment and systems.

# Discussion

A short review of MIL-STD-3034 is provided below.

## Scope of the MIL-STD-3034

MIL-STD-3034 is created for application in the US Navy, but the central concept could be re-used in any other operation relying on systems and equipment of high reliability requirements.

Paragraph 1.1 of the standard states: “This standard describes the Reliability-Centered Maintenance (RCM) methodology used for the determination of maintenance requirements. It applies to all levels of system or equipment grouping, and to all scheduled maintenance, whether equipment is in use, ready for use, or in standby or lay-up condition. RCM addresses the total scheduled maintenance program for an enterprise, irrespective of the maintenance echelon possessing the capability to perform the maintenance; that is, organizational, intermediate, and depot level maintenance. This standard provides the procedure to develop preventive, corrective, and inactive equipment maintenance (see 3.13.1, 3.13.2, and 3.13.5) within a planned maintenance management system.”

The standard provides a well structured set of definitions and requirements. Maintenance is an action of performing tasks at periodicities (periodic, situational, and unscheduled) to ensure the item’s functions are available until the next scheduled maintenance period. Maintenance is either preventive, corrective, or inactive (see 3.13.1, 3.13.2, and 3.13.5 of the standard).

Maintenance tasks are either time-directed, condition-directed, failure-finding, servicing, or lubrication (see 3.20.2, 3.20.5, 3.20.6, 3.20.7, and 3.20.8 of the standard). Failure modes and effects analysis (FMEA) is an important part of the RCM process, applied for iterative adaptation of the maintenance schedules based on demonstrated availabiltiy.

Item’s functions can be either active, passive, evident, or hidden (see 3.11.1, 3.11.2, 3.11.3, and 3.11.4 of the standard).

## Basic concept of the Reliability-Centered Maintenance (RCM) method

RCM is the process used to determine the maintenance requirements for new and in-service equipment or systems. The initial maintenance package provides the baseline for system expectations. As operational experience is developed (failure data, inspection results, etc.), the maintenance package will be monitored and improved as required.

The process is composed of up to twelve phases (see 5.1 of the standard):

a. Phase 1 – System partitioning and functional block diagram (FBD). Partitioning along major system and subsystem boundaries to facilitate analysis and specify analysis boundaries (scope) and approach (see 5.1.1).

b. Phase 2 – Functional failure analysis (FFA). Analysis of the functions of systems and subsystems and of the ways in which those functions can fail (see 5.1.2).

c. Phase 3 – Additional functionally significant item selection (AFSI). Selection of the additional functionally significant items (AFSIs) (see 5.1.3).

d. Phase 4 – Failure modes and effects analysis (FMEA). Analysis of the failure modes and effects of failure (see 5.1.4) of the FSIs.

e. Phase 5 – Decision logic tree analysis (LTA) (see 5.1.5).

f. Phase 6 – Servicing and lubrication analysis. Analysis of servicing and lubrication task requirements (see 5.1.6).

g. Phase 7 – Inactive equipment maintenance (IEM) task identification (see 5.1.7).

h. Phase 8 – Corrective maintenance task identification (see 5.1.8).

i. Phase 9 – Maintenance requirements index (MRI) (see 5.1.9).

j. Phase 10 – Maintenance requirement task definition (see 5.1.10).

k. Phase 11 – Maintenance procedure validation (see 5.1.11).

l. Phase 12 – Maintenance requirement card (MRC) and maintenance index page (MIP). Development and preparation of MRCs and formulation into MIPs (see 5.1.12).

Allthough a classical risk impact / probability matrix is not used, risk (criticality) analysis is actually described in Phase 4 -  Failure modes and effects analysis (FMEA, 5.1.4) and Phase 5 – Decision logic tree analysis (LTA, 5.1.5), resulting in failure criticality categorization into safety critical, mission critical, other regular, and hidden or infrequent functions (correspondingly A, B, C, or D). The RCM decision logic tree is provided in Figure 4 (page 20).

It is understood that a “development hierarchy” is established either in close cooperation with, or even by the developer of the product or system by partitioning it into several hierarchical levels. Failure modes are defined for each level. Implementation of such detailed analysis in non-military organization with limited resources may be rather costly.

Detailed guidance for documenting is provided, understandably US Navy specific; data collection forms and templates are provided. Useful guidelines are provided for documentation of maintenance procedures (see (s) on page 36).

## Conclusions

There is no doubt that MIL-STD-3034 is a useful reading material for anyone involved in drafting of maintenance policies or management systems; it is a topic for discussion whether it would be  feasible to recommend establishing a system with similar detailed requirements to the AtoN administrations.

General risk assessment guidelines are provided in the standard ISO 31000:2009 „Risk management - Principles and guidelines“. Risk analysis for technical systems from the aspect of safety is also described in IEC 61508 series („Functional safety of electrical/ electronic/ programmable electronic safety-related systems“) as well as in railway-specific standards like IEC 50126 and 50129.

# References

- United States military standard MIL-STD-3034 “RELIABILITY-CENTERED MAINTENANCE (RCM) PROCESS”, 21 January 2011.

# Action requested of the Committee

The Committee is requested to:

- Forward the document for review within WG1 for analysis and decision whether the concepts in MIL-STD-3034 can be re-used as an example in technology-related risk assessment at drafting an IALA Guideline for Risk Assessment Techniques in AtoN Design and maintenance.

MIL-STD-3034 “RELIABILITY-CENTERED MAINTENANCE (RCM) PROCESS”