AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

10 Rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  contact@iala-aism.org       Internet:  www.iala-aism.org

IALA Guideline No. ####

On

Guideline on the Application of AIS on Buoys

Edition 1

[Date issued]

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| 31/3/2011 | New document |  |
| 10/10/2011 | First revision |  |
| 17/04/2012 | Second revision |  |

Document Revisions	2

Index of Tables	4

Index of Figures	4

Guideline on the Application of AIS on Buoys	6

1	Introduction	6

2	Selection of the AIS unit	6

2.1	Messages	6

2.1.1	Message 6	6

2.1.2	Message 8	6

2.1.3	Message 21	7

2.1.4	Message 25	7

2.1.5	Message 26	7

2.1.6	Messages 12, 14 , 25 and 26	7

2.2	AIS AtoN type	7

2.2.1	Type 1	7

2.2.2	Type 2	7

2.2.3	Type 3	7

3	Primary considerations	8

3.1	Power consumption.	8

3.1.1	Special considerations between Type1 or 3? – Inter-session work – EEP 19	8

3.2	Transmission range.	8

3.2.1	Extraordinary coverage – Ducts or Tropospheric Refraction	9

3.2.2	Selection of VHF antenna	10

3.2.3	Comparison of different marine antenna	10

3.3	Additional services.	12

3.3.1	Message 8	12

3.3.2	Message 21  Regional bits - Monitoring capability (battery voltage, light status etc).	12

3.3.3	Satellite monitoring of AIS	12

3.4	Integrated AIS Units.	12

3.4.1	Integrated unit with 3rd party equipment	12

3.4.2	Integrated with Lantern and power system	12

3.5	Licensing by local licensing authority	13

3.5.1	ITU unification MMSI on web page – Inter-session work – EEP 19 - TRABAJADO	13

4	Physical Application	13

4.1	IP rating	13

4.1.1	Issues	13

4.1.2	IP enclosure rating	14

4.1.3	Pressure balance	14

4.2	Protection	14

4.2.1	Lightning.	14

4.2.2	Grounding	14

4.2.3	Physical damage, mounting against shock loading & vibration	14

4.3	Location of VHF antenna and GPS antenna receivers.	14

4.3.1	VHF antenna	14

4.3.2	GPS Antenna Receiver	15

5	Commissioning & Testing – To review at EEP19 October 2012	15

5.1	Range testing – Inter-session work – EEP 19	15

5.1.1	EQUIPMENT USED:	15

5.1.2	MEASURING INSTRUMENTS:	16

5.1.3	TESTING METHODOLOGY	16

5.1.4	Transmit power measurement	16

5.1.5	Data, curves, images and graphs obtained. Observations.	17

5.1.6	Conclusions	20

5.2	Frequency verification – Inter-session work – EEP 19	20

6	MaintenANCE & oPERATION	20

6.1	Programming	20

6.2	Spare transponders	21

6.3	Maintenance requirements – Inter-session work – EEP 19 – Done it	21

6.3.1	Technical Staff	21

6.3.2	Configuration Software	22

6.4	Training – Inter-session work – EEP 19 - TRABAJADO	23

6.4.1	Capabilities required for lanternists. (Maintainers)	23

6.4.2	Human, physical and technical restrictions on board.	23

6.4.3	Training documentation	23

7	Selection of equipment.	23

7.1	Issues to consider	23

8	General Considerations for integrating an ais-AtoN System	24

8.1	Impact on Buoy Performance	24

8.2	General Assembly	24

8.2.1	Hard wired or plug connections.	24

8.2.2	Preparation of inlets in watertight enclosures	25

8.2.3	Assembly of cable glands	25

8.2.4	Preparation of the AIS System	26

8.2.5	Sealing of external connections	27

8.2.6	Connections and Completion	28

8.3	General outlines for the assembly	28

9	Glossary of terms	29

10	rEFERENCES	30

Index of Tables

Table 1	Summary of optional AIS AtoN Station messages	6

Table 2	Spare transponders	21

Table 3	List of necessary components for installations	29

Table 4	Sample checklist	30

Index of Figures

Figure 1	Cross-section of the geoids	8

Figure 2	Normal situation – Reception coverage ≈ 35 km	9

Figure 3	Situation with ducts – Reception coverage ≈ 200 km	10

Figure 4	Measurement 1	11

Figure 5	Measurement 2	11

Figure 6	Measurement 3	11

Figure 7	Collision effect	15

Figure 8	Showing basic connection between systems	15

Figure 9	Measurement of power transmitted via direct connection through cable and signal attenuation.	16

Figure 10	Consumption [uA] vs. Samples obtained. Transponder A. Samples taken every 12 secs.	17

Figure 11	Consumption [A] vs. Samples obtained. Transponder A. Samples taken every 1,2 secs.	17

Figure 12	Consumption [A] vs. Samples obtained. Transponder B. Samples taken every 12 msecs.	18

Figure 13	Consumption [A] vs. Samples obtained. Transponder B (See table attached). Samples taken every 1,2  msecs.	18

Figure 14	Trial moment for Transponder B, AIS receiver connected to a notebook computer via RS-232, Multmeter for measuring and registering current, and cascaded attenuators for the transmitted signal to be displayed on the Spectrum analyzer.	18

Figure 15	Spectrum analyzer displaying transmission spectrum.	19

Figure 16	Spectrum of Transponder A.	19

Figure 17	Spectrum of Transponder B.	20

Figure 18	Fully assembled enclosure	25

Figure 19	Different sides of the perforated box	25

Figure 20	Different diameters of heat shrink tubes	26

Figure 21	Equipment with its protection	26

Figure 22	Complete Mixed IP Box	27

Figure 23	Sealing of antenna and power supply inputs.	27

Figure 24	Sealed cable gland with heat shrink	28

Guideline on the Application of AIS on Buoys

# Introduction

This document considers the application of employing AIS on buoys and is designed to offer guidance regarding specification, installation and maintenance.  This document should be considered as complimentary to higher level documents such as IALA Recommendation A-126 Edition 1.5

# Selection of the AIS unit

AIS has the capability of transmitting various messages.  The comprehensive list of messages available is in the IALA Recommendation A-126 Edition 1.5. The main messages that are of interest to the AIS as an AtoN provider are as follows;

## Messages

In addition to Aids to Navigation Report, Message 21, an AIS AtoN may also transmit Messages 6, 7, 8, 12, 13, 14, and 25. Note that Type 1 and Type 2 AIS AtoN stations, not having full AIS receiver capability, cannot send Messages 7 or 13.

| Msg ID | Message Name | Message Description | Application examples |
| --- | --- | --- | --- |
| 6 | Binary Addressed Message | Binary data for addressed Communication | Monitoring of AtoN lantern, power supply, etc. |
| 7 | Binary acknowledge message | Acknowledge of addressed binary message |  |
| 8 | Binary Broadcast Message | Binary data for broadcast communication | Meteorological and hydrological data |
| 12 | Addressed Safety Related Message | Safety related data for addressed communication | Warn AtoN malfunctioning |
| 13 | Safety related acknowledge message | Acknowledge of addressed safety related message |  |
| 14 | Broadcast Safety Related Message | Safety related data for broadcast communication | Warn AtoN malfunctioning |
| 21 | AIS AtoN message |  |  |
| 25 | Single slot binary message | Binary data for addressed or broadcast communication | Status report |

Summary of optional AIS AtoN Station messages

### Message 6

Message 6, Addressed Binary Message, can be employed by an AIS AtoN for sending AtoN status reports to the competent authority responsible for the AtoN. Useful data includes those for battery, lantern status, and solar power system charging current. The benefits for the competent authority include knowledge of equipment status, opportunity for preventative maintenance, early notification of faults, and ultimately increased availability. Such performance information can be fed back into the design process for AtoN systems.

### Message 8

Message 8 is a binary broadcast message. IMO has published a limited list of Message 8,

Application Specific Messages, for international use (SN.1/Circ.289). Competent authorities may use other Message 8 formats on a regional basis.

As an example, among the list of IMO Application Specific Messages is a message for

meteorological and hydrological data. Sensors on the AtoN provide this data to the AIS AtoN

Station, which in turn broadcasts this Message 8.

### Message 21

Defines the “Aids to Navigation Report”. AIS AtoN service enables AtoN providers to broadcast information on the following :

- Type of AtoN

- Name of AtoN

- Position of AtoN

- Position accuracy indicator

- Type of position fixing device

- On/Off position status

- Real and Virtual AtoN identification

- Dimension of the AtoN and reference positions

- Status of the AtoN systems

### Message 25

Message 25 is a single slot binary message that can for example be used to send encrypted

configuration data. See IEC 62320-2 for further details.

### Message 26

Message 26 may also be received, processed, and transmitted by an AIS AtoN station. Note that this message is not included in IEC62320-2.

### Messages 12, 14 , 25 and 26

There are two types of protocols for sending AtoN AIS messages.  They are Fixed Access Time Division Multiple Access (FATDMA) and Random Access Time Division Multiple Access (RATDMA).  These two protocols are set to ensure that messages from nearby AIS stations do not conflict.

## AIS AtoN type

### Type 1

This AIS AtoN Station is a transmit-only station, operating in FATDMA mode. Hence the slots used by the Type 1 AIS AtoN Station need to be reserved by a competent authority, using

Message 20, transmitted from an AIS station in the coverage area. The Type 1 unit must be

configured to use the slots reserved for it before being placed into service.

This is the simplest type of AIS AtoN station, likely to have low cost and low power consumption.

### Type 2

This AIS AtoN Station is similar to a Type 1, but has, in addition, an AIS receiver of limited capability which allows the Type 2 Station to be remotely configured via the AIS VDL. This receiver operates on a single AIS channel.

### Type 3

This AIS AtoN Station is more complex than the Type 1 and Type 2, and contains two AIS receiving processes that allow it to participate fully on the AIS VDL. This means that in addition to FATDMA, the Type 3 station can function in RATDMA mode.

The Type 3 station is therefore capable of:

- Autonomous operation, not requiring slot reservations (RATDMA);

- Autonomous operation using slots reserved by a competent authority, using message 20, transmitted from another AIS Station in the coverage area (FATDMA);

- Receiving and relaying AIS messages, including control and configuration messages for itself or for other AIS AtoN stations in a chain. See IEC 62320-2 for more details of chaining;

- Repeating AIS messages;

- Indirect synchronisation, using its receiving processes.

# Primary considerations

## Power consumption.

The power consumption needs to be balanced to the generation facility as well as the power consumption other AtoN such as lights etc. to deliver the required autonomy. The power consumption at the agreed configuration is recommended to be measured, rather than rely on the manufacturer’s generic data.  The power consumption will vary depending on the transmit frequency and the sleep interval. See point 5 Commissioning & Testing for further details. Refer to IALA guidance note 1039 on Designing Solar Power Systems for AtoN

### Special considerations between Type1 or 3? – Inter-session work – EEP 19

## Transmission range.

A typical transmission range is between 5 to 20 miles though this will increase with a higher aerial on the buoy and a high receiving station.    In areas of very heavy traffic, the volume of AIS transmissions may overload the base station which will reduce the range at which the base station will be able to cover.  With Type 1 AIS (FATDMA) in an area of heavy traffic, there may not be enough available slots to be allocated to enable transmission from the AIS station.

In a VHF bandwidth operation system, Transmission (Tx) and Reception (Rx) reach is understood to be tightly linked to the height of the antenna.  Therefore, location is of key importance, ensuring the highest position in the AtoN for safe installation and maintenance.

Considering Earth’s curvature, the straight distance between two points can be calculated by contemplating a supposedly smooth surface, that is, devoid of any flaws.  Nonetheless, the heights to be considered are those upon real surfaces plus the height of these above sea levels, in the exact same place.

Figure 1 shows a cross-section of the geoids by tracing a line tangent to the circumference, passing through the upper border of the point under consideration (i.e. the antenna) forming a right-angled triangle as in OAB, where:

Cross-section of the geoids

OA = rt + h1.

Where:

rt : Earth’s radius

h1: Height of antenna 1 plus height of position against sea level.

h2: Height of Antenna 2 plus height of position against sea level.

AB: Distance between point and horizon of this point

In this way, there can be calculated data-reception coverage of an AIS installed on a ship as compared to transmission coverage of an AIS installed on a floating beacon.  Knowing the height of the ship’s antenna in question, coverage can be estimated by adding the later to the height of the beacon’s antenna, as shown in the formula below in nautical miles:

In conclusion, the higher the Tx/Rx antenna are placed, the greater the range.

A1: Height above sea level of antenna installed on the AIS on ship ≈ 25 m.

A2: Height above sea level of antenna of the AIS installed on the buoy

Sample formula to estimate coverage reached by different types of floating beacons.

- Spar Buoy– Height of AIS-Aton - A2=10m    Range= 41.2 Km

- Maritime Buoy-type floating signal – Height of AIS-Aton - A2=4m    Range= 35.3 Km

### Extraordinary coverage – Ducts or Tropospheric Refraction

In certain zones, under specific environmental conditions, the troposphere may experience a meteorological phenomenon which creates ducts and channels enabling VHF frequencies to extend across greater distances.

This phenomenon takes place during certain weather conditions during which there are different refractive indexes which force electromagnetic waves to bounce back to the surface, thus broadly enhancing VHF coverage. This phenomenon should not be relied on to provide enhanced coverage.

Normal situation – Reception coverage ≈ 35 km

Situation with ducts – Reception coverage ≈ 200 km

### Selection of VHF antenna

The measurements and graphs shown above were obtained by means of an antenna analyzer AEA make, model Analyzer 140-525, which features an embedded frequency generator and a power meter to measure the Standing Wave Ratio (SWR).

The SWR is the ratio between maximum and minimum voltage of the standing wave. It may also be related to reflected and incident power, as shown below:

SWR= D+R / D-R

Where D is the number of divisions indicated by the instrument in a direct position, and R is the number of divisions indicated in a reverse position.

In case of perfect adaptation, SWR is 1 and all the power delivered by the system is radiated by the antenna, requiring that all installations endeavour to obtain the lowest possible SWR within the bandwidth or the frequency range in which the antenna operates.

VHF antenna to be installed for AIS applications is required to be marine VHF-type. Generally they have a central frequency is 156-157 MHz, and a bandwidth of between 6 and 7 MHz, ensuring good performance between 152 and 160MHz, and optimum work (SWR=1) in their central frequency.

Given that the frequency channels used by AIS are 161,975 MHz and 162,025 MHz, it is critical to use an antenna with a broader bandwidth or tuned to a frequency closer to that defined for the work of AIS.

### Comparison of different marine antenna

The installation of AIS equipment must be designed around the performance of the antenna, since marine antenna is exposed to metal, dielectric and human influences.

In order to make best choice of antenna to ensure a higher AIS efficiency and hence a greater coverage, there have been proposed three trials/measurements of different marine antenna associated to AISs installed on operational floating beacons.

As shown in graph Measurement 1, the measured antenna has a very low SWR (SWR around 1 is only achieved between 156 MHz and 157 MHz), though it lacks good response within the AIS frequency of 161,975 MHz and 162,025 MHz where that the SWR exceeds 10.

Measurement 1

In graph Measurement 2, this antenna has obtained better SWR figures for the AIS frequency with an SWR of 4.

Measurement 2

In graph Measurement 3, it is seen that this antenna is specially designed for AIS with a broader bandwidth than conventional marine antenna.  This allows for a SWR of 2 within the working frequency of AIS, a highly acceptable ratio in terms of installations.  Moreover, SWR remains close to 1 between 155 MHz and 159 MHz and gives a much flatter response than the previous ones, showing the high quality of the antenna.

Measurement 3

## Additional services.

### Message 8

AIS can be used to deliver additional services such as meteorological data and wave data. These can be incorporated into the e-Navigation services infrastructure. Accurate meteorological equipment can be very expensive and will require an analysis of the benefits over the cost of installation. Therefore, it may be a consideration to install the AIS on a Meteorological buoy rather than the other way round. Meteorological data can be difficult to verify and confirmation of the accuracy of this service needs to be assessed with any consequent liability.  An assessment of the number of users who are able to receive and display message 8 with Meteorological data needs to considered otherwise this service will be of little value.

One problem with this AIS capability is that AIS units do not always interface easily with meteorological transducers.

Different suppliers present alternatives to capture the meteorological information.

- The AIS AtoN may receive signals only from sensors supplied by the firm. This is very inconvenient for already installed networks due to the difficulty of changing sensors

- The AIS AtoN may require additional software.

- The AIS unit may require an interface to be designed and built by the user.

This means that in all cases an important development of software and in some cases hardware has to be carried out in-house.

As a general conclusion it can be said that it is not straightforward to transmit message 8 and that it should not be taken for granted that this will always be possible.

### Message 21  Regional bits - Monitoring capability (battery voltage, light status etc).

Message 21 can also be used to measure the energy source, photovoltaic level and the light system. This turns an AIS-AtoN into a highly efficient management tool when it comes to understanding the performance of the equipment, and in this way, the system itself.

The setting standards are detailed in Recommendation A-126 Edition 1.5 Item 4.8.4, and are to be established in the management software/AIS display.

In order to display the values being monitored by the AIS system (eg Battery voltage, Light on/off, Racon on/off ) suitable software is required. This software may be offered by either the AIS manufacturer, a standard AIS tracking package via the internet or bespoke software from a third party.

### Satellite monitoring of AIS

May be available in the future.

## Integrated AIS Units.

### Integrated unit with 3rd party equipment

AIS can be supplied as an integrated unit with 3rd party equipment such as lanterns, metrological or hydrological sensors. Monitoring data can include solar voltage, battery voltage, status of operating lamp or flasher, number of available lamps remaining (lamp changer), sun switch status, flash code.

### Integrated with Lantern and power system

AIS AtoN can be supplied complete with an integrated lantern, solar PV power supply with battery storage and all required controls and monitoring facilities.

## Licensing by local licensing authority

When an AtoN has an associated AIS, it becomes part of a dynamic network of information which is run by the users according to their own requirements.

The AIS may be Real, Synthetic or Virtual (Recommendation A-126 Edition 1.5 item 4.2), but in every case they must be properly registered with their Maritime Mobile Service Identity (MMSI) accordingly (Recommendation A-126 Edition 1.5, Item 4.3), which must be applied for with the National Telecommunications Authority.

Usually the organization with responsibility at national level gives a preliminary authorization which after time becomes definitive.

To obtain the MMSI for the first time in their country an interaction with the corresponding agency to define them is usually required. Each country will have their own licensing authority with particular fees and renewal arrangements which need to be followed.

### ITU unification MMSI on web page – Inter-session work – EEP 19 - DONE

The International Telecommunications Union (ITU) is the only organization entitled to register official information and updating coming from the Office of Telecommunications Management in the country which has jurisdiction over the AIS-AtoN.

In this way, the corresponding Office of Telecommunications Management is in charge of requesting the registration of the AIS-AtoN, thereupon notifying the ITU. Upon receiving the information from the Office of Telecommunications Management, MARS database will be updated.

Any queries to the AtoN MARS database may be addressed on the following web page:

http://www.itu.int/online/mms/mars/aidstonavigation_search.sh

Search criteria can be done by country or typing “99” in the “MMSI” field to obtain a full list with all stations notified.

# Physical Application

## IP rating

Electronic equipment installed on a Navigation Buoy will subject to severe environmental exposure.

In order to safeguard the integrity of the equipment, increase its lifespan and ensure its reliability, the installation must prevent the condensation cycle from starting. In order to achieve this, an IP(Ingress Protection) level of no less than IP66 should be specified

### Issues

AIS-AtoN equipment installation is exposed to sudden changes in temperature during the night, which allows water vapour present within the enclosure to condense and produce water drops which rapidly inhibit the protection conferred by the desiccating agent installed.

As the air gets cooler and releases water drops, the volume of air is reduced giving way to a vacuum inside the enclosure. If the air tightness of the enclosure does not keep humid air from seeping in, the day-night temperature oscillation creates a constant condensation of the water vapour that gets drawn in during the night by the vacuum effect inside the container.

This condensation takes place when the temperature gradient cools a mass of air up to the saturation point, which means that at this temperature a mass of air cannot hold water molecules in gaseous state, therefore releasing them as water drops.

Saturation can only happen given the following three factors:

- Temperature gradient (Temperature difference between the air inside and outside the IP enclosure);

- Water vapour (The higher the temperature of the air mass, the more water vapour it will carry);

- Air flow.

The removal of one of these three factors stops continuous condensation.

To this effect, it is fundamental that water-tightness be preserved, but above all, there should exist a state of balance between exterior and interior pressures eliminating air flow and breaking the cycle that generates condensation.

### IP enclosure rating

The enclosures and their connections should be rated to minimum IP66 and be protected from direct water spray. For more details see section 8.2.1

### Pressure balance

The pressure in the enclosure needs to be balanced with the outside in order to eliminate air flow between and thus eliminate condensation. This can be achieved with a propriety vapour barrier to the appropriate IP rating. For more details see section 8.1.2

## Protection

### Lightning.

In areas where Lightning strikes are considered to be a specific hazard, consideration should be given to protecting the AIS unit from this by installing Surge Protection.

The implementation of a surge protection to ensure the equipment against an atmospheric discharge is essential in any installation of electronic equipment afloat.
These atmospheric discharges affects stability of the equipment for the operation in the GPS signal acquisition and transmission.

The implementation of secure lightning protection requires good grounding to the water.

Block diagram of surge protection device

### Grounding

Block diagram of grounding device

### Physical damage, mounting against shock loading & vibration

Consideration needs to be given to ensure that the AIS unit is mounted such that it is free from vibration. A buoy can produce vibration in the upper superstructure from the motion of the waves. In order to alleviate this, the unit can be mounted lower down in a stronger area of the superstructure or mounted with anti vibration pads.

## Location of VHF antenna and GPS antenna receivers.

### VHF antenna

In the case of floating signals, installing VHF and GPS antenna at the uppermost section of a superstructure is a disadvantage from the structural point of view as it is vulnerable to damage if the buoy is struck by a passing ship or when the buoy is being recovered for maintenance.

Collision effect

### GPS Antenna Receiver

When planning the installation of a GPS antenna on a buoy, it is priority that it be clear of any vertical obstruction at all times, considering also the angles of vertical divergence.

It is important to assess the position of satellites for the geographic location.  This information is critical to locate the minimum number of satellites in the shortest time possible to assure effective transmission in slot allocation.

As long as the search sequence and reception of GPS signals is optimized for transmission, the timeframe of the equipment’s synchronization phase may be narrowed, with reduced energy use.

# Commissioning & Testing – To review at EEP19 October 2012 - DONE

The frequency of the sample period and the duration of the satellite sample can be adjusted by the user for specific locations & site conditions.

## Range testing – Inter-session work – EEP 19 – DONE

This is an example of a test for measuring voltage, current and power consumption from the power supply. It also gauges the transmit power on both AIS-AtoN systems at different operating conditions.

BASIC DIAGRAM:

Showing basic connection between systems

### Equipment used:

- AIS-AtoN A fully integrated by supplier and connected to a VHF antenna set at AIS 1 161.975 MHz and AIS 2 162.025 MHz frequency.

- AIS-AtoN B not integrated and connected to a VHF antenna set at AIS 1 161.975 MHz and AIS 2 162.025 MHz frequency, and the GPS antenna.

### Measuring instruments:

- GW Instek, 30 Vdc – 5 A constant voltage power supply.

- Agilent Multimeter up to 6 ½ digit.

- 3 dB and 10 dB attenuator.

- Agilent spectrum analyzer with a 100 KHz - 6 GHz range.

- ICOM. IC-R8500 receiver.

- Tektronix oscilloscope.

### Testing methodology

#### Regular operation

Within a period of 60 minutes both systems were tested with a nominal supply and a verified  SWR ≤ 1,5.

Transmit power as well as voltage, current and peak/standby power consumed from the power supply -battery- were measured.

#### Same conditions without GPS signal.

Ensuring normal operation over a testing period of 20 minutes, GPS reception was blocked out to keep the equipment from transmitting and being detected.

Voltage, current and peak/standby power consumed from the power supply -battery- were measured.

#### Operation with variable power supply.

Within a testing period of 30 minutes per voltage step, equipment operating threshold was determined by varying power supply at 500 mV from 13Vdc down to 11,2Vdc. Then below 11,2 Vdc  in 100mV steps.

Consumption peak and standby power as well as transmit power were measured.

### Transmit power measurement

In order to characterize the output power and the associated spectrum, a cable was connected from the VHF outlet of the equipment directly to a spectrum analyzer inserting cascaded attenuators -26 in all- so as to protect the input of the system.

GPS Antena

Transponder to be tested

Spectrum analyzer

Power supply

Cascaded attenuator

Measurement of power transmitted via direct connection through cable and signal attenuation.

### Data, curves, images and graphs obtained. Observations.

Consumption [uA] vs. Samples obtained. Transponder A. Samples taken every 12 msecs.

Consumption [A] vs. Samples obtained. Transponder A. Samples taken every 1,2 secs.

Consumption [A] vs. Samples obtained. Transponder B. Samples taken every 12 msecs.

Consumption [A] vs. Samples obtained. Transponder B (See table attached). Samples taken every 1,2  msecs.

Trial moment for Transponder B, AIS receiver connected to a notebook computer via RS-232, Multmeter for measuring and registering current, and cascaded attenuators for the transmitted signal to be displayed on the Spectrum analyzer.

Spectrum analyzer displaying transmission spectrum.

Spectrum of Transponder A.

Spectrum of Transponder B.

### Conclusions

After the tests, the following conclusions were reached:

- Both have a low average consumption rate. Current peaks, however, may reach 5 to 6 times higher.

Iavergage(Transponder A)= 26 mA.		Ipeak(Transponder A)= 2,241 A.

Iaverage(Transponder B)= 59 mA.		Ipeak(Transponder B)= 1,490 A.

- There is an insubstantial difference between the power levels of the spectrum at F1 and F2 frequency. The sum of both spectrum yields a total power of:

Total Power (Transponder A)= 27,366 dBm.

Total Power (Transponder B)= 27,625 dBm.

- When the supply voltage was diminished at a rate of 100 mV steps the following limits were obtained, beyond which the transponders ceased transmitting correctly.

Minimum supply voltage= 8,8 Vdc (AIS-AtoN A).

Minimum supply voltage = 9,5 Vdc (AIS-AtoN B).

- When the GPS antenna received no signal, Transponder consumption remained consistent with consumption of operation without transmission thus receiving no signals from the Transponder.

Consumption without GPS≈ 30 mA. (Transponder A).

Consumption without GPS ≈ 55 mA. (Transponder B).

## Frequency verification – Inter-session work – EEP 19

# MaintenANCE & oPERATION

## Programming

AIS AtoN units require programming for the following parameters

- MMSI

- Name of Navigation Aid

- AIS type (Type1, 2 or 3)

- Charted position

- Guard zone (Fence)

- For type 1: Transmission interval, Slot allocation

- AIS type (Real, Virtual or Synthetic)

This programming can be completed onshore or on station. The danger of programming ashore is that the buoy is not placed in the charted position and it will transmit an off station signal.

The issue with programming on station is the need to have programming equipment and trained staff on the ship to deliver this.

Confirmation of all on site programmed information needs to be recorded with the Competent Authority.

## Spare transponders

It is necessary to have spare transponders to replace those that fail, are damaged by passing ships, vandalized or out of service for any other reason.  The best way to maintain the service is to replace a malfunctioning transponder and to proceed to investigate the failure and eventually proceed to its reparation at the workshop

The spare number of transponder required depends on the installed number of them and an elevate compromised of level service. Assuming a high level of service the following list gives an indication

| Installed number of transponders | Recommended number of spare transponders |
| --- | --- |
| 1 | 1 |
| 2 to 3 | 1 |
| 4 to 6 | 2 |
| 7 to 10 | 3 |
| 11 to 16 | 4 |
| 17 to 50 | 25 % |

Spare transponders

## Maintenance requirements – Inter-session work – EEP 19 – DONE

Maintenance and implementation of AIS-AtoN equipment may vary depending on the following aspects:

- Characteristics of the device

- Implementation environment: fluvial or maritime

- Types of aids to navigation.

- Environmental conditions.

Considering these differences, it must be noted that when the equipment has been integrated by the supplier, there is a lower risk of failure to the internal connections, external antenna connection, balance of energy, etc.

User-implemented AIS equipment runs a higher risk of failing and producing a total signal loss.

Therefore, the purpose of this guideline is to consider the probability of failures occurring due to human error, requiring that the process be standardized, so as to minimize risk of failure.

### Technical Staff

One of the most common situations to be faced on board of the buoy tender while offering assistance in Signal Preventive Maintenance and Reparation tasks is that of repetitive work. This could cause slackening of attention among technical staff leading to oversight in signal and device verifications, or to a common understanding that the overall status of the signal is good, therefore neglecting details that could, in the medium-to-long term, lead to failure.

Considering the aforementioned and the fact that technical staff is responsible for verifying:

- Performance of the anchoring system

- Color and day marks

- Structural condition, accessory items, bindings.

- Signal positioning

- Power supply, wiring and connectors.

- Useful life of equipment, battery, lantern, photovoltaic panel, regulator, AIS, Racon, Tide Lantern.

- Cleaning of the signal and its surroundings; excessive vegetation.

- Light beam coverage, leveling, orientation.

It is recommended that the technical staff follow a checklist containing all the regular measuring and control standards in order to ensure reparation, maintenance and normal functioning of an AIS System on a navigational aid. The following parameters should be contained therein:

- AIS Aton verification.

- Condition of VHF antenna, of its flexibility and of the GPS if placed on the outside.

- Condition of wiring and connectors, stiffness, cracks, moisture and oxidation.

- SWR measurement.

- Power supply checking.

- Moisture inside the device.

- Fixing system.

- Transmission range and coverage measurement.

It mandatory that technical staff in charge of signal maintenance be properly trained in applied techniques and best practices. Since AIS maintenance and implementation on navigational aids is not a routine task, their performance is to be assessed and monitored within a short period of time.

This is because a small percentage of signals within the signaling system are equipped with AIS devices, strategically installed on key navigational aids, namely lighthouses, etc.

### Configuration Software

The technician is expected to be well acquainted with the configuration software. In situ mishandling is a very common problem in this area.

It is also quite frequent that after some years, the equipments installed will be comprised by several models of the same supplier as well as equipment from different suppliers.  Traceability of existing versions will lower the likelihood of confusing software configuration of AIS equipment.

This type of defect can be identified and corrected instantly whereas faulty assembly, short circuit or contact failure in antennae as well as moisture inside cable mesh and micro fissures in plastic cable jackets may not be detected soon enough during the testing period, leading to failure not long after startup.

## Training – Inter-session work – EEP 19 - DONE

### Capabilities required for lanternists. (Maintainers)

The technical staff associated with the maintenance and operation of AIS- Aton should have specific training that allows them to work independently in the resolution of faults that occur in AIS-AtoN.  The skills required to maintain and programme AIS AtoN units at sea are at a higher level than that required to maintain a basic lighted buoy.

### Human, physical and technical restrictions on board.

When a vessel renders support to a signal, technical personnel on board does not usually meet the best working conditions. This is due to different factors such as bumps, wave pounding, vibration, different movement frequencies between the vessel and the signal, climatology, limited access to labs or test rooms, and insufficient equipment and storage space.

It is also worth noting that due to the coverage of the signaling system, vessels with specialized technical crew will be deployed to meet the maintenance tasks of all the signals. With such a diversity of profiles among technical personnel and without a standardized working protocol, it is very difficult to guarantee that the intervention of any technician at random will ensure quality work.

Hence, it is for operative reasons that AIS equipment installation needs to be conducted by specially trained personnel, or in special workshops meant for such purposes.

### Training documentation

As with all training, manuals need to be plain, concise and reader-friendly, containing graphs and screen grabs.

# Selection of equipment.

When selecting a suitable AIS AtoN system, the following points should be considered to ensure that the unit performs well in service;

- Favourable cost-benefit ratio;

- Size of unit compared to available space;

- Simple to configure and interrogate.

- Upgrade easy to perform;

- After-sales service;

- Hardware and software capacity expansion;

- Different hardware solutions adaptable to the equipment base;

- External LED status indicators;

- Integrated device, easy to install;

- External connection, (Example: military-style).

- Low power consumption

## Issues to consider

- Protection against atmospheric discharge;

- Compatibility with GPS antenna. Complicated repair / maintenance on site.

# General Considerations for integrating an ais-AtoN System

The best method of work to achieve the flawless installation of an AIS-AtoN -whether integrated or not- is to ensure the highest quality of components, processes and people involved; moreover, to provide appropriate training for all the qualified personnel in charge at every stage of the deployment.

If the equipment is not fully integrated when delivered by the supplier, such service must be carried out by the installer, commonly making use of watertight enclosures which ensure the durability of the AIS equipment.

The purpose of this document is to offer guidelines for the perfect assembly of the watertight boxes mentioned above, and to establish sensible installation procedures, intended to ensure quality assembly and efficiency in the use of materials and resources, and in the avoidance of common errors.

## Impact on Buoy Performance

The installation of AIS on a small buoys of less than 2.5 meter diameter may impair the buoyancy and stability characteristics. The overall mass and centre of gravity of any additional items needs to be considered at the design stage.

By definition, the AIS unit requires a GPS receiver and VHF antenna at high level, this will require cables to pass the light unit and may impair lantern performance. All cable routes need to be considered at the design stage to minimise this.

## General Assembly

Standardized assembly patterns ensure enclosure capable of withstanding challenging weather conditions, thereby preserving the equipment within.

In a harsh environment, consideration should be given to use 2 watertight boxes (> IP66) to provide double protection in case of flooding of the external box and condensation in the internal one.

This point stresses the necessary balance of pressure between both boxes to avoid condensation in the internal box, where the device is contained.

It is recommended that the VHF and GPS antennae be installed before the final stage of assembly of the external box onto the corresponding signal.

Consideration should be given to use of UV stabilized cables and protection to excessive UV rays and heat.

### Hard wired or plug connections.

The weather conditions which AIS AtoN have to endure, may affect normal functioning thus shortening their lifespan.

It is of utmost importance that VHF and GPS wiring be well protected. In the case of GPS antenna, when they are not integrated in non-integrated equipment.

Deterioration and loss of flexibility and malleability of cables due to climate, UV rays and temperature gradient, plus mesh oxidation, lead to reduction or even loss of conductivity and increase of impedance.

Fully assembled enclosure

### Preparation of inlets in watertight enclosures

Ensuring a completely watertight enclosure is a challenging task given that the holes on the sides to let the cables in and to place cable glands or connectors are not original factory conditions.

Therefore, same caliber holes must be drilled into the front and/or sides of the boxes.

It is critical to seal all joints between the inlets and the corresponding cable glands and connectors using flexible glue so as to accompany the deformation derived from temperature oscillations.

In accordance with the installation guidelines, antennae must be plugged onto the same side in all boxes; likewise, power supply cables and earthing onto a different side.

Different sides of the perforated box

### Assembly of cable glands

Cable glands and/or connectors must be fitted carefully so that both the inner and outer rubber washers of the box do not get blocked and deteriorated when screwing the inner nut, making sure in this way that the whole set is firmly attached to the sides.

The clearance between glands and cables must be as insignificant as possible to ensure the minimum filtering of moisture.

A selection of different consecutive diameters may be necessary since there is no heat shrink tubing that will simultaneously adjust both gland and coaxial cable.

Different diameters of heat shrink tubes

### Preparation of the AIS System

Integrating different electronic devices requires that equipment and connections be firmly adjusted for the startup and fitting together of antennae and setup cables, thereby avoiding failure due to poor contact, loss of data and functional efficiency, and the possible ensuing breakdown of the equipment.

Inserting a rubber plate in the anchoring system of the AIS equipment has proven to be an effective way of softening vibration during the operation.

In some non-integrated installations it is recommended to fit a polarized key inserted between the power supply system and the input of the equipment. This key must remain inside the internal box.

Equipment with its protection

Complete Mixed IP Box

### Sealing of external connections

If the installation is performed using glands or connectors, moisture is liable to seep in, making necessary the use of heat shrink tubing. This component, which protects the cable connection of the external box, can be easily fitted; otherwise it can be stuffed with a paste that will seal the clearance between the heat shrink and the cable.

Once determined the stretch of heat shrink to be applied, the subsequent position of the cables must be calculated before the final adjustments are made.

When the heat shrinks have been properly fitted, the cables must protrude downwards -as shown on picture below- to enable smooth outpour of water.

Sealing of antenna and power supply inputs.

Sealed cable gland with heat shrink

### Connections and Completion

The gas shielding must be inserted in the input of the VHF antenna inside the internal watertight box keeping the connections isolated from any source of moisture.

Next, VHF and GPS antennae must be connected, verifying the SWR of the former, ensuring measurement around one (1) in line with point 3.2.3.

## General outlines for the assembly

- Follow Installation Chart shown on Annex n°1

- The necessary components must be recorded on the chart.

- The power supply system must always be inspected. Whether photovoltaic or not, redundancy must always be considered.

- Boxes must be mechanized to enable the access of the power supply connections and the VHF/GPS antennae. It is important that this process be recorded in the Installation Chart so as to keep track of intermediate controls.

- Once the external box has been anchored either to a superstructure, a floating AtoN or to its corresponding sledge -when it comes to fixed signals- the procedure must be recorded in the Installation Chart.

- When antenna cables have been fitted, the SWR of the VHF antenna must be measured and the water tightness of the cable glands must be visually verified and recorded in the Installation Chart.

- Having completed the installation and tested the system for at least one (1) week, the equipment will be ready to be installed.

| Components | Non-Integrated Equipment |
| --- | --- |
| Internal Box | Measurements |
| External Box | Measurements |
| Power Supply Cable | Cable prepared for open air exposure |
| Cable of VHF antennae | 50 ohms RG-58 coaxial cable prepared for open air installation |
| Ground cable | 5mm steel cable |
| GPS Antenna | Its voltage must be compatible with the output of the equipment |
| VHF Antenna | It must be set within frequency of channels 87 and 88 B |
| Protection against reverse polarity | Polarized key |
| Gas shielding | Gas shielding to be inserted in VHF antenna wiring |
| Connectors / Adapters | UHF / TNC / BNC |
| Sift-proof sealant | Flexible glue |
| Heat shrink tubes with watertight sealant | Consecutive stretches with different diameters |
| Cable glands / Connectors | Standalone |

List of necessary components for installations

# Glossary of terms

- AIS: Automatic Identification System

- ATON: Aid to Navigation

- IMO: International Maritime Organisation

- VDL: Very High Frequency Data Link:

- VHF: Very High Frequency

- FATMA: Fixed Access Time Division Multiple Access

- RADMA: Random Access Time Division Multiple Access

- IEC: International Electrical Commission

- ITU: International Communications Union

- SWR: Standing Wave Ratio

- IP: Ingress Protection

Sample Checklist

| Checklist for the verification of AIS on AtoN installation |  |  |
| --- | --- | --- |
| Signal |  |  |
| Person at workshop |  |  |
| Person on board |  |  |
| Item | Description | Agreement |
| Signal type |  |  |
| AIS serial number |  |  |
| Internal & External Boxes |  |  |
| Internal & External Connectors |  |  |
| VHF Antenna |  |  |
| GPS Antenna |  |  |
| Gas shielding |  |  |
| Grounding |  |  |
| Heat shrink tubes |  |  |
| Insulation of cables and antennae |  |  |
| Attachment to structure |  |  |
| Equipment testing time |  |  |
| Regulator |  |  |
| Battery and solar panel |  |  |
| Antenna SWR |  |  |
| Photography |  |  |
| Start date/Finish date |  |  |

Sample checklist

# rEFERENCES

IALA Recommendation A-126 Edition 1.5

IEC 62320-2

IALA guidance note 1039 - Designing Solar Power Systems for AtoN

ITU 1371

Circular 17