# Draft Information Paper

# AtoN AIS Implementation in Korea

# Background

This paper provides the information regarding to the status and operating conditions of AtoN AIS installed in Korea. The purpose of AtoN AIS experimental installation is to promote and enhance safety and efficiency of navigation.

Through not less than 2 years operation, it has been played an important role and verified the effectiveness. Nevertheless AtoN AIS didn’t spread in nationwide because of some of disadvantages and an expected problem.

# Summary

The Dae-san area in Korea has being operating not only major 3 ports having a various dolphins, mooring buoys, Poers and Berths but also a lot of fishery ports.

A natural base of the crystal clear sea and deep channel, more than 20 meters in depth, the port of Yeosu & Gwangyang is situated in an ideal location, and further, the island of the Myo-Do serves as a natural breakwater blocking high waves.

Around 300 AIS stations such as class A AIS station, class B AIS station, AIS base station, including almost 120 AtoN AIS stations are working together within 100km radius.

The AtoN AIS used in the experimental operation was designed as class 3 type AtoN AIS using RATDMA access scheme in accordance with ITU R.1371-3, IEC62320-2, IALA A-126.

Every AtoN AIS station was assigned an identification number with MMSI as maritime mobile service by a responsible administration in Korea. These MMSI numbers are formed as “99MID1XXX” according to ITU-R M.585-4 Annex 4.

The AtoN AIS station has been installed and being operating at every light houses, fixed AtoNs, floating AtoNs.

The figure below shows the AIS installed on the Yeosu & Gwangyang area. Around 100 TRS and CDMA and 40 AIS station are working together within 100km radius, respectively. The TRS and CDMA station instead of AIS station, as applied to the experiment has also purpose to transmit accurate positions of floating buoy, indicate if a floating buoy is off position, provide weather, tidal, and sea state data, monitor the status of a TRS and CDMA such as a voltage and current on the lamp.

Metrological buoy with AtoN AIS                               Floating AtoN AIS

AtoN AIS

The AtoN AIS, as applied to the experiment has a purpose to transmit accurate positions of floating AtoN, indicate if a floating AtoN is off position, provide weather, tidal, and sea state data, monitor the status of an AtoN such as a voltage and current on the lamp.

The function of an AtoN AIS

Message 21(AtoN report) is comply with that of ITU-R M.1371-3 exactly and message 8 is configured in accordance with IMO MSC Circ.236. Message 6 was recomposed to monitor a various voltage and current, setup and reset purpose.

Reporting interval of message 21 is setup every 3 min basically and other messages transmit every 6 min in succession of transmitting message 21. These ways are to reduce battery consumption. Each message’s interval can be changed remotely

In practice, no necessary to provide message 6 and message 8 faster than every 6 min.

Information for a virtual AtoN is transmitted from operation center using message 6 and the AtoN AIS station received a virtual AtoN information retransmits message 21 together with his own message 21.

Operating centre can get information not only from AtoN AIS station but also from ship’s AIS station navigating in vicinity of an AtoN facility. If a collision accident happen, it can be traced the vessel which was on a voyage

Information obtained from Message 6 and Message 8 could be utilized through analyzing tools as below.

Metrological and marine Information                                   AtoN Status

Monthly trend for wind direction and speed, current direction and speed, temperature, humidity, can be processed as useful information. It also can be provided to a passenger terminal.

AtoN Status information is most important to administration concerned. It can estimate the lifetime of a storage battery and troubles of a solar panel and the condition of the lamp.

Operating centre monitors in Dae-san area more than 120 AtoN facilities and vessels and arranges replacing a storage battery maintains a lamp.

# Discussion

- A fixed AtoN is desirable to use a fixed position data because sometime GPS has an fixed error and it has bein results in confusion

- It is founded most of ship’s AIS station couldn’t receive a metrological information through the message 8. The message 8 is normally using for broadcasting a binary text.