AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  contact@iala-aism.org       Internet:  www.iala-aism.org

IALA Guideline No. ####

On

Cost Engineering of Short Range AtoN and Asset Management

Edition 1

[Date issued]

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

# Table of Contents

Document Revisions	2

Table of Contents	3

Index of Tables	4

Index of Figures	4

1	Introduction	5

2	Purpose	5

3	Scope / Purpose	5

4	Overall concept of asset management	5

5	Financial	5

5.1	Cost benefit analysis	5

5.2	Whole life cost	5

5.3	Depreciation	5

5.4	Fixed/ variable costs	5

5.5	Momentum after change	6

6	Equipment Provision and Performance	6

6.1	Provision	6

6.2	Operation	6

6.3	Maintenance	6

6.4	Decommissioning	6

7	Service capability	6

7.1	Manpower	6

7.2	Logistics / Services	6

7.3	Ship	6

7.4	Helicopter	6

7.5	Transport	6

7.6	Non AtoN service buildings	6

7.6.1	Office	6

7.6.2	Buoys yards	6

7.6.3	Workshops	6

7.7	IT Service	6

8	Performance	6

8.1	Key performance indicators	6

8.2	Measuring Key performance indicators	7

8.3	Performance criteria for outsourcing	7

9	Safety	7

9.1	Personal safety	7

9.2	Third party safety	7

9.3	Safe design of systems and equipment	7

10	Environmental	7

10.1	Legal compliance	7

10.2	Cost of compliance	7

10.3	Environmentally compliant operations	8

10.4	Cost benefits of AtoN vs environmental marine accident	8

10.5	Effect of environment on operations – bird / seal colonies	8

11	Legal	8

11.1	Compliance with defined standards	8

11.2	Certification	8

12	Surplus assets	8

12.1	Heritage	8

12.2	Non AtoN	8

12.3	Artefacts	8

13	References	8

ANNEX A	Aton key performance indicators	10

ANNEX B	KPI measurement - availability	12

ANNEX C	Outsourcing contract - key performance indicators	14

APPENDIX 1	Appendix title	16

# Index of Tables

Table 1	Title required	3

# Index of Figures

Figure 1	Title required	3

Cost Engineering of Short Range AtoN and Asset Management

# Introduction

Strategic cost management has become an essential tool for all organisations. AtoN provision can be considered in the context of any service provision business.  In order to ensure that mariners are provided with the most cost effective AtoN service, cost management is essential.

Strategic cost management is a way to address cost concerns while preserving the key parts of the business.  While formulating the strategy for the accomplishment of organizational overall objectives, different cost drivers should be clearly identified.  Identification of key cost drivers help administrations to focus on key activities that will constitute almost 90% of the total costs.  In view of this, the importance of strategic cost management should not be underestimated.  This implies that organization should be installing an appropriate framework of strategic cost management to reduce its costs in key areas on which the success of organization is heavily dependent.

Cost management can be considered as the process of planning and controlling the budget of a business.  It is a form of management accounting that allows a business to predict impending expenditures to help reduce the chance of going over budget.  Many businesses employ cost management plans for specific projects, as well as for the over-all business model.  Management of cost related activities is achieved by collecting, analysing, evaluating, and reporting cost information used for budgeting, estimating, forecasting and monitoring costs.

# Purpose

The purpose of this guideline is to provide guidance on cost effective provision and management of Aids to Navigation (AtoN).

# Scope / Purpose

This guideline sets out methods for assessing the cost of AtoN provision and performance indicators to assist AtoN service providers ensure that mariners are provided with the most cost effective AtoN service.

This document should be considered as complimentary to other IALA quality recommendations and guidelines which are referenced in the relevant sections.

# Overall concept of asset management

# Financial

## Cost benefit analysis

## Whole life cost

## Depreciation

## Fixed/ variable costs

## Momentum after change

# Equipment Provision and Performance

## Provision

## Operation

## Maintenance

## Decommissioning

# Service capability

## Manpower

## Logistics / Services

## Ship

## Helicopter

## Transport

## Non AtoN service buildings

### Office

### Buoys yards

### Workshops

## IT Service

# Performance

## Key performance indicators

KPIs are commonly used by an organisation to evaluate its success or the success of a particular activity in which it is engaged.  Choosing the right KPIs is reliant upon having a good understanding of what is important to the organisation.  What is important often depends on measuring the performance of the organisation and the KPIs will depend on the department. KPIs useful to finance will be quite different than the KPIs assigned to maintenance, for example.  Because of the need to develop a good understanding of what is important, performance indicator selection is often closely associated with the use of various techniques to assess the present state of the business, and its key activities.  These assessments often lead to the identification of potential improvements; and as a consequence, performance indicators are routinely associated with performance improvement initiatives.

KPIs used by AtoN administrations vary depending on the way the business is run and the culture of operation.  Irrespective of whether an Administration carries out all its business using in-house resources or outsources some or all of its operations, KPIs provide an invaluable role in ensuring that the Administration achieves its goals.

The key performance indicators listed in Annex 1 are grouped under the different headings typically found in an Administration.  These are not definitive but are suggested as KPIs that would best suit AtoN providers and individual Administrations may use some, all or other KPIs to ensure cost effective management.

## Measuring Key performance indicators

Having selected appropriate KPIs, measurement will depend on the data required.  Collection of information can be implemented using a paper system, daily written logs, report sheets, computer reports, audit reports and so on.  In most cases Computerised Maintenance Management Systems (CMMS) will be used for collecting the data as close to source as possible.  It is also critical that any piece of data is collected only once but may be used by the CMMS for multiple purposes.

Whatever the KPI, a comprehensive written method for measurement is strongly advised. Measurement of AtoN Availability is outlined in IALA Recommendation O-130 and IALA Guideline 1037.  In order to ensure that measurement is consistent across an Administration or between Administrations, the method stated in Annex 2 could be used.  Similar measurement method statements could be provided for other KPIs in additional Annexes.

## Performance criteria for outsourcing

Annex 3 shows a list of criteria that could be considered when outsourcing.  It provides a useful checklist of parameters that should be included in an outsourcing performance contract.  KPIs are used to measure contractor performance over the contract period and can be linked to an annual performance payment, which along with the monthly base fee, forms the annual contract price.

# Safety

## Personal safety

## Third party safety

## Safe design of systems and equipment

# Environmental

## Legal compliance

## Cost of compliance

## Environmentally compliant operations

## Cost benefits of AtoN vs environmental marine accident

## Effect of environment on operations – bird / seal colonies

# Legal

## Compliance with defined standards

## Certification

cost and benefits

what benefit does certification provide in provision of the AtoN / asset.

# Surplus assets

## Heritage

## Non AtoN

## Artefacts

# References

Recommendation 0-130 On Categorisation and Availability Objectives for Short Range Aids to Navigation, Edition 2, June 2011

IALA Guideline 1037 On Data Collection for Aids to Navigation Performance Calculation, Edition 2, December 2009.

IALA Guideline 1075 On A Business Plan for the complementary use of a Historic Lighthouse, Edition 1, December 2009.

IALA Recommendation R-121 On the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz, Edition 1.1, December 2004.

Input paper EEP19/25 - Key Performance Indicators to IALA EEP Committee meeting 19.

IALA Recommendation O-132 On Quality Management for Aids to Navigation Authorities, Edition 2, December 2006

IALA Guideline 1034 On the Certification of Marine Aids to Navigation Products, Edition 2, December 2008.

Presentation Too Many Fixed Aids - Not Enough Money How to Set Priorities ?, Richard Moore, IALA Workshop on Short Range AtoN in the e-Navigation Era, 2012.

Presentation Life of plastic buoys, purchasing policy and cost management, Sipke Hoekstra, IALA Workshop on Short Range AtoN in the e-Navigation Era, 2012.

Presentation Key Performance Indicators for AtoN Provision and Management, Seamus Doyle, IALA Workshop on Short Range AtoN in the e-Navigation Era, 2012.

Note.

Those references highlighted in yellow cannot be used as references in the final document.  References need to be sources open to the worldwide reader.

Aton key performance indicators

Guidelines should have Annexes.  Appendices are attached to Annexes.

Note: this list should be verified and compared with the text for consistency. Other relevant KPIs could be added.

Customer Service

Number of periodic reviews per year to verify and prioritise AtoN required;

Number of marine accidents per year;

Availability of AtoN in accordance with IALA guidelines;

Number of AtoN inspection per year to ensure quality;

Financial

Cost percentage reduction on previous year;

AtoN unit cost (for comparable AtoN or using ‘standard’ AtoN;

Capital projects completed on time and budget;

Return on capital investment.

Equipment performance

Mean Time Between Failure;

Mean Time to Repair;

Planned maintenance programme on time and budget;

Innovation/ Improvement projects.

Manpower

Number of personnel per AtoN;

Number of back office staff as percentage of total workforce;

Training budget as percentage of total budget;

Employee turnover rate.

Safety

All accident frequency rate per year;

Lost time injury rate per year;

Sick Absence as percentage of total time per year.

Logistics/ Services

Ship working hours as percentage of total hours per year;

Helicopter flying hours used per year;

Helicopter operational flying hours as percentage of total helicopter hours flying per year;

Contractor transport cost compared with previous year;

Office / Workshop infrastructure cost per unit floor area;

IT Corporate Service availability.

Environmental

Number of prosecutions/ enforcement orders.

Legal

Number of audits confirming compliance with defined standards;

Cost of achieving/ retaining quality standards certification.

Heritage

Annual cost of non-operational heritage assets;

Gross and net income from alternative use of heritage assets;

Number of heritage assets.

KPI measurement - availability

The following sets out the method of calculating availability of AtoNs in accordance with IALA Recommendation O130 (AtoN excl DGPS, AIS) and IALA Recommendation R121 (DGPS). Reporting is over a 3 year period for short range aids and over a 2 year period for DGPS in accordance with the IALA recommendations.

- Short Range Aids - Lights, Sound Signals, Racons and AIS

For clarity, IALA Recommendation O130 defines AtoN performance parameters as follows:

Total Time (Total Operational Hours) is the time that an individual AtoN or a system of AtoN should be performing their specified function;

Down Time (Hours Outage) is the sum of the periods during which the AtoN of the system of AtoN are unable to perform their specific function.

It does not include those periods when the mariner has been notified of a discrepancy by prior publications through a Preliminary Notice to Mariners.

A failure is the malfunction of an aid to navigation or system of AtoN to display its proper characteristics or to be on its assigned position for its intended use by the mariner. As such, a failure of a technical function is not necessarily considered an aid to navigation discrepancy. For example, if the main power supply has failed but the light continues to function at normal intensity on standby power, this is not considered a failure, since the aid to navigation continues to provide its characteristics to the mariner.  The failure may be caused by equipment malfunction, or scheduled or unscheduled maintenance work.

( Hours outage x 100 )

% Availability (3 year rolling average) =	100 -  	_____________________

( Total operational hours )

where:

Hours outage = total hours that the AtoN category is not available for use (includes daylight hours for lights and clear weather hours for sound signals) but excluding periods when the mariner has been notified of a discrepancy by prior publications through a Preliminary Notice to Mariners;

Total operational hours = number aids x 365 x 24 x 3

Mean Time Between Failures =	( Number of Aids x 365 )

(MTBF)  (days)	(  Number of Outages )

where:

Number of Aids = total number of AtoN in the AtoN category;

Number of Outages = total number of AtoN outages in the AtoN category excluding planned outages when the mariner has been notified of a discrepancy by prior publications through a Preliminary Notice to Mariners.

Mean Time to Repair			= 	(   ______Hours Outage______	)

(MTTR) (days)					(	Number of Outages x 24	)

where

  - Hours outage = total hours that the AtoN category is not available for use (includes daylight hours for lights and clear weather hours for sound signals) but excluding periods when the mariner has been notified of a discrepancy by prior publications through a Preliminary Notice to Mariners

  - Number of Outages = total number of AtoN outages in the AtoN category excluding planned outages when the mariner has been notified of a discrepancy by prior publications through a Preliminary Notice to Mariners.

DGPS

IALA Recommendation R121 defines DGPS availability as the percentage of time that an aid, or system of aids, is performing a required function under stated conditions. The non-availability can be caused by scheduled and/or unscheduled interruptions. Performance is considered over a 2 year period.

DGPS Service availability

The IALA required availability for the GLA DGPS service is 99.8% (Service Availability) and is based on overlapping coverage between adjacent stations. In the event of failure of one transmitter, service to the mariner is maintained from the adjacent stations. A DGPS service failure is considered to occur only when adjacent stations have failed.  Availability is calculated based on adjacent station outages for each of the GLA DGPS service areas. The lowest figure is used and the availability for the service as a whole is then quoted as being equal to or better than this figure. It has been agreed that for DGPS, availability includes planned outages.

(      Seconds outage		)

% Availability of each Service Area  =	1 -  	________________________

(   Total operational seconds	)

where

  - Seconds outage = total number of seconds that overlapping coverage is not available for use in one DGPS service area and including planned maintenance outages and periods when the mariner has been notified of a discrepancy by prior publications through a Preliminary Notice to Mariners.

  - Total operational seconds = number of seconds in the review period (2 years).

  - Service availability is taken as the Service Area with the lowest availability.

Mean Time Between Failures for the DGPS service is calculated as for short range aids, except that planned outages for which NTM has been issued are included as outages.

Mean Time to Repair for the DGPS service is calculated as for short range aids, except that planned outages for which NTM has been issued are included as outages.

Outsourcing contract - key performance indicators

- Operational Performance Criteria

The following operational performance criteria are measured through the KPI process.

Standard of works;

AtoN performance levels;

Quality, safety and environment;

General reporting requirements;

Monthly, quarterly and annual reporting requirements;

Project reporting requirements;

Incident reporting requirements;

Corporate Performance Criteria

The following management plans are in place to ensure that the agreed contract outcomes are achieved and are also monitored through the KPI process.

Contract management plan:

Risk management plan;

Contract administration plan;

Audit program;

Quality, environment and safety management plan;

Transition and disengagement plans.

Human resource management plan:

Staff management plan;

Training management plan;

Occupational health and safety plan.

Engineering management plan;

Engineering design review;

Licensing and certification;

KPI Requirements

KPI’s are used to measure contractor performance over the contract period and are linked to an annual performance payment, which along with the monthly base fee, forms the annual contract price;

KPI’s are reviewed and agreed on an annual basis.

The annual contract review process allows for KPI’s to be altered to address issues or opportunities identified during the previous year.

Performance against the agreed KPI’s are reviewed, discussed and agreed on a quarterly basis.

AtoN Performance Levels

Maintenance and fault/failure restoration strategies are required to be in place to achieve the agreed availability and repair times;

Performance levels are calculated in accordance with AMSA and IALA guidelines and reported quarterly;

Equipment availability and failure restoration time is measured from the issue and cancellation of maritime safety information (Auscoast Warning or ReefVTS failure notification);

The contractor is not penalised for failures due to unforeseen events such as severe weather events, vandalism or natural disasters;

Quality, Safety and Environmental Performance

The contractor must maintain certification for quality, safety and environmental management systems.

Lagging indicators such as number of lost time injuries or medical treated injuries are recorded but are not linked to performance payments.  Incident and accident reporting information is a critical input to minimising future incidents.  Any indicators that encourage non-reporting should be avoided in performance payment based systems.

Positive Performance Indicators (PPI’s) or leading indicators are used to measure actions that the contractor takes to achieve its safety targets.

Good performance against PPI’s will lead to good safety outcomes. The following PPI’s could be considered for use in a KPI framework:

Number of OHS training & programs;

Incident reports - the numbers of days overdue: > 3 months;

Hazard reports - the number of days overdue: > 3 months;

Number of safety inspections/audits conducted;

Percentage of sub-standard conditions identified and corrected;

Percentage of employees with adequate OHS training.

Appendix title

APPENDIX Heading 1

Followed by Body Text

Appendix Heading 2

Followed by Body Text

Appendix Heading 3

Followed by Body Text Indent