AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  iala-aism@wanadoo.fr       Internet:  www.iala-aism.org

IALA Guideline No. ####

On

Bird Deterrents

Edition 1

December 2012

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| 17.4.12 |  |  |

# Table of Contents

Document Revisions	2

Table of Contents	3

Index of Figures	3

Bird Deterrents	4

1	SCOPE	4

2	AIM	4

3	Introduction	4

4	Negative effects of BIRD Fouling.	4

4.1	From an Availability and Structural Perspective	4

4.2	From an Occupation Health & Safety perspective.	5

5	Methods for Dealing with Bird Fouling Problems	5

5.1	Bird Spikes	5

5.2	Audible Bird Deterrents	6

5.3	Visual Bird Deterrents	7

5.4	Bird Rollers	7

5.5	Variation of Installation of Components and Fittings	8

5.6	Electrical Deterrents	8

6	Environmental Considerations	9

# Index of Figures

Figure 1	Examples of Bird Spikes	6

Figure 2	Audible Bird Deterrent installed on pontoon	7

Figure 3	Examples of Bird Rollers	7

Figure 4	Examples of Installation of cones on flat lantern surfaces	8

Figure 5	Examples of Electrical Deterrents	9

# Bird Deterrents

# SCOPE

The scope of this document is to provide information on the problem of bird fouling on AtoN, by focusing on identification of the detrimental effects of bird fouling and the possible use of effective bird deterrents.

# AIM

The aim of this guideline is;

To provide potential solutions to bird fouling.

To provide details of the detrimental effects of bird fouling on AtoN structures, power supplies and lighting systems and its overall impact on AtoN reliability.

To provide details on Occupational Health & Safety Concerns related to working in areas of excessive bird fouling.

To provide details of possible methods of bird deterrence and other remedial measures used to prevent bird fouling.

# Introduction

Bird fouling can be defined as the pollution or contamination of an area by bird droppings. In the context of this guideline it refers to the pollution or contamination of AtoN sites or associated structures. Bird fouling of an AtoN site has detrimental effects on the AtoN’s availability and reliability, on workplace health and safety and in general results in accelerated structural deterioration.

A bird deterrent can be defined as a device or object that deters a bird from landing on, or using an AtoN or AtoN structure for any purpose. The deterrence of a bird from an AtoN will effectively remove the issues associated with bird droppings.

Bird deterrents can be used in a number of different scenarios such as helipads, lanterns / AtoNs and associated structures, day marks, solar panels, buoys, facilities, other marine structures, fittings and components.

# Negative effects of BIRD Fouling.

## From an Availability and Structural Perspective

Bird fouling comes primarily from sea-birds landing and roosting, or attempting to land and roost, on an AtoN site. The discharge of faecal matter is what causes the bird fouling, but it can also be related to shedding of feathers, nesting debris and presence of rotting food. Bird fouling can have, among others, the following detrimental effects on AtoN sites;

Excessive birdlime coverage on lanterns or optics, causing obstruction of the light source, resulting in reduced nautical range or in severe cases, total outage of the AtoN.

Excessive bird lime coverage of solar panels, reducing the active area of the panel and severely limiting battery charging capacity, which can lead to negative effects on night time signalling functions of the lantern and may eventually lead to total battery discharge and subsequent outages of the AtoN.

Birdlime coverage on lighthouses or other daymarks can cause a change in the colour, severely affecting the ability of that AtoN to provide clear information to the user.

- Bird lime is highly caustic and can increase corrosion rates on AtoN structures, fittings and components, resulting in accelerated deterioration and reduced life span, higher maintenance costs and unsafe structures.

- Bird fouling on any site generally pollutes and contaminates, causing a number of associated issues.

## From an Occupation Health & Safety perspective.

Excessive bird fouling causes serious OH&S risks for personnel involved in routine maintenance or inspection. Whilst the OH&S risks are most commonly associated with the odour caused by bird lime build up, other risks are quite varied. A summary of some of the OH&S risks are provided below;

- Exposure to the excessive odour of bird lime build up can cause respiratory and other illnesses. Dried bird lime can become an air pollutant during cleaning or when disturbed.

- Numerous diseases are carried or caused by birds, from both their bodies and their droppings and serious risks arise from disease organisms growing in the nutrient rich accumulation of bird droppings, feathers and debris. Some of these diseases can be passed onto humans.

- Insects or parasites (ticks, mites, fleas, lice etc) that live on birds or their droppings may become a problem with particularly severe infestations. These can be passed onto humans with contact and can cause both minor and serious health issues.

- Bird lime can cause slippery conditions on AtoNs, especially when wet. This therefore increases risks associated with slipping, falling and generally magnifies all other risks normally associated with working at heights.

# Methods for Dealing with Bird Fouling Problems

There are many different methods of deterring birds, including the use of commercially available products and deterrent systems, the application of engineering solutions, structural changes or the revised installation methods.

The exact method should be tailored to suit a particular site, situation and in some cases, may need to be designed to suit the visitation habits of a particular species of bird.

Some examples of bird deterrents are detailed in the following sections.

## Bird Spikes

Bird spikes can be a cheap and effective way of deterring birds from landing on some structures. There are several commercial types of bird spikes available which can be installed on most parts of an AtoN structure, fittings or components.

The effectiveness of bird spikes depends on several factors.

The method of installation is critical and bird spikes must be installed to withstand the effects of wind, waves, bird collision and other environmental factors they may be exposed to in-situ. The level of adhesiveness must be considered to ensure that they remain insitu and do not break off, or are knocked off.

Bird spikes must also be designed and fabricated from durable materials, to withstand harsh environmental conditions.

The schedule of inspection and maintenance will also affect bird spikes as they are not necessarily items designed with a long life span in mind.

Many lantern brands now have permanent or semi permanent ‘in-built’ bird spikes, or components and fittings that allow installation of bird spikes. This can be considered when choosing equipment for a site where exposure to bird fouling may be an issue.

Examples of Bird Spikes

## Audible Bird Deterrents

Audible bird deterrents or bird scarers have been used in many locations, with varying levels of success. Experience shows it is particularly important to consider the species of bird being targeted when considering audible bird deterrents as a viable option.

Audible bird deterrents can include devices which omit bird calls or other noises that cause birds distress. These devices come in a variety of shapes and sizes and can omit noises at regular intervals or even fitted with remote sensors that ensure noises are only omitted when movement is sensed. The power supply requirements and electronic components may make these items unsuitable for remote area installations whereas they may prove to be the preferred choice in areas where power supply is regular and maintenance and inspections can be carried out regularly.

Suppliers of commercial bird deterrent systems can be sourced quite easily through internet searches and often provide services to identify the right type of product needed for a specific bird related problem.

Audible Bird Deterrent installed on pontoon

## Visual Bird Deterrents

There is a wide range of options and methods of using visual bird deterrents, and usually involves installation of a device or piece of equipment that is used to scare or deter a bird from landing, rather than physically preventing a landing. These types of devices are sometimes referred to as ‘scarecrows.’ Many interesting variations on this theme have been trialled around the world by AtoN authorities and service providers.

Results of past applications show that certain species of birds may, after some time, tend to habituate and become used to foreign items, thereby negating the usefulness of visual bird deterrents in some circumstances.

## Bird Rollers

Bird rollers are commercially available and have proven to be successful when used in the correct application. Bird rollers are usually installed on hand rails or other areas that may provide purchase or a landing for visiting birds. The rollers provide a moving and constantly rotating surface on which birds cannot land. Again it is important to consider all aspects of the installation, like the schedule of inspections and visiting, the remoteness of the location and the expected bird pressure.

Examples of Bird Rollers

## Variation of Installation of Components and Fittings

In the case of solar panels it is possible to change the method of mounting, i.e. mount them vertical to prevent the bird lime smear; however this requires careful consideration of the effects on power supply and reliability for the AtoN and will usually require additional solar panels to address power loss. In some cases the limited room available on a structure may prevent that option being feasible.

Installing cones on the top of flat lantern surfaces has also proved successful in some applications, as it effectively removes areas for bird purchase.

Depending on the species of bird causing the fouling, another consideration is changing the angle or elevations of possible landings. Practical observations on some types of sea birds, particularly seagulls indicate that they defecate in nearly horizontal bursts, whilst perched on steel buoy guard rails. So installation of a simple elevating platform may prove effective in redirecting the spray of bird faecal matter, thus removing issues with smearing on nearby lanterns or solar panels. This option needs careful consideration of the species of birds causing issues for a particular site and also the location of the set out of the different AtoN components.

Examples of Installation of cones on flat lantern surfaces

## Electrical Deterrents

- Various versions of electrical systems have also been trialled with good success. Electrical fence systems have the possibility of eliminating bird lime issues, providing the correct engineering and design considerations are taken into account. This process has proved especially successful when built into a structure’s design during the initial design phase, as it allows the designer to make considerations in minimizing bird purchases and also ensuring that installation of the electrical system is facilitated both in terms of space and power supply. Most components of these systems are commercially available and utilize the same equipment as standard electrical cattle fences. Full systems, specifically designed for use for bird control are also commercially available on the cattle fence control market.

Examples of Electrical Deterrents

# Environmental Considerations

Whilst the principal reason of a bird deterrent is to keep birds from landing or roosting on an AtoN component or structure, a fine line must be drawn to ensure that the method is not overly harmful to birds, for the simple reason that not all birds visiting the structure are necessarily pest species. It is possible that birds of high ecological value or a protected status may also attempt to use the structures.

A countries environmental or fauna legislations will usually stipulate the regulations to which an authority or service provider needs to act in relation to this subject. There may also be legislation or guidelines that identify bird species of concern.

Competent authorities and responsible service providers should also ensure that this issue is addressed in their environmental management system, so an attempt is made to identify interaction (and its impacts) with any endangered bird species or wildlife in general.