## EEP2 / 10 / 2

## Solar Superstructure for Light Buoys
Note and Question for EEP Committee
From Waterways and Shipping Office Wilhelmshaven

| Federal WaterWays and Shipping Administration Germany Waterways and Shipping Office Wilhelmshaven |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  Waterways and Shipping Office Wilhelmshaven  P.O. Box 2655  D-26366 Wilhelmshaven To the members of the IALA-Committee for Engineering, Environmental and Preservation of Historical Lighthouses – EEP   |  |  |  |  |  |  |
| Your reference and message from | My reference number 3-241.1/14 |  |  | 0049-4421-186-330 oder -186-0 |  | Date Monday, 17 March 2003 |
|  |  |  |  |  | Written by: Stefan Behrens mailto:behrens@whv.wsdnw.de |  |

Development of a Solar-Superstructure for light-buoys

Dear EEP-Committee-Member,

before we will meet again at the IALA headquarter for 2nd Session  I would like to make a request for your experience about a solar-superstructure for light-buoys.

Attached you will find a short description of what the Federal Waterways and Shipping Administration has developed, added with the topics I request for.

If you have any information please send them via Email or bring them to our next meeting in April.

If you have any questions do not hesitate to contact me.

I'm looking forward meeting you in St. German-en-Laye.

With best regards

# Stefan Behrens

Development of a Solarkompaktaufsatz (Solar-Superstructre) for floating Aids

The Waterways and Shipping Administration in Germany has up to now round about 1000 light-buoys in the north- and baltic sea in use. Most of them are ligthed by gas sea lanterns with a gas barrel in the buoy body, the other ones are lighted by electrical sea lanterns with incandescent lamps and battery or photovoltaic energy.

Gas-operated lighted buoys are expensive and a high maintenance is necessary. Gas-lantern and spare-parts are deliverable from only one company and it is not sure that they are available in the future. The next problem is that when a gas barrel is empty it has to be changed before the maintenance of the steel buoy is necessary. Because of safety reasons the changing of the barrel has been carried out on shore. So the complete buoy, even when the steel body has no damage or corrosion, has to be carried back to the workshop to change the gas barrel. This lightning system is not economic any more.

The recent development of lanterns with light emitting diodes - LED - offers here new possibilities. Especially for coloured light the LED’s produce much more light than it is possible with incandescent lamps. In combination with the photovoltaic energy this could be an economic solution for lighted buoys.

It was decided in 2000 to replace the gas-operated with the photovoltaic-operated system with LED-Lanterns until 2003/2004. The Traffic-Technology-Center, a part of German Federal Waterways and Shipping Administration in Koblenz has developed a Solar-Superstructure - SSS - for buoys which contains all parts of the lightning system such as the LED-lantern, photovoltaic panel, charge regulator, batteries and wires. The SSS is made out of not reinforced plastic – Polyethylene. It could be coloured in red, green and yellow. The disadvantage of the SSS is that it is very large and heavy in comparison to the gas-lantern. The weight of the complete SSS is approximately 180 kg. The batteries have a weight of approximately 120 – 150 kg. In 2001 25 SSS were produced for a testing program which still goes on. First results show that the system is working very well and reliable. Less problems occured. Biggest problem this time is the colour fastness of the red SSS. After a short time on station in the sunlight they are more orange or pink. The testing program will end at the end of this year.

Gas-lantern + SCT

The aim for the main service life of the steel buoys is longer than 3 years. During this time the maintenance should be minimal, only the mooring is checked once or twice a year and extensive marine growth may be cleaned of. The SSS containing photovoltaic and LED-Technic should be that reliable that the complete buoys with all its parts could keep on a particular station for more than 3 years. So it would not be necessary in the future to carry the buoy to the workshop because of a problem with the light meanwhile these 3 years.

The new SSS is fixed on the top of the steel buoy with 8 bolts, so it possible to change the SSS on board of the buoy tender/service vessel, but it is not possible to install the SSS when the buoy is floating on station. There are no other connections between the SSS and the buoy and no safety restriction in contrast to gas-operated lighted buoys.

The first SSS has only 4 eyes on top so it can only hang upright on a hook. In practice it has shown that it is not as easy as it should be to install the SSS on top of the buoy on board. It is difficult because the buoy is lying on board and so you can’t set the SSS down on the top. For a better handling on board some improvements were developed I won’t describe here.

Now the idea occurs to improve the SSS that way, that you can put down the SSS on top of the buoy when it floats on station and it secure itself automatically. An advantage could be that the changing of the SSS could be made with a smaller vessel with a lower lifting capacity. A company has developed and offered a potential technical solution. One of the SSS was equiped with this so called Quick-Closure-System at the end of last year. So up to now we have very less experience with it and at least some questions are not answered.

- Do we have the necessity for such a quick-closure-system, when we suggest that the SSS could leave on station more than 3 years?

- Is the quick-closure-system suitable and effective for the SSS which has to be on station in a rough environment (salty water, beating of the weaves, corrosion, ice)

- An SSS equiped with quick-closure-system is more expensive. Could this be more economic? Do we save costs because we can use smaller vessel to change the SSS?

The Federal Waterways and Shipping Administration has spend 2 years of development on the SSS and we want to finish it soon to replace the gas-operated lanterns as fast as we can. At this time the second question is very important for us and I would like to ask you if you have any experience with such a SSS and especially with a kind of a quick-closure-system.

If you have any Information and experience about similar systems in your administration or company it would be very kind if you could send them to me or bring them with you to our next IALA EEP2 meeting in April in St. German-en-Laye.

Stefan Behrens; SBL3  Wilhelmshaven, 29.7.26