| From: AIS Committee | AIS12/output/02 |
| --- | --- |
| To: VTS18, RNAV18, EEP2, ANM2 | Date: 07-03-2003 |

Liaison Note

Preliminary Draft Recommendation – AIS for Aids to Navigation

- Introduction

Intersessional work of the Technical Working Group of the IALA AIS Committee has resulted in a preliminary draft ‘Recommendation on AIS for AtoNs’ (see attached).

Although this document is not yet finalized, some parts of it are considered mature enough to be submitted to other IALA Committees for comments; in particular the contents of chapters 1-7.  Chapter 8 to the end of the document is not considered mature, and will need further drafting.

- Action Requested

The VTS, RNAV, ANM & EEP Committees are invited to provide comments on the preliminary draft so that the draft Recommendation can be finalised at AIS13 in September 2003.

The IALA Secretariat is asked to forward comments on the draft Recommendation from the VTS18, RNAV18, EEP2 and ANM2 to the Chairman of the AIS Technical Working Group of the AIS Committee (JOltmann@wsd-nord.de) by 30-06-03.

PANAMA - WORKING PAPER 03

14 February 2003 version 2

Preliminary DRAFT 0B

IALA RECOMMENDATION

ON

AIS FOR AIDS TO NAVIGATION

Working Towards Edition 1.0

14-02-2003

Table of Contents

1.	Application of AIS to aids to navigation	4

2.	Implementation methods	4

2.1.	AtoN AIS	4

2.2.	Synthetic AIS	4

2.3.	Virtual AtoN AIS	5

2.4.	MMSI numbers for Synthetic and Virtual AtoN AIS	5

3.	AIS Messages for Use with Aids to Navigation	5

3.1.	Message 21, Aids to Navigation Report	5

3.2.	Message 14, Safety Related Broadcast Message	8

3.3.	Message 8, Binary Broadcast Message	8

3.4.	Message 6, Addressed Binary Message	8

4.	AIS on Floating Aids to Navigation	8

5.	AIS on Fixed Aids to Navigation	9

6.	AIS on Offshore structures	9

7.	Radar Reference Target	9

8.	The AtoN AIS station	9

8.1.	Achieving the functionality of an AtoN AIS station	9

8.2.	Autonomous, continuous system	10

8.3.	Autonomous, non-continuous system	11

8.4.	Assigned time slot system	12

8.5.	General requirements for receivers and transmitters in an AtoN AIS station	13

8.6.	Configuration of AtoN AIS units	13

8.7.	Built-in integrity test	14

8.8.	Functional definition of the radio interface of the AtoN AIS unit	14

8.8.1.	General requirements of the physical layer	14

8.9.	Requirements and Recommendations for the TDMA Receivers of the AtoN AIS Station	15

8.9.1.	Message slot assignment	15

8.10.	Additional technical points	16

8.10.1.	Power consumption	16

8.10.2.	Antennas	16

8.10.3.	VHF range	16

8.10.4.	Monitoring	16

8.10.5.	Other considerations	16

9.	References	16

Annex, Message 21	17

#### Application of AIS to aids to navigation

The application of AIS to aids to navigation is a significant use of the AIS technology, and ITU has defined AIS Message 21 for exclusive use with aids to navigation. The primary purpose of applying AIS to aids to navigation is to promote and enhance navigation safety and efficiency by one or more of the following:

- Providing a positive and all-weather means of identifying an aid to navigation on AIS and ships’ radar displays

- Complementing existing signals from aids to navigation

- Transmitting accurate positions of floating aids (possibly corrected by DGNSS)

- Indicating if a floating aid to navigation is off station

- Providing reference points for a ship’s radar

- A complement to, or possible replacement of, racons

- Providing virtual aids to navigation

- Marking or delineating Tracks, Routes, Areas, and Limits (for example Areas To Be Avoided and Traffic Separation Schemes)

- Marking offshore structures (for example wind energy sites, oil or gas platforms)

- Providing weather, tidal, and sea state data.

A further set of benefits includes the following:

- Monitoring of aids to navigation status

- Tracking of a drifting floating aid

- Identifying ships involved in collisions with aids to navigation

- Gathering real-time information for condition monitoring

- Remotely controlling changes in AtoN parameters.

AIS for aids to navigation may be applied to both floating and fixed aids to navigation, and more than one AIS message format may be transmitted. The service provider has an obligation to verify the broadcast information and the correct operation of the AIS for AtoN.

An authority planning the application of AIS to AtoN, should bear in mind the message capacity of the VDL, and avoid unnecessary load, for example by considering selection of data to be disseminated

#### Implementation methods

AIS messages for an aid to navigation may be generated from information derived from the aid itself, and broadcast directly from the aid, or may be broadcast from an AIS unit not located at the AtoN.

These should be referred to by the following terms.

- AtoN AIS

  - Where the aid is equipped with an AIS unit designed to generate the appropriate AIS messages using local data from the aid

- Synthetic AIS

  - Where data from the AtoN is transferred to another location from where the AIS messages relating to the aid are sent

- Virtual AtoN AIS

  - Where the AIS message is an aids to navigation message but no real aid exists at the location indicated in the AIS message.

##### AtoN AIS

AtoN AIS, where an AIS unit is located at the AtoN, is described in detail later.

##### Synthetic AIS

For practical or economic reasons it may not be appropriate to fit an AIS unit to an AtoN and the Synthetic AIS approach may be taken. In Synthetic AIS, data from an AtoN is transmitted via a non-AIS data link. The data is received at a nearby base station or another AtoN AIS unit where it is converted into AIS format and broadcast as if it came from an AIS unit on the AtoN. The broadcast AIS data should only be based on regularly validated AtoN data.

##### Virtual AtoN AIS

It may be appropriate to create a virtual AtoN on a display, where none exists. In this case the symbol would appear on the display for a certain location, even though there is no AtoN there.  This message could be broadcast by a nearby base station or AtoN AIS station. The flag in Message 21 would clearly identify this as a Virtual AtoN. There are some cases where Virtual AtoNs could be useful, for example marking hazards to navigation on a temporary basis, until permanent aids can be established.

As with all AtoN AIS, a virtual AIS should only be established, or approved, by the competent authority for that area, taking account the needs of non-AIS-equipped craft.

##### MMSI numbers for Synthetic and Virtual AtoN AIS

Message 21 must include an MMSI number. For AtoN AIS, this number will be the MMSI number of the AtoN AIS unit. However, for Synthetic or Virtual AtoN AIS the question arises of whether the Message 21 MMSI number should be the MMSI number of the AIS station, such as a base station, which broadcasts the Message 21, or should be another MMSI number.

If the base station MMSI number is used, this might assist mobile stations to identify the source of the AtoN data carried in Message 21. However having the same MMSI number for a base station and a Synthetic or Virtual AtoN AIS might cause unforeseen problems for the data processing in mobile AIS units.

[This question needs further consideration before it can be resolved.

Input from mobile AIS unit manufacturers would be helpful.]

#### AIS Messages for Use with Aids to Navigation

The following AIS messages, as defined by ITU, Technical Characteristics for a Universal Automatic Identification System Using Time Division Multiple Access in the VHF Maritime Mobile Band, ITU-R M.1371-1

(Reference 1) may be applied to AIS for aids to navigation.

- Message 21, Aids To Navigation Message

  - Sent for use by vessels within range and by VTS centres

- Message 14, Safety Related Text Message

  - May be sent if a buoy moves off station or if the aid malfunctions in a manner as to cause its signal[s] to be in error

  - Note that this message could be sent automatically from an AIS unit on the buoy, or could be sent from a shore base station based on information received from the buoy. The latter system might be employed if human intervention prior to sending Message 14 is required; otherwise it is likely to be more reliable for the buoy AIS unit to send it.

- Message 8, Binary broadcast message, as approved for international, or regional use, for example

  - Weather, wave, tide, sea data.

  - Tracks, Routes, Areas, and Limits (for example Areas To Be Avoided and Traffic Separation Schemes)

- Message 6, Binary addressed message

  - For example, local use for AtoN monitoring data

#### Message 21, Aids to Navigation Report

Reference 1 paragraph 3.3.8.2.17 states that “This message should be transmitted autonomously at a reporting rate of once every three (3) minutes or it may be assigned by an Assigned Mode Command (Message 16) via the VHF data link, or by an external command.”

The default interval between transmissions of Message 21 is 3 minutes, but this may be increased or decreased at the discretion of the service provider, bearing in mind the capacity of the VDL and perhaps power consumption at the AtoN.

The main contents of Message 21, which is defined elsewhere, are:

- Type of aid to navigation;

- Name of the aid to navigation;

- Position;

- Position accuracy indicator;

- RAIM indicator;

- Type of position fixing device;

- Time stamp;

- Dimension of the aid to navigation and reference positions;

- 8 bits reserved for use by the regional/local authority (can include the technical status of the aid to navigation);

- Virtual aid to navigation target flag.

The Details of the composition of Message 21 are defined by Reference 1, and clarified by “IALA Technical Clarifications on Recommendation ITU-R M.1371-1” (Reference 2). Refer to Annex.

    - Message 21, Configuration

- The position derived from an EPFS (Electronic Position Fixing System) can be used in conjunction with a reference position and a ‘guard ring’ to monitor the position of the floating aids to navigation and to generate an ‘off station’ condition. This would be indicated I the AtoN AIS message and also be transmitted as a navigational warning message in the form of a Safety Related text message.

- When assigning the “Name of AtoN” field, the following priority should be used:

  - Charted name

  - National or international identification number

  - Description of special characteristics. (This content is up to the service provider, but examples are light flash character, range in nautical miles, elevation in metres.)

If more than one of a, b, c are used, they should be separated by comma(s).

- All positions must be given in WGS84 coordinates.

- The “Dimensions of AtoN field”

    - This field should indicate the dimensions of the AtoN object and not the dimensions of the area in which a floating aid can move (guard zone) or dimensions of a ”dangerous zone” around the AtoN.

    - For fixed AtoN, and offshore structures the orientation established by the dimension A should face true north.

    - For floating aids larger than 2 m x 2 m the dimensions of the AtoN should always be given as a circle, i.e. the dimensions should always be as follows: A=B=C=D>1. (This is due to the fact that an orientation of the floating aid is not transmitted.)

    - For objects smaller than or equal to 2 m x 2 m the values of the fields should be set to A=B=C=D=1.

    - When transmitting virtual AtoN information, i.e. the virtual Aids to Navigation flag is set to one (1), the dimension should be set to A=B=C=D=0 (=default). This should also be the case, when Type of AtoN is set to ”reference point”.

| Dimensions for a fixed AtoN |  |
| --- | --- |
|  | Distance in meters |
| A | 0 - 511 |
| B | 0 - 511 |
| C | 0 - 63 ; 63 = 63 m or greater |
| D | 0 - 63 ; 63 = 63 m or greater |

| Dimensions for a floating AtoN |  |
| --- | --- |
| A+B ≤ 2m | A=B=C=D=1 |
| A+B > 2m | A=B=C=D>1 |

    - Message 21, Data bits reserved for regional or local application

The eight data bits “Reserved for regional or local application” are defined at the regional or local level. If these bits are used for AtoN system monitoring, they could be used to address the following parameters in the manner indicated.

Table 1

| Bit number | Meaning |
| --- | --- |
| 1&2 | 00 = Lantern off (Day – normal) 01 = Lantern on (Night – normal) 10 = Lantern off (Failed) 11 = Lantern on (Faulty/reduced range/wrong character) Describes data content of Bit 2. 0 = Bit 2 indicates light condition day/night. 1 = Bit 2 indicates light condition normal/abnormal |
| 3 | 0=battery voltage OK, 1 = low battery voltage |
| 4 | 0=racon OK, 1=racon alarm |
| 5 |  |
| 6 |  |
| 7 |  |
| 8 |  |

#### Message 14, Safety Related Broadcast Message

An AtoN AIS station, which transmits Message 21, could also transmit at the competent authority's discretion Safety Related Broadcast Message (Message 14, text message), for example, on detecting that a floating Aid-to-Navigation has gone out of position or is malfunctioning.

#### Message 8, Binary Broadcast Message

An AtoN AIS could broadcast a Message 8, Broadcast binary message, carrying information about conditions in the vicinity of the AtoN.

Application specific messages intended for general use and with world-wide application should be transmitted in messages in the international branch of the application specific messages.  An example of this type of information is meteorological and hydrological information.

Messages intended for general use but in a limited area can preferably be transmitted in messages in the regional branch of the application specific messages.  An example of this is the ‘lock order message’ used by the St Lawrence Seaway. The interpretation of these messages must then be implemented in the display systems used by the ship in the region.

#### Message 6, Addressed Binary Message

An AtoN AIS station may use Message 6 to carry information for AtoN monitoring purposes, addressed to the AtoN service provider. An acknowledgement re-transmission procedure following transmission of Message 6 is optional in this case.

#### AIS on Floating Aids to Navigation

The position derived from an internal EPFS (Electronic Position Fixing System) can be used together with a reference position and a ‘guard zone’ to monitor the position of a buoy and to generate an ‘off station’ condition. This would be indicated in the AtoN AIS message 21 by setting the off-station flag.

IALA defines an Aid-to-Navigation as: "a device or system external to vessels designed and operated to enhance safe and efficient navigation of vessels and/or vessel traffic." The IALA Navguide stipulates: "A floating aid to navigation, which is out of position, adrift or during the night is unlighted, may itself become a danger to navigation. When a floating aid is out of position or malfunctioning, navigational warnings must be given." Therefore, a station, which transmits Message 21, could also transmit Safety Related Broadcast Message (Message 14) upon detecting that the floating Aid-to-Navigation has gone out of position or is malfunctioning, at the competent authority's discretion.

#### AIS on Fixed Aids to Navigation

AIS on a fixed Aid to Navigation such as a lighthouse could

- Broadcast its own messages, and optional local hydrological and meteorological data

- Forward messages from other AIS-equipped aids to navigation for monitoring purposes

- Broadcast Synthetic AIS or Virtual AtoN messages.

#### AIS on Offshore structures

The position of offshore structures is very important for safety and navigational purposes because they are seen as hazardous objects. Fixed offshore structures include offshore wind energy sites and floating offshore structures e.g. drilling rigs. Message 21, ‘Aids-to-Navigation Report’ may be the most appropriate message for reporting position and dimensions of an offshore structure when it is fitted with an AIS station. Other messages, such as Message 8, indicating, for example, an area to be avoided, could be broadcast.

#### Radar Reference Target

With the ever-increasing requirement for integrated display systems, the problems of aligning two or more systems on one display screen are increased. If two (preferably three) AIS stations can be installed on prominent, fixed radar targets within an area of special interest, such as a harbour or harbour approach, the AIS EPFS positions, radar echoes and chart symbols of each of these targets can be used to align the three displays. This can result in reduced ambiguity and a less cluttered display. AIS for aids to navigation can be a suitable technique for this purpose.

#### The AtoN AIS station

The AtoN AIS station needs to perform in accordance with the Reference 1 and Reference 2 (latest edition) in order to maintain harmony with the AIS system requirements.

The primary considerations for selecting the functional design of an AtoN AIS unit are power consumption and complexity, the latter being a direct influence on unit cost.

Most floating AtoN sites and many fixed sites have limited power availability due to physical constraints, and/or have a high cost of energy. An AtoN AIS unit derived directly from a ship AIS unit will likely have a power requirement beyond the practical or economic power available at the intended location site.

Besides the cost of energy, the cost of the AtoN unit itself is an important consideration for the service provider.

Other factors which influence the system selected to achieve AtoN AIS functionality are the AtoN type (where fixed AtoNs may for example require more functionality than smaller floating AtoNs), MTBF considerations, and the effect on the VDL.

##### Achieving the functionality of an AtoN AIS station

Three of the ways in which the AtoN AIS station functionality can be achieved are as follows.

- Autonomous, continuous system

- Autonomous non- continuous system

- Assigned time slot system.

##### Autonomous, continuous system

    - Functional block diagram

This system is realised in functional form by the method shown in Figure 1.

Figure 1: Functional block diagram of an autonomous continuous AtoN AIS station

Power Supply

BIIT

AIS RX

AIS RX

AIS TX

Autonomous continuous

AtoN Interface

DSC

Configuration

EPFD

Controller

Internal Sync Source

    - Description of functional elements

The functional elements of the autonomous continuous AtoN are required (compulsory) in the minimum configuration of an autonomous continuous AtoN :

- Receiver: The minimum configuration may consist of one receiver for one AIS channel operation.

- TDMA transmitter

- Controlling unit: The controller unit is the interface to the AtoN and allows for input and supervision of the AtoN parameters such as battery voltage, light status etc.
The controller unit also checks the AtoN position against the preset position and allowed swinging radius. The controller unit composes the output messages to the VDL.

- Internal sync source: The internal sync. source may be included in the EPFS

- EPFS: Electronic Position Fixing Source to determine the AtoN position.

- Built-In-Integrity-Test unit (BIIT)

- Power supply

The following functional elements are optional:

- Additional receiver(s).

- DSC functionality

    - Characteristics

The characteristics of this system are:

- Full functionality and fully compliant to ITU-R-M1371-1.

- Ability to work independently using RATDMA access scheme.

- Can utilize  FATDMA slot allocations(reserved by Base station)

- Includes input and output interfaces for transmitting AtoN messages and control/configuration of the aid

    - Applications

- Critical AtoN, no power limitation, good communications

- High capability/flexibility, range of messages

- Monitoring capability,

- Typically an onshore lighthouse with mains power, landline

##### Autonomous, non-continuous system

    - Functional block diagram

This system is realised in functional form by the method shown in Figure 2.

Figure 2: Functional block diagram of an autonomous non-continuous AIS base station

Controller

Power Supply

BIIT

AIS RX

AIS RX

AIS TX

Autonomous non-continuous

AtoN Interface

Internal Sync Source

DSC

Configuration

EPFD

Timing

device

    - Description of functional elements

The functional elements of the autonomous continuous AtoN are required (compulsory) in the minimum configuration of an autonomous continuous AtoN :

- Receiver: The minimum configuration may consist of one receiver for one AIS channel operation.

- TDMA transmitter

- Controlling unit: The controller unit is the interface to the AtoN and allows for input and supervision of the AtoN parameters such as battery voltage, light status etc.
The controller unit also checks the AtoN position against the preset position and allowed swinging radius. The controller unit composes the output messages to the VDL.

- Timing device: The timing device is responsible of scheduling the active/idle operation periods according to the setting made by configuration.

- Internal sync source: The internal sync. source may be included in the EPFS

- EPFS: Electronic Position Fixing Source to determine the AtoN position.

- Built-In-Integrity-Test unit (BIIT)

- Power supply

The following functional elements are optional:

- Additional receiver(s).

- DSC functionality

    - Characteristics

The characteristics of this system are:

- Ability to work independently using RATDMA access scheme.

- Can utilize  FATDMA slot allocations(reserved by Base station)

- Include input and output interfaces for transmitting AtoN messages and control/configuration of the aid.

- Ability to be non-continuous e.g. “sleep mode capabilities”

- Limited assignment capabilities

- Limited interrogation capabilities

- Allows for intelligent AtoN control “transmit only when there is AIS traffic” e.g. for power saving purposes

    - Applications

- Limited power, limited messages

- Communications limited

- Typically isolated minor lights, buoys outside port area

##### Assigned time slot system

    - Functional block diagram

This system is realised in functional form by the method shown in Figure 3.

Figure 3: Functional block diagram of an AIS base station

Controller

Power Supply

BIIT

AIS TX

Assigned time slot

AtoN Interface

Internal Sync Source

Configuration

EPFD

Timing

device

The functional elements of the autonomous continuous AtoN are required (compulsory) in the minimum configuration of an autonomous continuous AtoN :

    - Description of functional elements

- TDMA transmitter

- Controlling unit: The controller unit is the interface to the AtoN and allows for input and supervision of the AtoN parameters such as battery voltage, light status etc.
The controller unit also checks the AtoN position against the preset position and allowed swinging radius. The controller unit composes the output messages to the VDL.

- Timing device: The timing device is responsible of scheduling the active/idle operation periods according to the setting made by configuration.

- Internal sync source: The internal sync. source may be included in the EPFS

- EPFS: Electronic Position Fixing Source to determine the AtoN position.

- Built-In-Integrity-Test unit (BIIT)

- Power supply

    - Characteristics

The characteristics of this system are:

- Limited functionality

- No receive capability e.g. Can not be assigned via the VDL, dependent on FATDMA slot allocation from base station in the vicinity

- Include input interfaces for transmitting AtoN messages

- No interrogation capabilities

    - Applications

- Significantly reduced power requirement, cost & complexity

- Restricted application (need base station, dependent on base station for reliable operation)

- Limited message capability, cannot be controlled via VDL

- High density AtoN coverage

- Typically buoys in port area

##### General requirements for receivers and transmitters in an AtoN AIS station

Since the AtoN AIS is stationary, it need only be capable of operating on the set of channels as defined by local requirements.

The AIS frequencies used by an AtoN AIS unit may be a subset of the frequency range defined for AIS.

##### Configuration of AtoN AIS units

An AtoN AIS station should have an interface providing the means for configuration of

- Messages

- Reporting interval(s)

- Static data, e.g.

  - MMSI number

  - AtoN name

  - Type of AtoN

  - AtoN dimensions

For units used on floating aids, the nominal aid position in latitude and longitude must be entered as well as the radius of the acceptable position circle, outside which the off-station flag in Message 21 should be raised.

Where a Message 14 is to be sent automatically from the AtoN, the service provider must ensure that the Message is only sent when the relevant condition has been met [e.g. buoy off station].

The configuration method should be such that un-authorised or accidental re-configuration is prevented.

##### Built-in integrity test

The unit should have a built-in integrity test to detect failures that can disrupt the operation of the VDL, and should cease transmission if such a failure is detected. Table 2 applies.

The configuration interface, or another interface, should allow access to the unit for testing.

Table 2    Integrity alarm conditions

| Alarms description text | Reaction of the system |
| --- | --- |
| AIS: Tx malfunction | Stop transmission |
| AIS: Antenna VSWR exceeds limit | Continue operation |
| AIS: Rx channel 1 malfunction | Stop transmission on affected channel |
| AIS: Rx channel 2 malfunction | Stop transmission on affected channel |
| AIS: Rx channel 70 malfunction | Stop transmission on affected channel |
| AIS: general failure | Stop transmission |
| AIS: UTC clock lost | Continue operation using indirect or semaphore synchronisation |

#### Functional definition of the radio interface of the AtoN AIS unit

##### General requirements of the physical layer

Table 3 is derived in part from References 1 and 2. It indicates how any given paragraph of Recommendation ITU-R M.1371-1 (together with appropriate IALA Technical Clarifications), which is indicated by the paragraph number, applies to the AtoN AIS station. The meaning of the column indications is:

- 'C' compulsory;

- 'N' not allowed;

- 'R' recommended;

- 'O' optional.

[Table 3. Mapping of Functionality of Recommendation ITU-R M.1371-1 (together with appropriate Technical Clarifications) to AIS AtoN station

| Annex/ Para of M.1371-1 | Short description | AtoN | station |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | C | N |  | R |  | O |  |
| A2, 2 | Physical layer | X |  |  |  |  |  |  |
| A2, 2.1.1 A2, 2.1.2 A2, 2.1.3 | Physical layer, Parameters, General, Constants and Bandwidth except Table 2 PH.TXP | X |  |  |  |  |  | Full frequency range may not be required |
| A2, 2.1.1 | Table 2 PH.TXP High setting of transmit output power | X |  |  |  |  |  |  |
|  | Table 2 PH.TXP Low setting of transmit output power | X |  |  |  |  |  |  |
| A2, 2.1.4 | Transmission media | X |  |  |  |  |  |  |
| A2, 2.1.5 | Dual channel operation, 2 receivers |  |  |  |  |  | X |  |
|  | Transmission on 2 frequencies |  |  |  |  |  | X |  |
| A2, 2.2 | 25 kHz or 12.5 kHz setting | x |  |  |  |  |  | 25 kHz or 12.5 kHz may be used depending on local conditions |
| A2, 2.3 | Transceiver characteristics | x |  |  |  |  |  |  |
| A2, 2.4 @ | Modulation scheme | x |  |  |  |  |  |  |
| A2, 2.5 | Data transmission bit rate | x |  |  |  |  |  |  |
| A2, 2.6 | Training sequence | x |  |  |  |  |  |  |
| A2, 2.7 | Data encoding | x |  |  |  |  |  |  |
| A2, 2.8 | Forward error correction | x |  |  |  |  |  |  |
| A2, 2.9 | Interleaving | x |  |  |  |  |  |  |
| A2, 2.10 | Bit scrambling | x |  |  |  |  |  |  |
| A2, 2.11 | Data link sensing |  |  |  |  |  | X | Optional if a base station is reserving slots for an AtoN AIS station |
| A2, 2.12 @ | Transmitter settling time | x |  |  |  |  |  |  |
| A2, 2.13@ | Transmitter power high or low |  |  |  |  |  | X |  |
| A2, 2.14 @ | Shutdown procedure within 1.0 sec | x |  |  |  |  |  | Refer to section on shutdown procedure below |
| A2, 2.15 | Safety precautions | x |  |  |  |  |  |  |

]

##### Requirements and Recommendations for the TDMA Receivers of the AtoN AIS Station

The technical characteristics as specified in Table 4 should apply to the TDMA receivers. The requirements indicated by (*) are compulsory for all AtoN AIS Stations, while all other parameters are recommended as a minimum. They may be subject to further regional requirements.

Table 4: Required and recommended receiver characteristics

| Message slot assignment Receiver Parameters | 25kHz channels | 12.5kHz channels |
| --- | --- | --- |
| Sensitivity (*) | 20% PER for – 107 dBm | 20% PER for – 104 dBm |
| Co-channel rejection | -10 – 0 dB | -18 – 0 dB |
| Adjacent channel selectivity | 70 dB | 50 db |
| Spurious response rejection | 70 dB | N/A |
| Intermodulation response rejection and Blocking | 20% PER | N/A |

[An AIS AtoN unit typically operates in either RATDMA or FATDMA  mode. A unit operating in xxx mode can be assigned slot(s) by a base station, and in that case, one of the fields in Message 21 indicates in which mode the unit is operating.

No of slots per message

Complete after AIS12.]

#### Additional technical points

The following points are for the guidance of service providers.

##### Power consumption

At many AtoN sites, the provision of electrical power is expensive. The addition of an AtoN AIS unit will increase the power demand, and the competent authority should make or obtain appropriate computations of AIS system power needs. Choice of interval between AIS messages will affect power consumption of the AIS system, and so a balance is needed between frequent reports and power consumption.

##### Antennas

The AIS for AtoN unit will normally require two antennas, a position-fixing antenna and a VHF antenna for the AIS messages. Antennas are available where these two antennae are combined in a single assembly. This latter configuration may be a better choice for floating AtoN, where installation of antennae is not usually a simple choice.

##### VHF range

AIS signals obey the laws of VHF propagation; the height of the VHF antenna on the AtoN and the height of the ship antenna, and the geography of the transmission path, will determine the range achieved.

##### Monitoring

The service provider should consider whether the AtoN AIS needs to be monitored, and facilities may be provided for remote monitoring if required.

##### Other considerations

The following items should be considered when a service provider is planning to install on-AtoN AIS units.

- Supply voltage range

- Environmental protection

  - Ambient temperature

  - Humidity

  - Vibration

  - EMC

  - etc

- Input and output connections

- Installation and commissioning procedures

#### References

Reference 1

ITU, Technical Characteristics for a Universal Automatic Identification System Using Time Division Multiple Access in the VHF Maritime Mobile Band, ITU-R M.1371-1

Reference 2

IALA, Technical Clarifications on Recommendations ITU-R M.1371-1, Edition 1.3 or later. (The most recent edition should be used.)

#### Annex, Message 21

The following text and tables are taken from “IALA Technical Clarifications on Recommendation ITU-R M.1371-1”, edition 1.3. The reader should ensure that he uses the latest edition of this document.

This message should be used by an A-to-N AIS station. This station may be mounted on an Aid-to-Navigation or this message may be transmitted by a fixed station when the functionality of an A-to-N station is integrated into the fixed station. This message should be transmitted autonomously at a Reporting Rate of once every three (3) minutes or it may be assigned by an Assigned Mode Command (Message 16) via the VHF data link, or by an external command. This message should not occupy more than two slots.

TABLE 34

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message 21 |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. Refer to § 4.6.1; 0 - 3; default = 0; 3 = do not repeat any more. |
| ID | 30 | MMSI number |
| Type of Aids-to-Navigation | 5 | 0 = not available = default; 1 – 15 = Fixed Aid-to-Navigation; 16 - 31 = Floating Aid-to-Navigation; refer to appropriate definition set up by IALA; refer to Table 34bis. |
| Name of Aids-to-Navigation | 120 | Maximum 20 characters 6 bit ASCII, "@@@@@@@@@@@@@@@@@@@@" = not available = default. The name of the Aid-to-Navigation may be extended by the parameter "Name of Aid-to-Navigation Extension" below. |
| Position accuracy | 1 | 1  high ( 10 m; Differential Mode of e.g. DGNSS receiver) 0  low ( 10 m; Autonomous Mode of e.g. GNSS receiver or of other Electronic Position Fixing Device) ; Default = 0 |
| Longitude | 28 | Longitude in 1/10 000 min of position of Aids-to-Navigation (180 degrees, East  positive, West  negative. 181 degrees (6791AC0 hex) = not available = default) |
| Latitude | 27 | Latitude in 1/10 000 min of an Aids-to-Navigation (90 degrees, North  positive, South  negative, 91 degrees (3412140 hex) = not available = default) |
| Dimension/Reference for Position | 30 | Reference point for reported position; also indicates the dimension of an Aid-to-Navigation in metres (see Fig. 18 and § 3.3.8.2.3.3), if relevant. (1) |
| Type of Electronic Position Fixing Device | 4 | 0 = Undefined (default); 1 = GPS, 2 = GLONASS, 3 = Combined GPS/GLONASS, 4 = Loran-C, 5 = Chayka, 6 = Integrated Navigation System, 7 = surveyed.For fixed AtoNs and virtual/pseudo AtoNs, the surveyed position should be used. The accurate position enhances its function as a radar reference target. 8 – 15 = not used. |
| Time Stamp | 6 | UTC second when the report was generated by the EPFS (0 –59, or 60 if time stamp is not available, which should also be the default value, or 61 if positioning system is in manual input mode, or 62 if Electronic Position Fixing System operates in estimated (dead reckoning) mode, or 63 if the positioning system is inoperative) |
| Off-Position Indicator | 1 | For floating Aids-to-Navigation, only: 0 = on position; 1 = off position; NOTE – This flag should only be considered valid by receiving station, if the Aid-to-Navigation is a floating aid, and if Time Stamp is equal to or below 59. For floating AtoN the guard zone parameters should be set on installation. |
| Reserved for regional or local application | 8 | Reserved for definition by a competent regional or local authority. Should be set to zero, if not used for any regional or local application. Regional applications should not use zero. |
| RAIM-Flag | 1 | RAIM (Receiver Autonomous Integrity Monitoring) flag of Electronic Position Fixing Device; 0 = RAIM not in use = default; 1 = RAIM in use) |
| Virtual/pseudo AtoN Flag | 1 | 0 = default = real A to N at indicated position; 1 = virtual/pseudo AtoN, does not physically exist, may only be transmitted from an AIS station nearby under the direction of a competent authority. (2) |
| Assigned Mode Flag | 1 | 0 = Station operating in autonomous and continuous mode =default 1 = Station operating in assigned mode |
| Spare | 3 1 | Spare. Not used. Should be set to zero. |
| Name of Aid-to-Navigation Extension | 0, 6, 12, 18, 24, 30, 36, ... 84 | This parameter of up to 14 additional 6-bit-ASCII characters for a 2-slot message may be combined with the parameter "Name of Aid-to-Navigation" at the end of that parameter, when more than 20 characters are needed for the Name of the Aid-to-Navigation. This parameter should be omitted when no more than 20 characters for the name of the A-to-N are needed in total. Only the required number of characters should be transmitted, i. e. no @-character should be used. |
| Spare | 0, 2, 4, or 6 | Spare. Used only when parameter "Name of Aid-to-Navigation Extension" is used. Should be set to zero. The number of spare bits should be adjusted in order to observe byte boundaries. |
| Number of bits | 272 – 360 | Occupies two slots. |

Footnotes:

(1) When using 3.3.8.2.3.3 for Aids-to-Navigation the following should be observed:

For fixed Aids-to-Navigation, virtual and pseudo A-to-Ns, and for off-shore structures, the orientation established by the dimension A should point to true north.

For floating aids larger than 2 m * 2 m the dimensions of the Aids to Navigation should always be given approximated to a square, i.e. the dimensions should always be as follows A=B=C=D≠0. (This is due to the fact, that the orientation of the floating Aid to Navigation is not transmitted. The reference point for reported position is in the center of the square.)

A=B=C=D=1 should indicate objects (fixed or floating) smaller than or equal to 2m * 2m. (The reference point for reported position is in the center of the square.)

(2) When transmitting virtual/pseudo Aids to Navigation information, i.e. the virtual/pseudo Aids to Navigation Target Flag is set to one (1), the dimensions should be set to A=B=C=D=0 (default). This should also be the case, when transmitting "reference point" information (see Table 34bis).

This message should be transmitted immediately after any parameter value was changed.

Note on Aids-to-Navigation within AIS:

The competent international body for Aids-to-Navigation, IALA, defines an Aid-to-Navigation as: "a device or system external to vessels designed and operated to enhance safe and efficient navigation of vessels and/or vessel traffic." (IALA Navguide, Edition 1997, Chapter 7).

The IALA Navguide stipulates: "A floating aid to navigation, which is out of position, adrift or during the night is unlighted, may itself become a danger to navigation. When a floating aid is out of position or malfunctioning, navigational warnings must be given." Therefore, a station, which transmits Message 23 21, could also transmit Safety Related Broadcast Message (Message 14) upon detecting that the floating Aid-to-Navigation has gone out of position or is malfunctioning, at the competent authority's discretion.

Table 34bis  Type of A to N

The nature and type of AtoN can be indicated with 32 different codes, as shown below

|  | Code | Definition |
| --- | --- | --- |
|  | 0 | Default, Type of A to N not specified |
|  | 1 | Reference point |
|  | 2 | RACON |
|  | 3 | Structure off shore, such as oil platforms, wind farms, rigs. (Note: This code should identify an obstruction that is fitted with an Aid-to-Navigation AIS station.) |
|  | 4 | Spare |
| Fixed A to N | 5 | Light, without sectors |
|  | 6 | Light, with sectors |
|  | 7 | Leading Light Front |
|  | 8 | Leading Light Rear |
|  | 9 | Beacon, Cardinal N |
|  | 10 | Beacon, Cardinal E |
|  | 11 | Beacon, Cardinal S |
|  | 12 | Beacon, Cardinal W |
|  | 13 | Beacon, Port hand |
|  | 14 | Beacon, Starboard hand |
|  | 15 | Beacon, Preferred Channel port hand |
|  | 16 | Beacon, Preferred Channel starboard hand |
|  | 17 | Beacon, Isolated danger |
|  | 18 | Beacon, Safe water |
|  | 19 | Beacon, Special mark |
| Floating A to N | 20 | Cardinal Mark N |
|  | 21 | Cardinal Mark E |
|  | 22 | Cardinal Mark S |
|  | 23 | Cardinal Mark W |
|  | 24 | Port hand Mark |
|  | 25 | Starboard hand Mark |
|  | 26 | Preferred Channel Port hand |
|  | 27 | Preferred Channel Starboard hand |
|  | 28 | Isolated danger |
|  | 29 | Safe Water |
|  | 30 | Special Mark |
|  | 31 | Light Vessel / LANBY |

Note:

(1) The types of Aids to Navigation listed above are based on the IALA Maritime Buoyage System, where applicable.

(2) There is potential for confusion when deciding whether an aid is lighted or unlighted. Competent authorities may wish to use the regional/local section of the message to indicate this.

Rationale for Clarification:

1.) Rationale for introduction of parameter "Name of A-to-N Extension": IALA recognized that there are many cases, where longer designations for A-to-Ns are required (for details refer to the IALA AIS Guidelines). For a two-slot message, the maximum number of additional characters is 14. The rationale for not allowing more than 2 slots is to optimise the load of the VDL.

2.) Rationale for introduction of parameter "Virtual/pseudo A-to-N flag": In some situations it might be useful to transmit information on a physically non-existing AtoN. A method is required to differentiate between this type of virtual/pseudo AtoN targets from real AtoN objects. It is proposed that one of the spare bits in Message #21 be used as a "virtual/pseudo AtoN flag".

3.) Rationale for introduction of the Type of AtoN table: Recommendation ITU-R M.1371-1 requires so.

4.) Rationale for the introduction of the Assigned mode flag: Since the A-to-N AIS Station may be assigned a different reporting rate, the entering into assigned mode needs to be indicated.