DEVELOPMENT DEPARTMENT INFORMATION SHEET No. 127 Issue 01

(Document No. RD-INF-127-IT-03-01)

CIE/IALA Reportage April 2003

COMMERCIAL-IN-CONFIDENCE

The contents of this document must not be released outside the General Lighthouse Authorities without the authorization of the Research and Development Department.

PREPARED BY:	I. Tutt

Trinity House

East Cowes

1 April 2003

Isle of Wight

PO32 6RE.

# DISTRIBUTION

1 copy	Commissioners of Irish Lights, Engineer-in-Chief

1 copy	Northern Lighthouse Board, Director of Engineering

1 copy	Northern Lighthouse Board, Director of Operations

1 copy	Trinity House, Chief Executive

1 copy	Trinity House, Director of Operations

1 copy	Trinity House, Director of Finance

1 copy	Trinity House, Navigation Manager

1 copy	HoM, Trinity House, Harwich

1 copy	DMM, Trinity House, Harwich

1 copy	DMM, Trinity House, Swansea

1 copy	DMM, Trinity House, Penzance

1 copy	P.M. Hart, Department for Transport

1 copy	Library, East Cowes

CIE/IALA Reportage April 2003

- Introduction

This report includes information on Trinity House and General Lighthouse Authority work related to the CIE. It also includes work carried out in conjunction with the UK National Physical Laboratory and other work that may be of interest to IALA.

- CIE Symposium in Veszprem, Hungary

The International Commission on Illumination (CIE) Divisions 1 and 2 held a symposium in Veszprem, Hungary in August 2002. The main topic of interest was the Spatial and Temporal Aspects of Light and Colour Perception and Measurement.

The symposium brought together a large number of experts in the fields of vision, measurement, signalling and imaging, to discuss issues relating to the perception and measurement of light sources and images whose properties vary with time or space. Presentations covered a wide range of topics including the visibility of a flash of light, the detection of a moving target in road traffic simulation studies and changes in the performance of the human eye as a function of illumination level and age. A paper, “Methods of Analysing Flashing Lights within Trinity House Lighthouse Service” jointly written by Ian Tutt of Trinity House and Teresa Goodman of the UK National Physical Laboratory (NPL-UK), was presented by Teresa Goodman. It concerned the methods used by IALA to evaluate effective intensity and their shortcomings when applied to flashes from beacons with modern light sources. The paper also outlined the prospective work on pulsed light sources due to be carried out by NPL over the next two years.

It is clear that further research is necessary in many of the areas discussed and the lively debates that took place throughout the meeting demonstrated the high level of interest in carrying out this work.

Attached to this report are a copy of the paper presented by Teresa and a copy of a report by Dennis Couzin outlining an alternative method to those already used, “Between Blondel-Rey and Broca-Sulzer”.

- Viewing Trial of Emergency Wreck Buoy Beacons

There is a proposal within Trinity House to use emergency buoys that can be rapidly deployed to mark new wrecks in busy sea areas. These buoys will have both radar beacon (racon) and light beacon aids to navigation fitted. The challenge is to find a light beacon sufficiently conspicuous to enable fast identification of a new wreck without confusion.

To this end, three light beacons were viewed from a distance at night. Results showed that yellow and blue flashing lights used in conjunction gave a distinct signal. Further work is needed to synchronise the two lights to give alternating yellow and blue. An arrangement of reciprocal colours, such as these, appears to significantly reduce the risk of colour confusion. A copy of the Development Information Sheet No. 124 is attached.

- NPL-UK Pulse Light Source Study

Trinity House is assisting in the implementation of a pulsed light source study. This project, sanctioned by the UK Department of Trade and Industry, will look at existing methods and recommendations of the measurement of pulsed light sources. The result of the study is likely to be a ‘Best Practice Guide’ coupled with assistance in maintaining measurement traceability and recommendations on test equipment. The project is due for completion in 2004. Details are given in the CIE ‘Flashing Lights’ paper, section 5.

- ORM Club - LED Cluster FIG

Trinity House has joined a focussed interest group (FIG) run by the Optical Radiation and Measurement (ORM) club at NPL-UK. This group contains a range of experts from different fields including road, rail and marine transport, message board manufacturers and measurement laboratories. Minutes of the first meeting are attached. During the second meeting, held in March 2003, the IALA recommendation E-122 was proposed and accepted as relevant literature for the group to study.

- Documentation

| Project Brief: |  |  | Dev. File Ref. | DEV/9] |
| --- | --- | --- | --- | --- |
| Project Number |  |  | User File Ref. |  |
| Documents on disk |  | /tmp/in/input.doc |  |  |
| Data Directory |  |  |  |  |
| Attached Documents |  | Methods of Analysing Flashing Lights, Between Blondel-Rey and Broca-Sulzer, Dev Info Sheet 124, Minutes of LED Clusters FIG 04.12.02. |  |  |