Subject:	Buoy metallurgy

Agenda Item – Any Other Business

Author - Peter Christmas

Regional General Manager

Middle East Navigation Aids Service

PO Box 66

Manama

Bahrain

Tel:       (+973) 828541

Mob:     (+973) 9604838

E-mail:  peterc@menas.com.bh

Text:

We have some trouble with pitting on buoy bodies. Sometimes, this is so

severe that holes are found right through the steel body. The holes are so

neat that they look, quite seriously, as though someone has taken a

quarter-inch drill to them. They are all below the water-line.

Our buoys are prepared much the same as anyone else's:

1. Grit-blasted to SA 2.5

2. Two coats of Hempadur 15590 two compound epoxy primer

3. Tow coats of Hempadur 45150 two compound high-build epoxy paint

4. One coat of Hempetex chlorinated paint

5. Above the water-line One coat Hempetex enamel colour finish coat

5. Below the water-line Two coats of Hempel Classic Combic anti-fouling

The Gulf latitudes are 24 - 30 North, the water temperature can be anything

up to about 40C and it is quite saline. Barnacles are a problem, some being

very tenacious.

The bodies and towers are steel, but the backing-plates for the solar panels

are aluminium. (Could this be the problem?)

If anyone can shed any light, which would enable us to rectify the problem,

I would be eternally grateful.