# LED CLUSTERS FIG

## WORKING GROUP MEETING

4 December 2002, NPL

Present:	Hugh Barton, Scientifics Ltd 		Chair

Teresa Goodman, NPL		FIG Facilitator, Club Chairperson

Fiona Jones, NPL			ORM Club Manager

David Gibbs, NPL

Laura Crane, NPL

Mark Pleydell, Microsense Systems Ltd

Mike Clark, Bentham Instruments

Mark Johnson, Variable Message Signals Ltd

Malcolm MacGregor, BMAC Ltd

Colin Burton, Westinghouse Rail Systems Ltd

Phil Logan, Marl International Ltd

David Rudge, Westinghouse Rail Systems Ltd

David Thorndycraft, QinetiQ

John Ferguson, Spectroscopic & Analytical Developments Ltd

Alan Puffet, QinetiQ

Apologies:	Malcolm MacGregor, BMAC Ltd

Ian Tutt, Trinity Lighthouse Service

1. Introductions

Each attendee at the meeting introduced themselves, and described their area of work.

2. Discussion of issues relating to the use and measurement of LED clusters and arrays

- What sort of LED clusters are in use or under development, and for what applications?

The chairman had prepared a handout detailing the use of LED clusters in use and under development for rail and road transport, which was used as a basis for discussion. The detailed construction of LED clusters varies significantly, depending on the application, but it was agreed the chairman’s summary gave a good overview of the general types available or being developed for signalling, indication or messaging purposes. It was also mentioned that LEDs are being considered for general illumination purposes, particularly in applications such as corridors on board ships.

- Do any requirements specifications apply, e.g. BS EN 12368, RT/E/S/10062, or are LED clusters accepted on a like-for-like basis against conventional equipment?

The specifications listed by those present were:

- TR2136 (revision C) – highway specification for variable message signs

- European specification 12966, TR2206

- Traffic light specifications

- MOD specifications for arrays for aircraft lighting

- Maritime specifications

- Offshore helideck lighting

- Network of specifications for road and rail

It was mentioned that most specifications are not written specifically for LED clusters, but are intended to cover all technologies. In addition, there seems to be little attempt at harmonisation, whether within Europe or worldwide.

- How do these specifications deal with the required optical radiation quantities?

- Are there any optical radiation quantities which are useful predictors of performance for LED clusters, but are generally not specified?

It was agreed that the FIG members should look more closely at what optical radiation quantities are specified in these documents, in order to see what the industry is expected to comply with.

Items that are required to be quantified include the following (those marked * are useful indicators of performance but generally not specified):

- physical size

- colour – of light emitted from a cluster and from an individual element

- luminous intensity – straight on and at an angle

- luminance – of lit area viewed from close range and then over a defined area

- degradation with ambient sunlight

- phantom effect & swamping – both effects seen on bright sunny days

- uniformity across effective area

- contrast ratio

- contrast (several different definitions are used)

- electrical supply conditions

- luminance – as a function of angle *

- perceived colour in real environment (rather than darkroom) *

- effect of temperature

- effect of distance – road and rail traffic signals are measured at 10 &  25m and sea at hundreds of metres, but in laboratories the available space dictates distance *

- day, night and fog conditions *

- effective intensity for pulsed sources *

- visual effectiveness of a narrow band source compared with that of a broad band source of the same chromaticity *

- human perception, just noticeable differences *

- Are measurement organisations able to measure the required optical radiation quantities, and are these traceable to national measurement scales?

Those present felt that, with care, an adequate level of measurement accuracy can be achieved, but suggested that there is often a lack of appreciation of some of the major sources of measurement error (e.g. spectral mismatch errors for photometers). It was also felt that there are some shortfalls in the traceability chain. For example, some organisations use a transfer standard that is a different sort of device from the measurement artefact. Using a transfer standard of the same type of instrument as that to be measured minimises these errors. Some issues, such as changes in the spectral output during ‘warm-up’, have not yet been considered and cannot easily be measured at present.

It was mentioned that NPL is currently establishing a dedicated system for traceable measurements for LEDs, which should be ready next year. Those present thought this would be a valuable service.

- Are LED manufacturers able to measure the required optical radiation quantities?

There is currently no-one in the UK making LED chips, but there are many companies involved in LED packaging and distribution. The performance of LEDs is tested by manufacturers and distributors on a batch basis, who often sort a production run into a series of ‘bins’ based on colour or luminous intensity. Customers do not always have control of what they are buying, even if they select on the basis of a given ‘performance bin’ (ongoing improvements in LED efficiency mean that over a period of time the luminous intensity of LEDs selected to fall into a given ‘bin’ gradually drifts towards the upper limit of that ‘bin’).

- Is suitable measurement equipment available commercially?

Examples of equipment used include:

- goniometer, detector and input optics to set the angle

- photometer – silicon photodiode with photopic correction

- spectral measurements – a scanning monochromator with diffraction grating is used (generally gives best accuracy and repeatability)

- some equipment used for field measurements is customised by the user

- What issues are associated with the calibration and use of this equipment?

If a photometer is used to calibrate a LED, it must be characterised well in order to minimise the effect of spectral mismatch errors. The use of an f’LED figure of merit for photometers intended for use with LEDs was discussed and it was agreed this would be useful. Alignment of the LED with reference to the measurement axis can also be critical and is often not well-defined. There is currently no international recommendation regarding the methods to be used for determination of the effective intensity of pulsed sources; measurement equipment often ‘smooths’ the output, which may lead to incorrect results.

- Are there any existing specifications / procedures / guidelines / for measurement techniques, e.g. CIE 127? Are these appropriate?

CIE 127 deals with the measurement of luminous intensity for single LEDs and includes specifications for the alignment conditions and recommended measurement geometries.

- Are these published measurement specifications open to interpretation, or if they were universally accepted, would standard techniques emerge?

The CIE is working to encourage the development and adoption of standard measurement techniques, which are not open to interpretation. CIE 127 has helped to harmonise the way in which manufacturers specify the luminous intensity of single LEDs; it is hoped that other CIE publications in this field will be equally effective.

- Is there any work in progress elsewhere tackling the above issues, e.g. CIE Division 2?

Work is currently underway within several CIE Technical Committees to produce recommendations for measurements of partial flux, luminance, radiance and radiant intensity for single LEDs, and to consider issues relating to the measurement of LED clusters and arrays. There is currently no specification for measurements of pulsing LEDs, although NPL and CIE are working on this. CIE Division 4 is investigating colour boundaries for signalling purposes and Division 6 is involved in the specification of the safety of LEDs, particularly high luminance blue LEDs. It was mentioned that FAA have recently placed a contract relating to measurement specifications for LEDs used in aviation, but no details were available.

3. Role of the ORM Club FIG

- What are the requirements / issues that the FIG should address?

The main task of the FIG should be to provide an input to the CIE Committee and any other organisations which are later identified as being involved in the specification and measurement of LEDs. The aim should be to produce a best practice guide, or similar, that can be fed through to other organisations, particularly the CIE. This will speed up the establishment of internationally agreed best practice and will mean the UK is in the lead in this area.

- How should these be addressed?

It was agreed that a measurement intercomparison should be organised by the FIG. All participants should enter onto a table, circulated by email, the equipment they have for these measurements and available timeslots they would be able to perform the measurements. Hugh Barton will fill in his data first in order to give an indication of what is required. Marl International agreed to supply some artefacts (red, yellow, green and white 20 LED clusters) for the intercomparison. Participants should measure the key parameters in the way they normally would, stating how the measurements have been performed. It should be noted that this will mean that this is therefore not the usual sort of intercomparison, and there will be no ‘right’ answer. The drive circuits will be defined. Final details will be agreed by email discussion.

It was also suggested that a one-day ORM Club seminar should be held, where those present today (and appropriate other interested parties) should give a short presentation on their area of interest. This would provide an opportunity for wider feedback of industrial measurement requirements. A seminar on LED modelling software was also suggested.

The issue of the relationship between perceived colour and measured colour is also important. This will be a difficult issue to solve, but it was agreed that over the next few weeks all FIG members should review relevant literature of which they are aware.

- How should the FIG interact with other organisations or specification bodies?

The principal interaction will be with the CIE, via FIG members who are also active in CIE work. FIG members who are involved with other specification bodies should act as an interface between the FIG and these other bodies.

- Timetable for future work of ORM Club FIG

Intercomparison:

Set up & measurement requirements		end January 2003

Artefacts available				end February 2003

Begin to circulate artefacts			start March 2003

Literature review on perceived colours of LEDs	end February 2003

Next meeting						10.30am, 18 March, NPL

Preparation of brief progress report for CIE		end May 2003

# Other Actions

1. Email to Fiona Jones contact names of additional people who may be interested in the work of the FIG.

2. Email to Fiona Jones contact names for additional people who may wish to participate in intercomparison.

FMCJ

16/12/02