EEP2/WG5/WP2

IALA PRESERVATION MANUAL

PRESERVATION OF HISTORIC LIGHTHOUSES

Managing the consequences of technical changes

DRAFT

IALA – AISM   20ter rue Schnapper  -  78100 Saint Germain en Laye  -  France

Telephone : +33 1 34 51 70 01   -   Telefax : +33 1 34 51 82 05

E-mail : iala-aism@wanadoo.fr   -   Internet : http://www.iala-aism.org

(Draft)

VII  - Managing the consequences of technical changes

During the modernisation plan and process Authorities should seriously consider retaining redundant equipment on site as it contributes significantly to the heritage and tourism value of the light station property. Furthermore, wherever practical, authorities should attempt to integrate redundant equipment with the modern technology.

A presentation made by Philip Hyde and David Brewer of Trinity House (UK) at the 2002 IALA Conference is reproduced as an annex to this chapter Authorities and gives a good example of how to deal with this issue

Introduction

Consider the following stages

CONSIDER THE FOLLOWING STAGES

START

IDENTIFY STATION EQUIPMENT

ASSESS MODERNIZATION POTENTIAL

OPTIONS / SOLUTIONS TO CONSIDER TO ENSURE PRESERVATION

ASSESS CONSEQUENCES / IMPACT OF THIS CHANGE

FINISH

Note : Architecture is a major consideration throughout the process

- Identify station equipment (Inventory)

  - Optic system

  - Lantern

  - Tower

  - Dwellings – Living quarters

  - Outbuildings (Storage sheds, etc.)

  - Power source

  - Other (fog signal, DGPS, etc.)

  - Optic systems (examples)

- Motor

- Light source (lamp changers, burners…)

- Lens (dioptic, catadioptic)

- Reflectors and sectors ((dioptic, catadioptic)

- Pedestal

- Rotating system

  - Lantern – Structure and material

- Capola and glass

- Lantern balcony

- Weather vans

- Exhaust vent

  - Tower

- Material (wood, cast iron, stone, concrete, etc.)

- Tower balcony

- Railing

- Stairway

- Windows (window casing) and doors

- Counterweight device

- Vents

- Watch location (underneath the lantern)

- Watch room / Service room

  - Dwellings – Living quarters

- Materials

- Design / architecture

- Office equipment /Furniture

- Books / Stationary / Clocks

- Operating procedures

- Water supply / collection system

- Doors and windows

  - Outbuildings

- Storage sheds

- Fog signal alarm building

- Gun powder house / canon

- Outhouse / Backhouse

- Barns

- Outdoor bread oven

- Signalling station (mast, anemometer, weather station, etc.)

  - Power system

- Identify the source (oil, gas, etc.)

- Identify the plant

- Combustion engine

- Compressors

- Generators

- Coal stoves

- Old battery storage, containers

  - Other

- Fog signals (Diaphones, horns, explosive types, bells, whistles, etc.)

- Radio beacons

- Racons

- Telemetry

- Communications (radio, telephone)

- Lightning protection

- Uniforms

- Nautical charts

- Assess modernization potential

Note : You may wish to use parts of a unit on others, in order to preserve the better ones.

  - Optic system

- Potential exists for complete removal and replacement with modern technology

  - Lantern

- Possible that the lantern could be removed completely to reduce maintenance costs and improve reliability

  - Tower

- Cladding replacement

- Frames (doors and windows)

- Replacing or naze (?) railings and grates with modern materials

| STATION EQUIPMENT | MODERN. POTENTIAL (Y/N) | OPTIONS | SOLUTIONS/CONSEQUENCES |
| --- | --- | --- | --- |
| Optics |  |  |  |
| - Drive system (e.g. clock work system) | Yes (with electrical, AC or DC, drive system) | Gears and drive shaft can be retained on site | Rest of the equipment (clock work can be left on site for display or removed) |
| - Light source (e.g. lamp changer) |  |  |  |
| - Lens |  |  |  |
| - Rotating system |  |  |  |
| - Reflectors / Sectors |  |  |  |
| - …………. |  |  |  |
| Optics |  |  |  |

Annex

Consequences of Technical Changes in relation to

the Preservation of Lighthouses

Philip Hyde B.Sc. (Eng), C.Eng., M.I.C.E

Principal Engineering Manager

David Brewer

Director of Administration

Trinity House Lighthouse Service

Tower Hill

London EC3N 4DH

United Kingdom

Email:  philip.hyde@thls.org

The paper will review the current changes taking place as the result of re-engineering work in relation to lighthouse equipment and structures. It will illustrate the problems and the methods used to overcome them using recently completed Trinity House projects. It will deal with the current policies on the retention and disposal of redundant equipment and introduce the idea of conservation policies.

The paper will also present the Trinity House Lighthouse Service’s experience in practical uses for redundant lighthouse property with the example of Lizard Lighthouse.

# Introduction

From the day that the very first lighthouse was built, man strived to find ways of improving its efficiency and performance in order to benefit the mariner.

Fig. 1  Dungeness Lighthouse

The pace of change has never been so great than during the last century where world technology has developed so fast as to dictate the requirements of aids to navigation. Modern lighthouse equipment has kept pace through the use of modern technology and materials.

At one time lighthouse technology was at the forefront of man’s achievements but now it has lost this privilege and has had to adapt technical achievements, which have been developed in other fields. This is particularly the case in the electronic and communication field where obsolescence occurs within five to ten years due to the development in components and circuitry.

The physical space requirements to house aids to navigation have reduced with the use of modern high efficiency light sources and small focal length lens assemblies. The role of the traditional aid to navigation has also changed moving from their need to provide long range landfall marks to that of supplementing radio navigational aids and adopting more of an insurance or confirmatory role.

The majority of Lighthouses we see today were built to accommodate the equipment of the early 20th Century such as compressed air fog signals, generating plant and keeper’s accommodation.

In order to maintain an effective and cost effective service, Trinity House undertook a review of all its aids to navigation to ensure that they met the up-to-date needs of the mariner and identify where the new technology could be used to advantage.

A programme of automation was started in 1980 for the automation of all offshore lighthouses followed by the manned mainland stations. Advances in communications and reliability had reached a point where the removal of keepers would not be detrimental to the availability of the aids to navigation. The demanning of the lighthouses and the introduction of central control offered the greatest potential financial savings with a typical pay-back of the automation costs within three to four years and a predicted life of 15 years.

Upon the completion of this programme attention turned to solar conversion of offshore stations that not only provided further savings over constant running generators but also offered environmental advantages. However one effect of this was to remove any electrical power for the conditioning of the station accommodation.

# Consequences of Technical Changes

## The Effects on Equipment

The navigation light probably serves best to illustrate the speed of evolution. The first recorded lighthouse was The Pharos built around 285 B.C. and a wood fire was kept burning on top to warn and guide ships. This continued virtually unchanged except with coal being burnt. Then to encourage the coal to burn more brightly and protect it from the weather the top of the tower was enclosed using glass windows to allow the light to be seen. This was the start of the lantern as we know it today.

Fig. 2  Lizard Lighthouse – Lantern Pre 1812

The light source similarly evolved from the open burning of coal, to the use of candles, the use of reflectors to enhance the light and then on to oil wick burners and multi wick burners.

Fig. 3 – Multi wick burner

Developments continued increasing the intensity of the light sources using acetylene, paraffin vapour burners and up to today’s electric lamps. The search for improvement still continues today. The use of polished reflectors gave way to the use of glass lenses.

However, it is no longer towards increasing the range of the navigation lights but to improving their efficiency. Complimented by the use of radio aids to navigation there no longer exists the need for such generous long range lights, in fact ranges are being reduced. This in turn allows the solarisation of many offshore and floating aids to navigation.

The same philosophy applies to the other aids to navigation where efficiency versus power consumption is equally important if the solarisation route is to be followed.

So what are the consequences of technical change?

New equipment will replace old, methods and practices will change. The old equipment will join other equipment that we already have on or off site that are already considered as ‘artefacts’. Unfortunately not everything can be kept so some selection process must be introduced.  Their future must be reviewed in order to ensure that the Service needs are protected and information is not lost for future generations.

## Retention of Historic Artefacts and Equipment

There are no hard and fast rules as to how historic equipment and artefacts must be dealt with. Each must be considered in own right as to what it is, its possible reuse, location, rarity and history. The preferred sequence is as follows:

- Can the equipment be modified and reused?

If not, then is the history of the equipment station specific:

- Left in position as an exhibit

Left on station as an exhibit

These two options are the most important from Trinity House’s point of view

If not, then:

- Exhibit elsewhere.

- Remove to Stores for use to provide spares support

Scrap (last resort)

Fig. 4 Filament lamp and lamp changer

Such process will involve costs.

## Examples of What Can be Done

### Large Filament Lamps -

Large filament lamps are no longer available as they have become unique to lighthouse use. They were hand made and prohibitively expensive.

- In order to retain the large glass optics that require these large diameter light sources the following can be done:

- Install modern energy efficient lamps

- Use glass spreaders to increase divergence of the beam

Use lamp clusters to recreate the large size light source

Fig. 5  Glass spreaders

Fig. 5 Traditional glass optic

- Etch the glass envelope of the lamp(s)

- Use a glass spreader around the light source

### Traditional Glass Optics

Traditional glass optics, either rotating or fixed, can continue to be used after automation/modernisation. The options available are:

- If it is a rotating optic then use stepper motor drive motors to replace standard electric motors and gearboxes.

- To avoid damage from the sun.

- For drum lenses ensure that the lamp holder, lamp changer and wiring are properly shielded against the heat

- For rotating optics maintain rotation during the day.

- Mercury baths can be retained however a safe working procedure must be followed for the handling and cleaning of the mercury baths.

Fig. 5 Air receivers and fog signal equipment

### Fog Signals – Air Fog Signals

There is no alternative equipment that can provide the long-range fog signals. Trinity House have not been able to automate the traditional air fog signal equipment and as a result have been replaced with short-range electric signals.  Fog signal buildings complete with the compressors, air receivers and trumpets have become museum pieces. If preserved them public demonstrations can be arranged

Fog Bell

Fig. 6 Electric fog bell

Original carbon dioxide driven bells can be solarised using electric stepper motor drive and cam. An example of this can be seen at Trwyn Du Lighthouse in North Wales.

### Radio Direction finding Beacon

The system is no longer used but in the United Kingdom the transmitters are now used to transmit differential signals for GPS. Early examples of the equipment need to be kept for museum display.

### Radar Beacons

Again equipment has developed over the years. Early examples should be retained for exhibit purposes.

### Diesel Generators

Early models and types of engines should be retained.  In many cases they can be exhibited along side air fog signal equipment.

# Obsolete Equipment

Up to 20 years ago it was standard practice to remove and scrap all redundant equipment in some cases this included the large glass optics.  Some optics were dismantled, crated and removed from site. They were then given on loan to museums for public display.

The problem was that when a station was being re engineered the old equipment was consider as obsolete, it looked old fashioned and no longer had any use and often was in the way of the new equipment being installed. At that time the design drawings, photographs and manuals still exist, but it is amazing how quickly these details are destroyed or lost.

How should the decision be made as to what is kept or scraped?

Originally this was done on an informal basis left to the Engineer in charge of the project, often not even mentioned in the scheme. Cost of removal was not even considered and was lost under the heading  “Clear site”.

Today the profile of retaining artefacts has been raised to such an extent that a procedure has been included in Trinity House’s handover document so that every redundant piece of equipment is properly considered to determine whether it to be retained as an artefact,

returned to stores for use as spares or  for  potential re use, or just scraped.

Looking outside some of the offshore lighthouses it was obvious that the sea was used in the past as a dumping place with the remains of broken glass battery cases, wire, engine flywheels and even cannons on the seabed.

The smaller items perhaps faired less well as if they were attractive and fell pray of the souvenir hunter otherwise scraped. But again they tell a story and show a method and practice. It is too later to preserve a complete installation but perhaps somewhere sufficient components exist that would allow an exhibit to be assembled within  a museum environment.

Fig. 7 Example of window furniture lost from many stations

Policy Statement.

It is the duty of every organisation to have a clear policy statement in place to detail how redundant equipment should be dealt with and a procedure to be followed avoiding the decision being left to the whim of an individual person who may have other interests.

# Effect on Structures

Automation and re-engineering has led to the use of less space at the lighthouse properties. There is no need for family accommodation on mainland stations and the required day facilities for visiting personnel can easily be accommodated within the tower. On offshore rock stations, although accommodation was generally not so extensive, providing overnight accommodation for visiting personnel has still left some surplus accommodation.

The type of surplus accommodation can be divided into Living Accommodation and Equipment Space.

## Living accommodation

Trinity House is most fortunate that the majority of lighthouse cottages are situated in easily assessable areas and are connected to the main services – electricity and water. Some properties were sold off in the past to private individuals as Freehold and Leasehold but in all cases problems have been experienced in the failure of the new occupants to maintain the properties to Trinity House’s required standard. Unfortunately the corporate identity remains with the property and associates the service with a fall in standards!

Where the properties, and in some cases included the lighthouse tower, have been sold to a national organisation, such as the National Trust, it has not been a problem as they have maintained them to a high standard, as public attractions. However there is a limit to the number they want and can satisfactorily exploit.

Trinity House put together a business plan covering the development of visitor centres, focusing on the Lighthouse itself, and the public holiday letting of lighthouse cottages. This plan showed that it was economically viable, providing a financial return, yet retaining the properties in the Lighthouse estate. Work on implementing this is well underway with nine visitor centres up and running during 2001, all showing increasing visitor numbers, and the letting of cottages being introduction over the next two to three years. The handling of the hiring and the management of the day- to-day logistics of this latter operation have been put out to a third party specialising in letting holiday cottages.

The plan also allows for the adventurous use of some of the larger island sites where mains facilities are not available. This may include the idea of specialist holidays, such as bird watching.

Lighthouse cottages have a particular character having been built substantially to withstand the elements with heavy wooden internal finishes. They are located at some of the most picturesque and environmentally interesting locations around the English and Welsh coastlines.

Equipment Space

Fig. 8 Pig sties at Anvil Point Lighthouse

This usually consists of redundant fog signal and engine rooms that were often separate buildings housing the equipment. Other rooms within the tower that have became surplus to needs have been cleared and used for storage. Most stations have outbuildings such as oil stores, pigsties and workshops. Some have explosive magazines dating from the days of explosive fog signals.

The way the majority of these buildings were dealt with in the past was for the equipment to be scraped and the buildings demolished. The only reprieve for the building was if part of it continued to have a specific Service use, in which case the redundant equipment was pulled out and scraped but the building kept.

Luckily most of the smaller buildings have been left untouched over the years and provide an insight into life on the stations in years gone by. It is difficult to persuade maintenance that these need to be maintained but luckily they have survived. However where the Lighthouse is listed then they fall “within the curtilage of the building”, usually taken to mean within the boundary walls, then it is a legal responsibility on the Service to maintain them. Once the conservation initiative had been realised then such buildings and equipment are retained.

Fig. 9  The original plans of the Outbuildings at Anvil Point Lighthouse

Options available are:

### Retain the building with the equipment in place as an exhibit

Fig. 10 Moyes equipment, Hurst Point Lighthouse

This is quite easy to do by mothballing the equipment and ensuring that the building is properly conditioned. This is not so easy to do on island sites and the cost of maintaining the exterior and interior will continue to be a burden for years to come.

An example of this is the Acetylene Accumulator House at Hurst Point Lighthouse, on the south coast of England, which still houses the Moyes acetylene generators, the last surviving example of the equipment in the United Kingdom. Drawing and operating instructions are included in Appendix 3. Twenty years ago the importance of the equipment was realised and it was proposed to move it to the Lighthouse Museum at Penzance. This was not going to be easy the site can only be reached using a passenger ferry.  This was never carried out and so today the equipment remains in its original building which in itself form was a functional part of the equipment design. The plan now is to open it to the public as part of a visitor initiative jointly with English Heritage who own the adjacent Hurst Castle.  Such a joint venture may provide some income to offset the on-going maintenance costs. The building is Listed and as a result Trinity House has a legal responsibility to maintain it. Documentation concerning its operation has also survived but only because these were archived with the drawings.

In other cases complete air fog signal buildings have been made redundant through the introduction of short-range electric fog signals. The equipment has been retained, where possible, in working order so that they can be demonstrated to the public as their sound and loudness is quite unique and would otherwise be lost forever.  Such demonstrations form part of the visitor attraction hopefully raising funds for the preservation of the equipment. The engine room at Nash Point Lighthouse, South Wales is fine example where the fog signal building and the equipment form part of the lighthouse attraction.

### Retain the building for an alternative use after removal of equipment

If a building can still be used for a lighthouse use then it should be properly maintained and under go the minimum modifications to allow for its new use. At Nash Point Lighthouse a detached outbuilding in the yard adjacent to the tower, previously used as a workshop, has now been converted into an engine and control room for the station’s mains standby alternator set. This is a small stone building with a slated pitch roof, sliding sash windows and entrance doors at both ends. Only minor internal modifications were required and it is now well suited to its new use.

Fig. 11 Workshop at Nash Point Lighthouse. Now an engine room.

### Retain the Building

A decision may be made to retain the building for its historical value. For this reason many such buildings have survived as a store only later to be converted for a more useful purpose.

### Demolish the building

As a last resort the building may have to be demolished due to its structural condition. This was common in the years gone by but today all the other alternatives must be exhausted. Once demolished it is final. Also in the UK  the legal right for the existence of the building through the planning rules is lost. A structure that is Listed as being of special architectural interest or importance cannot be demolished without the approval of the Local Authority and the Secretary of State.

# Property Conservation

The maintenance of the Lighthouse estate is a significant task. Although at the time of each re-engineering project the light towers are brought up to standard through external painting, overhauling doors and windows and remedying any defects within the fabric of the building, on-going maintenance is essential. In days gone by there was a tendency to save costs by using building materials which were of an inferior standard to those previously used. Windows are a prime example where softwood woods were used instead of hardwood. The mistakes were soon realised, not only within the service but also within the building industry itself where the old skills and expertise have reappeared. Some changes such as replacing cast iron gutters with plastic reducing the need for painting and the problems of rust staining.

What is there to stop repairs being undertaken on a least cost basis using inferior material and unqualified operatives. The developments in modern building materials have tended to cater for the installation by the non-skilled labour. Part of the “Do it yourself brigade”!

A decision has to be made in order to balancing conservation against cost. The following  need to be considered

- The least cost route in the short term will most likely not be so in the long term.

- A responsibility exists to maintain the estate for future generations.

- A legal responsibility exists in the case of Listed building to maintain them in a satisfactory state of repair.

- Automation of lighthouses has removed the human presence on station capable of attending to minor defects before they become out of hand.

- Traditional aids to navigation are becoming less importance and may well no longer be required in twenty years time.

It must be recognised that some lighthouses figure more highly than others as being part of our heritage and are therefore of National importance and must be shared with the public. Reviewing the estate, a list of such stations has been identified, the most important is considered to be Lizard Lighthouse.

The lighthouse is situated on the Lizard headland, Southwest England and being the most southerly point of the British Isles. Its potential lies in it being a working lighthouse, its location, the size of the premises, its history, and the retained equipment on site. In order to record the station a conservation plan has been drawn up recording all features of historical and architectural interest. It also contains policy statements on repair and new works.

Fig. 12

The policy contains statements that can equally be applicable to the Service in general. The implications of adopting such policies Service wide should not be dismissed lightly as they will have a significant cost implication.

A full list of the Policies drawn for Lizard Lighthouse is contained in Appendix 1.

The other purpose of the Plan is to focus on the cultural significance of the lighthouse and the policies that need to be put in place to retain this. It also provides a detailed schedule of items of significance. This considers the main components of the structure under the following headings.

- Detailed area/location of component

- Element - with description of any detail and loss of detail

- Documentation, and its implications; indications for further research (further information if not documented). Drawing numbers refer to Trinity House numbering

- Significance: [S1]= Exceptional significance:

[S2] = Considerable Significance = some significance:

[S3] = Some Significance:

[S4] = Little Significance:

[S-] = Detrimental to site

- Condition and Vulnerability (and case for removal of detail if of negative significance)

- Policy proposed, and /or any proposals for conservation or re-presentation.

An example is included within Appendix 2

It would seem logical to produce global polices and undertake the preparation of schedules of items of significance for the whole lighthouse estate. The degree to which this is done will be dictated by the architectural and historic importance of each station. It also allows a schedule of items to be drawn up as part of the planned maintenance of the station identifying what must and what would be “nice to do” if finances become available. This can also be linked into formal condition surveys of the individual properties, something not routinely done within Trinity House.

Global policies for service properties should cover at least the following headings: (Reference in brackets refers to Lizard Policies in Appendix 1)

- an effective approach to repair (Policy 2)

- the required standard of services installations and other minor works (Policy 3)

- the surveying of property and the actioning of the necessary remedial work (Policy 5)

- the extent of site and boundaries (Policy 7)

- the standards of detailing in relation to the effects of the elements (Policy 9)

- Protection against fire (Policy 10)

- the presumption against the commissioning of new buildings on the site where existing can be used (Policy 12)

- the long term consequences of any proposed alterations (Policy 13)

- the identification of any activity or practice that should be avoided or continued (Policies 14,15,16…)

- consideration of all aspects of the natural environment in relation to the site and its surrounds (Policy 18)

- the survival of any surviving original details (Policy 21)

- the identification of any existing activities that need to be maintained (Policies 21-25)

Although these policies are to generic, some will inevitably become station specific.

# Summary

The paper has attempted to set out the reasons for the decline in the need for Traditional Aids to Navigation and the resulting effects on the Lighthouse Estate. A duty exists to record and maintain the lighthouse heritage for future generations. Procedures and policies have to be produced to ensure this so that we can move away from the decisions being left to ‘the whim of an individual person who may have other interests’.

Maintaining a lighthouse estate that now exceeds the requirements of the Service has significant financial implications and presents the need to look for alternative uses. The preparation of conservation plans for the significant historic lighthouses within the estate provides a basis for seeking heritage funding and recording  the features and maintenance requirements. By adopting a series of generic conservation policies the process can be extended to the rest of the Service, something that should be the desire of any competent Lighthouse Authority.

The setting up of the I.A.L.A. Panel for the Preservation of Lighthouses, Aids to Navigation and Related Equipment of Historical Interest confirms that this is of international concern.

# Appendix 1

Policy 1

The Lizard Lighthouse is to be conserved where ever possible either as a working lighthouse, which part of it currently remains, or conserved as it was at the point where it ceased to function fully as such, which might be taken as the date when keepers were withdrawn [1998].

Policy 2

All repairs are to be undertaken to a conservation specification utilising the original materials of construction unless a clear case to the contrary can be made. They shall be professionally designed, resourced and inspected by persons with appropriate building conservation experience and training,  and executed by contractors with appropriate conservation skills.

Policy 3

All building services installation work shall be professionally, and carefully designed, specified and inspected, and resourced that the conservation implications of the intervention, however small scaled, are fully taken into account. In particular, all service runs, drilled holes and chases are to be agreed in advance

Policy 4

All future major or structural building work should be formally assessed following the thinking and policies set out in this plan

Policy 5

The Authority will continue to monitor the condition of this historic station on a regular basis to produce a written quinquennial condition survey and act where appropriate on its recommendations.

Policy 6

Specific management measures and adaptations will be made to maximise accessibility to as much of the station as is possible, for all persons including those with physical disabilities, within the constraints imposed by the safety and security requirements of the station (including any operational requirements), and unavoidable physical limitations to parts of the station (eg steps in the light towers).

Policy 7

The site area and the boundaries walls are to be maintained in their present (historic) form

Policy 8

The Lighthouse Authority will seek to maintain, formally or informally, effective consultations with all the stakeholders for the Lizard Point area to ensure the maximum coordination of the objectives of the several organisations, owners and bodies involved

Policy 9

To preserve this historic station from damage the highest standards of detailing against the weather should be adopted at this site including in the following areas:

- wind loads on roof structures

- wind uplift on the roof coverings, ridges and flashings

- securing and handling of windows and doors in extreme weather

- regular checks on the structural integrity of exposed elements, especially:

    - chimneys

    - lantern and wind vane

- maintenance of lightning conductor system with extension to Dwellings. Reinstate

- protection to W tower

- condensation and humidity

- reduction of risk of fire spread in strong winds

Policy 10

The best fire prevention systems and advice should be procured in consultation with the Fire Service.

Policy 11

No further property is to be sold off or acquired without a full assessment in conjunction with the completed Conservation Plan for the station in question

Policy 12

There will be a presumption against new buildings being commissioned on the lighthouse site where existing accommodation might be used instead.

Policy 13

Assessment of any proposed adaption or alteration work to the site should look beyond any immediate effect to include an assessment of the impact of any possible further developments that might reasonably evolve from these changes

Policy 14

The approved Trinity House colour scheme will be adhered to in all respects by tenants and staff

Policy 15

There will be a presumption against the fixing of signs to the exterior of building by either the Lighthouse Authority or tenants.

Policy 16

Avoid parking near light towers and control parking on site generally.  Use boundary walls to the full to contain parking.

Policy 17

All options will be considered to maintain the in service use of traditional light/optic

Policy 18

Attention will be paid to all aspects of the natural environment on and around the site, and best conservation practice in maintaining and enhancing it will be taken as a starting point in respect of the planning and execution of any alteration or maintenance work that might affect that environment.  If the site merits it, an ecological audit of the site and/or surround areas may also be commissioned.

Policy 19

Everything will be done to maintain the two towers at the Lizard symmetrically as far as this can be done.

Policy 20

To maximize the possibility that their character will be retained, the dwellings should if possible remain in residential use, whether it be for custodial staff, local people or holiday accommodation.

Policy 21

Any work on the dwellings will ensure that any surviving original detail is retained in situ in some way and that in addition to this, it is used to help increase the legibility of the station (in terms of clarifying the two main phases of work).

Policy 22

The previous history of adaption should if possible continue, viz the rear extensions, which are less formally composed, will continue to bear the brunt of necessary adaptations while the two storey S part of each dwelling remains as intact as possible

Policy 23

The Engine House should remain as it was intended, an Engine House

Policy 24

At least one of the exhibited engines will be kept in working order, to include the use of one of the historic circulating water tanks for cooling.

Policy 25

The 1897 foghorn will be kept in working order

# Appendix 2 – Example of the ‘Detailed schedule of significance

# Appendix 3 – Details of the Moyes Patent ‘Acetylite’  Generators

Definitions

The following word definitions have been for the purposes of this paper:

| Freehold | A property where the owner of the property also owns the land on which it stands. |
| --- | --- |
| Leasehold | A property where the occupier of the property does not own the land on which it stands. |
| Listed | A building that has been placed on the list of buildings, by the Local Authority, as being of special Architectural or historic interest. |
| Automation | Is the process of automating a Lighthouse following which the lighthouse keepers are removed. |

# Bibliography

Conservation Plan for The Lizard Lighthouse

Prepared by: Frans Nicholas RIBA Chartered Architects & Historic Buildings Consultants

Trinity House Lighthouse Service

Milestones in Lighthouse Engineering

By Ebbe Almqvist & Kenneth Sutton-Jones

Published by Pharos Marine Ltd/Automatic Power Inc.

Lighthouses & Maritime Aids of the World

By Kenneth Sutton-Jones

Published by B & T Publications