| From: EEP Committee | EEP2/output/05 |
| --- | --- |
| To: AIS Committee (AIS TWG) | Date: April 10, 2003 |

Liaison Note

Preliminary Draft Recommendation AIS for AtoN

- Introduction

The EEP Committee reviewed the Liaison Note from the AIS Committee, requesting comments on the Preliminary Draft Recommendation on the use of AIS for AtoN.

- Comments

The following comments were discussed and are provided to the AIS Committee to take into consideration in their work on the document.

- In section 1, it was noted that the applications “provide a positive and all weather means of identifying an aid to navigation on AIS and ship’s radar displays” and “providing reference points for ships radar” are not practical unless carriage of AIS/radar compatible equipment is mandatory.

- In section 2.2, it is essential that the position and status of a floating aid be continuously monitored if synthetic AIS information about the aid is transmitted.

- In 3.1.1.3, it was felt that, while GPS position co-ordinates are in WGS84, AIS systems should be capable of presenting position data in the local datum standard.

- In section 3.4, a standard format for Message 6 (Binary address message) needs to be defined for use by AtoN service providers.

- In 8.2, it was felt that there is a need to define a hardware interface to AtoN systems that may be connected to the AIS equipment. A direct I/O interface should be specified and should include both voltage and current signals. It was suggested that an RS232 interface with automatic transparent generation of message 6 signals in the AtoN could be included.