EEP20	Input paper

Agenda item	11.1

Task Number	94 (EEP19)

Author(s) / Submitter(s)	Aivar Usk (Cybernetica AS)

Online buoy heel angle monitoring

# Summary

The paper describes two contemporary methods for online monitoring of floating platform dynamics, particularly the heel angles of navigational buoys, that are applicable for verification of the fulfilment of AtoN availability objective established for the navigational signals originating from floating aids with limited vertical divergence of the signal light beam.

## Purpose of the document

The document is offered for the consideration of the IALA EEP Committee for inclusion of the below Paragraph 3 “VERIFICATION OF AVAILABILITY OF THE FLOATING AID SIGNAL” as Paragraph 9 in the Edition 2 of the IALA Guideline No. 1065 On Aids to Navigation Signal Light Beam Vertical Divergence due to be reviewed at EEP20. The Annex A „Buoy heel angle monitoring example: Estonia“ is provided for information.

## Related documents

IALA Guideline No. 1065 On Aids to Navigation Signal Light Beam Vertical Divergence. Edition 1, May 2009.

# Background

In order to achieve low power consumption, LED buoy lanterns tend to be designed with a rather narrow vertical beam profile (with FWHM typically at 5° to 15°) that may result in severely degraded range of the navigational light signal when the host platform is not perfectly upright. In most cases the authorities responsible for provision of floating AtoN are unaware of actual postural behaviour of deployed buoys. Technologies have recently emerged on the market that enable cost efficient measurement and online monitoring of heel angles of floating aids by the means of power efficient compact solid state acceleration sensors (accelerometers) and corresponding mathematic algorithms. At the same time, the cost of digital machine-to-machine communications in GSM/GPRS and 3G cellular networks has decreased, rendering online monitoring of AtoN at medium data rates a significantly lower burden for AtoN authorities in comparison with the past. Other cost efficient data links suitable for such purpose may emerge in the future. As a result, online monitoring of heel angles and even dynamics of floating AtoN becomes feasible, considering the positive impact of resulting situational awareness on marine navigation safety.

# VERIFICATION OF AVAILABILITY OF THE FLOATING AID SIGNAL

IALA documents that provide guidance on establishing targets for, collecting data on, and calculating levels of key performance indicators of AtoN service ([2], [3] and [4]) outline AtoN availability as the primary performance indicator. In order to maintain awareness regarding the extent of actual fulfilment of the aid availability objectives established for lighted floating aids, it is essential to systematically collect information on heel angles of the corresponding floating platforms. Although technical status of typical monitored floating aids that lack inclination measurement capability may be reported as fully operational and on position, excessive dynamic heeling or even moderate static heeling may reduce the actual visibility range significantly, depending on the vertical divergence profile of the light beam of a particular AtoN.

Technologies that enable cost efficient measurement and online monitoring of heel angles of floating aids by the means of power efficient compact solid state acceleration sensors (tri-axial g-sensors) and digital communication networks are available on the market. Obtained buoy heel angles can be cross-checked either in-situ or on the shore side with the vertical divergence profiles of the carried AtoN lights in order to determine inclination induced degradation of visibility range at any given time when the light was expected to be operational. When averaging of sensor data is implemented prior to analysis, application of a reasonably selected time window is recommended, to avoid loss of heeling maxima with significant duration. It is recommended to perform heel angle monitoring in a continuous manner to be able to register inclination extremes prior to averaging.

The length of the time periods during which the light signal of a significantly heeling AtoN was not available to the mariners within expected nominal range should be considered a temporary excessive inclination failure and used as downtime of the light in availability calculations. A threshold for registering an excessive inclination failure depends primarily on the vertical beam profile and spare luminous intensity of the light. Such thresholds can be implemented in buoy telematics equipment to activate immediate dispatching of a warning message to the monitoring centre when exceeded, and applied to offline statistical analysis for determination of actual availability of the AtoN light signal.

In addition to provision of important input data to availability verification, online monitoring of floating AtoN dynamics can provide benefits such as collision detection and timely identification of buoys heeling either due to structural damage, or incompatibility of the buoy hull or mooring with dominating environmental conditions in the area of deployment (winds, waves, or tidal effects).

## Buoy heel angle monitoring with in-situ calculations

In order to be able to perform online monitoring of buoy heel angles, a buoy must be equipped with proper acceleration sensors and telematics equipment with calculation capabilities that provides a data communication channel to a remote monitoring centre for data collection and immediate analysis. While utilizing on-board electronic inclinometers merely for measurement and storage of heel angle data for collection upon retrieval of such devices is a possibility, it is not recommended due to infrequent planned physical access to navigational buoys of higher categories that are a most likely platform for heel angle monitoring and tend to be unseasonal.

Due to the limited electrical power that can be provided to the equipment of secondary mission importance aboard a buoy, in-situ buoy dynamics analysis is usually limited to heel angle calculations and collision detection. Both the accelerometer sensors and calculation algorithms employed may be selected based on the “as low as reasonably possible” power consumption requirement that usually results in a compromise in precision and speed. Calculation of the inclination angle referenced to the 90° upright vertical axis of the floating platform can be accomplished rather efficiently using a compact micromechanical tri-axial accelerometer, based on distribution of the g-force on the three perpendicular axes. Although precision of the calculations is not very high at small inclination angles, this imperfection can be overlooked from the aspect of monitoring for heel angles that degrade the visibility range of the navigational light signal. It is recommended to calibrate each sensor on the host platform in upright position where possible.

Communication bandwidth required for uploading heel angle information from such AtoNs is rather small: typical scenario foresees sending of averaged heel angle data (a few bites) to a remote monitoring operations centre within regular reporting sessions in accordance with established reporting schedule. In addition, warnings would be sent in case of trigger events such as detection of a heel angle exceeding a pre-configured threshold (or one of several thresholds), or a collision. Instantaneous results of heel angle calculations may be subjected to averaging either for the whole period between status reports, or for a shorter period in order to obtain samples with better representation of short-term behaviour that can be further averaged on the shore side.

Additional local functions may be activated when certain thresholds are exceeded. For instance, flashing may be switched off when a critical heel angle is exceeded (a buoy is expected to have submerged under ice load, or laid down in horizontal position for maintenance or storage).

## Buoy dynamics monitoring with shore side calculations

Once a floating platform is fitted with an accelerometer and a suitable communication channel is available, streaming of raw acceleration measurement data to a shore side monitoring system becomes possible. While a much more sophisticated analysis of floating platform dynamics is possible when employing powerful computing resources, it would be a waste of communications bandwidth and outstation energy merely for obtaining heel angle information of higher precision.

Equipment for in-situ heel angle monitoring is expected to offer capabilities for activation of raw acceleration measurement data streaming from acceleration sensors to the monitoring centre either continuously, at pre-configured time intervals for required duration, or on demand. Such data in the form of time-synchronized series of measurements for each of the three movement axes sampled at pre-configured data acquisition rate present an opportunity for detailed floating platform stability calculations.  Among possible uses are buoy hydrostatics analysis, research of floating platform behaviour in particular environmental conditions, and even wave regime monitoring with limited precision and range depending on the properties of the buoy hull. Due to the significant data volumes that may result from streaming acceleration data to the monitoring centre it is recommended to establish a data management policy prior to activation of such measurements on the larger scale.

# References

- IALA Guideline No. 1065 On Aids to Navigation Signal Light Beam Vertical Divergence. Edition 1, May 2009

- IALA Guideline No. 1037 On Data Collection for Aids to Navigation Performance Calculation, Edition 2, December 2009.

- IALA Guideline No. 1035 To Availability and Reliability of Aids to Navigation. Theory and Examples. Edition 2, December 2004

- IALA Recommendation No. 0-130 On Categorisation and Availability Objectives for Short Range Aids to Navigation, Edition 2, June 2011

# Action requested of the Committee

The EEP Committee is requested to review and update the contents in Working Group 4 as necessary for inclusion in the IALA Guideline 1065.

- Buoy heel angle monitoring example: Estonia

- Buoy heel angle monitoring principles employed in Estonia

The Estonian Maritime Administration started regular monitoring of heel angles and encountered collisions of deployed navigational buoys in 2010. The corresponding e-navigation testbed was created within the EfficienSea project, based on technologies supplied by Cybernetica AS. Telematics modules that were used for provision of regular AtoN status monitoring were upgraded with tri-axial accelerometer functionality and corresponding calculation algorithms; the existing GSM/GPRS/3G cellular phone network based remote monitoring infrastructure was updated to support both platform heel angle and raw acceleration measurement data uploads.

At present time, 100% of floating aids of the Estonian Maritime Administration (180 buoys of all categories) are subjected to regular heel angle monitoring. A telematics module inside the buoy lantern conducts constant measurement and averaging of buoy inclination angles with typical averaging window of 3 minutes even when the light is in standby mode. Regular reporting frequency is different from buoy to buoy, spanning from 96 minute to 640 minute registration interval that has been selected depending on AtoN significance.

When the telematics module detects an averaged host buoy heel angle value greater than the excessive inclination threshold, reports shall be sent to the monitoring centre at a shortened time interval (from 3 minutes for synthetic AIS buoys to 20 minutes, depending on AtoN settings) until the platform returns to upright position, or until disabled by the monitoring centre. A second, critical inclination threshold is used for detection of a situation predating submersion of the buoy under ice pressure; after the monitoring centre is notified, the light is switched off to save power.

Temporary excessive inclination failure periods obtained are used in floating AtoN availability calculations. Based on the acceleration data streaming capability of the telematics modules, field trials of wave height estimation by regular navigational buoys are currently underway.

Figure 1 Recorded heel angles of navigational buoys no. 186, 535, 572, 574, 614, 660, 837 and 839 during 2012. The orange peak marks maintenance of buoy no. 837 on May 5’Th.

Figure 2 10-bin histogram of distribution of the heel angle reports of all 173 monitored buoys of the Estonian Maritime Administration during the year 2012. Basic averaging: 3 minutes, further averaged to 1 hour on the server side if submitted at a higher reporting frequency.