EEP21	Input paper

Agenda item	11.1 / 11.2

Task Number	86 (EEP20)

Author(s) / Submitter(s)	Aivar Usk (Cybernetica AS)

Guideline 1041 amendments: Multiple Source Sector Lights

# Summary

The paper provides information on contemporary modular sector lights for inclusion in the IALA Guideline No. 1041 On Sector Lights that is in the process of being updated at EEP21.

## Purpose of the document

The document is offered for the consideration of the IALA EEP Committee for inclusion of the below Paragraph 3 “Multiple Source Sector Lights” as Paragraph 5.4 in the Edition 3 of the IALA Guideline No. 1041 due to be reviewed at EEP21. Additional amendments to other paragraphs of the Guideline are proposed in Annex 1 and Annex 2.

## Related documents

IALA Guideline No. 1041 On Sector Lights, working paper EEP20/WG4/WP3 / EEP21-11.2.2.

# Background

Modular LED sector lights have been developed and manufactured at Cybernetica since 2006. We are willing to share this experience with the AtoN community - information below is our contribution to standardizing sector lights, and is intended for publication by IALA with any changes or amendments implemented by relevant experts during EEP21.

# paragraph 5.4 “multiple Source Sector Lights”

## Multiple Source Sector Lights

Multiple Source Sector Lights (MSSL) are designed for application at particular AtoN sites. All sectored light signals originate from dedicated light sources, tailored and pre-adjusted to site specific requirements at the factory. Sector/cut screens are used to determine the sectors required in the same manner as in case of Single Source Sector Lights.

Multiple Source Sector Lights are typically built in one of the three light source configurations:

- Integrated Multiple Source Sector Lights – horizontally distributed light sources of an MSSL are housed inside a single enclosure on as many vertical tiers as necessary in order to provide the required sector signals; such lights are usually implemented in case of low luminous intensity requirements, employing a single LED/lens block per sector with typical luminous intensities from tens to hundreds of cd;

- Modular Integrated Multiple Source Sector Lights – instead of LED/lens blocks, field replaceable LED modules with integrated lenses are used inside a common weather proofed enclosure, allowing to achieve luminous intensities up to 2000 cd and above;

- Modular Distributed Multiple Source Sector Lights - LED modules with integrated lenses are installed inside a lantern house in a horizontally distributed arrangement, close to windows in order to eliminate reflections; luminous intensities in several thousands of cd and above can be achieved. The modules are operated as a single sector light by either a single or a redundant control module.

Use of a MSSL is particularly energy efficient when only a few sector signals of low to medium intensity and low to medium boundary uncertainty are required.

Principle of Operation

Multiple Source Sector Lights produce the sector signals using several LED light sources equipped with individual lenses and beam forming screens that are distributed in horizontal plane as off-centre point sources. An example of placement of LED/lens blocks on one tier of an integrated MSSL is provided in Figure x.

Figure x Examples of placement of modules and LED/lens blocks on one tier of integrated MSSL

Depending on the light source and cut screen geometry implemented for separating the sector signals, usable segment of an MSSL starts at a distance L1 that is typically from tens to a couple of hundreds of meters from the AtoN position (Figure x). A blind patch of the size of the cut screen (D, typically not exceeding 100 mm) extends to infinity, increasing the sector boundary region slightly.

Figure x Visibility example of light signals generated by a Multiple Source Sector Light

Beginning of the usable segment of an MSSL can be estimated using the following formula:

L1 = A x D / d

where

A = Distance from light source to cut screen in [mm];

D = Horizontal extent of the cut screen between sector light sources in [mm];

d = Horizontal dimension of light source in [mm].

The Light Source

Although in theory it is possible to build Multiple Source Sector Lights using incandescent light sources and filters, this technology is currently implemented using LED light sources.

Light signal modules employed inside a MSSL may be of varying complexity, ranging from a single unprotected LED/lens assembly to a fully weather proofed LED/lens module with several vertical LED arrays, redundant circuitry and low-level modulation input for control of flashing.

Angle of Uncertainty

Multiple Source Sector Lights are suitable for sites where high precision of sector borders is not required. Angle of uncertainty depends on a particular implementation with typical values in the range of 0.2° to 0.6°, with longer distances from light sources to cut screens (larger enclosure diameters) providing resolution improvements.

The mathematical relationship between the factors is as follows:

 = Angle of uncertainty [minutes of arc] ≤ 3750 / A

where

A = Distance from light source to cut screen in [mm].

Angle of uncertainty should be the input parameter for MSSL design for preliminary estimation of feasibility of MSSL application.

Advantages

- Once manufactured to site specification, installation on AtoN site requires minimum efforts

- Sectors with widely varying luminous intensities can be implemented inside a single enclosure

- Different sectors can utilise different rhythmic characters

- Sectors with overlapping borders displaying alternating colour signals with isophase rhythmic character at the boundary can be created using a multi-tiered construction

- LED modules with dual (redundant) circuitry can be employed for increased dependability

- Power consumption of optimally combined LED arrays of different sectors is typically lower than in case of single-source-per-tier screened omnidirectional solutions

- Distributed modular design allows to utilize existing lantern house at the AtoN site, bypassing shadowing effect of window frames

- In case of a modular construction, light source replacement can be accomplished efficiently onsite when necessary and does not require re-adjustment of sectors

- Modular design allows stocking of spare modules for extended life cycle support

Disadvantages

- Once manufactured to site specification, sector configuration cannot be altered

- Not available off-the-shelf - engineering costs are involved in the design process of each sector light

# References

- IALA Guideline No. 1041 On Sector Lights, Edition 2, December 2008.

# Action requested of the Committee

The EEP Committee is requested to review and update the contents in Working Group 4 as necessary for inclusion in the IALA Guideline 1041.

# ANNEX 1	Proposed Table 3 with updated contents

- Comparison of the difference technologies of sector lights

| Type | Practical Night Time Range | Angle of uncertainty / Boundary width | Max. Sector width | Typical Min. Sector width |
| --- | --- | --- | --- | --- |
| Point Source | Up to 20 M | High (0.25º) | 360º | 1º |
| Multiple Source | Up to 15 M | High (0.25º to 0.6 º) | 90º | 1º |
| Projected | Up to 30 M | Very low (1 minute of arc, 0.017º) | 25º | 0.2 |
| Slot | Up to 30 M | Low Boundary width is constant with range | 45º | 1º |
| Rotating sector light | Up to 20 M | 1º | 360º | 7º |

Note: added input is shown on yellow background.

# ANNEX 2	Proposed amendment to Paragraph 5.6.4: Advantages/Disadvantages of projector sector lights

Advantages

- Lowest boundary uncertainties down to and below 1 minute of arc achievable

- Once manufactured to site specification, installation on AtoN site requires minimum efforts

- Sectors with well-balanced (equal) luminous intensities can be implemented

- Different sectors can utilise different rhythmic characters

- Sectors with overlapping borders displaying alternating colour signals with isophase rhythmic character within the precisely defined sector boundary area can be created using a two-tiered construction

Disadvantages

- Limited achievable width of sectors

- Once manufactured to site specification, sector configuration may be difficult to alter