- EEP3 / 6e) / 2

- DRAFT IALA GUIDELINE

AVAILABILITY AND RELIABILITY

- THEORY AND EXAMPLES

- IALA ENGINEERING COMMITTEE

October 2000

- INTRODUCTION

One of the conclusions of the IALABATT 2 Workshop held in St. Germain en Laye in 1993 was that IALA should as a matter of urgency prepare a standard method of calculating and defining the load profile format for those aids to navigation which utilise batteries.

This task was added to the Work Program for the IALA Engineering Committee at the conclusion of the 1994 IALA Conference in Honolulu.  The continued need for this methodology was reconfirmed in the Spring of 1997 at IALABATT 3 Workshop.

- TABLE OF CONTENTS

1	Introduction	4

1.1	Background (purpose and scope)	4

1.2	Relationship between reliability and availability	4

2	A short introduction to probability theory	7

2.1	Axioms of mathematical probability	7

2.2	Addition rule for mutually exclusive events	7

2.3	Addition rule for arbitrary events	7

2.4	Conditional probability	8

2.5	Multiplication rule (Theorem 4)	9

3	Reliability of systems	10

3.1	Introduktion	10

3.2	Relationsship between preventive maintenance and reliability	11

3.3	Blocks in series	12

3.4	Blocks in parallel, passive redundancy	13

3.5	Blocks in parallel, active redundancy	14

4	Proof of formulas	16

4.1.1.	BLOCKS IN SERIES	16

4.1.2.	PASSIVE REDUNDANCY WITHOUT REPAIR	18

4.1.3.	PASSIVE REDUNDANCY, WITH REPAIR :	19

4.1.4.	AKTIVE REDUNDANCY WITHOUT REPAIR:	20

4.1.5.	ACTIVE REDUNDANCY WITH REPAIR OF A FAILED ITEM:	22

4.1.6.	RELASIONSHIP BETWEEN PREVENTIVE MAINTENANCE AND RELIABILITY	24

5	Examples	25

5.1	COMPUTATION OF FAILURE RESPONSE TIME AND MTTR	25

5.2	Computation of MTTR	26

6	MTTR AND NUMBER OF FAILURES PER YEAR, COMPATIBLE WITH OBJECTIVES	29

7	Terminology	30

ABBREVIATIONS	31

# Introduction

## Background (purpose and scope)

In spite of the efforts made within IALA, it has not proved possible to assess the importance of an aid on a rational and universally accepted and applicable basis.

Notwithstanding this difficulty one can seek to produce guidance applicable to the availability of aids, firstly, because examination of the data concerning the operation of aids to navigation gathered by IALA revealed the possibility of such an approach and, secondly, because the needs for such guidance is evident.

Changing conditions of navigation, and the development of new aids, are leading to a reassessment of what were considered to be established policies and to a search for the foundations of a more effective use of resources. Engineers encountered difficulties in defining the level of sophistication of an aid when no objectives were laid down other than navigational requirements.

The guidelines, which it is sought to produce, are simple. They consist of paying special attention to operational performance and of recording data permitting the calculation of an indicator of service rendered. The value of this indicator should be set for each project and cases of aids for which the indicator value is unsatisfactory with regard to the objectives should be closely examined.

It is accepted that this examination could lead to the aid concerned being abolished in order to concentrate available resources on a reduced number of aids providing an acceptable level of service.

The following cases are treated:

- blocks in series

- passive redundancy without repair

- passive redundancy with repair

- active redundancy without repair

- active redundancy with repair

The outlined methodology may be used to calculate the predicted reliability of a single aid to navigation consisting of several statistically independent subsystems each with its own level of reliability expressed as Mean Time Between Failure (MTBF).

The methodology may also be used to calculate the predicted reliability of a system of aids consisting of a number of individual aids.

The outlined mathematical model may furthermore be used by system designers to calculate the required reliability of systems and subsystems to ensure that the availability objectives set by management can be obtained.

## Relationship between reliability and availability

Use of the "AVAILABILITY" parameter is a good method for defining the level of service a navigator can expect from an aid to navigation. Furthermore, the numerical value of availability can be used to determine, in precise quantifiable terms, the sum total of all the relevant characteristics of the design, engineering, procurement and quality assurance procedures involved in the provision of aids to navigation, together with the necessary logistics and maintenance.

DEFINITION:

The AVAILABILITY is the probability that an aid or a system of aids to navigation is performing a required function under stated conditions at any randomly chosen instant in time.

It should be emphasized that, within the framework of what follows, availability concerns only the ability of an aid to operate as advertised in nautical documents and does not consider external factors such as reduced visibility.

It is true to say that at the design stage the intensity of a light will be chosen according to visibility conditions prevailing locally.

It is also true to say that a light of say 1000 candelas (cd) operating as advertised will have the same luminous output irrespective of the visibility and that it is the duty of the mariner to adjust his expectations and behavior according to the weather conditions.

The parameter "AVAILABILITY" is intended to be used as a management tool, not as a measure of an act of God. In other words, because he meteorological visibility is greater than 10 NM 90% of the time one year and 95% of the time the next year, the Lighthouse Authority should not be automatically satisfied with its performance.

The availability (A) may be calculated by dividing the total time during which the aid has been operating correctly (Up Time) by the total time during which the function to be performed by the aid should have been fulfilled (Mission Time).

A=UpTimeMissionTime	eq. 1

Percentage availability is also used. It is defined as 100 times the availability:

A%=100A	eq. 2

An appropriate period of time should be used for the calculation of the availability of a single aid. This period will have to be sufficiently long, say longer than 2 years, if the calculated availability is intended to represent the long-term aid performance.

DEFINITION:

System RELIABILITY expressed as Mean Time Between Failures (MTBF) is the average period of time wherein the probability of system failure approaches or equals 1 ????

In order to achieve a reasonable value for MTBF the system of interest must be observed over a large number of system failures. ????

The unique parameter used hereafter to characterize the reliability is the Mean Time Between Failures (MTBF) which is also the parameter used in data collection by most Lighthouse Authorities.

The availability of an aid to navigation may also be calculated by dividing the MTBF by the sum of the MTBF and the Mean Time to Repair (MTTR).

A=MTBFMTBF + MTTR	eq. 3

If we rewrite eq. 3 to obtain MTBF we get:

MTBF=MTTRA(1−A)	eq. 4

which is useful for system designers to calculate the required reliability (MTBF) of systems and subsystems to ensure that the availability objectives set by management can be obtained with a given MTTR.

A typical failure scenario will look like this:

# A short introduction to probability theory

## Axioms of mathematical probability

If E is any event in a sample space S, then:

0≤P(E)≤1	(Axiom 1)

To the entire sample space S, there corresponds:

P(S)=1	(Axiom 2)

If A and B are mutually exclusive events, then:

P(A∪B)=P(A)+P(B) 	(Axiom 3)

This may be represented graphically as follows:

If the sample space is infinite, we must replace Axiom 3 with:

If E1, … Em are mutually exclusive events, then:

P(E1∪E2∪....∪Em)=P(E1)+P(E2)+....+P(Em)	(Axiom 3*)

## Addition rule for mutually exclusive events

If E1, … Em are mutually exclusive events, then:

P(E1∪E2∪....∪Em)=P(E1)+P(E2)+....+P(Em)	(Theorem 1)

## Addition rule for arbitrary events

If A and B are any events in a sample space S, then:

P(A∪B)=P(A)+P(B)−P(A∩B)	(Theorem 2)

This may be represented graphically as follows:

Furthermore, an event and its complement EC are mutually exclusive and E  EC = S.

Using axioms 3 and 2 we thus have:

P(E∪EC)=P(E)+P(EC)=1

This may be represented graphically as follows:

This yelds

Probabilities of an event and its complement EC in a sample space S vare related by the formula:

P(E)=1−P(EC)	(Theorem 3)

This formula may be used if the computation of P(EC) is simpler that P(E).

## Conditional probability

Often it is required to find the probability of an event B under the condition that event A occurs. This probability is called the conditional probability of B given A and is denoted by P(BA).

In this case A serves as a new (reduced) sample space, and that probability is the fraction of P(A) which corresponds to A  B. Thus:

P(B|A)=P(A∩B)P(A)	[P(A)  0]

Similarly, the conditional probability of A given B is:

P(A|B)=P(B∩A)P(B)	[P(B)  0]

Solving the last two equations (7 and 8) for P(A  B), we obtain:

## Multiplication rule (Theorem 4)

If A and B are events in a sample space S and P(A)  0, P(B)  0 then:

P(A∩B)=P(A)P(B|A)=P(B)P(A|B)

If the events A and B are such that:

P(A∩B)=P(A)P(B)

they are called independent events. Assuming P(A)  0, P(B)  0, we see from (7-9) that in this case:

P(A|B)=P(A)P(B|A)=P(B)

which means that the probability of A does not depend on the occurrence or nonocurrence of B and conversely. This justifies the terminology.

## Poisson Distribution

It seems reasonable to use Poisson……

The discrete distribution with the probability function:

f(x)=μxx!e−μ

is called the Poisson distribution, named after Siméon Denis.Poisson (1781-1840).

( Mean  = 5 )

# Reliability of systems

## Introduktion

In spite of the value of the methods whereby the availability of the total equipment is expressed as a function of the availability of its components and the MTBF is derived from that availability, we have concentrated on more simple and straightforward methods whereby the predicted MTBF of an equipment is calculated as a function of the MTBF of its components.

This has been possible only because of the very special nature of aids to navigation for which on the one hand the time to respond to a failure generally depends more on the local conditions than on the technology, and on the other hand the time to respond to any discrepancy of an aid from its advertised characteristics relies on a policy which differs from that relevant to time to respond to the failure of an equipment which has not resulted in a change of the advertised characteristics.

In aids to navigation business, there is generally only one redundant element. For the sake of simplicity and clarity attention is restricted to this case although the formulae can often be generalized to a system with a higher level of redundancy.

Some basic assumptions have to be made (……in order to comply with the basic rules of probability theory):

- All systems of interest may be modeled by a number of subsystems (blocks) connected in series or parallel or as a combination of both.

- The various blocks are statistically independent.

- For a given block the MTTR is far shorter than the MTBF.

- The number of blocks is few which in practice means that the difference between the MTBF computed as the mean of up times and the MTBF computed as the ratio between mission time and the number of failures can be neglected. Under these hypotheses the difference between the MTBF and the mean time to failure MTTF, can also be neglected.

- The probability that a device block starts when activated or the probability that a switching device operates correctly will often be is assumed to be constant. (this hypothesis will be restated when used).

- A repaired item is equal to a new item in terms of MTBF.

- An item put into service from time to time (e.g. a standby element) has the same MTBF in terms of its own mission time, as one operating continuously.

Probability laws are continuous with a large spectrum. This eliminates some very peculiar cases encountered when the probability laws concentrate on few values.

## Relationsship between preventive maintenance and reliability

If the life duration of a component follows a probability law for which the more probable duration periods are concentrated around a given value m (see the next figure) then it is clear that the replacement of the component after T hours will be beneficial in terms of enhanced reliability but in the case of expensive components there might be better ways to solve the problem, and a cost benefit analysis should be carried out.

f(t)

- The on site MTBF of a component which is regularly changed after T hours of operation is equal to:

MTBF=MTBF0+T1−p0p0	eq. 5

where MTBF0 is the average working time of a component that fails before T hours of operation, and p0 the probability that such a situation occurs.

## Blocks in series

- The system operates if and only if each of the blocks is in good running order.

The operating time may be presented as follows:

The MTBF of the system (MTBFSYS) is then given by the following formula:

1MTBFSYS=1MTBFX+1MTBFY+.............+1MTBFi

where MTBF i is the Mean Time Between Failures of the i´th block.

The MTTR of the system (MTTRSYS) is given by the following formula:

MTTRSYS=MTBFSYSn=1iMTTRnMTBFn

## Blocks in parallel, passive redundancy

Block Y is put into service only if block X fails, and block X is repaired only after failure of block Y   (PASSIVE REDUNDANCY. WITHOUT REPAIR)

The succession of up times and down times is as follows:

The MTBF of the system (MTBFSYS) is then given by the following formula:

MTBFSYS=MTBFX+(1−p)*MTBFY

where p is the probability that Y doesn't start operating correctly when X fails:

- PASSIVE REDUNDANCY, WITH REPAIR

- The block Y is put into service only if the block X fails and operates only while X is under repair.

The MTBF of the system (MTBF SYS) is then given by the following formula:

MTBFSYS=MTBFX+ MTTRXp+(1−p)∗MTTRXMTBFY

where p is the probability that Y does not start operating correctly when X fails.

## Blocks in parallel, active redundancy

AKTIVE REDUNDANCY WITHOUT REPAIR WHEN ONLY ONE BLOCK HAS FAILED

X and Y run at the same time when X or Y alone is enough to produce the function required from S (a failure of the system occurs only when X and Y are out of order).

If X and Y are identical then if the time between failure was fixed (i.e. failures don't occur at random) MTBFSYS would be equal to MTBFX and the redundancy would not carry advantages.

Active redundancy will prove beneficial if the dispersion of the running times is of some importance.

If X and Y are identical with a constant failure rate MTBFSYS is given by the following formula:

MTBFSYS=3−2p2MTBFX

where, p, is the probability that the second block becomes ineffective as soon as a failure occurs on one block.

- ACTIVE REDUNDANCY WITH REPAIR OF A FAILED ITEM

X and Y operate simultaneously when X or Y alone would be enough to ensure that the function required from the system S is performed.

The operating times may be represented as follows:

MTBFSYS is given by the following formula:

MTBFSYS=11MTTRX+1MTTRY1−ISIS     where   IS=MTTRXMTBFX+MTTRX∗MTTRYMTBFY+MTTRY

# Proof of formulas

### BLOCKS IN SERIES

We shall assume that all blocks stop operating when one of them is out of order and that the equipment has been operating for a very long period of time T and has had a great number of failures, N, comprising Nl failures of block 1, N2 failures of block 2 …. Ni failures of block i.

The total number of failures (N) in a system consisting of i blocks all in series may be expressed as follows:

N=n=1iNn

- The MTBF of the system is equal to

TN=TN1+ N2+.....+ Ni

i.e. the total time divided by the total number of failures.

Let MTBF i be the MTBF of block number i

MTBF i=  TNi

then

NT=N1T+N2T+....+NiT

that is to say

1MTBFSYS=1MTBFX+1MTBFY+.............+1MTBFi

The invert of the MTBF of a system with all blocks in series is equal to the sum of inverses of the MTBF of each block.

A very popular demonstration of this formulae is given hereafter:

The probability that two independent events occur during a trial is equal to the product of the individual probability of each event. Then the probability that a system with i blocks in series does not fail during a period of time t is equal to

R(t)=e−(λX t)+e−(λY t)+......+e−(λi t)

If each block has a constant failure rate.

It results from the property of the exponential that

R(t)=e−n=1i(λn * t)

so, the total failure rate SYS of the system is equal to

R(t)=n =1iλn * t

- Note: With the same notations as before

MTTRSYS=n =1iNn MTTRnN=n=1iTMTBFn∗ MTTRnTMTBFSYS

And thus:

MTTRSYS= MTBFSYS * n =1iMTTRnMTBFn

### PASSIVE REDUNDANCY WITHOUT REPAIR

The succession of up times and down times is as follows:

By definition

MTBFSYS=1nn=1i(Xn+Yn)

where n is large and is the total number of failures of the system. So:

MTBFSYS=1nn=1iXn+1nn=1iYn=MTBFX+ MTBFY

If Y has a probability equal to, p, not to start operating correctly when X falls then it is only in 100 * ( 1-p ) % of the cases that the operating time of Y will be added to that of X.

It follows that :

MTBFSYS=MTBFX+(1+p)MTBFY

### PASSIVE REDUNDANCY, WITH REPAIR :

Let n be the number of failures of X during a long period of time equal to n * (MTBF X + MTTR X).

The block Y will have to operate during the down times of X. The total down during the period in question will be equal to NX * MTTR X, and there will be on average a failure of Y, (and so of the system) every MTBF Y hours (Counted on the basis of the down time of X).

Then the number of failures NSYS of the system S will be very close to

NSYS Approx=NX* MTBFXMTBFY

- MTBFSYS=NX * (MTBFX+ MTTRX)NSYS≈NX * (MTBFX+MTTRX)NSYS ApproxBy definition:

- Thus follows :

- MTBFSYS≈MTBFX2MTTRX+MTBFXIf X an Y have the same statistical characteristics:

- If we assume that there is a probability p that the block Y does not start when activated then the same demonstration as before leads to the following formula:

NSYSApprox=Np+N(1−p)MTTRXMTBFY

And

MTBFSYS≈N ( MTBFX+ MTTRX)NSYSAapprox=MTBFX+MTTRXp+(1−p)MTTRXMTBFY

* Strictly speaking this should be noted MTTFSYS, and the demonstration stands only when after repair of y both X and Y are in a good running order (under basic hypothesis, the approximation is very good).

### AKTIVE REDUNDANCY WITHOUT REPAIR:

If p = 0 the proof is rather simple:

X and Y being statistically independent then P(S  t) = P(X  t) * P(Y  t)

If in addition X and Y are identical with a constant failure rate then:

P(S≤t)=1−e−λt1−e−λt=1−2e−λt+e−2λt

- The reliability function R(t) is then equal to:

- R(t)=2e−λt+e−2λt

- and

- MTBFSYS=0∞2e−λt+e−2λtdt=32∗1λ    where   1λ=MTBF

- That is to say

- MTBFSYS=32MTBFX

If d 0 the proof is more complicated and is only given hereafter for those familiar with the theory of probability.

- Let FX and FY be respectively the states where X and Y are in good running order at t.

- The event "the failure of one element does not cause that of the other" will be quoted as FB.

- Let FX,FY  and  FB be the events complementary to FX, FY FB respectively.

Then by applying the theorem of "total propabilities" (1) it follows that the probability P(S t) of a failure of the system occurring before t is given by:

P(S≤t)=P(FB)P(FX∪FY)+P(FB)P(FX∩FY)=P(FB)P(FX)+P(FY)−P(FX)P(FY)+P(FB)P(FX)P(FY)P(FB)=p   and   P(FB)=(1-p)P(FX)=(1−e−λt)   and   P(FY)=(1−e−λt)0∞P(S≤t) dt =MTBFSYS

A simple calculation shows that

MTBFSYS=3−2p2MTBFX

One should note that the same method would permit calculation of MTBFSYS when the 2 blocks do not have the same failure rate.

### ACTIVE REDUNDANCY WITH REPAIR OF A FAILED ITEM:

The operating times may be represented as follows:

Let us calculate IS = (1 - A SYS) where A SYS is the here AS is the availability og the system S:

IS=MTTRXMTBFX+MTTRX∗MTTRYMTBFY+MTTRY

As by definition :

IS=MTTRSYSMTBFSYS+MTTRSYS

by looking at the operating diagram one can notice that the model for calculating MTTRSYS is equivalent to one with blocks in series where time to repair, is substituted for operating time (up time) and vice versa and where it is assumed that a block continues to operate even when the other has failed.

Having done that, the basic hypotheses are not preserved since mean time to repair is greater than MTBF, and then the demonstration given in 2.1 can't be applied.

However, one can show that the following formula is still valid :

1MTTRSYS=1MTTRX+1MTTRY

and so

MTBFSYS=11MTTRX+1MTTRY1−ISIS

- If X and Y are identical    MTTRSYS=MTTRX2   and

- MTBFSYS=MTTRX21+MTBFXMTTRX2−1

- The above formula can be generalized to the case where i blocks are in active redundancy.

If the i blocks are identical the formulae are:

and

MTBFSYS=MTTRXi1+MTBFXMTTRXi−1

Notes:

- A more efficient policy being in general applied then the whole system S has failed; the above MTTRSYS should be considered only as a mean to calculate MTBFSYS.

- Under the basic hypothesis it can be shown that the probability that the system S still operates at time t is close to e−tMTBFSYS

### RELASIONSHIP BETWEEN PREVENTIVE MAINTENANCE AND RELIABILITY

The proof is quite easy for those familiar with the theory of probability:

By definition:   P0=0tF(t)dt

Let Pq be the probability that the equipment fails after having been replaced q times then

Pq=P0(1−P0)q

since q components have been changed after having been operating correctly during T hours and the q + l component failed before T hours of operation.

- MTBF0=oTtf(t)dtoTf(t)dt

- And

- MTBF=i=0∞(iT+MTBF0)Pi=MTBF00∞Pi+T0∞iPi

- In this expression  0∞Pi=1  and   0∞iPi=P00∞i(1−P0)i=1−P0P0

- It follows that

- MTBF=MTBF0+T1−P0P0

# Examples

## COMPUTATION OF FAILURE RESPONSE TIME AND MTTR

METHODOLOGY TO ASSESS FAILURE RESPONSE TIME (FRT). IMPACT ON SERVICE AVAILABILITY

- Computation of failure response time

Failure Response Time (FRT) can be derived from the availability (A) relationship between MTBF and MTTR :

- Equation (1)

Solving for MTTR yields :

- Equation (2)

Use as an example the following case where assumed values are given:

-A = 0,998 .. -MTBF = 14000 h.

-Mean Time To Report (MTT Report) = 5 h

-Mean Time To Prepare (MTT Prepare) = 4 h.

-Mean Time To Transport (MTT Transport) = 11,5 h

(Includes possible loss of time due to bad weather)

-Mean Time To Repair on site (MTT Repair on site) = 2 h.

Recall that :

MTTR = Mean Failure Response Time (MFRT) + MTT Report + MTT Prepare + MTT Transport + MTT Repair on site.

Equation (3)

Inserting the assumed values yields :

- MTTR = 5 + 4 + 11,5 + 2 + MFRT MTTR = 22,5 + MFRT

Substituting this value for MTTR in equation (2) yields :

22,5 + MFRT = 14 000 (1 -0,998) ; 22,5 + MFRT= 28,1; MFRT= 28,1-22,5 0,998

MFRT = 5,6 hours.

- Conclusion

For the given values of MTBF and repair times, a Mean Failure Response Time of 5,6 hours selected by management would permit the lighthouse service to maintain the stated level of service (A = 0, 998) to the mariner.

If the computed value for MFRT is negative, it means the MTBF is too low or the total MTTR is too high to provide the stated availability. The MTBF situation can be helped by specifying higher quality equipment, increasing equipment redundancy, or increasing preventive maintenance. The MTTR situation can be helped by reducing the various time components in the MTTR total.

It is possible that a certain aid to navigation does not seem to be very important and as a result may have been assigned a low availability and a large MFRT. Due to a failure of this aid, a vessel may be misled, run aground and be heavily damaged and severely injure the environment. The resulting cost could be enormous. A lighthouse service should keep this in mind when setting an availability objective.

## Computation of MTTR

COMPUTATION OF MEAN TIME TO REPAIR FOR SELECTED REDUNDANCY AND REPAIR POLICIES

Consider the two aid systems represented by the block diagrams below, one without redundancy and the other with redundancy. Using the relationships developed in Part I Chapter 2 of this Guide, MTTR's are computed for differing redundancy, maintenance policies, and number of planned maintenance visits annually.

For each case the availability objective (A) is chosen to be 0,995, and the MTBF of each light system block is given to be 4000 hours. The 4000 hours interval is nominally one year for these daylight controlled lights. Computed values for MTTR are shown in the table below. A more detailed presentation of each computation is shown on the following pages of this annex.

NO REDUNDANCY			PASSIVE REDUNDANCY

- MAXIMUM MTTR TO ACHIEVE AVAILABILITY OBJECTIVE OF 0,995.

This simple example shows that maintenance policies and the existence of a standby equipment will affect considerably the MTTR objective necessary to meet an availability objective.

This is of paramount importance because the cost of maintenance is likely to be far higher when the Mean Time To Repair a failure is set at 20 hours than when it is set at 300 hours (note that in both cases the number of visits to the aid, per year, is the same) .One reason for this is that more flexibility is given to the maintenance force and that there are fewer panic maintenance operations.

It should also. be realized that paying visits every 2000 h or 500 h to an aid may not be excessively costly. This will be the case, for example, if the distance between the aids marking an area is short compared with that between an aid and the maintenance facility and if it 15 accepted that a visit is paid to each aid when travel to one of them is rendered necessary by circumstances.

- Detailed computation of MTTR values presented in the above table

The general technical hypotheses are given as follows :

- availability objective, A: 0,995 ;

- 2 technical choices : no redundancy (normal light), passive redundancy (normal & standby);

- MTBF of each light (normal & standby light) = 4000 h.

Equation terminology :

d is the probability that the standby light does not start operating correctly when the normal light fails. MTBF and MTTR denote general availability attributes.

Addition of a suffix denotes corresponding subsystem values :

MTBFX is MTBF for subsystem X

MTTRY is MTTR for subsystem Y

MTBFS is MTBF for entire system

MTTRS is MTTR for entire system

The general formulas for cases are:

Availability (A) = MTBF  / (MTBF + MTTR)

MTTR = MTBF x (1-A) / A

MTBFS = (MTBFX + MTTRX) / (d + (d + 1) x (MTTRX / MTBFY) )

With d = 0, the above formula is reduced to:

MTBFS = (MTBFY / MTTRX) x (MTBFX + MTTRX)

- CASE 1

A. No redundancy.

MTTR = MTBF x (1-A) / A

MTTR = 1.000 x (1-0,995)  / 0,995

MTTR =20,1 hours.

B. Redundancy ; repairs take place only when the two lights (normal & standby) are extinguished. This is a category of passive redundancy without repair

MTBFS = MTBF'X + (1- d) MTBFY Formula 2.2 ;

assume d = 0

MTBFS = 4000 + (1-0) 4000 = 8000 h.

MTBFS = 8000 hours.

- MTTRS = MTBFS X (1-A) / A

- MTTRS = 8000 X (1-0,995) / 0,995 MTTRS = 40,2 hours.

- CASE 2 :

Redundancy repairs take place at a regular visit or when the two lights (normal & standby) are extinguished visit planned at selected intervals. This is a category of passive redundancy with repair. The formulas below are used to solve the problem.

MTTR = V (Visit interval) / 2 (See 2.8 in Part I)

MTBFS = (MTBFY / MTTRX) x (MTBFX + MTTRX) Formula 2.3 with d = 0

- A. One visit every 2000 h

MTTRX = 2000 / 2 = 1000 h.

MTBFS = (4000 / 1000) X (4000 + 1000) = 20 000 h .

MTTR = 20.000 x (1-0.995) / 0.995

MTTR = 100 hours.

8. One visit every 1000 h.

- The same procedure is followed as in A

MTTRX = 1000 / 2 = 500 h.

MTBFS = (4000 / 500) X (4000 + 500) = 36000 h

MTTR = 36000 X (1-0,995) /0,995

MTTR = 180 hours.

C. One visit every 588 h.

- The same procedure is followed as in A

MTTRX = 500 /2 = 250 h.

MTBFS = 4000/250) x (4000 + 250) = 68.000 h

MTTR = 68.000  X  (l - 0,995) / 0,995

MTTR = 340 hours.

CASE 3 : Redundancy ; no regular visit, Mean Time To Repair is the same when only one light is out and when the two lights (normal & standby) are extinguished. This is a category of passive redundancy with repair.

- MTBFS = (MTBFY / MTTRX) x (MTBFX +MTTRX) Formula 2.3 with d = 0

MTBFS = (4000 / MTTRX) X (4000 + MTTRX)

- A = MTBFS / (MTBFS + MTTR)

0,995 = MTBFS / (MTBFS + MTTR)

The MTTR of the system is given to be equal to MTTRX in this case, so:

0,995 = MTBFS / (MTBFS + MTTRX) or

MTBFS = (MTBFS + MTTRX) x 0,995 or

MTBFS (1-0,995) = 0,995 MTTRX

- MTBFS = 0,995 MTTRX / 0,005 = 199 MTTRX

- 199 MTTRX = (4000 / MTTRX) x (4000 + MTTRX) or

199 MTTRX x MTTRX = 16.000.000 + 4000 MTTRX or

199 (MTTRX) 2 - 4000 MTTRX - 16.000.000 = 0 or

divide by 199, we have :

(MTTRX) 2 - 20,1 MTTRX - 80.402 = 0

Delta = (-20,1 ) 2 - 4 x 1 x (-80.402) = 322.012

SQR = Square Root

SQR (Delta) = SQR (322.012) = 567,46

MTTRX2 = (-(-20,1) - (567,46)) / 2 = -273,68 (not accepted)

MTTRX'1= (-(-20,1) + (567,46)) / 2 = 293,78 (accepted)

MTTR = 293,78 hours (approximately 300 hours)

# MTTR AND NUMBER OF FAILURES PER YEAR, COMPATIBLE WITH OBJECTIVES

To draw the tables a Poisson distribution was assumed both for the time between failures and for the time to repair.

- Maximum acceptable mean time to repair

Maximum acceptable Mean Time to Repair (MTTR) for a given objective MTTRo, of the MTTR and a 10% probability of rejecting unduly the hypothesis that repair times comply with the objective.

- NUMBER OF FAILURES …..

((TABLE))

Maximum acceptable Mean Time to Repair (MTTR) for a given objective MTTRo, of the MTTR and a 1% probability of rejecting unduly the hypothesis that repair times comply with the objective.

- NUHBER OF FAILURES….

((TABLE))

- Maximum acceptable number of failures per year

Probability for having at the most x failures in one year.

((TABLE))

Example of a table that can be extracted from the previous one :

Maximum acceptable number of failures per year for a given objective MTBFo of the Mean Time Between Failures and a = 1%  probability of rejecting unduly the hypothesis that the times between failures comply with the objective.

MTBFo unit. = year

((TABLE))

# Terminology

Failure Response is the action taken by a lighthouse service after a failure has been reported. It may take the form of an immediate repair action, a repair action within a certain period of time, another action such as a notice to mariners, or a combination of the latter with one of the former.

Failure Response Time (FRT) is the time wherein a lighthouse service must react to a failure. The FRT depends on the intended availability (A) of an aid or system of aids, the risk which may be caused by a failure (such as economic consequences to a port, environmental damage and involved cost, etc.), and the type of failure.

Aid Availability established by a service depends in part on the attributes of the aid itself. This includes the importance of the aid, determined by its purpose, type and number of users, configuration of the waterway, and equipment on the aid. The selected level of availability will also depend on other aids or information sources in the vicinity, maintenance and Mean Time Between Failure (MTBF), and Mean Time To Repair (MTTR). Several methods exist to determine the importance of an aid including computation' according to a quantitative methodology, or from practice or experience in the local area of interest. Pilots and VTS communication centers/stations can be valuable information sources.

Mean Time Between Failures (MTBF) is simply the length of time between aid failures. Maintenance and MTBF are interactive. If more preventive (planned) maintenance is rendered, the MTBF increases.

Mean Time To Repair (MTTR) is the time it takes to restore an aid to normal operation after it fails. MTTR is composed of reporting time (or time to detect), preparation time, transportation time, repair time on site, and failure response time.

Mean Time To Report varies, depending on whether the aid is manned or remotely controlled ir not, the time will depend on the attentiveness of mariners who can make reports from vessels passing a failed aid.

Mean Time To Prepare is the time required for preparations such as alerting the crew, collecting tools and materials, making the tender ready to sail, etc.

Mean Time To Transport is the time required to transit from the base to the failed aid and includes time lost due to sea and weather conditions.

Mean Time To Repair on site is the time required on site for the repair crew to actually restore a failed aid to its original function.

# ABBREVIATIONS

A		Ampere

D		Solar declination angle (in degrees)

D1 Apr		Solar declination angle on 1 April

D31 Oct		Solar declination angle on 31 October

EDL		Daily load

EDL/max		Maximum daily load

EDLapr-oct	Maximum daily load between April and October

EDL/nov-mar	Maximum daily load between November and March

Esurge		Surge factor

FL		Flash character < Esurge

h/day		Hours per day

Hdarkness	Hours of possible darkness

Hdarkness1Apr	Hours of possible darkness on 1 April

Hdarkness31Oct	Hours of possible darkness on 31 October

Hdaylight		Hours of possible daylight

Iavg		Average current

L		Latitude (in degrees)

LED		Light emitting diode

mA		milli-Ampere

n		Julian date

N		North

S		South

V		Voltage

W		Watt

Wh/day	Watt hours per day

Ws		Watt seconds