FRAMEWORK AND ACTION LIST FOR PRODUCTION

OF "GREEN GUIDE" TO ATON OPERATIONS

United States Coast Guard Input

Per EEP2/WG3/WP2, the USCG was assigned to develop sections of the Green Guide in the following areas:  battery disposal, synthetic buoys/moorings, impacts on habitats, and lamp disposal.  This document is a first step in that process.  It brings together the responses to those four areas from members who submitted their Environmental Surveys as of EEP2, April 2003.

This document consists of two parts.  The first part summarizes the problems and solutions provided by the members, with "poetic license" taken in combining similar responses.  The second part is a collation of the "raw data" as submitted by the members on their survey forms.

Please direct any questions or comments to:

Mr. Wayne Danzik

United States Coast Guard

Commandant (G-SEC-2B)

2100 Second Street, SW

Washington, DC 20593-0001  USA

wdanzik@comdt.uscg.mil

PART 1  -  SUMMARY OF PROBLEMS AND SOLUTIONS

BATTERY DISPOSAL

Problems

Different types of battery and electrolyte are used, requiring separate storage and handling for the subsequent disposal of the different types.

Batteries contain toxic and hazardous materials such as heavy metals, acids, and alkalies.  Disposing of them requires special handling.  Leakage of these materials could harm the environment.

Batteries can enter the environment through accidental loss, vandalism, or deliberate disposal.  This creates a waste remediation problem in the water or on land at an ATON site.

Solutions

Switch to rechargeable solar batteries.  These batteries can be recycled, and this can often be done through the battery supplier.

Ensure that disposal of non-recyclable batteries is carried out by licensed contractors or waste disposal authorities who provide documentation of proper disposal at authorized waste facilities.

Implement engineering solutions to minimize the chances of battery loss through collisions or vandalism.

Synthetic Buoys/Moorings

Problems

Some synthetic materials do not lend themselves to recycling and must be disposed of as industrial waste.  This creates extra cost, and adds to the environmental waste stream when these materials are deposited in landfills (many of these materials are non-biodegradable).  Also, some materials create toxic air emissions when they are cut up or burned.

Old moorings that are discarded on site could present a tangling hazard to marine life, or foul the propellers of passing vessels.

Solutions

When possible, utilize buoys and moorings made from environmentally-friendly materials that are fully recyclable.

IMPACTS ON HABITATS (MARINE, PLANT, BIRDLIFE)

Problems

Water pollution and marine litter from ATON activities.  Some examples would include on-site battery disposal, abandonment of synthetic line, spills of diesel fuel, spilling concrete or sewage when building lighthouse foundations on site.

Biocide-based antifouling paint on buoys is toxic to marine life.

ATON maintenance activities can interfere with or harm marine life and habitats.  Migratory birds nest on some stations, making it problematic to service the ATON without disturbing the nests.  ATON servicing vessels can strike marine animals while working in their habitat.  Lighthouses can be situated in areas where rare flora and fauna is found.

It is sometimes necessary to clear trees and brush when ATON structures become obscured.

Solutions

Extend the maintenance intervals of ATON hardware and equipment to the greatest extent possible to minimize the requirement for intrusive servicing visits.

Use ATON hardware and equipment that has less potential for environmental damage (e.g., solarization versus primary batteries, diesel, or submarine cable).

Avoid interfering with nests or colonies.  Consider engineering measures such as installing separate nesting platforms on ATON sites.

Jet buoys at sea or near the areas where growth is acquired.

Consider the use of non-biocide-based antifouling systems when proven technologies become available.

Perform concrete work at a yard on shore, rather than pouring on site.

LAMP DISPOSAL

Problems

Marine lamps could potentially contain elements that are dangerous to the environment, and thus create disposal problems.  Fluorescent and mercury lamps contain toxic materials.  Sodium and neon lamps are not ozone friendly, and require special handling and disposal.

Solutions

Lamps are available that are made of inert materials (krypton gas with tungsten filaments) and can be disposed of as standard waste.

Lamps can be recycled or disposed of through the manufacturer or a licensed contractor.

PART II  -  COLLATION OF SURVEY RESPONSES

The members' responses in this part are shown verbatim, except for minor editing for clarity.  They are labeled as follows to indicate the member responding:

DK	=	Denmark

FL	=	Finland

GE	=	Germany

HK	=	Hong Kong

IR	=	Ireland

JA	=	Japan

SW	=	Sweden

UK	=	United Kingdom

US	=	United States

BATTERY DISPOSAL

Problems

- UK:  Acids, heavy metals.

- UK:  Different types of battery and electrolyte.

- UK:  Increasing use of batteries for float charge on solar and remote mains stations.

- UK:  Separate storage for subsequent disposal of different types.

- FL:  Approximately 5MWh of primary alkaline batteries used in a year on buoys.

- FL:  End up in landfill (non-toxic).

- DK:  If mercury in batteries, must dispose of them through special channels.

- IR:  Lead, cadmium, and mercury in lead-acid, nickel-cadmium, and primary batteries are toxic materials.

- JA:  In Japan CG, strong alkali (over pH 125) is used as the electrolysis solution for the air battery (primary battery), while strong acid (under pH 2) is used for the lead battery (secondary battery).  These are used as the power source for beacons.  They have the strong possibility of causing harmful effects in case of leakage to fish, shellfish, and aquatic plants, thus causing increased expenditure to the Japan CG from trying to take proper action.

- US:  The USCG operates more than 12,000 lighted ATON structures and nearly 5,000 lighted ATON buoys.  Although close to 98% of these aids are now solar powered and use rechargeable solar batteries, prior to the 1980s we relied heavily on various types and sizes of "one-time-use" primary batteries.  These batteries contained mercury coated zinc in some of their parts.  As was common practice in the "old days," these batteries were sometimes discarded in the water or on the land near the ATON site.  Although this practice was banned in the mid-1970s, the batteries that had previously been disposed of remained to be dealt with.  Also, despite prohibiting the deliberate disposal of batteries on site, batteries continued to be lost through accidents or vandalism, a problem that persists to this day.

- HK:  Battery (primary and secondary) for supplying energy to the marine lanterns.

Solutions

- UK:  Supplier takes away on a new for old basis.

- UK:  Redundant and time expired gel and liquid electrolyte batteries are disposed of via a licensed contractor who provides controlled waste documentation.  Scrap value realized (20 pounds per sonnenschein).

- UK:  Beco cells recycled by supplier against a discount on new product.

- FL:  A development project has been started to replace primary batteries with secondary batteries that would be charged on shore (solar panels are not useable because of moving ice fields).  Secondary batteries are returned to manufacturer.

- GE:  Used batteries are collected and stored at the workshop.  A contractor provides the waste disposal according to the rules from time to time.

- SW:  All environmentally-harmful batteries are dealt with by the supplier or alternatively by the local waste disposal authorities.

- DK:  Use mostly primary batteries for floating ATON.  These are mercury-free and may be disposed of through normal channels.

- IR:  Utilize licensed waste disposal contractors and return to manufacturer.

- JA:  They should be packaged in the specified mode of packing during the transportation to the repository site.

- JA:  Consigned to the nationally-authorized carrier and disposal processor.

- JA:  Permanent disposal of each individual battery is confirmed together with securing enough budgets in order to realize the above through incorporation of a managerial system.

- HK:  For batteries containing liquid electrolyte, the electrolyte should be drained out and collected by a licensed waste collector at a licensed waste disposal facility.  The drained battery casings (including rechargeable types) should be disposed of at the EPD's landfill as a chemical waste.

- US:  The single biggest improvement has been the transition from primary batteries to solar power systems with secondary batteries.  These are lead-acid batteries, similar to automobile batteries.  They have a service life of up to 6 years (compared with 1 to 2 years for the old primaries), and they are completely recyclable.  Some USCG suppliers will even pick up old batteries for recycling when they deliver a shipment of new ones.

- US:  In 1994, the USCG implemented a "cradle to grave" serial number tracking system for all ATON batteries.  We created this nationwide system to ensure that all ATON batteries can be accounted for until they are released from USCG control for proper disposal or recycling.  Batteries used after 1994 are identified individually through a USCG-affixed serial number.  Responsibility for maintaining the tracking system rests with the unit exercising custody or control over the battery.  The system requires the use of a battery log which tracks individual batteries and appropriate property transfer documents which ultimately release USCG control of the batteries for disposal or recycling.

- US:  When batteries accidentally make their way into the water (from knockdowns, bad weather, vandalism, carelessness), the event is reported to Federal, state, and local authorities and a project is initiated to recover them.

- US:  More than 800 ATON structures are knocked down annually.  These are typically single pile wood structures that are destroyed through collisions with passing vessels.  When a structure of this type is hit, the pile might break off and float away, the battery box might separate from the pile, or the battery might be thrown from the battery box into the water.  This could result in the loss of upwards of 1,000 batteries per year, which the USCG would have the responsibility to retrieve.  However, we have implemented several simple but effective measures on these structures to minimize the loss of batteries when structures are knocked down.  These measures are mandatory on all ATON structures with histories of frequent knockdowns:

The battery box is reinforced with steel end brackets, a top bracket, and a bottom "strong-back."  The top bracket and the strong-back secure the battery to the structure platform.  The end brackets, on both ends of the box, keep the battery from being thrown from the platform;

The upper portion of the pile is tethered with chain to the lower end of the pile below the mud line.  This prevents the upper portion of the pile from floating away from the charted position if it is broken off during a collision.

The platform (usually a wooden "A-frame" with wooden decking) is reinforced with steel angles to help hold it together during a collision.

The platform is tethered with chain to the upper portion of the pile to ensure that it remains connected to the pile and on station with the other wreckage.

- US:  The USCG has initiated a training program on battery handling and disposal procedures.  All personnel involved in ATON maintenance operations receive an awareness briefing, and all ATON units are required to view a training video which demonstrates battery accounting and disposal procedures.

Synthetic Buoys/Moorings

Problems

- JA:  Fiber Reinforced Plastic (FRP) and polyurethane rubber used in some of the buoy floating frame are unqualified for recycling and must be consigned to industrial waste scrap processor, which also requires disposal and transport costs.

- IR:  Cutting up would produce dust, burning produces toxic particles.

- UK:  Non-biodegradable materials such as fiberglass, plastic, and epoxy materials, some containing isocyanates.

- US:  The USCG uses two types of synthetic buoys.  The first consists of a hull made from rotationally-molded polyethylene plastic, filled with a closed-cell polyurethane foam.  The second is made from extruded sheets of ionomer resin foam, which are wrapped around a central plastic core and heat sealed in layers to form the shape of the buoy hull and daymark.  Neither of these buoy types is easily recyclable.  With regard to the plastic buoy, the polyethylene shell could be recycled, but the tightly adhering polyurethane foam makes it problematic.  The ionomer foam requires expensive, specialized equipment that very few U.S. companies have available.  As a result, our synthetic buoys are disposed of as bulk waste, and usually wind up in landfills.

- US:  The USCG does not use synthetic moorings extensively.  Out of more than 20,000 buoy stations, only 40 to 50 use synthetic line.  These are deep water (400 to 800 feet) or high current locations (5 knots or higher, 45 to 135 feet) where a traditional chain mooring would submerge the buoy.  The material is typically double-braided nylon.  When these moorings are replaced, the old mooring should be recovered.  If it is simply discarded in the water, it could present a tangling hazard to marine life, or could foul propellers of passing vessels.

Solutions

- JA:  At current state, users should follow the law when disposing.  To the supplier, it is requested that the materials and structure that are reusable be adopted.

- JA:  Synthetic moorings are not used, nor are there plans for using for some time to come.  Reusable innocuous 100% copper is used only for this purpose and there are no problems.

- SW:  All types of plastic spar and lightbuoys are today manufactured in environmentally-friendly material and are fully recyclable when decommissioned.  If a buoy is subject to fire, no hazardous or environmentally hostile gases or products are produced.  The supplier will take the buoy back when it is discarded.

- SW:  Synthetic line--delivered for recycling.

- FL:  Polyethylene and styrofoam recycled separately.

- FL:  Synthetic line not recycled yet.

- US:  The USCG's new buoy tenders have an on-board hydraulic winch that we use to retrieve synthetic moorings.  Old moorings are inert and are disposed of as bulk waste.

IMPACTS ON HABITATS (MARINE, PLANT, BIRDLIFE)

Problems

- UK:  Water pollution.

- UK:  Marine litter.

- UK:  Controlling access and movement of materials outside the limitations of lighthouses which can be Site of Special Interest (SSI), and where rare flora etc. can be found.

- FL:  Maintenance visits especially disturb bird nesting.

- FL:  Tree and brush growth in front of ATON causes a lot of work and expense.

- DK:  Biocide based antifouling on floating aids and buoy tender vessels.  We use copper-based antifouling at the moment.

- IR:  Migratory bird colonies and nesting sites on some stations.  Some stations on migratory routes result in birds flying into the lantern.

- JA:  There is the possibility of affecting marine products when creating lighthouse foundations and boat slips on site by spilling cement pastes and sewage during placement of the hydro-concrete and underwater drilling.

- US:  Antifouling paint on buoys, by its very nature, is toxic to marine growth.

- US:  The activities of ATON servicing vessels could interfere with or even harm (strike/run over) marine animals when working in their habitat.

- US:  Problems related to on-site battery disposal, abandonment of synthetic line, potential spills of diesel fuel can impact marine life and habitats.

- US:  Of particular concern to the USCG is the need to avoid harming any endangered or threatened species, such as migratory birds .  A prime example would be the osprey, a Federally-protected sea bird that has a proclivity for nesting on ATON structures.  We must go to great lengths not to remove or even disturb their nests, making it problematic to service the aids.

Solutions

- UK:  Buoys are jetted at sea or near areas where growth acquired--most growth is removed at this stage.

- UK:  Relevant policies and practices properly set out in environmental supplement to the property manual.  Manual used as reference for each site.

- FL:  Continuous work on making maintenance interval longer.  Landing on protected bird nesting areas prohibited in nesting time.  ATON maintenance visits are excused.

- FL:  With satellite navigation the leading lines for positioning buoys are no longer necessary, which reduces the need for cutting vegetation.

- SW:  Today technology creates a limited impact on our environment.

- IR:  Avoid interference with colonies.

- JA:  By adopting an on-shore yard, placement of concrete becomes unnecessary, thus reducing the chance to cause any effects to the marine products.  Also, the site work will be mitigated which will then improve on quality control while costs are also reduced.

- US:  With regard to antifouling paint, there are currently no proven materials on the market that could be substituted for the copper-based coatings that are now the industry standards.  Various products are in development in the coatings industry, such as "release" (abhesive) paints to which growth cannot stick, and the USCG will consider using these as their effectiveness is demonstrated.

- US:  With regard to potential harm to marine life through the activities of the servicing vessels (i.e., hitting animals or disturbing their habitat), the best solution is to minimize the servicing visits required for aid stations.  The USCG strives for this through engineering design solutions that extend the service intervals of hardware and equipment, minimize the interim maintenance visits required for a given aid, and eliminate the potential for environmental damage associated with the equipment (i.e., solarization versus primary batteries, diesel, or submarine cable).

- US:  Sometimes unique solutions are required on a case-by-case basis.  For example, we often build separate, higher nesting platforms on ATON structures or add extensions on lantern stands to keep the birds from nesting on the signal equipment.  Our units survey their aids during nesting season to determine which ones would benefit from this sort of intervention.

LAMP DISPOSAL

Problems

- IR:  Fluorescent and mercury lamps contain toxic materials.

- UK:  Sodium and neon lamps are not ozone friendly, and require special handling and disposal.  Fluorescent tubes from buildings and vessels--disposal.

- US:  Marine lamps could potentially contain elements that are dangerous to the environment, and thus create disposal problems.

Solutions

- HK:  They can be returned to manufacturers.

- IR:  Licensed contractor.

- SW:  Delivered for recycling.

- GE:  A contractor provides the disposal of lamps.

- UK:  Licensed contractor disposal arrangement.

- US:  USCG ATON lamps are made of inert materials (krypton gas with tungsten filaments) and are disposed of as standard waste.  Although a small amount of lead solder is used in assembly, the quantity of lead is extremely small, similar to what is found in household light bulbs and automobile headlights.