GUIDELINES ON THE METHODOLOGY OF COLLECTING INFORMATION ON RELIABILITY AND FAILURES OF ATON EQUIPMENT

- Introduction

A typical mission statement of a lighthouse authority is to deliver a reliable and cost effective network of aids to navigation of the benefit and safety of mariners and the protection of the marine environment. In order to achieve this it is necessary to collect data on AtoN equipment performance.

These guidelines provide details of methods that can be used to collect information on the reliability and failures andCost?? of AtoN equipment. It covers general aspects with an overview of sources, measures and information that may be involved when collecting information.

It should be noted that meaningful data must include the data on successes (operation without failures) as well as data on failures and faults. In other words, these Guidelines are not intended to be only a failure reporting guideline.

- Scope

These Guidelines will inform the AtoN service provider on how data collection and reporting methods can be applied within their organisation to monitor all equipments.

It is considered that, if these guidelines are followed, accuracy and completeness of reporting are enforced and data collected can be used to help improve the quality of the monitored equipments on a medium- to long-term basis. Moreover, such an effort will facilitate the interchange of information between user and suppliers.

This Guideline does not recommend how maintenance support should be organised. However, it will be self evident that some items will be repaired on site whilst others may only be replaced on site and repaired at a central facility or returned to the supplier for repair. Field data may be obtained at each of those stages.

In order to obtain maximum efficiency from the collection of data, it is suggested that the programmes of reporting, analysis and dissemination of results be closely co-ordinated. This may mean close liaison between departments who contribute to the collection and analysis of the data.

- The need for data collection

Any data collection methods must aim to provide the information required to enable the correct decisions to be taken in order to reach specified objectives. Performance indicators are management tools that can be used to measure, analyse and monitor the performance of a network of aids to navigation and or specific systems and equipment. The performance indicators in the IALA Navguide are, reliability, availability, continuity, redundancy, integrity, failure, mean time between failures (MTBF), mean time to repair (MTTR) and failure response time. The scope of this Guideline is the collection of data on reliability and failures of AtoN equipment.

It is considered that an AtoN system comprises the AtoN signal equipment plus ancillary components such as power supplies. Availability is a useful indicator of the level of service provided by individual or defined groups of aids to navigation because it is representative of all the considerations within the control of the authority that have gone into providing and maintaining the facility. These include:

- Quality assurance procedures;

- Design and systems engineering;

- Procurement;

- Installation;

- Maintenance of procedures;

- Failure response;

- Logistic arrangements.

While availability is the essential measure of the service provided to mariners, it is necessary to measure the other performance indicators in order to ensure that a lighthouse authority is operating an efficient and cost effective service.

The specific objectives of the field data collection and presentation are as follows:

- to provide information of the actual performance level of the equipments monitored to enable management reporting of availability of AtoNs, and as information for operations and planning, maintenance support, cost, training of personnel, etc.;

- to indicate a possible need for the improvement of:

-	AtoNs already installed and in operation, or

-	further equipments to be delivered;

- to compare the specified or predicted characteristics of the equipment(s) with the actual field performance;

- to improve future designs;

- to improve related processes;

- to improve predictions (data bases and procedures);

- to inform the supplier about the performance of items on a regular or on a single occasion basis;

- to have a common reporting basis.

4	Sources and means of data collection

In the following, the various information sources are described and the methods for systematically collecting information are outlined.

  - Sources of data

The following sources of data on reliability and failures of AtoN equipments are generally available:

- Mariner report;

- Keeper report;

- Remote monitoring;

- Maintenance activities;

- Repair activities on site, in workshop or at suppliers;

- Scheduled inspections

  - Means of collecting data

It is not intended to recommend any particular format for the recording medium (e.g. paper based or computer data base), however it should be recognized that early consideration of the format is necessary and important in setting up an effective data collection scheme and also aids subsequent successful processing.

Frequently the recording of data will be by manual means but automated and interactive data collection systems may be also considered. The advantages to be gained from holding data in a form suitable for processing by an electronic data processing system include easy and accurate  access and updating of information and the possibility of performing new extended analyses.

Data may be collected by one or several of the following reporting means.

    - Operation reporting

Data reporting should be supported by information on the use of the items such as the number of Aids and Aids mission time. Cognisance should be taken of the IALA categories of AtoNs.

    - Failure reporting

At any level, failure reporting is dependent on the fault coverage test resources used at the considered level: cases such as "fault not found" or "right when tested" should be clearly mentioned.

Failure reporting should cover all failures that have been observed. They should also contain sufficient information to identify failures. Failures considered to be attributable to any maintenance action should be so noted. It should be noted that failure of ancillary equipment or individual components within an AtoN system might not cause complete failure of the AtoN.

The failure reporting should be sufficiently comprehensive to cover the requirements of detailed investigation of an individual failure and the resulting fault. Where economic reasons or lack of resources make it impractical to collect all of the failure data indicated, the data required to calculate availability should be considered as the essential minimum.

    - Maintenance reporting

The maintenance report should contain all information relevant to the action taken to restore the item.

Maintenance activities can be divided into breakdown (AtoN performance is compromised), corrective (component failure – AtoN performance not compromised) and preventative (scheduled to prevent failures). When there is need to distinguish between corrective maintenance and preventive maintenance reporting if no replacements or repairs are made, the action can be classified as a preventive maintenance report. If a preventive maintenance action results in unscheduled replacement or repair, the report may be treated as a corrective maintenance report even though the item has in fact not failed in operation.

  - Data storage, updating and checking procedures

Independently of the structure chosen for the data storage, data should be checked at the time of input so as to ensure validity.

5	Performance measures

The selection of the data to be collected is very dependent on the kind of performance measures to be evaluated/estimated.

Field data reporting may have to be limited by economic necessity to the minimum necessary to meet the requirement, whilst recognising that collection systems should be capable of future expansion.

It is likely that certain data may be needed for more than one purpose, and careful consideration can therefore result in the most cost-effective data collection scheme.

The performance indicators identified in Section 3 that are considered are listed as follows.

5.1	Availability performance

AtoN outage time per Aid and category of Aid

AtoN mission time per Aid and category of Aid

AtoN availability per Aid and category of Aid

5.2		Reliability performance

AtoN mission time

Number of outages

Mean operating time between failures

5.3	Maintainability performance

5.3.1	Time related performance

Cumulative outage time

Number of outages

Mean time to repair

6	Data required

Consideration of the foregoing performance measures defines the need for a system that provides for the collection of documented data covering:

- the identity of items or population of items under observation;

- operational conditions (remote or mainland site);

- performance monitoring.

For each individual item, sufficient information has to be recorded to clearly identify the item itself.

Depending on the item under consideration (e.g. equipment, printed circuit board, component, personnel), and on the depth and kind of analysis of collected data, the necessary item identification data shall be used.

The item identification should also allow the analysis of the relationships between the items for which data is collected.

  - Data for performance indicators

For each failure the following information should be collected.

- Outage date and time

- Date and time restored

- Aid type and category

- Station

  - Data for reliability improvement

In order to improve the reliability of systems it is necessary to collect data additional to the data required for performance indicators.

For each failure the following additional information should be collected.

- Type of item failed (e.g. battery, generator, lantern etc.)

- Item identification (e.g. model number, serial Number, etc.)

- Root cause of failure

  - Misuse

  - Maintenance induced

  - Lightning

  - Corrosion

  - Fatigue

  - Lubrication

  - No fault found

  - Right when tested

  - Collision

  - Component failure (specify component)

  - Infant mortality etc.

- Action taken: Replacement, repair, adjustment, modification, lubrication, etc.

7	Data analysis

The performance indicators in Section 5 can be calculated in accordance with the IALA Guide to the Availability and reliability of Aids to Navigation and the IALA Recommendation on the Availability Objectives for Traditional Aids to Navigations (this is still a draft and may have a different title in the end) and the IALA Navguide.

The data can be analysed to show the reliability of individual subsystems and components and to identify the principle causes of unreliability. This will facilitate a planned approach to maintenance with predictable maintenance free operation periods for stations. In addition, reliability information can influence future purchasing decisions and improve system design.